3GPP TSG-RAN WG4 Meeting #103-e    							        					R4-2210294

Electronic Meeting, May 09th – May 20th, 2022

Agenda item:			9.8.3

Source:	Moderator (Samsung)

Title:	Email discussion summary for [103-e][222] NR_HST_FR2_RRM_2

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

In RAN Plenary #89-e, the RAN4-led work item of NR support for high speed train (HST) scenario in FR2 has been approved [RP-202118] (which has been further revised to [RP-210800] with editorial revisions and updates on time schedule).

As big CR [R4-2206016] agreed in RAN4#102-e and further approved in RAN#95-e, the relevant RRM core requirement for FR2 HST scenarios are introduced and enhanced over existing FR2 RRM requirements. Specifically, the enhancements for FR2 HST are introduced on cell re-selection, RRC connection mobility control, gradual timing adjustment, SSB based radio link monitoring and beam failure detection, intra-frequency measurement and L1-RSRP/SINR measurement. The new requirements of one-shot large UL timing adjustment and TCI state switch delay are specified for FR2 HST scenarios.

In this email thread, the following agenda items will be discussed:

- 9.8.3   RRM performance requirement

Based on the discussion papers is submitted under agenda 9.8.3. It is suggested to have the following target of 1st and 2nd round email discussion:

- 1st round: Initial discussion based on papers for RRM performance requirement.

- 2nd round: Achieve agreement as much as possible, and set up the planning for the RRM performance part of FR2 HST work item.

# Topic #1: RRM Performance Requirements for FR2 HST

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207733 | Qualcomm, Inc. | Proposal 5: Consider the following tests to verify FR2 HST RRM requirements: • Introduce two tests to cell identification requirement in non-DRx mode, one for set 1 and the other for set 2 requirements. • Introduce one test to verify the TCI state switch delay compliance. |
| R4-2208155 | CATT | Proposal 1: the test case list is proposed as following table: Core requirements; Test cases. Idle mode: cell reselection for intra-frequency cells; Define test case in A. 7.1.1.X for cell reselection for HST FR2 to verify cell reselection delay. DRX= 0.32. Connected mode: RRC re-establishment; Add one test case in A.7.3.2.X to verify re-establishment delay. Tq is changed from 2.5 *64*Tc to 4.5 *64*Tc; In current test cases, the Tp and Tq adjustment rate are not tested for exact Tq. There is no need to add new test case to verify Tq enhancement in HST FR2.. One shot large UL timing adjustment; The capability of one shot large UL timing adjustment is still FFS. Prefer to add one test case.. RLM/BFD requirements for reduced N for set1 and set2; For RLM, add one test to verify Out-of-sync Test for set1 and DRX = 40ms in A.7.5.1.X For BFD, add one test to verify BFD and link Recovery Test based on SSB for set2 and DRX = 40ms in A.7.5.5.X. MAC-CE based TCI state switch delay; Add one test in A.7.5.8.X. Intra-frequency cell identification when without MG; Add SA event triggered reporting test case in A.7.6.1.2.X to verify cell search requirements for [set1] and [set2]. DRX = 40ms. L1-RSRP measurement; Add one test case in A.7.6.3.2.X for SSB based with DRX = 40ms. L1-SINR scheduling availability; Do not add test case. |
| R4-2208348 | OPPO | Proposal 1: With the same side conditions, legacy accuracy requirements could be reused for FR2 HST. Proposal 2: Define RRM test cases for both set-1 with 2 Rx beams and set-2 with 6 Rx beams. Proposal 3: To verify the gradual timing adjustment requirements, the existing test cases defined in clause A.7.4.1 can be used as the starting point and the adjustment value for DL timing needs further discussion. Proposal 4: One-shot large timing adjustment could be tested along with TCI state switch. |
| R4-2208847 | Samsung | Observation 1: The impacted RRM core requirements for FR2 HST can be summarized as below table. Proposal 1: The RRM performance test cases are expected to be introduced or modified for the corresponding core reuiqrement as below: TC Index; Necessity of New Test Case; Corresponding Core Requirement; Core CR Responsibility; TC CR Responsibility. TC1; Reuse test case of existing TC A.7.1.1.1, but updated detection and evaluation periods for PC6 UE.; 4.2, for cell re-selection for FR2 PC6; ZTE. TC2; Reuse test case of existing TC A.7.3.2.1.1, but updated time period for re-establishment for PC6 UE; 6.2 for re-establishment to NR intra-frequency cell delay requirement for FR2 PC6; Ericsson. No new test cases are needed. Note: FR2 PC6 requirement can be verified by existing TC A.7.4.1.1; 7.1.2.1 for Tq for FR2 PC6; Nokia. TC3; New test case is needed for one shot large UL timing adjustment; 7.1.2.3 for new requirement for FR2 PC6; Nokia. TC4-TC7; Reuse test case of existing TC A.7.5.1.1 – A.7.5.1.4, but updated time periods for PC6 UE.; 8.1.2 for SSB based RLM requirement for FR2 PC6; CATT. TC8-TC9; Reuse test cases of the existing TC A.7.5.5.1 – A.7.5.5.2, but updated time periods for PC6 UE.; 8.5.2 for SSB based BFD requirement for FR2 PC6; CATT. TC10; New test case is needed for MAC CE based TCI state switching delay for FR2 PC6; 8.10.3A for MAC-CE based TCI state switch delay for FR2 PC6; Samsung. TC11-TC12; Reuse test case of existing TC A.7.6.1.1 – A.7.6.1.2, but updated time periods for PC6 UE.; 9.2.5 for NR intra-frequency measurements without measurement gaps for FR2 PC6; Nokia. TC13-TC14; Reuse test case of existing TC A.7.6.3.1 – A.7.6.3.2, but updated time periods for PC6 UE.; 9.5 for SSB based L1-RSRP measurement, and scheduling restriction for FR2 PC6; ZTE. TC15; Reuse test case of existing TC A.7.6.6.2, but updated time periods for PC6 UE.; 9.8 for L1-SINR measurement with SSB based CMR and dedicated IMR configured, and scheduling restriction for FR2 PC6; Huawei. |
| R4-2208965 | Huawei, Hisilicon | Proposal 1: The legacy accuracy of L1 RSRP, SS-RSRP and SS-RSRQ can be reused for L1 and L3 measurement in FR2 HST. Proposal 2: Test cases for FR1 HST are suggested as in Table 1. |
| R4-2208966 | Huawei, Hisilicon | This contribution provides the simulation results of L1 and L3 measurement accuracy for FR2 HST. |
| R4-2209098 | Ericsson | Proposal 1: Requirements on measurement accuracy shall be checked and verified with HST FR2 channel. • L1 measurements (L1-RSRP/RSRQ/SINR) under HST FR2 channel: AWGN with [19444] Hz • L3 measurements (RSRP/RSRQ) under HST FR2 channel: AWGN with [19444] Hz Proposal 2: Develop tests cases for all the above requirements. Proposal 3: Above test configurations shall be defined. Proposal 4: Add RRM tests for HST FR2, with parameters and configuration listed in Table 1. |
| R4-2209525 | Nokia, Nokia Shanghai Bell | We propose RAN4 to agree on the link simulation assumptions for L1 and L3 measurement accuracy outlined in Sections 2.1 and 2.2. |
| R4-2209526 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 discusses and agrees with the proposed list of RRM tests in Table 1 for the FR2 HST RRM enhanced requirements. |
| R4-2210214 | Nokia, Nokia Shanghai Bell | Reserved for simulation results for measurement accuracy for FR2 HST. |

## Open issues summary and 1st round view collection

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-Topic 1: Scope of RRM Performance Requirements

[Moderator] Multiple companies provided their views on the scope of RRM performance requirements for FR2 HST UE, and it is discussed as follows by breaking down into individual issues.

Issue 1-1: Necessity of Test Cases for Cell Re-selection Requirement

- [Background] Based on the approved CR [5, R4-2206016], cell reselection requirement for FR2 PC6 UE is introduced for RRM core requirement.

- Proposals:

  - Proposal-1 (Samsung): Reuse test case of existing TC A.7.1.1.1, but update TC with the detection and evaluation periods for PC6 UE.

  - Proposal-2 (CATT/Huawei/Ericsson/Nokia): New test case for A.7.1.1.X, Cell reselection to FR2 intra-frequency NR case for PC6 UE configured with highSpeedMeasFlagFR2-r17, DRX cycle=0.32s:

    - 2 tests:1 for unidirectional and 1 for bi-directional (Ericsson).

    - SMTC period = 20 ms, Propagation condition: AWGN (serving cell) and AWGN with 9722 Hz frequency offset (neighbour cell) (Nokia)

- Recommended WF

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | Idle mode is not very common in FR2 HST. We may not need to have a dedicated test for idle mode requirement. |
| Nokia | Support Proposal 2 since the requirement cell re-selection is enhanced for FR2 PC6 UE for HST. Next, RAN4 should discuss the details of the test case. For Proposal 2, we suggest revising the frequency offset for the AWGN propagation condition as follows: - Propagation condition: AWGN (serving cell) and AWGN with 19444 Hz frequency offset (neighbour cell) |
| Samsung | The targets of the proposal 1 and 2 are the same. Here in P1, we are considering the reuse existing test cases of A.7.1.1.1, but just give the necessary changes to avoid setup the new test cases. If companies think independent TC can be more clear, we are also okay to that. The propagation condition change as proposed by Nokia is okay to us. |
| Ericsson | We suggest to add a clause for HST FR2, which is same approach as existing HST performance tests. |
| CATT | We support Proposal 2 to add new test case to verify the enhanced requirements for FR2 HST new PC6 UE. For the number of test case of 1 or 2, we don’t have strong view. slightly prefer to 2 tests for full test coverage. |
| OPPO | We do not think separate test cases for both uni-directional and bi-directional are needed in proposal-2. The detection and evaluation periods are the same. |
| Huawei | In general support option 2. However we are not sure whether to test uni-directional and bi-directional scenario. When cell reselection happens, UE is at cell edge, the channel mode UE observed is AWGN with 19444Hz/ 9722Hz rather than HST channel. |

Issue 1-2: Necessity of Test Cases for Re-establishment Delay Requirement

- [Background] Based on the approved CR [5, R4-2206016], re-establishment to NR intra-frequency cell delay requirement for FR2 PC6 UE is introduced for RRM core requirement.

- Proposals:

  - Proposal-1 (Samsung): Reuse test case of existing TC A.7.3.2.1.1, but update TC with the time period for re-establishment for PC6 UE.

  - Proposal-2 (CATT/Huawei/Ericsson): New test case for A.7.3.2.1.X, Intra-frequency RRC Re-establishment in FR2 for PC6 UE configured with highSpeedMeasFlagFR2-r17.

    - 2 tests:1 for unidirectional and 1 for bi-directional (Ericsson).

    - SMTC period = 20 ms, Propagation condition: AWGN (Nokia).

  - Proposal-3 (QC): Do not introduce this test due to the similarity of enhancement, changing Rx sweep number, which can be verified by neighboring cell and L1-RSRP measurement tests.

- Recommended WF

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | The functionality difference to legacy requirement is verified by neighboring cell detection test, and no need for additional tests. |
| Nokia | Since the requirement is enhanced for FR2 PC6 UE, Proposal 2 can be used as a baseline. |
| Samsung | Similar comment as Issue 1-2, the targets of the proposal 1 and 2 are the same. If companies think independent TC can be more clear, we are also okay to that. |
| Ericsson | Re-establishment requirement comprises delays from different derivations, we suggest to verify it. |
| CATT | We support Proposal 2 to add new test case to verify the enhanced re-establishment requirements for FR2 HST. |
| Huawei | Proposal 2. Fine to have test on Re-establishment, our concern is whether to test uni-directional and bi-directional. The same comments as 1-1. |

Issue 1-3: Necessity of Test Cases for Timing Related Requirement

- [Background] Based on the approved CR [5, R4-2206016], timing related requirement for FR2 PC6 UE is introduced for RRM core requirement, including:

  - Gradual timing requirement for FR2 PC6 UE

  - One shot large UL timing adjustment for FR2 PC6 UE

- Proposals:

  - Proposal-1 (OPPO/Samsung/CATT): No new test cases are needed for gradual timing requirement for FR2 PC6 UE. Note: FR2 PC6 requirement can be verified by existing TC A.7.4.1.1.

  - Proposal-2 (Ericsson/Nokia): New test case for gradual timing requirement.

  - Proposal-3 (CATT/Samsung/Huawei/Ericsson/Nokia): New test case is needed for one shot large UL timing adjustment.

    - 2 tests:1 for unidirectional and 1 for bi-directional (Ericsson).

  - Proposal-3a (OPPO, QC): One-shot large timing adjustment could be tested along with TCI state switch.

- Recommended WF

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | There is no legacy test for gradual timing adjustment, are we going to invent one for FR2 HST, and why it is necessary given that there is no legacy test? One shot large UL timing adjustment is after TCI state switch, and therefore it can be tested by TCI state switch test. |
| Nokia | Support Proposals 2 and 3. We are open to discuss Proposal 3a. Based on our understanding, Proposal 3a suggests combining the test cases for one-shot large timing adjustment and for MAC-CE based TCI State Switch Delay instead of testing each separately. In response to QC’s comment on “there is no legacy test for gradual timing adjustment”, we want to refer to TS 38.133, A.7.4.1.1.2: 4) The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 Table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first detected path (in time) of DL SSB. Skip this step for test 2 with DRX configured. The above Step 4) directly refers to the gradual timing adjustment (Table 7.1.2.1-1), which is a test case. |
| Samsung | For gradual timing adjustment, same response as Nokia, as we suggested in our discussion paper, test case A.7.4.1.1 can be reused, and no new test case needed here. For one shot UL timing adjustment, we are open to both P3 and P3a. |
| Ericsson | Regarding gradual timing requirement, we support Option2 and 3. Regarding One shot large UL timing adjustment, we support Option3a. |
| CATT | For gradual timing adjustment, we support not to add new test case. For new one shot UL timing adjustment, new test case is needed to verify as Proposal 3. |
| OPPO | Support proposal-1 for gradual timing requirements. In the existing test cases, the test system will adjust the timing of the DL path by 8*64*Tc in test1 or 4*64*Tc in test2. We understanding only 8*64*Tc in test1 is needed since Tq is changed to 4.5*64*Tc for FR2 HST. We are also open to discuss the DL timing adjustment after the threshold for one-shot large timing adjustment is reached. The test system shall adjust the timing of the DL path by values given in Table A.7.4.1.1.2-1 Table A.7.4.1.1.2-1 Adjustment Value for DL Timing SCS of SSB signals (kHz); Adjustment Value. Test1; Test2. 240; +8*64Tc; +4*64Tc. Support proposal-3a for one-shot large timing difference since it will only apply in case of TCI state switch. |
| Huawei | Prefer proposal 3. A bit confused with QC’s comments. Gradual timing adjustment is verified with Te. If we would like to test one step timing adjustment, the similar test can be set. We are open to discuss proposal 3a if we can find a good scheme to well test. |

Issue 1-4: Necessity of Test Cases for SSB-based RLM and BFD

- [Background] Based on the approved CR [5, R4-2206016], SSB-based RLM and BFD requirements for FR2 PC6 UE are introduced for RRM core requirement.

- Proposals:

  - Proposal-1 (Samsung): Reuse test case of existing TC A.7.5.1.1 – A.7.5.1.4 for SSB-based RLM, but update the time periods for PC6 UE.

  - Proposal-2 (Samsung): Reuse test cases of the existing TC A.7.5.5.1 – A.7.5.5.2 for SSB-based BFD, but update the time periods for PC6 UE.

  - Proposal-3 (Huawei/Ericsson/Nokia): Two new test cases for SSB-based RLM:

    - A.7.5.1.X	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode for FR2 Power Class 6 UE (Huawei)

    - A.7.5.1.X	Radio Link Monitoring In-sync Test for FR2 Pcell configured with SSB-based RLM RS in non-DRX mode for FR2 Power Class 6 UE (Huawei)

    - Two TCs for DRX mode (Nokia/Ericsson), with two tests each (1 for uni-directional and 1 for bi-directional) (Ericsson)

  - Proposal-4 (Huawei): One new test case for SSB-based BFD:

    - A.7.5.5.X	Beam Failure Detection and Link Recovery Test for FR2 Pcell configured with SSB-based BFD and LR in non-DRX mode for PC6 UE, DRX = 80ms.

  - Proposal-5 (CATT): For RLM, add one test to verify Out-of-sync Test for DRX = 40ms in A.7.5.1.X.

  - Proposal-6 (CATT/Ericsson/Nokia): For BFD, add one test to verify BFD and link Recovery Test based on SSB for DRX = 40ms in A.7.5.5.X.

    - 2 tests:1 for unidirectional and 1 for bi-directional (Ericsson)

    - SMTC period = 20 ms, Propagation condition: FFS (Nokia)

  - Proposal-7 (QC): Do not introduce this test due to the similarity of enhancement, changing Rx sweep number, which can be verified by neighboring cell and L1-RSRP measurement tests.

- Recommended WF

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | The functionality difference to legacy requirement is verified by L1-RSRP test, and no need for additional tests. |
| Nokia | Proposals 3 and 6 are a good starting point. |
| Samsung | We are also okay with QC’s proposal here. For RLM and BFD, the TC (if any) for PC6 is intended to test the same beam sweeping function as L1 RSRP measurement test, so we can skip the TCs here. |
| Ericsson | We support Proposal 6, but proposal 7 also makes sense. Essentially, we don’t need to test all cases, but only necessary cases with significant effects. RLM, BFD and L1-RSRP have not significant difference from RRM test perspective. |
| CATT | We support to add new test case for RLM and BFD as proposal 3/5/6. But unlike so many test cases in R15, just add one or two necessary cases for each. |
| OPPO | Since the impacts on L1 measurements, including RLM, BFD, L1-RSRP are similar, we agree to define one or two TCs rather than testing all of them. |
| Huawei | Support option 3 and option 4. And option 7 is also fine to save test number. |

Issue 1-5: Necessity of Test Cases for MAC-CE based TCI State Switch Delay Requirement

- [Background] Based on the approved CR [5, R4-2206016], MAC-CE based TCI state switch delay requirement for FR2 PC6 UE are introduced for RRM core requirement.

- Proposals:

  - Proposal-1 (CATT/Samsung/Huawei/Ericsson/Nokia): New test case is needed for MAC CE based TCI state switching delay for FR2 PC6: A.7.5.8.X, MAC-CE based active TCI state switch for FR2 PC6 UE

    - 2 tests:1 for unidirectional and 1 for bi-directional (Ericsson)

    - DRX cycle = 40 ms, SMTC period = 20 ms, Propagation condition: AWGN (Nokia)

- Recommended WF

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | One shot large timing adjustment should be enabled, and no need to test bi-directional model. |
| Nokia | Support Proposal 1. We are open to discuss uni- and bi-directional cases. |
| Samsung | For tests on both bi- and uni-directional, we don’t see the strong necessity due to the unified requirement defined. |
| Ericsson | Support Proposal-1, one question is channel is AWGN only or shall with frequency offset? Regarding unidirectional or bi-directional, we’re ok to choose unified requirement but the untified requirement, e.g. the timeline of TCI State Switch can follow typical HST scenario. |
| CATT | Support Proposal 1. |
| OPPO | Support the test cases for MAC-CE based TCI switch, and the accuracy for one-shot large timing adjustment can be tested along with it. Since the uni-directional is the typical scenario for one-shot large timing adjustment, we prefer to only consider uni-directional case. The delay requirements are irrelevant to DRX cycle, so DRX should be OFF. |
| Huawei | Support proposal 1. Test with AWGN with frequency offset 9722Hz is suggested. |

Issue 1-6: Necessity of Test Cases for L3 Measurement Requirement

- [Background] Based on the approved CR [5, R4-2206016], NR intra-frequency measurements without measurement gaps for FR2 PC6 are introduced for RRM core requirement.

- Proposals:

  - Proposal-1 (Samsung): Reuse test case of existing TC A.7.6.1.1 – A.7.6.1.2, but updated time periods for PC6 UE.

  - Proposal-2 (Huawei): New test case for A.7.6.1.X, SA event triggered reporting test without gap under non-DRX for PC6 UE

  - Proposal-2a (CATT/Nokia): Add SA event triggered reporting test case in A.7.6.1.2.X to verify cell search requirements. DRX = 40ms.

    - SMTC period = 20 ms, Propagation condition: AWGN (serving cell) and AWGN with 9722 Hz frequency offset (eighbor cell) (Nokia).

- Recommended WF

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | Support proposal 2, non-DRx is more common in FR2 HST since it is CPE device. |
| Nokia | Proposal 2a can be used as basis for further discussions for down selecting test cases. |
| Samsung | Same as QC, prefer to prioritize non DRX case. New TC for PC6 is also okay to use, since the purpose is the same as P1. |
| Ericsson | Support Proposal -2a. |
| CATT | Support Proposal 2 and 2a. open to discuss the detailed parameters. |
| OPPO | Support proposal 2. |
| Huawei | Proposal 2 with non-DRX is more preferred. |

Issue 1-7: Necessity of Test Cases for L1-RSRP/SINR Measurement Requirement

- [Background] Based on the approved CR [5, R4-2206016], NR Layer-1 measurements for FR2 PC6 are introduced for RRM core requirement, including:

  - SSB based L1-RSRP measurement, and scheduling restriction for FR2 PC6

  - L1-SINR measurement with SSB based CMR and dedicated IMR configured, and scheduling restriction for FR2 PC6

- Proposals:

  - Proposal-1 (Samsung): Reuse test case of existing TC A.7.6.3.1 – A.7.6.3.2 for L1-RSRP measurement, but update the time periods for PC6 UE.

  - Proposal-2 (Samsung): Reuse test case of existing TC A.7.6.6.2 for L1-SINR measurement, but update the time periods for PC6 UE.

  - Proposal-3 (Huawei/CATT/Nokia): New test case for A.7.6.3.2, SSB based L1-RSRP measurement when DRX is used for PC6 UE.

    - DRX cycle = 40 ms, SMTC period = 20 ms, Propagation condition: AWGN with 9722 Hz frequency offset (Nokia)

  - Proposal-4 (Nokia): New test case for A.7.6.6.2, SSB based L1-SINR measurement when DRX is used for PC6 UE.

    - DRX cycle = 40 ms, SMTC period = 20 ms, Propagation condition: AWGN with 9722 Hz frequency offset (Nokia)

  - Proposal-5 (CATT): Don’t add new test case for L1-SINR scheduling availability.

  - Proposal-6 (Ericsson): New test cases for scheduling availability requirement:

    - Scheduling availability of UE performing L1-RSRP measurement on FR2, configured with [highSpeedMeasFlagFR2-r17]

    - Scheduling availability of UE performing L1-SINR  measurement on FR2, configured with [highSpeedMeasFlagFR2-r17]

- Recommended WF

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | Support proposal 3 but in non-DRx mode. |
| Nokia | Support Proposals 3 and 4. We are open to discuss down selecting test cases. |
| Samsung | We are okay to use P3 and P4 as starting point. Non-DRX to be used instead. |
| Ericsson | We agree on proposal 3,4,6. We are open to discuss the necessity of proposal6. |
| CATT | Support Proposal 3. P4 is similar. We are open to discuss the detailed parameters. |
| OPPO | Support proposal 3. The delay requirements for L1-SINR, if introduced, are the same as L1-RSRP, so we prefer to select one test case. |
| Huawei | Support option 3 and 4. Open for small DRX or non-DRX. |

### Sub-Topic 2: RRM Test Configuration and Applicability

Issue 2-1: Applicability of Set-1 and Set-2 of enhanced RRM requirements

- [Background] Considering the typical deployments for FR2 HST, two sets of enhanced RRM core requirements in terms of number of Rx beams were defined in previous meetings. Both sets of Rx beams are mandatorily supported by FR2 HST UE in case of mix deployments. And which set of enhanced RRM requirements should apply is up to network signalling.

- Proposals:

  - Proposal-1 (OPPO): Define RRM test cases for both set-1 with 2 Rx beams and set-2 with 6 Rx beams.

  - Proposal-2 (QC): Test set 1 and 2 in different tests to reduce total number of tests.

- Recommended WF

  - Initial discussion provided in the 1st round discussion, view collected on the proposal.

| Company | Comments |
| --- | --- |
| Nokia | Proposal 1 can be used as baseline. We suggest to discuss the applicability on a case-by-case basis rather than making a common agreement for all test cases. |
| Samsung | Seems in previous Issues, companies are okay to have both two sets of requirement to be tested with different NW signaling configured. Here we can firstly accept the general agreement as P2, and then to discuss set-1 or set-2 to be used case by case. |
| Ericson | Support Proposal-1. In 2nd round, we can take a list to discuss test cases allocated with set 1 and set 2, case by case. |
| CATT | We support to discuss it case by case. |
| OPPO | We are fine with both options, set-1 and set-2 can be discussed case by case. |
| Huawei | In general fine to test set1 and test 2. Proposal 2 is a good way to reduce test number. |

Issue 2-2: Test configuration

- Proposals:

  - Proposal-1 (Ericsson): consider test configurations:

    - Channel model – HST channel for bi-directional and ui-directional

    - Uni- directional or bi-directional: Define tests for uni- directional or bi-directional if necessary.

    - Doppler shift: [19444] Hz

    - DRX cycles shall be limited to 80ms, or 40ms depends on test cases.

    - SMTC and measurement gap configuration shall be limited also.

- Recommended WF

  - Initial discussion provided in the 1st round discussion, view collected on the proposal.

| Company | Comments |
| --- | --- |
| QC | No need to specify uni or bi-directional model since the channel model of neighboring cell is AWGN with [19444]Hz shift for both deployment models. We should test non-DRx mode, SMTC = 20ms and no gap configuration is needed. |
| Nokia | Support Doppler shift: [19444] Hz. The rest of the configuration parameters can be discussed when individual test cases are specified. |
| Samsung | For bi-directional test case, we would like to clarify that it is still based on OTA 1AoA test setup but just the Doppler frequency offset is introduced in cell/beam-2? If not, prefer to consider uni-directional test case only. |
| Ericsson | We can agree on one of directional test cases, but maybe it also needs case by case check? |
| CATT | We can discuss the test configuration in each test case. |
| Huawei | As we commented above, channel model may be distinguished with the tests. For cell reselection/handover/measurement, AWGN with frequency offset is recommended. For other tests, we can further discuss. |

### Sub-Topic 3: Work Splitting for RRM Test Case CR Drafting

[Moderator] For FR2 HST RRM core requirement, the work splitting among companies is of importance to complete the core part, and the work splitting for RRM test case CR drafting is also needed to be discussed.

Issue 3-1: Work Load Splitting for RRM Test Case CR Drafting

- Proposals:

  - Proposal-1 (Samsung): Work load splitting for RRM performance test cases to be planned with the following table:

| TC Index | Necessity of New Test Case | Corresponding Core Requirement | Core CR Responsibility | TC CR Responsibility |
| --- | --- | --- | --- | --- |
| TC1 | Reuse test case of existing TC A.7.1.1.1, but updated detection and evaluation periods for PC6 UE. | 4.2, for cell re-selection for FR2 PC6 | ZTE |  |
| TC2 | Reuse test case of existing TC A.7.3.2.1.1, but updated time period for re-establishment for PC6 UE | 6.2 for re-establishment to NR intra-frequency cell delay requirement for FR2 PC6 | Ericsson |  |
|  | No new test cases are needed. Note: FR2 PC6 requirement can be verified by existing TC A.7.4.1.1 | 7.1.2.1 for Tq for FR2 PC6 | Nokia |  |
| TC3 | New test case is needed for one shot large UL timing adjustment | 7.1.2.3 for new requirement for FR2 PC6 | Nokia |  |
| TC4-TC7 | Reuse test case of existing TC A.7.5.1.1 – A.7.5.1.4, but updated time periods for PC6 UE. | 8.1.2 for SSB based RLM requirement for FR2 PC6 | CATT |  |
| TC8-TC9 | Reuse test cases of the existing TC A.7.5.5.1 – A.7.5.5.2, but updated time periods for PC6 UE. | 8.5.2 for SSB based BFD requirement for FR2 PC6 | CATT |  |
| TC10 | New test case is needed for MAC CE based TCI state switching delay for FR2 PC6 | 8.10.3A for MAC-CE based TCI state switch delay for FR2 PC6 | Samsung |  |
| TC11-TC12 | Reuse test case of existing TC A.7.6.1.1 – A.7.6.1.2, but updated time periods for PC6 UE. | 9.2.5 for NR intra-frequency measurements without measurement gaps for FR2 PC6 | Nokia |  |
| TC13-TC14 | Reuse test case of existing TC A.7.6.3.1 – A.7.6.3.2, but updated time periods for PC6 UE. | 9.5 for SSB based L1-RSRP measurement, and scheduling restriction for FR2 PC6 | ZTE |  |
| TC15 | Reuse test case of existing TC A.7.6.6.2, but updated time periods for PC6 UE. | 9.8 for L1-SINR measurement with SSB based CMR and dedicated IMR configured, and scheduling restriction for FR2 PC6 | Huawei |  |

- Recommended WF

  - Table to be updated after issues in Sub-Topic 1 are resolved, based on which the work splitting for RRM test cases CR drafting is planned.

| Company | Comments |
| --- | --- |
| Nokia | The recommended WF is fine. |
| Samsung | Discussion on Sub-Topic 1 firstly as suggested by WF. |
| CATT | We should conclude the test case list firstly. |

### Sub-Topic 4: Measurement Accuracy Requirement for FR2 HST UE

Issue 4-1: Measurement accuracy for FR2 HST UE:

- Proposals:

  - Proposal-1 (Huawei/OPPO): The legacy accuracy of L1 RSRP, SS-RSRP and SS-RSRQ can be reused for L1 and L3 measurement in FR2 HST.

- Recommended WF

  - Initial discussion provided in the 1st round discussion, and companies’ view are collected.

| Company | Comments |
| --- | --- |
| QC | Support proposal 1. |
| Nokia | Pending simulation result contributions from companies. |
| Samsung | Based on Huawei paper’s observation, by assuming the same number of measurement occasions as NR Rel-15 evaluation, i.e., L1 measurement evaluation is based on 1 sample @-3dB and L3 measurement is based on 3 samples@-6dB, the measurement accuracy is evaluated by resulting in the same accuracy is achievable. The evaluation assumption can be the critical factor here, and if AWGN with higher Dopper shift is used, we believe Huawei’s results should be representative. But we are okay to have further evaluation to confirm this. |
| Ericsson | Same view with Nokia, we suggest to add simulation task for this issue in next meeting. |
| CATT | FFS |
| OPPO | If same side conditions, e.g. Es/Iot, are assumed, the accuracy requirements can be reused. We are also fine to evaluate by simulations. |
| Huawei | Proposal 1 is based on our simulation results. It is good if companies provides more evaluations. |

Issue 4-2: L1/L3-measurement simulation result and alignment:

- The following simulation results are provided:

  - R4-2208966 (Huawei):

- L1 measurement evaluation is based on 1 sample @-3dB and L3 measurement is based on 3 samples@-6dB. The simulation results of L1 SS-RSRP measurement accuracy are provided in Table 1. - Table 1. Measurement accuracy of delta L1 SS-RSRP (1 sample) Propagation condition; SINR [dB]; SS-RSRP (SCS=120KHz). 5%; 50%; 95%; 50% - 5%; 95% -50%. AWGN; -3; -0.70; 0.39; 1.44; 1.09; 1.05. AWGN 9722Hz; -3; -0.86; 0.31; 1.33; 1.17; 1.02. AWGN 19444Hz; -3; -1.18; 0.07; 1.21; 1.25; 1.14. Propagation condition; SINR [dB]; SS-RSRP (SCS=240KHz). 5%; 50%; 95%; 50% - 5%; 95% -50%. AWGN; -3; -0.80; 0.38; 1.43; 1.18; 1.05. AWGN 9722Hz; -3; -0.82; 0.34; 1.37; 1.16; 1.03. AWGN 19444Hz; -3; -0.88; 0.29; 1.37; 1.17; 1.08. - The simulation results of L3 SS-RSRP/SS-RSRQ measurement accuracy are provided in Tables 2 and 3. - Table 2. Measurement accuracy of delta L3 SS-RSRP (3 samples) Propagation condition; SINR [dB]; SS-RSRP (SCS=120KHz). 5%; 50%; 95%; 50% - 5%; 95% -50%. AWGN; -6; -0.66; 0.34; 1.29; 1.00; 0.95. AWGN 9722Hz; -6; -0.74; 0.25; 1.16; 0.99; 0.91. AWGN 19444Hz; -6; -1.08; -0.01; 0.99; 1.07; 1.00. Propagation condition; SINR [dB]; SS-RSRP (SCS=240KHz). 5%; 50%; 95%; 50% - 5%; 95% -50%. AWGN; -6; -0.68; 0.36; 1.25; 1.04; 0.89. AWGN 9722Hz; -6; -0.69; 0.33; 1.32; 1.02; 0.99. AWGN 19444Hz; -6; -0.77; 0.28; 1.21; 1.05; 0.93. - Table 3. Measurement accuracy of delta L3 SS-RSRQ (3 samples ) Propagation condition; SINR [dB]; SS-RSRQ (SCS=120KHz). 5%; 50%; 95%; 50% - 5%; 95% -50%. AWGN; -6; -0.66; 0.33; 1.22; 0.99; 0.89. AWGN 9722Hz; -6; -0.69; 0.23; 1.09; 0.92; 0.86. AWGN 19444Hz; -6; -1.08; -0.02; 0.94; 1.06; 0.96. Propagation condition; SINR [dB]; SS-RSRQ (SCS=240KHz). 5%; 50%; 95%; 50% - 5%; 95% -50%. AWGN; -6; -0.66; 0.34; 1.17; 1.00; 0.83. AWGN 9722Hz; -6; -0.67; 0.32; 1.21; 0.99; 0.89. AWGN 19444Hz; -6; -0.75; 0.26; 1.15; 1.01; 0.89.

- Recommended WF

  - Simulation results to be captured in the reserved Tdoc R4-2210214, for simulation result alignment purpose.

| Company | Comments |
| --- | --- |
| Nokia | Support the recommended WF. A question for clarification of the above simulation results presented in R4-2208966: what were the simulation assumptions used? |
| Ericsson | Same question as Nokia comments. |
| Huawei | The simulation assumption is mostly based on that of FR1 HST except SCS and channel to cope with FR2. If needed we can take L3 measurement simulation assumption in this meeting. |
| Nokia2 | Here, we provide preliminary simulation results for L3 SS-RSRP and L1-SINR. The simulation assumptions are similar to those in Issue 4-3 and 4-4 for L3 SS-RSRP and L1-SINR, respectively. L3 SS-RSRP measurement accuracy, SINR = -6 dB, AWGN with 9722 Hz offset SCS; SSB burst periodicity; Number of Samples; 5th percentile; 50th percentile; 95th percentile. 5 ms; 1; -1.81; 0.54; 2.03. 3; -1.15; 0.11; 1.13. 5; -0.97; 0; 0.82. 10 ms; 1; -1.51; 0.43; 2.15. 120 kHz; 3; -1.02; 0.07; 1.16. 5; -0.95; -0.05; 0.78. 20 ms; 1; -1.43; 0.47; 2.1. 3; -0.98; 0.16; 1.21. 5; -0.88; 0.03; 0.87. 5 ms; 1; -1.58; 0.42; 2.11. 3; -1.12; 0.11; 1.14. 5; -0.92; -0.01; 0.85. 240 kHz; 10 ms; 1; -1.54; 0.63; 2.02. 3; -1.05; 0.21; 1.21. 5; -0.9; 0.07; 0.97. 20 ms; 1; -1.47; 0.56; 2.08. 3; -0.97; 0.09; 1.14. 5; -0.88; 0; 0.84. L3 SS-RSRP measurement accuracy, SINR = -6 dB, AWGN SCS; SSB burst periodicity; Number of Samples; 5th percentile; 50th percentile; 95th percentile. 5 ms; 1; -1.14; 1.02; 2.56. 3; -0.66; 0.81; 1.84. 5; -0.66; 0.78; 1.58. 10 ms; 1; -1.04; 1.04; 2.67. 120 kHz; 3; -0.56; 0.8; 1.81. 5; -0.51; 0.78; 1.54. 20 ms; 1; -1.1; 1.09; 2.55. 3; -0.64; 0.86; 1.88. 5; -0.59; 0.75; 1.68. 5 ms; 1; -1.04; 1.04; 2.67. 3; -0.56; 0.8; 1.81. 5; -0.55; 0.77; 1.54. 240 kHz; 10 ms; 1; -1.1; 1.09; 2.55. 3; -0.64; 0.86; 1.88. 5; -0.59; 0.75; 1.68. 20 ms; 1; -0.8; 0.97; 2.68. 3; -0.63; 0.82; 2.05. 5; -0.6; 0.76; 1.82. L1-SINR, SCS = 120 kHz, AWGN with 9722 Hz offset Scenario; CMR; IMR; Samples (M); 5% (dB); 50% (dB); 95% (dB). Case 1; CSI-RS; N/A; 1; -1.5; -0.11; 1.13. Case 2; SSB; NZP-IMR; 1; -1.82; -0.13; 1.24. Case 3; SSB; ZP-IMR; 1; -1.83; -0.13; 1.26. Case 4; CSI-RS; NZP-IMR; 1; -1.56; -0.14; 1.17. Case 5; CSI-RS; ZP-IMR; 1; -1.42; -0.13; 1.07. |

Issue 4-3: Simulation assumption further alignment for L3 measurement

- Proposals:

  - Proposal-1 (Ericsson): Requirements on measurement accuracy shall be checked and verified with HST FR2 channel.

    - L3 measurements (RSRP/RSRQ) under HST FR2 channel: AWGN with [19444] Hz

  - Proposal-2 (Nokia): L3 measurement simulation is provided:

In the simulation, two cells are considered, which are Cell 1 (serves as an interfering cell and unknown to UE) and Cell 2 (serves as the cell under measurement and known to UE). For L3 measurement accuracy, the CDF curves to be provided are as follows: - Delta SS-RSRP = (estimated SS-RSRP – ideal SS-RSRP) [dB] - Delta SS-RSRQ = (estimated SS-RSRQ – ideal SS-RSRQ) [dB] - Delta SS-SINR = (estimated SS-SINR – ideal SS-SINR) [dB] Table 1: General parameters Simulation parameters; Comments/values. Carrier frequency for Cell 1 and Cell 2; 30 GHz. DRX; No. RRH transmit antenna configuration for SS blocks; 1 tx or single layer transmissions. UE receive antenna configuration; 2 rx. Data and control channel subcarrier spacing; SSB SCS 240 kHz/120 kHz, data SCS 120 kHz. Measurement period (in number of measurement samples); 5 (other values should be considered such as 1 and 3). - Subcarrier spacing SSB; 240 kHz/120 kHz. - Number of SS blocks per SS burst set, K; 1 up to 8 is possible (a subset can be used). - SS burst periodicity; 5 ms and other values, e.g., 10ms, 20 ms. Table 2: Cell-specific parameters Parameter; Unit; Cell 1; Cell 2. NR RF Channel number; -; Channel 1; Channel 1. NR-PSS and NR-SSS sequences; -; To be indicated by companies; To be indicated by companies. PBCH and DMRS power offset with respect to NR-PSS and NR-SSS; dB; 0; 0. Data and control PSD relative to NR-PSS and NR-SSS; dB; 0; 0. RB Utilization; %; 100; 100. Data Modulation; -; QPSK; QPSK. Slot length; -; 14 symbols; 14 symbols. CP Length; -; Normal; Normal. Frequency Offset relative to UE frequency reference; Hz; 0; 0. 1)Relative Delay of 1st Path (synchronous); µs; 0; [CP/2]. Es/IoT; dB; 6.4; 1.25; FSS: -0.75, 0.25; (SINR: -6dB; FFS: -8dB, -7dB). Propagation conditions; -; AWGN with 9722 Hz offset between desired and interfering cells.

- Recommended WF

  - Initial discussion provided in the 1st round discussion, and companies’ view are collected.

| Company | Comments |
| --- | --- |
| QC | No need to discuss this issue if proposal 1 in issue 4-1 is agreed. We need at least analytically justification on how lower speed can lead to accuracy requirement difference, but not higher speed as in FR1 HST before spending time on simulation discussion. |
| Nokia | To align Proposals 1 and 2, we propose to revise the frequency offset of 9722 Hz to 19444 Hz, which is - AWGN with 19444 Hz offset between desired and interfering cells |
| Samsung | Suggest to use SINR = -6dB only, and no need to consider “FFS: -8dB, -7dB”. Doppler frequency offset changed to 19444Hz is okay. |
| Ericsson | We suggest to reserve task to check simulation result in next meeting based on the assumption. |
| Huawei | Both frequency offset of 9722 Hz and 19444 Hz can be considered for considering uni- or bi-directional case. |

Issue 4-4: Simulation assumption further alignment for L1 measurement

- Proposals:

  - Proposal-1 (Ericsson): Requirements on measurement accuracy shall be checked and verified with HST FR2 channel.

    - L1 measurements (L1-RSRP/RSRQ/SINR) under HST FR2 channel: AWGN with [19444] Hz

  - Proposal-2 (Nokia): L1 measurement simulation is provided:

In the simulation, one cell is considered. - Table 3: General parameters Simulation parameters; Comments/values. Carrier frequency for Cell 1; 30 GHz. channel; AWGN. BS transmit antenna configuration; 1 tx. UE receive antenna configuration; 2 rx. Subcarrier spacing; 120 kHz. Carrier frequency offset; 9722 Hz for uni-directional. Frequency tracking; TRS with periodicity = 10 ms. - Table 4: L1-SINR accuracy evaluation simulation assumptions for CMR only Simulation parameters; Comments/values. Data channel subcarrier spacing; The same as CSI-RS subcarrier spacing. Measurement period (in number of measurement samples); 1 sample. CSI-RS periodicity; 80 slots. Number of PRBs; 48. Density; 3. Side condition (SNR) on CMR; -3 dB. - The simulation assumptions in Table 4 are also applicable to SSB and CSI-RS based L1-RSRP accuracy evaluation. - Table 5: L1-SINR accuracy evaluation simulation assumptions for SSB-based CMR + NZP-IMR Simulation parameters; Comments/values. Data channel subcarrier spacing; The same as SSB and CSI-RS subcarrier spacing. Measurement period (in number of measurement samples) for CMR/IMR; 1 sample. Channel measurement resource (CMR); SSB. Interference measurement resource (IMR) configuration; CSI-RS. periodicity; CMR periodicity (80 slots) = IMR periodicity. Ideal SINR; -3dB. Number of PRBs for IMR; 48. Density for IMR; 3. Side condition (SNR) on CMR; 0 dB. Side condition (SNR) on IMR; 0 dB. - Table 6: L1-SINR accuracy evaluation simulation assumptions for SSB-based CMR + CSI-IM Simulation parameters; Comments/values. Data channel subcarrier spacing; The same as SSB and CSI-RS subcarrier spacing. Measurement period (in number of measurement samples) for CMR/IMR; 1 sample. Channel measurement resource (CMR); SSB. Interference measurement resource (IMR) configuration; CSI-IM. periodicity; CMR periodicity (80 slots) = IMR periodicity. Ideal SINR; -3dB. Number of PRBs for IMR; 48. Density for IMR; 3. Side condition (SNR) on CMR; -3 dB. Side condition (SNR) on IMR; N/A. - Table 7: L1-SINR accuracy evaluation simulation assumptions for CSI-RS-based CMR + NZP-IMR Simulation parameters; Comments/values. Data channel subcarrier spacing; The same as SSB and CSI-RS subcarrier spacing. Measurement period (in number of measurement samples) for CMR/IMR; 1 sample. Channel measurement resource (CMR); CSI-RS. Interference measurement resource (IMR) configuration; CSI-RS. periodicity; CMR periodicity (80 slots) = IMR periodicity. Ideal SINR; -3dB. Number of PRBs for IMR; 48. Density for CMR/IMR; 3. Side condition (SNR) on CMR; 0 dB. Side condition (SNR) on IMR; 0 dB. - Table 8: L1-SINR accuracy evaluation simulation assumptions for CSI-RS-based CMR + CSI-IM Simulation parameters; Comments/values. Data channel subcarrier spacing; The same as SSB and CSI-RS subcarrier spacing. Measurement period (in number of measurement samples) for CMR/IMR; 1 sample. Channel measurement resource (CMR); CSI-RS. Interference measurement resource (IMR) configuration; CSI-IM. periodicity; CMR periodicity (80 slots) = IMR periodicity. Ideal SINR; -3dB. Number of PRBs for IMR; 48. Density for CMR/IMR; 3. Side condition (SNR) on CMR; -3 dB. Side condition (SNR) on IMR; N/A. For L1 measurement accuracy, the CDF curves to be provided are as follows: - Delta L1-RSRP = (estimated L1-RSRP – ideal L1-RSRP) [dB] - Delta L1-SINR = (estimated L1-SINR – ideal L1-SINR) [dB]

- Recommended WF

  - Initial discussion provided in the 1st round discussion, and companies’ view are collected.

| Company | Comments |
| --- | --- |
| QC | Same comment as issue 4-3 for L1-RSRP. |
| Nokia | To align Proposals 1 and 2, we propose to revise the frequency offset of 9722 Hz to 19444 Hz, which is - Carrier frequency offset: 19444 Hz for uni-directional |
| Ericsson | We suggest to reserve task to check simulation result in next meeting based on the assumption. |

## Companies views’ collection for 1st round

### Open issues

[Moderator] View collection under each issue in Section 1.2 above.

### CRs/TPs comments collection

N/A.

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Issue 1-1: Necessity of Test Cases for Cell Re-selection Requirement Discussion summary on 1st round: - Majority view confirm the necessity of the test case, and new test case is introduced as Proposal 2. - For uni- vs. bi-direcitonal condition: Three companies agree to have 1 test only (either unidirectional or bidirectional) to save test effort, but no strong preference observed. For other companies, no strong view observed for this sub-bullet in P2. Tentative agreements: - TC for Cell Re-selection Requirement: - New test case for A.7.1.1.X, Cell reselection to FR2 intra-frequency NR case for PC6 UE configured with highSpeedMeasFlagFR2-r17, DRX cycle=0.32s: - 1 test defined from either unidirectional or bi-directional: - FFS unidirectional or bi-directional. - SMTC period = 20 ms, - Propagation condition: AWGN (serving cell) and AWGN with [9722] or [19444] Hz frequency offset (neighbour cell) Recommendations for 2nd round: - Discussion on above tentative agreement. - Note: if AWGN is assumed, uni-directional should result in [9722] Hz frequency offset while bi-directional means [19444] Hz. - If Test setup is 1AoA based, the only difference between uni- and bi-directional should be the value of Doppler frequency offset. Companies may want to check this in 2nd round. |
|  | Issue 1-2: Necessity of Test Cases for Re-establishment Delay Requirement Discussion summary on 1st round: - Majority views (4 companies) agree with Proposal 2. One company prefer to reuse existing test case but is also okay with P2. And one company disagree to introduce re-establishment delay requirement. Tentative agreements: - TC for Re-establishment Delay Requirement: - New test case for A.7.3.2.1.X, Intra-frequency RRC Re-establishment in FR2 for PC6 UE configured with highSpeedMeasFlagFR2-r17. - 1 test defined from either unidirectional or bi-directional: - FFS unidirectional or bi-directional. - SMTC period = 20 ms, - Propagation condition: AWGN. Recommendations for 2nd round: - Discussion on above tentative agreement. - Note: if AWGN is assumed, what is the difference between uni-directional and bi-directional condition for the TC. Companies may want to check this in 2nd round. |
|  | Issue 1-3: Necessity of Test Cases for Timing Related Requirement Discussion summary on 1st round: - Gradual timing adjustment requirement for FR2 PC6 UE: - Requirement is confirmed to be tested in TC A.7.4.1.1. - Whether or not new test cases are needed need FFS: - Needed: Nokia, Ericsson - Reuse A.7.4.1.1: Samsung, CATT, OPPO (need to confirm both tests are needed or not), - One shot large UL timing adjustment for FR2 PC6 UE: - 1 company think it can be tested by TCI state switching delay test: QC - All other companies prefer P3 or P3a. Tentative agreements: - TC for Timing Related Requirement: - TC for gradual timing adjustment requirement for FR2 PC6 UE: - Option 1: Need new test case (similar as A.7.4.1.1) - Option 2: Reuse A.7.4.1.1, and no need to define new test case. - TC for one shot large UL timing adjustment for FR2 PC6 UE - Define new test case to verify one shot large UL timing adjustment requirement. Recommendations for 2nd round: - Discussion on above tentative agreement. - Preference on Option 1 and 2 by further checking A.7.4.1.1 - Details about TC for one shot large UL timing adjustment for FR2 PC6 UE is encouraged. |
|  | Issue 1-4: Necessity of Test Cases for SSB-based RLM and BFD Discussion summary on 1st round: - Either define or not define BFD/RLM test case dedicated for FR2 PC6: - No new test is needed: QC, Samsung, Ericsson, Huawei - New BFD/RML test cases are needed: Nokia, Ericsson, CATT, OPPO Tentative agreements: - TC for SSB-based RLM and BFD: - Option 1: No new SSB-based RLM/BFD test cases dedicated for FR2 PC6. - Option 2: New TCs defined: - SSB-based RLM: two new TCs: - A.7.5.1.X Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS for FR2 Power Class 6 UE - FFS DRX or non-DRX - FFS unidirectional or bi-directional - A.7.5.1.X Radio Link Monitoring In-sync Test for FR2 Pcell configured with SSB-based RLM RS for FR2 Power Class 6 UE - FFS DRX or non-DRX - FFS unidirectional or bi-directional - SSB-based BFD: - One new TC to verify BFD and link Recovery Test based on SSB for DRX = 40ms in A.7.5.5.X. - FFS DRX or non-DRX - FFS unidirectional or bi-directional - SMTC period = 20 ms, Recommendations for 2nd round: - Discussion on above tentative agreement. - If Option 2 is preferred, further discussion on detailed condition and parameters. |
|  | Issue 1-5: Necessity of Test Cases for MAC-CE based TCI State Switch Delay Requirement Discussion summary on 1st round: - Majority support P1 and detailed preference is also provided in test condition and configuration. Tentative agreements: - TC for MAC-CE based TCI State Switch Delay Requirement: - New test case is needed for MAC CE based TCI state switching delay for FR2 PC6: A.7.5.8.X, MAC-CE based active TCI state switch for FR2 PC6 UE - FFS unidirectional or bi-directional - DRX cycle = 40 ms, SMTC period = 20 ms, - Propagation condition: AWGN Recommendations for 2nd round: - Discussion on above tentative agreement. - Note: if AWGN is assumed, what is the difference between uni-directional and bi-directional condition for the TC. Companies may want to check this in 2nd round. |
|  | Issue 1-6: Necessity of Test Cases for L3 Measurement Requirement Discussion summary on 1st round: - Clear majority goes to P2 or P2a to introduce new test cases. - At least three companies show clear preference on non-DRX case, and one company is okay with both non-DRX and DRX. Two companies prefer P2a with DRX configuration. Tentative agreements: - TC for L3 Measurement Requirement: - New test case is needed for A.7.6.1.X, SA event triggered reporting test without gap under non-DRX for PC6 UE - SMTC period = 20 ms, - Propagation condition: AWGN (serving cell) and AWGN with 9722 Hz frequency offset (neighbour cell) Recommendations for 2nd round: - Discussion on above tentative agreement. - It is based on the combined P2 and P2a, for non-DRX and other condition from P2a. |
|  | Issue 1-7: Necessity of Test Cases for L1-RSRP/SINR Measurement Requirement Discussion summary on 1st round: - P3 and P4 for L1-RSRP and L1-SINR gains the majority support. Tentative agreements: - TC for L3 Measurement Requirement: - New test case for A.7.6.3.2, SSB based L1-RSRP measurement when DRX is used for PC6 UE. - DRX cycle = 40 ms, SMTC period = 20 ms, Propagation condition: AWGN with 9722 Hz frequency offset - New test case for A.7.6.6.2, SSB based L1-SINR measurement when DRX is used for PC6 UE. - DRX cycle = 40 ms, SMTC period = 20 ms, Propagation condition: AWGN with 9722 Hz frequency offset Recommendations for 2nd round: - Discussion on above tentative agreement. |
| Sub-Topic 2 | Issue 2-1: Applicability of Set-1 and Set-2 of enhanced RRM requirements Discussion summary on 1st round: - Discussion is based on the below P1 and P2: - P1: Define RRM test cases for both set-1 with 2 Rx beams and set-2 with 6 Rx beams - P2: Test set 1 and 2 in different tests to reduce total number of tests. - Majority companies are okay with the case-by-case manner. Tentative agreements: - Applicability of Set-1 and Set-2 of enhanced RRM requirements: TC category (if confirmed in Sub-Topic 1); Applicability of set1 and set2 (either set1 with 2RX beams or set2 with 6RX beams, or both). TC for Cell Re-selection Requirement; TBD. TC for Re-establishment Delay Requirement; TBD. TC for Timing Related Requirement; N/A (not differentiated by set1 and set2). TCs for SSB-based RLM and BFD; TBD. TC for MAC-CE based TCI State Switch Delay Requirement; TBD (for target TCI is unknown, SSB-based TCI switching to a RS with new QCL-TypeD reference). TC for L3 measurement; TBD. TC for L1-RSRP/SINR Measurement Requirement; TBD. Recommendations for 2nd round: - Discussion on above tentative agreement. - Preference is encouraged to be provided in the table. |
|  | Issue 2-2: Test configuration Discussion summary on 1st round: - Discussion on other aspects of test configuration: - Channel model: AWGN with or without Doppler shift is used in each TC discussion. - Uni- vs Bi-directional, and Doppler - DRX, SMTC cycle: configuration is discussed in each TC case-by-case. Tentative agreements: - Channel model for RRM test cases: - if AWGN is assumed, uni-directional should result in [9722] Hz frequency offset while bi-directional means [19444] Hz. - If Test setup is 1AoA based, the only difference between uni- and bi-directional should be the value of Doppler frequency offset. Recommendations for 2nd round: - Discussion on above tentative agreement. |
| Sub Topic-3 | Issue 3-1: Work Load Splitting for RRM Test Case CR Drafting Discussion summary on 1st round: - All companies agreed: - Work split table to be updated after issues in Sub-Topic 1 are resolved, based on which the work splitting for RRM test cases CR drafting is planned. - Tentative agreements: - N/A. Recommendations for 2nd round: - Work split table to be updated after issues in Sub-Topic 1 are resolved, based on which the work splitting for RRM test cases CR drafting is planned. |
| Sub Topic-4 | Issue 4-1: Measurement accuracy for FR2 HST UE Discussion summary on 1st round: - Majority companies prefer to further study and confirm/disconfirm by providing more simulation results, rather than based on one company’s observation. Tentative agreements: - Measurement accuracy for L1 and L3 measurement: - FFS - Companies are encouraged to provide simulation results in next meeting. Recommendations for 2nd round: - Confirm the above agreement and no more discussion needed in 2nd round. |
|  | Issue 4-2: L1/L3-measurement simulation result and alignment Discussion summary on 1st round: - Simulation results provided based on 2 companies’ inputs. Tentative agreements: - N/A. Recommendations for 2nd round: - Simulation results alignment to be prepared in 2nd round. |
|  | Issue 4-3: Simulation assumption further alignment for L3 measurement Discussion summary on 1st round: - Comments and revisions are provided based on Nokia’s P2. Tentative agreements: - Confirm the simulation assumptions provided in R4-2209525, except: - FFS AWGN with 9722Hz and 19444 Hz offset between desired and interfering cells, respectively for uni- and bi-directional cases - use SINR = -6dB only, and no need to consider “FFS: -8dB, -7dB”. - Recommendations for 2nd round: - Suggest to revise Nokia’s LLS assumption Tdoc (R4-2209525) for approval. - Discussion on above tentative agreement. - Can only the worst case (bi-directional with 19444Hz) be considered? |
|  | Issue 4-4: Simulation assumption further alignment for L1 measurement Discussion summary on 1st round: - Comments and revisions are provided based on Nokia’s P2. Tentative agreements: - Confirm the simulation assumptions provided in R4-2209525, except: - FFS AWGN with 9722Hz and 19444 Hz offset between desired and interfering cells, respectively for uni- and bi-directional cases Recommendations for 2nd round: - Suggest to revise Nokia’s LLS assumption Tdoc (R4-2209525) for approval. - Discussion on above tentative agreement. - Can only the worst case (bi-directional with 19444Hz) be considered? |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

## Summary for 1st round

## Discussion on 2nd round (if applicable)

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on RRM performance requirement for FR2 HST | Samsung |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2209525 | Link simulation assumptions for L1 and L3 measurement accuracy for FR2 HST scenarios | Nokia and NSB | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Samsung | Wang, He (Jackson) | h0809.wang@samsung.com |
| Nokia, Nokia Shanghai Bell | Anthony Lo | Anthony.Lo@nokia.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)