3GPP TSG-RAN WG4 Meeting # 103-e												      R4-2210313

Electronic Meeting, 09 – 20 May 2022

Agenda item:			9.12.1, 9.12.2

Source:	Moderator (THALES)

Title:	Email discussion summary for [103-e][307] NTN_Solutions_General

Document for:	Information

# Introduction

This discussion summary document captures general issues related to RAN4 RF part Rel-17 NR NTN WI, including system parameters, NTN class/Type, and regulatory discussions, including exemplary bands. It contains a summary of the contributions under sections and subsections of Agenda Items 9.12.1, 9.12.2 at TSG-RAN WG4 #103-e, together with identified key open issues and recommends topics/questions to be handled via email discussions. The goal of this document is to provide recommendation on prioritization of discussion.

Please also note the draft TSG-RAN WG4 #103-e meeting agenda with respect to NTN topic:

-------------------------------------- Items led by other working group ----------------------------------------------------

9.12	Solutions for NR to support non-terrestrial networks (NTN)	[NR_NTN_solutions]

9.12.1	General	[NR_NTN_solutions-Core]

*T-docs related to NTN UE features for system parameters and RF can be submitted under this AI.

9.12.2	Coexistence aspects	[NR_NTN_solutions-Core]

9.12.3	Satellite Access Node RF requirements 	[NR_NTN_solutions-Core]

9.12.3.1	TX requirements for radiated characteristics	[NR_NTN_solutions-Core]

9.12.3.2	RX requirements for radiated characteristics 	[NR_NTN_solutions-Core]

9.12.3.3	Tx requirements for conducted characteristics	[NR_NTN_solutions-Core]

9.12.3.4	Rx requirements for conducted characteristics	[NR_NTN_solutions-Core]

9.12.4	Satellite Access Node RF conformance testing 	[NR_NTN_solutions-Perf]

9.12.4.1	General and work plan	 [NR_NTN_solutions-Perf]

9.12.4.2	Conductive conformance Testing 	[NR_NTN_solutions-Perf]

9.12.4.3	Radiated conformance Testing 	[NR_NTN_solutions-Perf]

9.12.5	UE RF requirements 	[NR_NTN_solutions-Core]

9.12.5.1	TX requirements	[NR_NTN_solutions-Core]

9.12.5.2	RX requirements 	[NR_NTN_solutions-Core]

9.12.6	RRM core requirements	[NR_NTN_solutions-Core]

* R2-2204114 LS on measurement gaps enhancements for NTN

9.12.6.1	General	[NR_NTN_solutions-Core]

9.12.6.2	GNSS-related requirements 	[NR_NTN_solutions-Core]

9.12.6.3	Mobility requirements 	[NR_NTN_solutions-Core]

9.12.6.4	Timing requirements	[NR_NTN_solutions-Core]

9.12.6.5	Measurement procedure requirements	[NR_NTN_solutions-Core]

9.12.7	RRM performance requirements	[NR_NTN_solutions-Perf]

9.12.8	Demodulation requirements	[NR_NTN_solutions-Perf]

9.12.8.1	General 	[NR_NTN_solutions-Perf]

9.12.8.2	Satellite Access Node demodulation requirements 	[NR_NTN_solutions-Perf]

9.12.8.2.1	PUSCH requirements	 [NR_NTN_solutions-Perf]

9.12.8.2.2	PUCCH requirements 	[NR_NTN_solutions-Perf]

9.12.8.2.3	PRACH requirements 	[NR_NTN_solutions-Perf]

9.12.8.3	UE demodulation requirements	 [NR_NTN_solutions-Perf]

9.12.8.3.1	PDSCH requirements	 [NR_NTN_solutions-Perf]

For informative purpose, RAN4#103-e E-meeting Arrangements and Guidelines proposed the following schedule:

A total of 15 TDocs have been submitted under AIs 9.12.1 and 9.12.2, but only 10 Tdocs have been identified for discussion in [103-e][307] NTN_Solutions_General (please also see the Appendix for the details, with all the observations/proposals):

| TDoc Number | TDoc Type | Title | Company | Status | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- | --- |
| R4-2208246 (not uploaded) | discussion | Remaining issue for General aspects for SAN | CATT | reserved | Discussion | 9.12.1 |
| R4-2208333 | pCR | Draft Text Proposal to Update TR 38.863 Chapter 3,6 and 8 | Samsung | available | Approval | 9.12.1 |
| R4-2208334 | pCR | Draft Text Proposal to Update TR 38.863 structure | Samsung | available | Approval | 9.12.1 |
| R4-2208640 (for approved TPs) | draft TR | Draft TR 38.863 v0.4.0 | Samsung | reserved | Agreement | 9.12.1 |
| R4-2208641 (for approved TPs) | draft TS | Draft TS 38.101-5 v0.2.0 | Samsung | reserved | Agreement | 9.12.1 |
| R4-2209676 (not uploaded) | pCR | TP to TS 38.108: TS corrections; general parts | Huawei, HiSilicon | withdrawn | Approval | 9.12.1 |
| R4-2208889 | pCR | TP to TR 38.863 - Updates | Ericsson | available | Approval | 9.12.1 |
| R4-2210189 (for approved TPs) | draft TS | Draft TS 38.108 v0.2.0 | THALES | reserved | Agreement | 9.12.1 |
| R4-2209992 | pCR | Draft text proposal for Clause 3 - TS 38.101-5 | THALES | available | Approval | 9.12.1 |
| R4-2209994 | pCR | pCR for Clause 3.3 Abbreviations - TS 38.108 | THALES | available | Approval | 9.12.1 |
| R4-2209995 | pCR | pCR for Clause 4.3 Requirement reference points - TS 38.108 | THALES | available | Approval | 9.12.1 |
| R4-2209990 | pCR | pCR for Annex D - TS 38.108 | THALES | available | Approval | 9.12.1 |
| R4-2209917 | Work Plan | NR_NTN_solutions work plan | THALES | available | Endorsement | 9.12.1 |
| R4-2209997 | pCR | Draft text proposal for Clause 6.1 Coexistence Figures - TR 38.863 | THALES | available | Approval | 9.12.2 |
| R4-2210228 | pCR | Draft text proposal for Clauses 6.4 and 6.5 Corrections Typos - TR 38.863 | THALES | available | Approval | 9.12.2 |

Moderator note1: There are 5 pCRs to TR 38.863, which the moderator proposes to discuss in the dedicated folders from 1st round and 2nd round.

Moderator note2: There are 3 pCRs to TR 38.108, which the moderator proposes to discuss in the dedicated folders from 1st round and 2nd round.

Moderator note3: There are 1 pCRs to TR 38.101-5, which the moderator proposes to discuss in the dedicated folders from 1st round and 2nd round.

Moderator note4: The TS/TR reserved numbers are used to update the respective TS/TR at the end of the meeting with agreed TPs/pCRs.

Identified topics and issues for the 1st round:

- Topic #1: pCRs/CRs

  - Issue 1-1-1: pCR to TR 38.863 – see R4-2208333 (Samsung)

  - Issue 1-1-2: pCR to TR 38.863 – see R4-2208334 (Samsung)

  - Issue 1-1-3: pCR to TR 38.863 – see R4-2208889 (Ericsson)

  - Issue 1-1-4: pCR to TR 38.863 – see R4-2209997 (THALES)

  - Issue 1-1-5: pCR to TS 38.863 – see R4-2210228 (THALES)

  - Issue 1-2-1: pCR to TS 38.108 – see R4-2209994 (THALES)

  - Issue 1-2-2: pCR to TS 38.108 – see R4-2209995 (THALES)

  - Issue 1-2-3: pCR to TS 38.108 – see R4-2209990 (THALES)

  - Issue 1-3-1: pCR to TS 38.101-5 – see R4-2209992 (THALES)

- Topic #2: NTN Satellite Work Plan

  - Issue 2-1-1: NR_NTN_solutions work plan for RAN4 RF

  - Issue 2-2-1: NR_NTN_solutions work plan for RAN4 RRM

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

# Topic #1: pCRs/CRs

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-Doc Number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2208333 | Samsung | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. Draft Text Proposal to Update TR 38.863 Chapter 3,6 and 8 |
| R4-2208334 | Samsung | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. Draft Text Proposal to Update TR 38.863 structure |
| R4-2208889 | Ericsson | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. TP to TR 38.863 – Updates |
| R4-2209992 | THALES | TP to TS 38.101-5 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clause 3 - TS 38.101-5 |
| R4-2209994 | THALES | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. pCR for Clause 3.3 Abbreviations - TS 38.108 |
| R4-2209995 | THALES | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. pCR for Clause 4.3 Requirement reference points - TS 38.108 |
| R4-2209990 | THALES | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. pCR for Annex D - TS 38.108 |
| R4-2209997 | THALES | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clause 6.1 Coexistence Figures - TR 38.863 |
| R4-2210228 | THALES | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clauses 6.4 and 6.5 Corrections Typos - TR 38.863 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description: pCRs to TR 38.863

Open issues and candidate options before e-meeting:

Issue 1-1-1: pCR to TR 38.863 – see R4-2208333 (Samsung)

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [103-e][307] folders in both Round 1 and Round 2.

Issue 1-1-2: pCR to TR 38.863 – see R4-2208334 (Samsung)

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [103-e][307] folders in both Round 1 and Round 2.

Issue 1-1-3: pCR to TR 38.863 – see R4-2208889 (Ericsson)

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [103-e][307] folders in both Round 1 and Round 2.

Issue 1-1-4: pCR to TR 38.863 – see R4-2209997 (THALES)

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [103-e][307] folders in both Round 1 and Round 2.

Issue 1-1-5: pCR to TR 38.863 – see R4-2210228 (THALES)

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [103-e][307] folders in both Round 1 and Round 2.

### Sub-topic 1-2

Sub-topic description pCR to TS 38.108

Open issues and candidate options before e-meeting:

Issue 1-2-1: pCR to TS 38.108 – see R4-2209994 (THALES)

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [103-e][307] folders in both Round 1 and Round 2.

Issue 1-2-2: pCR to TS 38.108 – see R4-2209995 (THALES)

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [103-e][307] folders in both Round 1 and Round 2.

Issue 1-2-3: pCR to TS 38.108 – see R4-2209990 (THALES)

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [103-e][307] folders in both Round 1 and Round 2.

### Sub-topic 1-3

Sub-topic description pCR to TS 38.101-5

Open issues and candidate options before e-meeting:

Issue 1-3-1: pCR to TS 38.101-5 – see R4-2209992 (THALES)

- Proposals

  - Option 1: Agree pCR

- Recommended WF

  - Continue discussion/correct pCR if needed in dedicated [103-e][307] folders in both Round 1 and Round 2.

## Companies views’ collection for 1st round

### Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Example 1

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| Ericsson | Issue 1-1-4: The figure 6.1-1 should be updated, removing : " far-away", "severely impacted" and "transparent" Issue 1-2-1: We have also a tdoc to update the same clause (R4-2208888), tdocs should be merged. Issue 1-2-2: Why the last sentence is removed? Issue 1-2-3: PDSCH should not be used when testing ACS. Issue 1-3-1: We have also a tdoc to update the same clause (R4-2208886), tdocs should be merged. |
| CATT | Issue 1-2-1: CACLR should be removed since we only consider single carrier operation. Issue 1-2-2: same question as Ericsson. If 8 is not the typical number for SAN. Then it seems we can adapt the wording a little bit rather than deleting it. Issue 1-3-1: Some abbreviations are related to MIMO or CA and should be deleted as well. |
| THALES | Issue 1-1-3: We do not agree with the phase, please see comment in the R4-2208889 dedicated folder. Simulation results are clear enough, and the satellite beams might overlap with terrestrial networks, therefore the phrase seems to be misleading. THALES2: Would something like this be reasonable? Note: The RAN4 coexistence analysis assumes NTN Satellite operation is assumed to be outside a Terrestrial Network operating in an adjacent to the channel/band used for Terrestrial operation. However, the NTN satellite and terrestrial coverage may overlap. THALES3: or something like this: Note: Satellite operation is used complementarily to Terrestrial Network operation, and in separate location when they operatesatellite operates in an adjacent channel/band of terrestrial network. For instance, when NTN UE connected to satellite, NTN satellite UE is assumed to connect and transmit to the satellite when it is outside a Terrestrial Network operating in an adjacent channel/band. The satellite beam might still overlap the terrestrial network. Issue 1-1-4: To Ericsson: Sure, we can do that. Issue 1-2-1: To Ericsson: Yes, sure, we can merge R4-2209994 with R4-2208888. Can Ericsson take the lead of this merge? To CATT: fine for us. Could CATT provide corrections directly in the document? Issue 1-2-2: To Ericsson: The reason is simple, since we think we should not limit the active antenna of a SAN type 1-O with minimum 8Tx/Rx in the 38.108 specification. THALES2: However, if Ericsson insists, we could also keep the phrase. Please feedback if you have different opinion. Issue 1-2-3: To Ericsson: On the contrary, 38 dB ACS value used to compute the interfering signal mean power (see e.g. pCR to TS 38.108 R4-2210030 and current TR 38.863) is obtained from Coexistence Scenario 6 where TN BS (DL) is interfering with NTN Satellite (Satellite UL, Satellite RX). In this particular case, PDSCH should be used when testing Satellite ACS. Issue 1-3-1: To Ericsson: Yes, sure, we can merge R4-2209992 with R4-2208886 (or vice-versa). In this particular case, THALES was the lead of the initial document. Could Ericsson provide corrections to R4-2209992? To CATT: fine for us. Could CATT provide corrections directly in the document? |
| Hughes/EchoStar | Issue 1-1-3: We cannot agree to the “Note: Satellite operation is assumed to be outside a Terrestrial Network operating in an adjacent channel/band.” The adjacent co-existence analysis NTN-TN already took care of this matter. Issue 1-3-1: Looks OK |
| Qualcomm | Issue 1-1-3: the intention of adding the Note is to solve the UE-UE co-existence issue discussed in NTN UE RF thread? |
| Ericsson | Issue 1-1-3: To Thales and Hughes-EchoStar: In last RAN4#102-e, there was an agreement (R4-2207346, topic#3) to capture this note (exact same wording) in the TR, and not TS 38.108/38.101-5. I hope no one is coming on any past agreement then. We are open to improve the wording if we could find some better one that would please everyone but, if not, we will have to stick to the agreement. To Qualcomm: No, this was not the intention. This tdoc only captures the agreement made in R4-2207346 Issue 1-2-1: To Thales: yes, we can take the lead on this TP. Issue 1-2-3: To Thales: But ACS requirement is a general requirement, not only valid for scenario 6. What’s the concern using UL characteristics then? Issue 1-3-1: To Thales: Please check our tdoc R4-2208886. |
| MediaTek | Issue 1-1-1: Note that SAN is not in the abbreviations. May be good to add that. Issue 1-1-2: With this change, the conclusions of the coexistence study give the perception now that they are only valid for S-band. However, the generic requirements are also applicable for requirements in other FR1 bands in our understanding, at least L band. Did I misunderstand something here? Issue 1-1-3: Adjacent band? We did coexistence analysis for adjacent channels. For the UE-UE coexistence that is a different issue with different solutions discussed. Issue 1-3-1: Suggest to add “Satellite Access Node” to the definitions and reference the spec where it is defined. |
| Hughes/EchoStar | Issue 1-1-3: We cannot agree to the Note. It undermines the adjacent band co-existence analyses that were done to ensure the band can be deployed in the same areas as adjacent bands while ensuring not more than 5% service degradation. If we have this note then satellite system can only be deployed in the ocean or in deep unserved areas where there is completely no terrestrial systems (e, g n1 is deployed in most countries). It is understood that it cannot be deployed where there is overlapping band such as in the US, but it is ridiculous for 3GPP to assume that it cannot be deployed in adjacent band areas. Again, to be clear - that was the whole purpose of the adjacent band co-existence analysis to ensure the band can be deployed in the same areas while ensure not more than 5% service degradation. |
| Omnispace | Issue 1-1-3 Strongly oppose to the addition of the Note: Satellite operation is assumed to be outside a Terrestrial Network operating in an adjacent channel/band.” There is no technical reason why this should be included as co-existence studies between TN-NTN networks carried out in the past have show that there are no issues. This is a political statement rather than a technical one! |
| HISPASAT | Issue 1-1-3 We DO NOT agree with the Note (Satellite operation is assumed to be outside a Terrestrial Network operating in an adjacent channel/band) included in this issue, no technical reason as previously demonstrated to indicate such statement. |
| Huawei | Issue 1-2-2: If the only concern is about the minimum number of 8, we can further discuss it. But I’m not sure if it’s necessary to remove all the sentence. Issue 1-2-3: I think Annex D is for specific configuration of signal. For ACS test, the main issue is the interference level. Choosing PUSCH or PDSCH will not bring much deviation from test perspective. Thus, for the specific test configuration, it’s enough to choose one kind of configuration, i.e. PUSCH. That is my understanding. |
| Samsung | Response to MediaTek’s questions Issue 1-1-1: SAN will be added. Issue 1-1-2: It is the agreement in RAN4 #98e meeting, both L-band and S-band are exemplary bands and relative requirements will be studied and captured in the TR. The requirements in Chapter 7 cover both L-band and S-Band (hereby, n255, n256 respectively) It was also agreed that 2GHz is used only as the frequency band for co-existence studies. So, it doesn’t imply the results in Chapter 6 would apply to S-Band only. However, the title of Session 6.5.1 could be changed as “Coexistence in 2GHz” since the original text “Coexistence in S-band (2GHz)” may cause similar confusion to readers indicating the results is for S-band. The proposed text in conclusion part is “to take due consideration of contents in Chapter 6 and Chapter 7” which we believe is a quite soft expression. Hope such clarification and modification could ease the concern or confusion. |
| ZTE | Issue 1-1-3: We support that, currently from n1 dl interfering NTN DL, it requires 1.5km isolation regions. We could see the problem from NTN N256 UL interfering TN band n34. isolation is also needed. Issue 1-2-2: We are fine to remove that since 8 is coming from 8 MIMO layer from LTE spec, however MIMO is not supported on SAN, we are fine to remove it. Issue 1-2-3: There is no PDSCH for ACS requirements; |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2208333 | See comments received for Issue 1-1-1. |
|  | Company B |
| R4-2208334 | See comments received for Issue 1-1-2. |
|  | Company B |
| R4-2208889 | See comments received for Issue 1-1-3. |
| R4-2209997 | See comments received for Issue 1-1-4. |
|  | Removed “far-away”, “severely” and “transparent” |
| R4-2210228 | No Comments Received on FTP or in Draft Summary |
| R4-2209994 | See comments received for Issue 1-2-1. |
| R4-2209995 | See comments received for Issue 1-2-2. |
| R4-2209990 | See comments received for Issue 1-2-3. |
| R4-2209992 | See comments received for Issue 1-3-1. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1-3 | Tentative agreements: Note: Satellite operation is used complementarily to Terrestrial Network operation, and in separate location when they operatesatellite operates in an adjacent channel/band of terrestrial network. For instance, when NTN UE connected to satellite, NTN satellite UE is assumed to connect and transmit to the satellite when it is outside a Terrestrial Network operating in an adjacent channel/band. The satellite beam might still overlap the terrestrial network. Candidate options: Recommendations for 2nd round: |
| Sub-topic #1-2-2 | Tentative agreements: Remove the phrase as indicated in R4-2209995 (Requirement reference points – TS 38.108). 8 is coming from 8 MIMO layer from LTE spec, however MIMO is not supported on SAN. Candidate options: Recommendations for 2nd round: |
| Sub-topic #1-2-3 | Tentative agreements: 38 dB ACS value was obtained for a specific coexistence case when BS (Tx) is interfering with satellite (Rx). 38 dB ACS value is further used to derive the interferer signal mean power for the ACS test. In order to be consistent with the coexistence analysis result (worst case) from Scenario 6 (see TR 38.863), PDSCH should be used in TS 38.108 when testing Satellite ACS. Candidate options: Recommendations for 2nd round: |
| Sub-topic #1-3-1 | Tentative agreements: Ericsson and CATT to verify/provide further corrections to R4-2209992 if required. Candidate options: Recommendations for 2nd round: |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Topic #2: NTN Satellite Work Plan

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2209917 | THALES | RAN4 work plan for endorsement NR_NTN_solutions work plan RAN4 ## WG RAN4 RF The following RAN4 RF work plan for NR support non-terrestrial network WI is proposed: May 2022, RAN4#103-e - Agree on the RF core requirements (UE and “BS” requirements) for NTN; - Agree on exemplary band(s) specific requirements ; - Finalize demodulation requirements; - Finalize RF conformance testing; - Endorse CRs; - Approve the TS skeleton and work split on RF conformance testing for TS 38.181 (exact split to company can be done be the TS rapporteur); - General discussion on conformance testing and approve the guidance; - Discussion on demodulation performance part. August 2022, RAN4#104 meeting - Further discuss the remaining issue for conformance test; - Discussion for each test cases and approve the test cases as much as possible; - Agree work split for demod performance testing part (exact split to company can be done by the TS rapporteur). November 2022, RAN4#105-e-meeting - Further discussion on RF conformance part testing and approve the TPs; - Further discussion on demodulation part testing and approve the TPs; - Approve all the TPs for all the remaining issues. ## WG RAN4 RRM The following RAN4 RRM work plan for NR support non-terrestrial network WI is proposed: May 2022, RAN4#103-e - Agree on the RRM core requirements for NTN; - Agree if there is any specific requirement associated to the selected exemplary bands and simulations results; - Agree CRs. August 2022, RAN4#104, TBD - Discuss and decide RRM test cases and related parameters; - Discuss and specify measurement accuracy; November 2022, RAN4#105, TBD - approve the CRs for TS 38.133 for the RRM performance part. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description: RAN4 RF Work Plan

Open issues and candidate options before e-meeting:

Issue 2-1-1: NR_NTN_solutions work plan for RAN4 RF

- Proposals

  - Option 1: Agree with proposal

  - Option 2: Comments, suggestions for timeline/other subjects, any kind of recommendation - please provide them in the tables

- Recommended WF

  - TBA

### Sub-topic 2-2

Sub-topic description: RAN4 RRM Work Plan

Open issues and candidate options before e-meeting:

Issue 2-2-1: NR_NTN_solutions work plan for RAN4 RF

- Proposals

  - Option 1: Agree with proposal

  - Option 2: Comments, suggestions for timeline/other subjects, any kind of recommendation - please provide them in the tables

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

Example 1

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| Ericsson | Issue 2-1-1: Why RF conformance testing should be finalized in this meeting, while it’s also proposed to further discuss this part in future meetings? We should better have a separate plan for UE demod and SAN demod. |
| CATT | Issue 2-1-1: it is important to approve 38.181 skeleton considering the tight timeline to complete conformance part. We are ok with the work plan in general. But the sentence “Finalize RF conformance testing “ need to be deleted as commented by Ericsson |
| THALES | New proposal(s): RAN4 work plan for endorsement NR_NTN_solutions work plan RAN4 ## WG RAN4 RF The following RAN4 RF work plan for NR support non-terrestrial network WI is proposed: May 2022, RAN4#103-e - Agree on the RF core requirements (UE and “BS” requirements) for NTN; - Agree on exemplary band(s) specific requirements ; - Finalize demodulation requirements; - Finalize RF conformance testing; - Endorse CRs; - Approve the TS skeleton and work split on RF conformance testing for TS 38.181 (exact split to company can be done be the TS rapporteur); - General discussion on conformance testing and approve the guidance; - Discussion on demodulation performance part. August 2022, RAN4#104 meeting - Discuss UE RF conformance testing - Discuss SAN RF conformance testing - Further discuss the remaining issue for conformance test; - Discussion for each test cases and approve the test cases as much as possible; - Agree work split for SAN and UE demod performance testing part (exact split to company can be done by the TS rapporteur). November 2022, RAN4#105-e-meeting - Further discussion on SAN RF conformance part testing and approve the TPs; - Further discussion on UE RF conformance part testing and approve the TPs; - Further discussion on SAN demodulation part testing and approve the TPs; - Further discussion on UE demodulation part testing and approve the TPs; - Approve all the TPs for all the remaining issues. ## WG RAN4 RRM The following RAN4 RRM work plan for NR support non-terrestrial network WI is proposed: May 2022, RAN4#103-e - Agree on the RRM core requirements for NTN; - Agree if there is any specific requirement associated to the selected exemplary bands and simulations results; - Agree CRs. August 2022, RAN4#104, TBD - Discuss and decide RRM test cases and related parameters; - Discuss and specify measurement accuracy; November 2022, RAN4#105, TBD - approve the CRs for TS 38.133 for the RRM performance part. |
| Qualcomm | Issue 2-1-1: Share the similar view as Ericsson. It would be better to have a split plan for Demod. ## WG RAN4 RF ## WG RAN4 RRM ## WG RAN4 Demod |
| MediaTek | It would be best to split the Core remaining finalization from the Performance objectives, and yes splitting like others have suggested would be good. For Core work maybe it is best just to refer to just refer to finalizing the issues in the exception sheet agreed at plenary? |
| ZTE | Tend to agree with Ericsson, it’s better to discuss the demod plan and RRM plan in the corresponding thread. In addition, based on the latest agreement on F2F decision on RAN4 meeting, we might have Aug and Oct e-meeting. In this meeting, it’s impossible to complete the NTN conformance testing and we might need more discussion, the current work plan for SAN conformance testing is too aggressive since we are still discussing the skeleton for SAN conformance testing. |
| Huawei | The adjusted work-plan proposal is to be discussed in the second round. Agree on the split of RF/demod/RRM. After the first round we may already see what can/cannot be completed during this May meeting. It would be good to incorporate it in the final version to make the plan more realistic. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-3-1 | Tentative agreements: RAN4 work plan for endorsement NR_NTN_solutions work plan RAN4 ## WG RAN4 RF The following RAN4 RF work plan for NR support non-terrestrial network WI is proposed: May 2022, RAN4#103-e meeting - Agree on the RF core requirements (UE and “BS” requirements) for NTN; - Agree on exemplary band(s) specific requirements ; - Endorse CRs; - Approve the TS skeleton and work split on RF conformance testing for TS 38.181 (exact split to company can be done be the TS rapporteur); - General discussion on conformance testing and approve the guidance; August 2022, RAN4#104-e meeting - Discuss UE RF conformance testing - Discuss SAN RF conformance testing - Discussion for each test cases and approve the test cases as much as possible; October 2022, RAN4#104-bis-e meeting - Further discussion on SAN RF conformance part testing and approve the TPs; - Further discussion on UE RF conformance part testing and approve the TPs; - Approve all the TPs for all the remaining issues. November 2022, RAN4#105, TBD - Approve all the TPs for all the remaining issues. ## WG RAN4 RRM The following RAN4 RRM work plan for NR support non-terrestrial network WI is proposed: May 2022, RAN4#103-e meeting - Agree on the RRM core requirements for NTN; - Agree if there is any specific requirement associated to the selected exemplary bands and simulations results; - Agree CRs. August 2022, RAN4#104-e meeting - Discuss and decide RRM test cases and related parameters; - Discuss and specify measurement accuracy; October 2022, RAN4#104-bis-e meeting - Approve the CRs for TS 38.133 for the RRM performance part. November 2022, RAN4#105, TBD - Approve the CRs for TS 38.133 for the RRM performance part. ## WG RAN4 Demod The following RAN4 Demod work plan for NR support non-terrestrial network WI is proposed: May 2022, RAN4#103-e meeting - Finalize demodulation requirements; - Discussion on demodulation performance part. August 2022, RAN4#104-e meeting - Agree work split for SAN and UE demod performance testing part (exact split to company can be done by the TS rapporteur). October 2022, RAN4#104-bis-e meeting - Further discussion on SAN demodulation part testing and approve the TPs; - Further discussion on UE demodulation part testing and approve the TPs; - Approve all the TPs for all the remaining issues. November 2022, RAN4#105, TBD - Approve all the TPs for all the remaining issues. Candidate options: Recommendations for 2nd round: |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| R4-221xxxx | Way Forward on NTN_Solutions_General | THALES | Document # R4-221xxxx WF [103-e][307] NTN_Solutions_General |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2208333 | R4-221xxxx | Draft Text Proposal to Update TR 38.863 Chapter 3,6 and 8 | Samsung | Agreeable | Document updated/revised |
| R4-2208334 | R4-221xxxx | Draft Text Proposal to Update TR 38.863 structure | Samsung | Agreeable | Document updated/revised |
| R4-2208889 | R4-221xxxx | TP to TR 38.863 - Updates | Ericsson | To be revised | Discussion Ongoing Topic recommended for further discussion |
| R4-2209997 | R4-221xxxx | Draft text proposal for Clause 6.1 Coexistence Figures - TR 38.863 | THALES | Revised | Document updated/revised |
| R4-2210228 | N/A | Draft text proposal for Clauses 6.4 and 6.5 Corrections Typos - TR 38.863 | THALES | Agreeable | No comment Received |
| R4-2209994 | N/A | pCR for Clause 3.3 Abbreviations - TS 38.108 | THALES | Merge with R4-2208888 | Document updated/revised |
| R4-2209995 | R4-221xxxx | pCR for Clause 4.3 Requirement reference points - TS 38.108 | THALES | To be revised (if required), but it seems Agreeable | Topic recommended for further discussion |
| R4-2209990 | R4-221xxxx | pCR for Annex D - TS 38.108 | THALES | To be revised (if required) | Discussion Ongoing Topic recommended for further discussion |
| R4-2209992 | R4-221xxxx | Draft text proposal for Clause 3 - TS 38.101-5 | THALES | To be revised (if required), but it seems Agreeable (Merge abbreviations from R4-2208886 into R4-2209992) | Topic recommended for further discussion |
| R4-2209917 | R4-221xxxx | NR_NTN_solutions work plan | THALES | Revised | Discussion Ongoing Topic recommended for further discussion |
| R4-2210313 | R4-221xxxx | Email discussion summary for [103-e][307] NTN_Solutions_General | THALES | To be revised for 2nd Round | To be noted after the 2nd round |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| THALES | Dorin Panaitopol |  |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Appendix: Companies contribution summary

Contribution summaries for [103-e][307] NTN_Solutions_General thread are as follows:

| TDoc Number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2208246 (not uploaded) | CATT | N/A |
| R4-2208333 | Samsung | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. Draft Text Proposal to Update TR 38.863 Chapter 3,6 and 8 |
| R4-2208334 | Samsung | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. Draft Text Proposal to Update TR 38.863 structure |
| R4-2208640 (for approved TPs) | Samsung | For submission at the end of the meeting, after incorporating approved TPs Draft TR 38.863 v0.4.0 |
| R4-2208641 (for approved TPs) | Samsung | For submission at the end of the meeting, after incorporating approved TPs Draft TS 38.101-5 v0.2.0 |
| R4-2209676 (not uploaded) | Huawei, HiSilicon | N/A |
| R4-2208889 | Ericsson | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. TP to TR 38.863 – Updates |
| R4-2210189 (for approved TPs) | THALES | For submission at the end of the meeting, after incorporating approved TPs Draft TS 38.108 v0.2.0 |
| R4-2209992 | THALES | TP to TS 38.101-5 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clause 3 - TS 38.101-5 |
| R4-2209994 | THALES | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. pCR for Clause 3.3 Abbreviations - TS 38.108 |
| R4-2209995 | THALES | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. pCR for Clause 4.3 Requirement reference points - TS 38.108 |
| R4-2209990 | THALES | TP to TS 38.108 to be discussed in the dedicated 1st round & 2nd round folders. pCR for Annex D - TS 38.108 |
| R4-2209917 | THALES | RAN4 work plan for endorsement NR_NTN_solutions work plan RAN4 |
| R4-2209997 | THALES | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clause 6.1 Coexistence Figures - TR 38.863 |
| R4-2210228 | THALES | TP to TR 38.863 to be discussed in the dedicated 1st round & 2nd round folders. Draft text proposal for Clauses 6.4 and 6.5 Corrections Typos - TR 38.863 |