3GPP TSG-RAN WG4 Meeting # 103-e													R4-2210335

Electronic Meeting, 9th May – 20th May 2022

Agenda item:			9.19.5

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [103-e][329] NR_RedCap_Demod

Document for:	Information

# Introduction

This email discussion threads discusses the UE demodulation and CSI reporting requirements for RedCap. The target of the email discussion is summarized as follows:

- 1st round:

  - Solve the remaining open issues

  - Collect the simulation results

- 2nd round:

  - Continue to solve the remaining open issues.

  - Discuss the detailed simulation assumptions.

  - Draft CR work split

When updating this document, please remember to:

- At the beginning of the first round, the moderator shares the summary in /ftp/tsg_ran/WG4_Radio/TSGR4_103-e/Inbox/Drafts/[103-e][329] NR_RedCap_Demod/Round 1/Summary_329_1st_v00.docx

- After update by company A: Summary_329_1st_v01_companyA.docx

- After update by company B: Summary_329_1st_v02_companyA_companyB.docx

- After update by company C: Summary_329_1st_v03_companyB_companyC.docx

# Topic #1: General topics for UE demodulation and CSI reporting requirements

## Companies’ contributions summary

### General (9.19.5.1)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2209055 | Ericsson | Summary of simulation results |
| R4-2209056 | Ericsson | Update of work plan and CR work split proposal |
| R4-2209057 | Ericsson | Skeleton of CR for UE demodulation and CSI reporting requirements for RedCap |
| R4-2209058 | Ericsson | Proposal 1: Define RedCap demodulation and CSI reporting requirements in TS38.101-4 with the suffix ‘I’, as same as TS38.101-1/2. Observation 1: According to RAN1 feature list and RAN#95-e decision, supporting half-duplex FDD is per-band capability, i.e., UE is not mandated to support full-duplex FDD if UE support half-duplex FDD. Proposal 2: For the requirement specification of FDD 1Rx UE, consider full-duplex FDD and half-duplex FDD with applicability rule. Proposal 3: Specify the following applicability table for RedCap UE supporting half-duplex FDD. UE feature/capability; Test type; Test list; Applicability notes. Half-duplex FDD operation type A for RedCap UE; FR1 Half-duplex FDD; PDSCH; All tests in Clause [5.2I.1.1]; The requirements apply for UE supporting 1Rx. FR1 Half-duplex FDD; PDCCH; All tests in Clause [5.3I.1.1]; The requirements apply for UE supporting 1Rx. FR1 Half-duplex FDD; PBCH; All tests in Clause [5.4I.1.1]; The requirements apply for UE supporting 1Rx. FR1 Half-duplex FDD; SDR; All tests in Clause [5.5I.1]; The requirements apply for UE supporting 1Rx. UE feature/capability; Test type; Test list; Applicability notes. Half-duplex FDD operation type A for RedCap UE; FR1 Half-duplex FDD; CQI; All tests in Clause [6.2I.1.1]; The requirements apply for UE supporting 1Rx. FR1 Half-duplex FDD; PMI; All tests in Clause [6.3I.1.1]; The requirements apply for UE supporting 1Rx. Proposal 4: Define FRC of UE demodulation and CSI reporting requirements for RedCap 1Rx UE in FDD by assuming the half-duplex FDD operation, that is, DDDSU. |
| R4-2209705 | Nokia, Nokia Shanghai Bell | Proposal 1: Select Option 1: Consider Full-duplex FDD only. |

### PDSCH/SDR requirements (9.19.5.2.1)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207810 | Apple | Proposal #1: Do not define additional PDSCH demod requirements or requirements with 256QAM for RedCap UE. |
| R4-2209059 | Ericsson | Proposal 1: If RAN4 defines 256QAM demodulation requirements for RedCap UE in FR1, set MCS22 for 1Rx case. |
| R4-2209706 | Nokia, Nokia Shanghai Bell | Proposal 1: Focus on definition of minimum set of requirements, discussed in Topic #2, to verify the mandatory features. RAN4 to potentially discuss other requirements once mandatory requirements are stable and pending remaining performance part TUs. Observation 1: Support for 256QAM is optional feature for RedCap UE in FR1. Proposal 2: Do not specify 256QAM demodulation requirements. |
| R4-2209707 | Nokia, Nokia Shanghai Bell | Simulation results |
| R4-2209797 | MediaTek inc. | Proposal 1: Use MCS20 to define 256QAM demodulation requirements for RedCap UE. |
| R4-2209832 | Huawei, HiSilicon | Proposal 1: Consider Full-duplex FDD only for FDD tests for 1 Rx UE. Proposal 2: Not define any additional PDSCH demodulation requirements other than those agreed in last RAN4 meeting in Rel-17 for RedCap. Proposal 3: Specify 256QAM demodulation requirements for FR1 only with reusing the existing requirements of TS38.101-4 5.2.2.1.1-3 Test 1-3 for FDD 15kHz and TS38.101-4 5.2.2.2.1-3 Test 1-3 for TDD 30kHz. Proposal 4: Reuse the existing test procedure and test setup with removal unfeasible test parameters from Rel-15 SDR test. Proposal 5: For FR1, define SDR requirements with configuration 1T1R for UE supporting 1 layer and 2T2R for UE supporting 2 layers. For FR2, define SDR requirements for 2 layers only with configuration 2T2R. |
| R4-2209833 | Huawei, HiSilicon | Simulation results |
| R4-2210146 | Qualcomm Incorporated | Simulation results |

### PDCCH/PBCH requirements (9.19.5.2.2)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207811 | Apple | Proposal #1: Use AL 4 for PDCCH requirements for RedCap UE. |
| R4-2209060 | Ericsson | Proposal 1: Define both AL4/AL8 for PDCCH demodulation requirements for RedCap. Observation 1: PDSCH demodulation test uses PDCCH AL8 with 2Tx to transmit DCI 1_1. Proposal 2: If RAN4 need to choose either AL4 or AL8, choose AL4 for FR1 FDD/TDD and FR2 TDD. PDCCH AL8 performance can be guaranteed during the PDSCH demodulation tests. Proposal 3: Set REG bundle size to 2 for FR1 TDD PDCCH demodulation requirements. Proposal 4: Set CORESET duration to 1 for FR1 TDD PDCCH demodulation requirements. |
| R4-2209708 | Nokia, Nokia Shanghai Bell | Observation 1: AL16 is specified for RRM requirements and shows strong performance gains of >7dB, when compared to AL4 with the configurations of current PDCCH requirements. Proposal 1: Specify PDCCH demodulation requirements for AL4, AL8 and AL16 for FR1 FDD 15 kHz and FR1 TDD 30 kHz, both for 1 Rx and 2 Rx requirements, and AL4, AL8 for FR2 TDD 120 kHz, for 2 Rx requirements and corresponding test cases. |
| R4-2209709 | Nokia, Nokia Shanghai Bell | Simulation results |
| R4-2209798 | MediaTek inc. | Observation 1: It is possible to configure CORESET 1 symbol duration for TDD in 20MHz channel bandwidth. Proposal 1: Use 1 CORESET symbol to define test cases for TDD in 20MHz channel bandwidth. |
| R4-2209834 | Huawei, HiSilicon | Proposal 1: Use AL4 for case with 1TX and use AL8 for case with 2TX. |
| R4-2210146 | Qualcomm Incorporated | Simulation results |

### CQI requirements (9.19.5.3.1)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207812 | Apple | Proposal #1: Choose SNR points [0,1] dB for CQI reporting in QPSK range and [12,13] dB for CQI reporting in 64QAM range for 2x1 in AWGN. Proposal #2: Choose SNR points [2,3] dB for CQI reporting in QPSK range and [14,15] dB for CQI reporting in 64QAM range for 2x2 in AWGN. Proposal #3: Choose SNR [4,5] dB for SNR in 16QAM range and SNR [12,13] dB for SNR in 64QAM range for 2x2 in fading channel. Proposal #4: Choose SNR points 3dB lower than 2RX for requirements in fading channel with 1RX. Proposal #5: Reuse the same test metrics and pass/fail criteria as existing WB CQI reporting tests in fading channel for RedCap UE. |
| R4-2209061 | Ericsson | Proposal 1: For the static condition CQI reporting test with CQI table 1, set two SNR test points corresponding to CQI indexes of 16QAM and 64QAM for RedCap 1Rx and 2Rx UEs. Proposal 2: For the static condition CQI reporting test with CQI table 1 for 2Rx UEs (rank 2), set SNR=8/9dB and SNR=14/15dB. Proposal 3: For the static condition CQI reporting test with CQI table 1 for 1Rx UEs (rank 1), set SNR=5/6dB and SNR=11/12dB. Proposal 4: Reuse the existing test metrics of CQI reporting test in fading condition for RedCap 1Rx/2Rx UEs, i.e., - A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time - The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ. - When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to 0.02. Proposal 5: For the wideband CQI reporting test with CQI table 1 in fading condition, set SNR=6/7dB and 12/13dB for RedCap 2Rx UE (both FDD 15kHz and TDD 30kHz). Proposal 6: Set α=5%, γ=1.05 for RedCap 2Rx UE (both FDD 15kHz and TDD 30kHz). Proposal 7: For the wideband CQI reporting test with CQI table 1 in fading condition, set SNR=9/10dB and 15/16dB for RedCap 1Rx UE (both FDD 15kHz and TDD 30kHz). Proposal 8: Set α=5%, γ=1.05 for RedCap 1Rx UE (both FDD 15kHz and TDD 30kHz). |
| R4-2209799 | MediaTek inc. | Proposal 1: For static channel CQI reporting test with CQI table 1, we suggest setting SNR=8/9dB and SNR=14/15dB for both 1Rx and 2Rx cases. Proposal 2: For wideband CQI reporting test in fading channel with CQI table 1, we suggest setting SNR=9/10dB and SNR=15/16dB for 1Rx case and SNR=6/7dB and SNR=12/13dB for 2Rx case. |
| R4-2209835 | Huawei, HiSilicon | For CQI test under AWGN channel: Proposal 1: Define the codebookSubsetRestriction for 2T1R to avoid the equivalent channel matrix is 0. Proposal 2: Set codebookSubsetRestriction to 000001 if H=[1 1] is used. Observation 1: The SINR for 2T2R with rank 1 is 3dB higher than that for 2T2R rank 2 under static channel condition. Proposal 3: Simulate the case for 2T2R with rank2 to find the tested SNR point and subtract 3dB for case with 2T1R with rank 1 for CQI test. Proposal 4: Define the SNR points with [8,9] dB and [14, 15]dB for case with 2T2R rank2 and define the SNR points with [5,6] dB and [11, 12]dB for case with 2T1R rank1. For CQI test under fading channel: Proposal 5: Define CQI requirements on [3 or 4, 9 or 10] dB for 2RX and [7 or 8, 13 or 14] dB for 1RX under fading channel. Proposal 6: Use following test metric for CQI test under fading channel: - BLER with following CQI>0.02 - Prob (Median CQI-1<CQI< Median CQI+1)>20% - γ>1.05 |

### PMI/RI requirements (9.19.5.3.2)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207813 | Apple | Observation #1: The same TP gain metric can be re-used for RedCap UE requirements for 2RX and 1RX for FDD and TDD. Proposal #1: Reuse the TP gain metric of 1.3 for RedCap requirements with 2RX and 1RX. Proposal #2: Do not define RI reporting requirements for RedCap UE. |
| R4-2209062 | Ericsson | Proposal 1: Reuse the existing test metrics of PMI reporting test for RedCap 1Rx/2Rx UEs (both FDD 15kHz and TDD 30kHz), i.e., - The minimum requirements are specified in terms of the ratio γ=tue/trnd, where tue is 90% of the maximum throughput observed at SNRUE using the precoders configured according to UE reports, and trnd is the throughput measures at SNRUE with random precoding. Proposal 2: Set γ=1.3 for RedCap 1Rx/2Rx UEs (both FDD 15kHz and TDD 30kHz). Proposal 3: Not define RI tests for RedCap. |
| R4-2209710 | Nokia, Nokia Shanghai Bell | Observation 1: MIMO operation is not one of the reduced capabilities of the RedCap UE. Proposal 1: Select Option 1: Define RI reporting tests for RedCap 2Rx UE. |
| R4-2209800 | MediaTek inc. | Proposal 1: Consider using the following MCS and throughput ratio γ to define requirements on PMI for RedCap UE. - FDD, 1Rx: MCS 13, γ = 1.3 - TDD, 1Rx: MCS 13, γ = 1.3 - TDD, 2Rx: MCS 15, γ = 1.3 |
| R4-2209836 | Huawei, HiSilicon | Observation 1: For case with FDD 4T1R, the SNR @90% of max TP with follow PMI is 8.4dB. Gama=2.14 Observation 2: For case with TDD 4T1R, the SNR @90% of max TP with follow PMI is 4.9dB. Gama=2.7 Observation 3: For case with TDD 4T2R, the SNR @90% of max TP with follow PMI is 3.07dB. Gama=1.9 Proposal 1: Not define RI requirements for RedCap UE. |

## Open issues summary

### Sub-topic 1-1 Specification structure

Issue 1-1-1: Spec structure of UE demodulation and CSI reporting requirements for RedCap

Background (R4-2209058): RF main session agreed to use suffix ‘I’ to define UE RF requirements for RedCap in TS38.101-1/2

- Proposal (Ericsson)

  - Define RedCap demodulation and CSI reporting requirements in TS38.101-4 with the suffix ‘I’, as same as TS38.101-1/2. The proposed spec structured in provided in R4-2209057.

- Recommended WF

  - Discuss the proposal is acceptable or not from demodulation requirements point of view.

  - If agreeable, 2nd round discusses the CR work split.

Agreements in GTW (2022-05-10)

- Agreement:

- Further discuss below two options considering specification drafting effort, spec structure and test applicable rule drafting:

  - Option 1: New Suffix “I”

  - Option 2: No new suffix with existing structure.

### Sub-topic 1-2 Common issues for UE demodulation and CSI reporting requirements

Issue 1-2-1: UL/DL scheduling for FR1 FDD for 1Rx UE

Background: According to RAN1 feature list and RAN#95-e decision, supporting half-duplex FDD is per-band capability, i.e., UE is not mandated to support full-duplex FDD if UE support half-duplex FDD.

- Proposals

  - Option 1 (Ericsson): Define FRC of UE demodulation and CSI reporting requirements for RedCap 1Rx UE in FDD by assuming the half-duplex FDD operation, that is, DDDSU.

    - Specify the applicability rule for RedCap UE supporting between half-duplex FDD and full-duplex FDD.

  - Option 1 (Ericsson): Define the single test case set of UE demodulation and CSI reporting requirements for RedCap 1Rx UE in FDD, which is applicable for both half-duplex FDD UE and full-duplex FDD UE. The applied FRC is based on half-duplex FDD operation, that is, DDDSU.

    - If RedCap UE support only HD-FDD in a FDD band, this UE is tested with HD-FDD mode.

    - If RedCap UE support only FD-FDD in a FDD band, this UE is tested with FD-FDD mode.

    - If RedCap UE support both FD-FDD and HD-FDD in a FDD band, this UE is tested with FD-FDD mode.

  - Option 2 (Nokia, Huawei): Consider Full-duplex FDD only for FDD tests for 1 Rx UE.

- Recommended WF

  - Collect inputs.

Agreements in GTW (2022-05-10)

- Agreement:

Introduce demodulation/CSI requirements covering both FD-FDD and HD-FDD.

- DDDSU applied for HD-FDD

- Existing pattern applied for FD-FDD

- Same demodulation requirement applied for FD-FDD and HD-FDD with different FRCs

- For CSI requirements: Further discuss the CSI feedback scheduling pattern applicable for both FD-FDD and HD-FDD

Test applicable rule:

- If RedCap UE support only HD-FDD in a FDD band, this UE is tested with HD-FDD mode otherwise UE is tested with FD-FDD mode

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1: Spec structure of UE demodulation and CSI reporting requirements for RedCap Issue 1-2-1: UL/DL scheduling for FR1 FDD for 1Rx UE Others: |
| Ericsson | Issue 1-1-1: Spec structure of UE demodulation and CSI reporting requirements for RedCap We support the proposal since it is aligned with UE RF requirements TS38.101-1/2. More clarification on our proposal - The new requirements (e.g, FR1 1Rx tests) is specified by creating the clauses with suffix ‘I’. - Create the applicability table for RedCap UE, where lists the applicable test cases both new requirements and reuse requirements. - No new clauses are added for the ‘reused’ test cases (e.g., FR2 tests). Example of applicability table for FR1 1Rx UE feature; Test type; Test list. RedCap UE (supportOfRedCap-r17); FR1 FDD; PDSCH; All tests in Clause 5.2I.1.1 (for new tests). FR1 TDD; PDSCH; All tests in Clause 5.2I.1.2 (for new tests). FR1 FDD; PDCCH; …. Example of applicability table for FR1 2Rx UE feature; Test type; Test list. RedCap UE (supportOfRedCap-r17); FR1 FDD; PDSCH; All tests in Clause 5.2I.2.1 (for new tests) Clauses 5.2.2.1.1 (Tests 1-3 and 1-4) (for reused tests). FR1 TDD; PDSCH; All tests in Clause 5.2I.2.2 (for new tests). FR1 FDD; PDCCH; …. Example of applicability table for FR2: UE feature; Test type; Test list. RedCap UE (supportOfRedCap-r17); FR2 TDD; PDSCH; Clause 7.2.2.2.1 (Tests 1-1, 1-2, 2-6) (for reused tests). FR2 TDD; PDCCH; …. FR2 TDD; PBCH; Clause 7.4.2.2 (Table 7.4.2.2-2 Tests 1 and 2) (for reused tests). Issue 1-2-1: UL/DL scheduling for FR1 FDD for 1Rx UE We support the proposal. As we discussed in our paper, there could be a UE which supports HD-FDD only. Therefore we think it is important for RAN4 to ensure the HD-FDD UE demodulation performance as same as RF/RRM requirements. We have clarified our proposal above. We think this proposal reuses the method for LTE Cat-M1 tests and it does not increase the total number of test cases for RedCap UEs. Others: |
| Huawei | Issue 1-1-1: Spec structure of UE demodulation and CSI reporting requirements for RedCap The proposal is generally OK for us. Especially the test applicability tables clearly list all applicable test cases for RedCap UE that is same as other UE capabilities and no need to define those reused test cases. Issue 1-2-1: UL/DL scheduling for FR1 FDD for 1Rx UE We support Option 2. There is already reduced throughput from the supported CBW, number of antenna and modulation order compared to normal eMBB UE for RedCap UE, to keep the competitiveness compared with LTE Cat.4 from reduced complexity but still relative high throughput to meet real application request, we don’t think there are RedCap UE to support HD-FDD with further reduced throughput and complex NW scheduling in the real network. We doubt whether there is RedCap UE that support both HD-FDD and FD-FDD, we didn’t figure out the motivation that one RedCap UE supports HD-FDD for certain band but it already has the capability to support FD-FDD. RAN1/2 and RF/RRM core part maybe just define all related specification from specification completeness point of view, but demodulation requirements are used to test real product in the market, it is no much meaningful to define requirements that are not useful. For Option 1, it is strange that UE supporting FD-FDD is configured with TDD pattern “DDDSU” that is not aligned with real scheduling in NW. |
| Nokia | Issue 1-1-1: Spec structure of UE demodulation and CSI reporting requirements for RedCap The proposal is fine. The given applicability tables for FR1 and FR2 provide a good overview on the 38.101-4 changes. Issue 1-2-1: UL/DL scheduling for FR1 FDD for 1Rx UE Based on RAN#95e decision, that HD-FDD is declared per band, we update our position, that both FD-FDD and HD-FDD tests are needed in FR1. The selected RMC in option 1 covering the slot pattern “DDDSU” is applicable for HD-FDD. For FD-FDD, RAN4 needs to take decision whether the same RMC with the same slot pattern applies or whether another RMC with higher Tput is required. The latter means different demodulation requirements (SNR points) for FD-FDD and HD-FDD RedCap UE, whilst CSI, PMI and RI reporting requirements can still be common. Thus, for different demodulation requirements, simulation effort is higher and specification complexity is increased. |
| Apple | Issue 1-1-1: Spec structure of UE demodulation and CSI reporting requirements for RedCap We don’t see the necessity to introduce a new subclause of requirements x.xI for demod requirements with RedCap. In RF spec they have several suffixes for different requirements for NR-U, V2X etc, but we don’t necessarily follow the same for demod requirements. We have a sub clause for 1RX requirements and 2RX requirements, where the requirements for RedCap UE with 1RX and 2RX can be specified. The applicability can be captures in 5.1.1.3 to list the requirements applicable to UE that supports RedCap. Also, it is very confusing to the spec reader if we jump to suffix I after suffix C for demod requirements. Issue 1-2-1: UL/DL scheduling for FR1 FDD for 1Rx UE Our understanding is that RedCap UE supports either HD-FDD or FD operation in a FDD band. Also, our understanding is that the demod requirements don’t change for HD_FDD and FD operation, but only the FRC would be different. If we define requirements for both HD-FDD and FD, the UE would need to pass requirements based on the operating mode it supports in the band. We don’t see how the 3rd bullet in the proposal is possible. |
| Qualcomm | Issue 1-1-1: Spec structure of UE demodulation and CSI reporting requirements for RedCap Can the proponent of this proposal clarify as to why we need a separate clause for redcap? Is it possible to instead use the existing 1Rx/2Rx clauses? Issue 1-2-1: UL/DL scheduling for FR1 FDD for 1Rx UE In general, we are okay to define demod requirements for HD FDD. However, our understanding is that HD or FD modes of operation depends on FRC, so we are not sure how we can guarantee that it could only be tested for FD for a given band, e.g., where only HD FDD is supported. (Bullet#3 for Option 1). Can the proponent of this proposal clarify? |
| Ericsson2 | Issue 1-2-1: UL/DL scheduling for FR1 FDD for 1Rx UE Regarding the CSI feedback scheduling pattern applicable for both FD-FDD and HD-FDD, we will propose the following CSI report and CSI-RS scheduling: - CQI reporting test in static/fading condition (periodic CSI reporting) - CSI-RS periodicity and offset: 5/1 - CSI-Report periodicity and offset: 5/4 - Comment: slot offset is changed from 0 to 4 because of the pattern of DDDSU) - CQI/RI/PMI delay: 8 ms - PMI reporting tests (aperiodic CSI reporting) - CSI request: 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0. - Comment: To keep the aperiodic report slot offset to 4, and UL slot is every 5 slots, CSI request should be changed from mod(i, 5)=1 to mod(i, 5)=0). - Aperiodic Report Slot Offset: 4 slots - CQI/RI/PMI delay: 6 ms It important to keep CQI/RI/PMI delay as the existing requirements. We propose to discuss this configuration in the 2nd round. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2209056 (Ericsson) | Title: Update of work plan for RedCap demodulation performance part |
|  | Huawei: the updated work plan is fine for us. |
|  | Nokia: Work plan looks fine. Apple: Work plan is fine with us, but we don’t agree on the spec structure. |
|  | MediaTek: We are OK with the updated work plan. Qualcomm: We are okay with the updated work plan. |
| R4-2209057 (Ericsson) | Title: draft big CR: Introduction of UE demodulation and CSI reporting requirements for RedCap |
|  | Huawei: The specification structure is under discussion in Issue 1-1-1, after it is finalized, the specific section numbering for RedCap can be reflected in the CR work splitting, all draft CRs submission can follow the work plan from next August meeting. |
|  | Nokia: We agree with Huawei. Further discussion on issues 1-1-1 and 1-2-1 needs to be carried out first before endorsing the spec structure. Apple: TBD based on Issue 1-1-1 discussion. We assume this is only a placeholder to show the spec structure rather than to be endorsed. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1-1: Spec structure of UE demodulation and CSI reporting requirements for RedCap | Agreements in GTW (2022-05-10) - Agreement: - Further discuss below two options considering specification drafting effort, spec structure and test applicable rule drafting: - Option 1: New Suffix “I” - Option 2: No new suffix with existing structure. Candidate options: - Option 1: New Suffix “I” - https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_103-e/Inbox/Drafts/%5B103-e%5D%5B329%5D%20NR_RedCap_Demod/Round%201/REV_R4-2209057%20RedCap%20spec%20structure%20Option%201.docx - Option 2: No new suffix with existing structure - https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_103-e/Inbox/Drafts/%5B103-e%5D%5B329%5D%20NR_RedCap_Demod/Round%201/REV_R4-2209057%20RedCap%20spec%20structure%20Option%202.docx Recommendation for 2nd round: According to the 1st round discussion, both the options work. Several companies prefer Option 2. The moderator proposes that Option 2 is adopted if no companies have strong preference for Option 1. The moderator would like to check if Option 2 is acceptable. |
| Issue 1-2-1: UL/DL scheduling for FR1 FDD for 1Rx UE | Agreements in GTW (2022-05-10) - Agreement: Introduce demodulation/CSI requirements covering both FD-FDD and HD-FDD. - DDDSU applied for HD-FDD - Existing pattern applied for FD-FDD - Same demodulation requirement applied for FD-FDD and HD-FDD with different FRCs - For CSI requirements: Further discuss the CSI feedback scheduling pattern applicable for both FD-FDD and HD-FDD Test applicable rule: - If RedCap UE support only HD-FDD in a FDD band, this UE is tested with HD-FDD mode otherwise UE is tested with FD-FDD mode Recommendation for 2nd round: Regarding the CSI feedback scheduling pattern applicable for both FD-FDD and HD-FDD, the moderator would like to check the proposal from Ericsson is accepted: - CQI reporting test in static/fading condition (periodic CSI reporting) - CSI-RS periodicity and offset: 5/1 - CSI-Report periodicity and offset: 5/4 - Comment: slot offset is changed from 0 to 4 because of the pattern of DDDSU) - CQI/RI/PMI delay: 8 ms - PMI reporting tests (aperiodic CSI reporting) - CSI request: 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0. - Comment: To keep the aperiodic report slot offset to 4, and UL slot is every 5 slots, CSI request should be changed from mod(i, 5)=1 to mod(i, 5)=0). - Aperiodic Report Slot Offset: 4 slots - CQI/RI/PMI delay: 6 ms |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2209056 (Ericsson) | To be revised. Moderator recommends to capture the CR work split in the revision. |
| R4-2209057 (Ericsson) | Postponed |

## Discussion on 2nd round (if applicable)

# Topic #2: UE demodulation requirements

## Companies’ contributions summary

See 1.1

## Open issues summary

### Sub-topic 2-1 PDSCH demodulation requirements

Issue 2-1-1: Define 256QAM demodulation requirements (for FR1 only) or not

Background: 256QAM is optional feature for RedCap UE (both 1Rx and 2Rx)

- Proposals

  - Option 1 (MediaTek?, Huawei): Specify 256QAM demodulation requirements for FR1 only

  - Option 2 (Apple, Nokia): Not to Specify 256QAM demodulation requirements

- Recommended WF

  - Collect inputs

Agreements in GTW (2022-05-10)

- Agreement:

Introduce 256QAM requirement for 2Rx

FFS whether introduce requirement for 1Rx

Issue 2-1-2: MCS for 256QAM demodulation requirements for 1Rx UE (if agreed)

- Proposals

  - Option 1 (Ericsson): MCS22

  - Option 2 (MediaTek): MCS20

  - Option 3: MCS24 (reuse the same MCS as the existing requirements)

- Recommended WF

  - Discuss in the 2nd round depending on the conclusion on Issue 2-1-1

Issue 2-1-3: Additional PDSCH demodulation requirements

Background (WF: R4-2207206 agreed in RAN4#102-e):

Option 1: Focus on definition of minimum set of requirements, discussed in Topic #2, to verify the mandatory features. RAN4 discuss other requirements once it is stable, and the performance part TU is allowed.

Option 2: Not define the additional PDSCH demodulation requirements other than the candidates discussed in 2.1, in Rel-17 RedCap

- Proposals

  - Option 1 (Nokia): Focus on definition of minimum set of requirements, discussed in Topic #2, to verify the mandatory features. RAN4 to potentially discuss other requirements once mandatory requirements are stable and pending remaining performance part TUs.

  - Option 2 (Huawei): Not define any additional PDSCH demodulation requirements other than those agreed in last RAN4 meeting (moderator: RAN4#102-e) in Rel-17 for RedCap

- Recommended WF

  - Collect inputs considering the updated work plain in R4-2209056.

Agreements in GTW (2022-05-10)

- Agreement: Option 2 agreed.

### Sub-topic 2-2 SDR requirements

Issue 2-2-1: Test setup for SDR requirements

- Proposals (Huawei)

  - Reuse the existing test procedure and test setup with removal unfeasible test parameters from Rel-15 SDR test.

  - For FR1, define SDR requirements with configuration 1T1R for UE supporting 1 layer and 2T2R for UE supporting 2 layers. For FR2, define SDR requirements for 2 layers only with configuration 2T2R.

- Recommended WF

  - Agree with the proposals.

### Sub-topic 2-3 PDCCH demodulation requirements

Issue 2-3-1: Aggregation level(s) for PDCCH demodulation requirements

Background (WF: R4-2207206 agreed in RAN4#102-e):

Option 1: Specify PDCCH demodulation requirements with AL4 and AL8.

Option 2: Specify PDCCH demodulation requirements with AL4 or AL8.

- Proposals

  - Option 1 (Apple): AL4

    - Option 1a (Ericsson): AL4 with the assumption PDCCH AL8 performance can be guaranteed during the PDSCH demodulation tests.

  - Option 2 (Ericsson, Huawei): AL4 and AL8

  - Option 3 (Nokia): AL4/8/16 for FR1, AL4/8 for FR2

- Recommended WF

  - Moderator suggests not to discuss AL16 option again according to the agreements in RAN4#102-e.

  - Collect inputs.

Issue 2-3-2: Test setup for PDCCH FR1 TDD 30kHz (symbol duration)

Observation: It is possible to configure CORESET 1 symbol duration for TDD in 20MHz channel bandwidth.

- Proposal (MediaTek, Ericsson)

  - Set CORESET duration to 1 for FR1 TDD PDCCH demodulation requirements.

- Recommended WF

  - Agree with the proposal

Issue 2-3-3: Test setup for PDCCH FR1 TDD 30kHz (REG bundle size)

Observation: REG bundle size 6 cannot be configured for with interleaver size 3 if RB size is set to 48RB

- Proposal (Ericsson)

  - Set REG bundle size to 2 for FR1 TDD PDCCH demodulation requirements.

- Recommended WF

  - Agree with the proposal

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1-1: Define 256QAM demodulation requirements (for FR1 only) or not Issue 2-1-2: MCS for 256QAM demodulation requirements for 1Rx UE (if agreed) Issue 2-1-3: Additional PDSCH demodulation requirements Issue 2-2-1: Test setup for SDR requirements Issue 2-3-1: Aggregation level(s) for PDCCH demodulation requirements Issue 2-3-2: Test setup for PDCCH FR1 TDD 30kHz (symbol duration) Issue 2-3-3: Test setup for PDCCH FR1 TDD 30kHz (REG bundle size) Others: |
| Ericsson | Issue 2-1-3: Additional PDSCH demodulation requirements We slight prefer Option 2, considering the total RAN4 workload. Issue 2-2-1: Test setup for SDR requirements Support the recommended WF. Issue 2-3-1: Aggregation level(s) for PDCCH demodulation requirements We prefer Option 2. But we are also ok with Option 1 (option 1a), as AL8 performance could be implicitly ensured through PDSCH demodulation tests. Issue 2-3-2: Test setup for PDCCH FR1 TDD 30kHz (symbol duration) Support the recommended WF. Issue 2-3-3: Test setup for PDCCH FR1 TDD 30kHz (REG bundle size) Support the recommended WF. |
| Huawei | Issue 2-1-1: Define 256QAM demodulation requirements (for FR1 only) or not We support option 1. 256QAM is an important feature which could increase throughput and spectrum efficiency especially for high SNR. Hence we think it is necessary to define the requirements Issue 2-1-2: MCS for 256QAM demodulation requirements for 1Rx UE (if agreed) We proposed to reuse MCS 24 from Rel-15. But considering companies’ concern that the SNR is too high for 1RX, we can compromise to option 2. MCS20. Issue 2-1-3: Additional PDSCH demodulation requirements We support option 2. Considering this is first release of RedCap and workload for RAN4, we propose to focus on basic requirements. Issue 2-2-1: Test setup for SDR requirements Support the recommended WF. Issue 2-3-1: Aggregation level(s) for PDCCH demodulation requirements Both Option 1 and Option 2 are fine for us. We don’t have strong views if the total number of test cases is not increased. Issue 2-3-2: Test setup for PDCCH FR1 TDD 30kHz (symbol duration) Support the recommended WF. Issue 2-3-3: Test setup for PDCCH FR1 TDD 30kHz (REG bundle size) Support the recommended WF |
| Nokia | Issue 2-1-1: Define 256QAM demodulation requirements (for FR1 only) or not We support option 2. Issue 2-1-2: MCS for 256QAM demodulation requirements for 1Rx UE (if agreed) We support the recommended WF. No strong view on the preferred MCS (if agreed). Issue 2-1-3: Additional PDSCH demodulation requirements We can compromise to option 2 (Huawei). Issue 2-2-1: Test setup for SDR requirements We support the recommended WF. One aspect to be clarified is the support of SDR test for HD-FDD UE in FR1, depending on issue 1-2-1, i.e. whether there is a common requirement for FD-FDD and HD-FDD, or separate SDR requirements. Issue 2-3-1: Aggregation level(s) for PDCCH demodulation requirements We support option 3. As mentioned in R4-2209708, we see AL16 as important to be included, as it better covers cell-edge scenario. Also, RLM performance is specified based on AL16. Otherwise, option 2 is preferred over option 1. Issue 2-3-2: Test setup for PDCCH FR1 TDD 30kHz (symbol duration) We support the recommended WF. Issue 2-3-3: Test setup for PDCCH FR1 TDD 30kHz (REG bundle size) We support the recommended WF. |
| Apple | Issue 2-1-1: Define 256QAM demodulation requirements (for FR1 only) or not We support option 2 to only define requirements with mandatory features at this point. We could possibly re-use the existing requirement for 2RX RedCap UE with 256QAM if needed . With 1RX the operating SNR would be much higher for 256QAM, we don’t see a suitable use case for 1RX with 256QAM Issue 2-1-2: MCS for 256QAM demodulation requirements for 1Rx UE (if agreed) TBD based on consensus on Issue 2-1-1 and simulation results and feasible operating SNR. Issue 2-1-3: Additional PDSCH demodulation requirements We support option 2. Also in our understanding 256QAM is also optional feature and should be treated the same as other optional features. Issue 2-2-1: Test setup for SDR requirements We are fine with the proposal. Issue 2-3-1: Aggregation level(s) for PDCCH demodulation requirements Based on simulation results we propose AL8 for FDD and AL4 for TDD in FR1 for both 2RX and 1RX. For FR2, either AL4 or AL8 is fine with us. RLM requirements are specified based on separate evaluation and don’t depend on demod requirements defined in our understanding. Issue 2-3-2: Test setup for PDCCH FR1 TDD 30kHz (symbol duration) We are fine with the proposal. Issue 2-3-3: Test setup for PDCCH FR1 TDD 30kHz (REG bundle size) We are fine with the proposal. Others: |
| MediaTek | Issue 2-1-1: Define 256QAM demodulation requirements (for FR1 only) or not Although 256QAM is an optional feature, we think it is important to verify higher throughput for RedCap UE. Therefore, we support Option 1 to specify 256QAM demodulation requirements. Issue 2-1-2: MCS for 256QAM demodulation requirements for 1Rx UE (if agreed) We prefer to use MCS20 and we are OK to determine MCS in the second round. Issue 2-3-1: Aggregation level(s) for PDCCH demodulation requirements According to the results from companies, the SNR requirements for FDD, AL4 is from 12.28 to 14.6 dB, which is very high. We are not sure whether it is suitable to define PDCCH requirements with such configuration. Therefore, we propose to define only AL8 for FDD. For TDD, we are OK to define requirements with AL4. Issue 2-3-2: Test setup for PDCCH FR1 TDD 30kHz (symbol duration) Support the recommended WF. Issue 2-3-3: Test setup for PDCCH FR1 TDD 30kHz (REG bundle size) Support the recommended WF. |
| Qualcomm | Issue 2-1-1: Define 256QAM demodulation requirements (for FR1 only) or not We support Option 1. This will allow us to achieve higher throughput for redcap UE with 1 Rx. Issue 2-1-2: MCS for 256QAM demodulation requirements for 1Rx UE (if agreed) Option 2. MCS20 seems to be a reasonable choice for 256QAM. Issue 2-2-1: Test setup for SDR requirements We are fine with the recommended WF. Issue 2-3-1: Aggregation level(s) for PDCCH demodulation requirements We support Option 2, i.e., specify PDCCH demodulation requirements with both AL4 and AL8. We are okay to exclude any test with very high SNR requirement since defining PDCCH requirement at such a high SNR would run the risk of missing PDSCH as this SNR falls within the PDSCH operating regime. Issue 2-3-2: Test setup for PDCCH FR1 TDD 30kHz (symbol duration) We support the recommended WF. Issue 2-3-3: Test setup for PDCCH FR1 TDD 30kHz (REG bundle size) We support the recommended WF. |

### CRs/TPs comments collection

Not applicable

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1-1/2-1-2: 256QAM demodulation requirements (for FR1 only) | Agreements in GTW (2022-05-10) - Agreement: Introduce 256QAM requirement for 2Rx FFS whether introduce requirement for 1Rx Candidate options: - Option 1: Define 256QAM demodulation requirements for 1Rx in FR1 - Option 1a: MCS20 - Option 1b: Same MCS as 2Rx test case, that is, MCS24 - Option 2: Not to define 256QAM demodulation requirements for 1Rx in FR1 Recommendation for 2nd round: Continue the discussion |
| Issue 2-1-3: Additional PDSCH demodulation requirements | Candidate options: - Option 1: Focus on definition of minimum set of requirements, discussed in Topic #2, to verify the mandatory features. RAN4 to potentially discuss other requirements once mandatory requirements are stable and pending remaining performance part TUs. - Option 2: Not define any additional PDSCH demodulation requirements other than those agreed in last RAN4 meeting (moderator: RAN4#102-e) in Rel-17 for RedCap Agreements in GTW (2022-05-10) - Agreement: Option 2 agreed. Recommendation for 2nd round: No discussion |
| Issue 2-2-1: Test setup for SDR requirements | Agreements - Reuse the existing test procedure and test setup with removal unfeasible test parameters from Rel-15 SDR test. - For FR1, define SDR requirements with configuration 1T1R for UE supporting 1 layer and 2T2R for UE supporting 2 layers. For FR2, define SDR requirements for 2 layers only with configuration 2T2R. Recommendation for 2nd round No discussion |
| Issue 2-3-1: Aggregation level(s) for PDCCH demodulation requirements | Candidate options: - Option 1 (Ericsson, Huawei): AL4 for all the scenarios - Ericsson: AL8 is used for PDSCH test - Option 2 (Ericsson, Huawei, Nokia, Qualcomm): AL4 and AL8 - Qualcomm: OK to exclude any test with very high SNR requirement - Option 3 (Nokia): AL4/8/16 for FR1, AL4/8 for FR2 - Option 4 (Apple, MediaTek): AL8 for FR1 FDD, AL4 for FR1 TDD, AL4 for FR2 TDD - MediaTek: Required SNR for AL4 is verify high for FDD case. Tentative agreements: Some companies have concern for the required SNR level for FR1 FDD AL4 test case. Duplex CBW/SCS; Antenna configuration; Aggregation level; Channel model; Average. FDD 10MHz/15kHz; 1x1; AL4; TDLA30-10; 13.3. 2x1 Low; AL8; TDLA30-10; 3.1. TDD 20MHz/30kHz; 1x1; AL4; TDLC300-100; 6.1. 2x1 Low; AL8; TDLC300-100; 0.8. 1x2 Low; AL4; TDLC300-100; 1.0. 2x2 Low; AL8; TDLC300-100; -2.2. Reviewing the latest simulation summary, the moderator proposes the following tentative way forward as a compromise. - 1Rx: - Define AL8 test for FR1 FDD - Define AL4 test for FR1 TDD - 2Rx: - Define AL4 and AL8 tests for all the scenarios Recommendation for 2nd round: Check if the tentative agreement proposed by the moderator is accepted. |
| Issue 2-3-2/2-3-3: Test setup for PDCCH FR1 TDD 30kHz | Agreements: - Set CORESET duration to 1 for FR1 TDD PDCCH demodulation requirements - Set REG bundle size to 2 for FR1 TDD PDCCH demodulation requirements Recommendation for 2nd round: No discussion |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #3: CSI reporting requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

See 1.1

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1 CQI reporting requirements in static condition

Issue 3-1-1: Test points for CQI reporting test in static condition for 2Rx (FR1 FDD and TDD)

- Proposals

  - Option 1 (Apple): 2/3dB and 14/15dB

  - Option 2 (Ericsson, MediaTek, Huawei): 8/9dB and 14/15dB

- Recommended WF

  - Set at least 14/15dB for the upper SNR test point.

  - Collect inputs for the lower SNR test point.

Issue 3-1-2: Test points for CQI reporting test in static condition for 1Rx (FR1 FDD and TDD)

- Proposals

  - Option 1 (Apple): 0/1dB and 12/13dB

  - Option 2 (Ericsson, Huawei): 5/6dB and 11/12dB

  - Option 3 (MediaTek): 8/9dB and 14/15dB

- Recommended WF

  - Collect inputs

Issue 3-1-3: Test setup for CQI reporting test in static condition

- Proposals (Huawei)

  - Define the codebookSubsetRestriction for 2T1R to avoid the equivalent channel matrix is 0.

  - Set codebookSubsetRestriction to 000001 if H = [1 1] is used.

- Recommended WF

  - Agree with the proposals

### Sub-topic 3-2 CQI reporting requirements in fading condition

Issue 3-2-1: Test points for CQI reporting test in fading condition for 2Rx (FR1 FDD and TDD)

- Proposals

  - Option 1 (Apple): 4/5dB and 12/13dB

  - Option 2 (Ericsson, MediaTek): 6/7dB and 12/13dB

  - Option 3 (Huawei): 3/4dB and 9/10dB

- Recommended WF

  - Collect inputs to narrow down the SNR test point candidates.

Issue 3-2-2: Test points for CQI reporting test in fading condition for 1Rx (FR1 FDD and TDD)

- Proposals

  - Option 1 (Apple): 7/8dB and 15/16dB (3dB higher than 2RX requirements)

  - Option 2 (Ericsson, MediaTek): 9/10dB and 15/16dB

  - Option 3 (Huawei): 7/8dB and 13/14dB

- Recommended WF

  - Collect inputs to narrow down the SNR test point candidates.

Issue 3-2-3: Test metric and pass/fail criteria for CQI reporting test in fading condition for 1Rx/2Rx UE (both FDD and TDD)

- Proposals (Apple, Ericsson, Huawei)

  - Reuse the same test metrics and pass/fail criteria as existing WB CQI reporting tests in fading channel for RedCap UE

    - A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time

    - The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ.

    - When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

    - Set α=20%, γ=1.05

- Recommended WF

  - Agree with the proposals

### Sub-topic 3-3 PMI reporting requirements

Issue 3-3-1: Test metric and pass/fail criteria for PMI reporting test for 1Rx/2Rx UE (both FDD and TDD)

- Proposals (Apple, Ericsson, MediaTek):

  - Reuse the same test metrics and pass/fail criteria as existing PMI reporting tests for RedCap UE

    - The minimum requirements are specified in terms of the ratio γ=tue/trnd, where tue is 90% of the maximum throughput observed at SNRUE using the precoders configured according to UE reports, and trnd is the throughput measures at SNRUE with random precoding.

    - Set γ=1.3

- Recommended WF

  - Agree with the proposals

### Sub-topic 3-4 RI reporting requirements for 2Rx UE

Issue 3-4-1: Whether to define RI reporting requirements for RedCap 2Rx UEs

- Proposals

  - Option 1 (Nokia): Define RI reporting requirements

  - Option 2 (Apple, Ericsson, Huawei): Not define RI reporting requirements

- Recommended WF

  - Moderator would like to ask whether option 2.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 3-1-1: Test points for CQI reporting test in static condition for 2Rx (FR1 FDD and TDD) Issue 3-1-2: Test points for CQI reporting test in static condition for 1Rx (FR1 FDD and TDD) Issue 3-1-3: Test setup for CQI reporting test in static condition Issue 3-2-1: Test points for CQI reporting test in fading condition for 2Rx (FR1 FDD and TDD) Issue 3-2-2: Test points for CQI reporting test in fading condition for 1Rx (FR1 FDD and TDD) Issue 3-2-3: Test metric and pass/fail criteria for CQI reporting test in fading condition for 1Rx/2Rx UE (both FDD and TDD) Issue 3-3-1: Test metric and pass/fail criteria for PMI reporting test for 1Rx/2Rx UE (both FDD and TDD) Issue 3-4-1: Whether to define RI reporting requirements for RedCap 2Rx UEs Others: |
| Ericsson | Issue 3-1-1: Test points for CQI reporting test in static condition for 2Rx (FR1 FDD and TDD) Option 2. We have a bit concern to set SNR=2/3dB because the variance of reported CQI index becomes large due to the low SNR. OK to set SNR=14/15dB for the upper SNR test points. Issue 3-1-2: Test points for CQI reporting test in static condition for 1Rx (FR1 FDD and TDD) Option 2, which is 3dB lower than 2Rx case because it is 1Rx test reports CQI index with rank 1 although 2Rx test reports CQI index with rank 2. To make the discussion simple, alternatively, we propose to set SNR test points X dB lower than 2Rx case, where X = [3]. Issue 3-1-3: Test setup for CQI reporting test in static condition Support the recommended WF. Issue 3-2-1: Test points for CQI reporting test in fading condition for 2Rx (FR1 FDD and TDD) Based on the companies proposal, we propose to set: - Lower SNR test points: 4/5dB (middle of options 1, 2, and 3) - Higher SNR test points: 12/13dB (proposed in option 1 and 2) Issue 3-2-2: Test points for CQI reporting test in fading condition for 1Rx (FR1 FDD and TDD) To make the discussion simple, we propose to set SNR test points 3 dB higher than 2Rx case. Issue 3-2-3: Test metric and pass/fail criteria for CQI reporting test in fading condition for 1Rx/2Rx UE (both FDD and TDD) Support the recommended WF. Issue 3-3-1: Test metric and pass/fail criteria for PMI reporting test for 1Rx/2Rx UE (both FDD and TDD) Support the recommended WF. Issue 3-4-1: Whether to define RI reporting requirements for RedCap 2Rx UEs We are ok with option 2. We also understand the motivation of option 1 that it is important to verify RedCap 2Rx UE reports RI=2. CSI reporting functionality requires to reports the recommended MIMO rank, but gNB can still schedule PDSCH with rank 2 even if RedCap UE reports RI=1 if it is capable of maxNumberMIMO-LayersPDSCH > 1. And RAN4 has already agreed to define PDSCH demodulation requirements with rank 2 (64QAM 0.5 Rank 2 and SDR tests). We think it is possible to ensure the RedCap UE receiver performance with rank 2 even if we don’t define RI reporting tests. |
| Huawei | Issue 3-1-1: Test points for CQI reporting test in static condition for 2Rx (FR1 FDD and TDD) Prefer Option 2. From our simulation results, SNR proposed in both options make sense, but SNR=2/3dB is related to QPSK, we prefer to choose 8/9 dB related to 16QAM, which is aligned with Rel-15 test. Issue 3-1-2: Test points for CQI reporting test in static condition for 1Rx (FR1 FDD and TDD) As per our analysis in our contribution, with linear MIMO detection processing, the post SNR for 2T1R with rank 1 is 3dB lower than 2T2T with rank 2. Hence, we support to set SNR for 1RX UE is 3dB lower than that for 2RX. Issue 3-1-3: Test setup for CQI reporting test in static condition Support the recommended WF. Issue 3-2-1: Test points for CQI reporting test in fading condition for 2Rx (FR1 FDD and TDD) OK with Ericsson’s suggestion: - Lower SNR test points: 4/5dB (middle of options 1, 2, and 3) - Higher SNR test points: 12/13dB (proposed in option 1 and 2 Issue 3-2-2: Test points for CQI reporting test in fading condition for 1Rx (FR1 FDD and TDD) OK with Ericsson’s suggestion to make discussions simple: - Lower SNR test points: 7/8dB (middle of options 1, 2, and 3) - Higher SNR test points: 15/16dB (proposed in option 1 and 2) Issue 3-2-3: Test metric and pass/fail criteria for CQI reporting test in fading condition for 1Rx/2Rx UE (both FDD and TDD) Support the recommended WF. Issue 3-3-1: Test metric and pass/fail criteria for PMI reporting test for 1Rx/2Rx UE (both FDD and TDD) Support the recommended WF. Issue 3-4-1: Whether to define RI reporting requirements for RedCap 2Rx UEs Support option 2. It is not necessary to define RI test since it is only limited to be applicable for 2RX UE. We agree with Ericsson that performance with rank 2 has been verified in SDR test and CQI test and no need to be verified again in RI test to keep balance of number of test cases for 1Rx and 2Rx UE. |
| Nokia | Issue 3-1-1: Test points for CQI reporting test in static condition for 2Rx (FR1 FDD and TDD) We support option 2. Issue 3-1-2: Test points for CQI reporting test in static condition for 1Rx (FR1 FDD and TDD) We support option 2 and Ericsson’s proposal based on offset [3] dB vs. test points for 2 Rx. Issue 3-1-3: Test setup for CQI reporting test in static condition We support the recommended WF. Issue 3-2-1: Test points for CQI reporting test in fading condition for 2Rx (FR1 FDD and TDD) We support option 2. Issue 3-2-2: Test points for CQI reporting test in fading condition for 1Rx (FR1 FDD and TDD) We support option 2. Issue 3-2-3: Test metric and pass/fail criteria for CQI reporting test in fading condition for 1Rx/2Rx UE (both FDD and TDD) We support the recommended WF. Issue 3-3-1: Test metric and pass/fail criteria for PMI reporting test for 1Rx/2Rx UE (both FDD and TDD) We support the recommended WF. Issue 3-4-1: Whether to define RI reporting requirements for RedCap 2Rx UEs We support option 1. As pointed out in R4-2209710, RI is still part of Redcap UE mandatory capabilities, hence RI reporting needs to be tested. We potentially see a strong negative TPUT performance impact if no requirements for RI reporting are specified, or rather such functionality is not implemented to increase spectral efficiency. Thus, RI reporting requirements for 2 Rx need to be specified. |
| Apple | Issue 3-1-1: Test points for CQI reporting test in static condition for 2Rx (FR1 FDD and TDD) We are fine with option 2 based on majority view. Issue 3-1-2: Test points for CQI reporting test in static condition for 1Rx (FR1 FDD and TDD) Based on our results and using lower SNR point corresponding to 16QAM for 2RX, we propose 6/7dB for lower SNR and 11/12 dB for higher SNR. We don’t think we can lower SNR by 3dB for 1RX compared to 2RX. Issue 3-1-3: Test setup for CQI reporting test in static condition We are fine with the proposal. Issue 3-2-1: Test points for CQI reporting test in fading condition for 2Rx (FR1 FDD and TDD) Either option 1 or option 2 is fine with us. We are fine with Ericson’s proposal above. Issue 3-2-2: Test points for CQI reporting test in fading condition for 1Rx (FR1 FDD and TDD) Either option 1 or option 2 is fine with us. We are fine with Ericson’s proposal above. Issue 3-2-3: Test metric and pass/fail criteria for CQI reporting test in fading condition for 1Rx/2Rx UE (both FDD and TDD) We support the proposals and recommended WF. Issue 3-3-1: Test metric and pass/fail criteria for PMI reporting test for 1Rx/2Rx UE (both FDD and TDD) We support the proposals and recommended WF. Issue 3-4-1: Whether to define RI reporting requirements for RedCap 2Rx UEs We support option 2 to not introduce RI requirements for RedCap UE. As raised by Ericsson and Huawei, the performance with rank 2 would also be verified with other CQI and demod/ SDR requirements. Others: |
| Qualcomm | Issue 3-1-1: Test points for CQI reporting test in static condition for 2Rx (FR1 FDD and TDD) For initial result, we support Option 2. Issue 3-1-2: Test points for CQI reporting test in static condition for 1Rx (FR1 FDD and TDD) For initial result, we support Option 2. Issue 3-1-3: Test setup for CQI reporting test in static condition We support the recommended WF. Issue 3-2-1: Test points for CQI reporting test in fading condition for 2Rx (FR1 FDD and TDD) For initial result, we support Option 2. Issue 3-2-2: Test points for CQI reporting test in fading condition for 1Rx (FR1 FDD and TDD) For initial result, we support Option 2. Issue 3-2-3: Test metric and pass/fail criteria for CQI reporting test in fading condition for 1Rx/2Rx UE (both FDD and TDD) We support the recommended WF. Issue 3-3-1: Test metric and pass/fail criteria for PMI reporting test for 1Rx/2Rx UE (both FDD and TDD) We support the recommended WF. Issue 3-4-1: Whether to define RI reporting requirements for RedCap 2Rx UEs We support Option 2. Others: |

### CRs/TPs comments collection

Not applicable

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 3-1-1: Test points for CQI reporting test in static condition for 2Rx | Candidate options: - Lower SNR test points - Option 1 (Ericsson, Huawei, Nokia, Apple, Qualcomm): 8/9dB - Higher SNR test points - Option 1 (Ericsson, Huawei, Nokia, Apple, Qualcomm) 14/15dB Tentative agreements: Set SNR=8/9dB for the lower test point. Set SNR=14/15dB for the higher test point. Recommendation for 2nd round: Check the tentative agreement is accepted. |
| Issue 3-1-2: Test points for CQI reporting test in static condition for 1Rx | Candidate options: SNR test points X dB lower than 2Rx case - Lower SNR test points: - Option 1 (Ericsson, Huawei, Nokia, Qualcomm): X = 3dB - Option 2 (Apple): X = 2dB - Higher SNR test points - Option 1 (Ericsson, Huawei, Nokia, Apple, Qualcomm): X = 3dB Tentative agreements: SNR test points X dB lower than 2Rx test case - For Lower SNR test point - Option 1: X = 3dB - Option 2: X = 2dB - For Higher SNR test points - X = 3dB Recommendation for 2nd round: Discuss X = 2 or 3 for lower SNR test points. Encourage companies to check with the simulation results considering SNR=8/9dB for 2Rx case (rank 2) |
| Issue 3-1-3: Test setup for CQI reporting test in static condition | Agreements: - Define the codebookSubsetRestriction for 2T1R to avoid the equivalent channel matrix is 0. - Set codebookSubsetRestriction to 000001 if H = [1 1] is used. Recommendation for 2nd round: No discussion |
| Issue 3-2-1: Test points for CQI reporting test in fading condition for 2Rx | Candidate options: - Lower SNR test points - Option 1 (Ericsson, Huawei, Apple): 4/5dB - Option 2 (Nokia, Qualcomm): 6/7dB - Higher SNR test points - Option 1 (Ericsson, Huawei, Nokia, Apple, Qualcomm): 12/13dB Tentative agreements: Lower SNR test points - Option 1: SNR=4/5dB - Option 2: SNR=6/7dB Higher SNR test points: - SNR=12/13dB Recommendation for 2nd round: Continue the discussion for lower SNR test points. One compromise could be SNR=5/6dB, these SNR test points include both the options. |
| Issue 3-2-2: Test points for CQI reporting test in fading condition for 1Rx | Candidate options: - Option 1 (Ericsson, Huawei, Nokia, Apple, Qualcomm?): Set SNR test points 3 dB higher than 2Rx case. Tentative agreements: - Set SNR test points 3 dB higher than 2Rx case. Recommendation for 2nd round: Check if the tentative agreement is accepted. |
| Issue 3-2-3: Test metric and pass/fail criteria for CQI reporting test in fading condition for 1Rx/2Rx UE | Agreements: Reuse the same test metrics and pass/fail criteria as existing WB CQI reporting tests in fading channel for RedCap UE - A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time - The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ. - When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to 0.02. - Set α=20%, γ=1.05 Recommendation for 2nd round: No discussion |
| Issue 3-3-1: Test metric and pass/fail criteria for PMI reporting test for 1Rx/2Rx UE | Agreements: - Reuse the same test metrics and pass/fail criteria as existing PMI reporting tests for RedCap UE - The minimum requirements are specified in terms of the ratio γ=tue/trnd, where tue is 90% of the maximum throughput observed at SNRUE using the precoders configured according to UE reports, and trnd is the throughput measures at SNRUE with random precoding. - Set γ=1.3 Recommendation for 2nd round: No discussion |
| Issue 3-4-1: Whether to define RI reporting requirements for RedCap 2Rx UEs | Candidate options from GTW (2022-05-10): - Option 1 (Nokia, CMCC): Define RI reporting requirements - Need to ensure the performance for RI reporting. SDR and demodulation requirements cannot verify RI reporting performance because the rank is fixed during the tests. - RI reporting is not optional feature. - Option 2 (Apple, Ericsson, Huawei, MediaTek): Not define RI reporting requirements - CSI reporting from UE is the recommended value. Network can still schedule rank 2 even if UE report rank 1. RAN4 has already agreed to define demodulation and SDR requirements covering rank 2, which can guarantee rank 2 performance. Tentative agreements: - Option 1: Define RI reporting requirements - Option 2: Not define RI reporting requirements Moderator: If the RI test is introduced, we need the following changes. - CQI table from Table 2 to Table for FR1 FDD/TDD tests - Channel bandwidth from 40MHz to 20MHz for FR1 TDD test Recommendation for 2nd round: Continue the discussion considering the comments during GTW. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on RedCap UE demodulation and CQI reporting requirements | Ericsson | Capture the agreements and update the simulation assumption. |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2209056 |  | Update of work plan for RedCap demodulation performance part | Ericsson | Revised | Capture the draft CR work split plan |
| R4-2209057 |  | draft big CR: Introduction of UE demodulation and CSI reporting requirements for RedCap | Ericsson | Postponed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |
| Apple | Manasa Raghavan | manasa.raghavan@apple.com |
| MediaTek | Licheng Lin | licheng.lin@mediatek.com |
| Huawei, HiSilicon | Ke Li | like54@hisilicon.com |
| Nokia | Juergen Hofmann | juergen.hofmann@nokia.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)