3GPP TSG-RAN4 Meeting #103-e 																R4-2210493

E-meeting, 09th May – 20th May, 2022

Agenda item:			9.12.6.2 & 9.12.6.4 & 9.12.7

Source:	Moderator (Xiaomi)

Title:	Email discussion summary for [103-e][224] NR_NTN_solutions_RRM_2

Document for:	Information

Introduction

The scope of this email discussion contains the maintenance for GNSS-related requirements (9.12.6.2) and UE timing requirements (9.12.6.4) and NTN RRM performance requirements (9.12.7). All the submitted TDocs in this agenda were reviewed and the relevant observations and proposals are included in this email discussion. The following topics will be discussed according to the submitted TDocs.

The timeline for 1st and 2nd round email discussions can be referred in TDoc of “RAN4#103-e E-meeting Arrangements and Guidelines”

In providing comments, companies are encouraged to:

- Be concise

- Provide comments on all topics/sub-topics of interest to them

- Ensure that their comments are inserted in the latest version of the document by checking the folder before uploading

- Use “Track changes” to help identify added comments/changes

Topic #1: Maintenance for GNSS-related requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2209639 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 to set a minimum requirement for UEs using GNSS for timing and frequency pre-compensation in NR operation. - Option 1: UE is required to be able to update GNSS while in connected mode while performing transmission/reception towards the RAN. - Option 2: UE is not required to be able to update GNSS while in connected mode while performing transmission/reception towards the RAN. UE may inform the validity timer of the GNSS fix to the RAN. - Option 3: Other. |

Open issues summary and Companies views’ collection for 1st round

GNSS fix in RRC_Connected mode

Issue 1-1: Clarification on GNSS fix in RRC_Connected mode for timing and frequency pre-compensation in NR operation.

- Option 1: (Nokia)

  - UE is required to be able to update GNSS while in connected mode while performing transmission/reception towards the RAN.

- Option 2: (Nokia)

  - UE is not required to be able to update GNSS while in connected mode while performing transmission/reception towards the RAN. UE may inform the validity timer of the GNSS fix to the RAN.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | We do not quite understand why this needs to be discussed. It looks like two different NTN items are mixed up here, IoT NTN and NR NTN. In NR-NTN, it is our understanding that GNSS fix does not get interrupted by normal data transmission or receiving. |
| CMCC | We share similar views with QC, based on our understanding, NR NTN UE has the capability to update the GNSS while performing transmission/reception. |
| Ericsson | Option 1: UE is required to be able to update GNSS while in connected mode while performing transmission/reception towards the RAN. |
| Apple | We cannot agree on either options. The GNSS updating is up to UE implementation and it doesn’t need to be specified in RRM spec as long as UE can meet the RRM/demod/RF requirement respectively. |
| Huawei | We understand option 1 is the default assumption for NTN. RAN4 has discussed the GNSS issue before, and the conclusion was that RAN4 postpone the discussion on RRM impact due to UE Internal Coexistence between GNSS receiver and NR UL transmitter until RF session has the conclusion. |
| MTK | We are not clear about what requirement should be specified and its necessity. |
| LGE | Updating GNSS is up to UE implementation, so we think no further clarification is needed. |
| CATT | In UE implementation, we think it will be option 1. But in RRM spec, we agree it is not necessary to define UE behavior as long as UE can meet RRM requirements. |
| OPPO | Agree with LGE and CATT, GNSS fix is up to UE implementation. |
| Xiaomi | We disagree with these options, in my understanding, whether UE can acquire GNSS fin in RRC_Connected was discussed only in IoT NTN WID, not apply in NTN WID. |
| Nokia | Based on the overall position of the companies, it seems the GNSS fix is assumed to not be interrupted in connected mode. As such, it is assumed UE is capable of updating the synchronization timing and frequency at the beginning of every UL slot the UE is transmitting on. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

- Issue 1-1: Clarification on GNSS fix in RRC_Connected mode for timing and frequency pre-compensation in NR operation.

|  | Status summary |
| --- | --- |
| Issue 1-1 | All the companies think no need to have such discussion and it is assumed that NR NTN UE has the capability to update the GNSS while performing transmission/reception. Tentative agreements: - No further clarification GNSS fix in RRC_Connected mode is needed. Recommendations for 2nd round: - No further discussion in the 2nd round |

Discussion on 2nd round (if applicable)

Topic #2: Maintenance for UE timing requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2208101 | Xiaomi | DraftCR on UE timer accuracy for NR_NTN |
| R4-2208360 | OPPO | Proposal 1: For Application time of TA adjustment, the following text proposal can be introduced for TS 38.133: 7.3C.2.1 Timing Advance adjustment delay UE shall adjust the timing of its uplink transmission timing at time uplink slot n+ k+1+K_offset for a timing advance command received in time slot n, and the value of k is defined in clause 4.2 in TS 38.213 [3], K_offset is a parameter configured by higher layer as specified in [TS 38.213 clause 4.2]. The same requirement applies also when the UE is not able to transmit a configured uplink transmission due to the channel assessment procedure. |
| R4-2208361 | OPPO | Draft CR to the timing requirements for NR NTN |
| R4-2208471 | MediaTek inc. | Draft CR on Introduction of Timing advance requirement for NTN |
| R4-2208653 | Ericsson | Observation 1: RAN1 are working on clarifications in TS 38.211 and TS 38.213 regarding N_”TA,adj” ^”common” and N_”TA,adj” ^”UE” . Proposal 1: Add any clarifications regarding N_”TA,adj” ^”common” and N_”TA,adj” ^”UE” in RAN1 specification. Observation 2: The legacy RAN4 requirement, the DL reference timing can only focus on the DL reception timing of first path of reference cell without considering the UL transmission. Proposal 2: No need to add further clarification downlink timing of the reference cell. |
| R4-2208995 | Huawei, Hisilicon | Proposal 1: The definition of common TA (N_”TA,adj” ^”common” ) and UE specific TA (N_”TA,adj” ^”UE” ) in TS 38.211 can be reused in RAN4, and there is no need to further clarify. Proposal 2: The definition of downlink timing of reference cell used for legacy UE can be reused for NTN UE. |
| R4-2208996 | Huawei, Hisilicon | DraftCR on UE transmit timing requirements for NTN R17 |
| R4-2209639 | Nokia, Nokia Shanghai Bell | Proposal 2: Capture the text proposal in TP #1 ------------------------------------ TP#1 TS 38.133----------------------------------------------- ### 7.3C.1 Introduction The timing advance is initiated from gNB to NTN UE in NR SA operation modes, with MAC message that implies the adjustment of the timing advance, as defined in clause 5.2 of TS 38.321 [7]. Operations over NTN constitutes an exception to this rule, whenever the UE is required to either apply or different of 0. In this case the timing advance is initiated and maintained at the UE. For NTN operations, the gNB may provide assistance to correct the timing advance initiated by the UE using MAC messages. For UE initiated operations, if UE considers GNSS is a reliable synchronization reference, the UE shall meet timing accuracy requirement as specified in this document and frequency accuracy requirement as specified in TS38.101-5 [xx]. -------------------------------- end of TP#1------------------------------------------------------------------- |
| R4-2209639 | Nokia, Nokia Shanghai Bell | In this contribution we propose the following: Proposal 1: RAN4 to adopt Option 1 and use the agreed RAN1 definition and assumptions for both N_(TA,UE-specific) and N_(TA,common). Proposal 2: RAN4 should not mandate the use of a specific reference propagator model in the UE for the estimation of the N_(TA,UE-specific) or N_(TA,common). And have the following observations: Observation 1: According to RAN1 agreements, the reference target time for UL reception at the target satellite is given by the reference DL time at the satellite plus the network controlled offset, indicated in the common delay parameters. Observation 2: According to RAN1 agreements, the expected ehaviour of the UE is such the values of N_(TA,UE-specific) and N_(TA,common) are maintained by the UE, in order to maintain the received time of the first symbol of the UL slot, within an error margin T_e from the reference target time for UL at the satellite. Observation 3: In RAN1 discussion it was left for FFS what is the UE ehaviour when the aggregate N_(TA,UE-specific) + N_(TA,common) increase between consecutive slots the UE is scheduled to transmit on. Observation 4: In Option 2, the proposed definition for N_(TA,common) is the same as the one already agreed in RAN1. However, the use of the “F3 + F4” requires further clarification. |
| R4-2207960 | Qualcomm Incorporated | Proposal 1: In test cases, for UE transmit timing and timing advance error measurements, a time reference is defined as the downlink timing of the reference cell minus (N_TA + N_{TA,UE-specific} +N_{TA,common} + N_{TA,offset}) x T_c where • Reference timing of downlink is the DL slot corresponding to UL slot index where UE transmits the UL signal/channel. • Reference timing of N_{TA,UE-specific} is S3 + S4, where o for S3, the slot when the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and Eckstein Hechler based propagator model o for S4, the slot when the DL transmission corresponding to the reference timing of downlink is supposed to arrive at the target satellite based on actual received time of the slot and provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and Eckstein Hechler based propagator model • Reference timing for N_{TA,common}, F3+F4, is derived according to N_{TA, common} related parameters broadcasted within a validity duration. • Note that downlink frame boundary should also be adjusted according to open-loop TA control related parameters provided by serving cell. |
| R4-2208103 | Xiaomi | Proposal 1: The definition of N_(TA,UE-specific) and N_(TA,common) shall follow the definition agreed in RAN1, and the value of N_(TA,UE-specific) and N_(TA,common) used to determine the reference timing should be the value without estimation/calculation error. Proposal 2: The existing definition of downlink timing defined in TS38.133 is reused in NTN scenario. |
| R4-2208423 | CMCC | Observation 1: How to define NTA,UE-specific and NTA,common related parameters and how to reflect them in the test may need the feedback from TE vendor Proposal 3: For the test requirement, several updates should be included based on current test requirement: 1. The formula (NTA + NTA_offset) ×Tc ± Te should be updated to (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN, the parameter Te should be updated to Te_NTN. 2. The clarification of NTA,common and NTA,UE-specific are needed, which are: – The NTA,UE-specific and NTA,common are ideal value, no estimation or calculation error will be included. – Reference timing for NTA,UE-specific and NTA,common is the slot when UL transmission is supposed to arrive at the target satellite based on true satellite position. Proposal 5: The parameter Koffset should be defined and the delay requirements should be revisited accordingly based on legacy requirements. Such as: – The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. k+1+2μKoffset slots after the reception of the timing advance command, where k=5, Koffset=[TBD]. |

Open issues summary and Companies views’ collection for 1st round

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Maintenance for UE transmit timing requirements

Issue 2-1-1: The clarification on NTA,UE-specific and NTA,common.

- Option 1: (Ericsson)

  - Add any clarifications regarding NTA,adjcommon and NTA,adjUE in RAN1 specification.

- Option 2: (Huawei, Nokia)

  - The definition of common TA (NTA,adjcommon ) and UE specific TA (NTA,adjUE ) in TS 38.211 can be reused in RAN4, and there is no need to further clarify in RAN4.

- Option 2a: (Xiaomi)

  - The definition of NTA,adjcommon and NTA,adjUE shall follow the definition agreed in RAN1, and the value of NTA,adjcommon and NTA,adjUE used to determine the reference timing should be the value without estimation/calculation error.

- Option 3: (Qualcomm)

  - In test cases, for UE transmit timing and timing advance error measurements, a time reference is defined as the downlink timing of the reference cell minus (N_TA + N_{TA,UE-specific} +N_{TA,common} + N_{TA,offset}) x T_c where

    - Reference timing of downlink is the DL slot corresponding to UL slot index where UE transmits the UL signal/channel.

    - Reference timing of N_{TA,UE-specific} is S3 + S4, where

      - for S3, the slot when the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model

      - for S4, the slot when the DL transmission corresponding to the reference timing of downlink is supposed to arrive at the target satellite based on actual received time of the slot and provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model

      - The reference propagator model shall be defined in RAN4 in such a way that those Ues using more accurate propagator model than the reference model are not penalized. The reference model can be determined based on companies’ input. And Eckstein Hechler based propagator model can be one of the candidate models.

    - Reference timing for N_{TA,common}, F3+F4, is derived according to N_{TA, common} related parameters broadcasted within a validity duration.

    - Note that downlink frame boundary should also be adjusted according to open-loop TA control related parameters provided by serving cell.

- Option 4: (CMCC)

  - For the test requirement, several updates should be included based on current test requirement:

    - The formula (NTA + NTA_offset) ×Tc ± Te should be updated to (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN, the parameter Te should be updated to Te_NTN.

    - The clarification of NTA,common and NTA,UE-specific are needed, which are:

      - The NTA,UE-specific and NTA,common are ideal value, no estimation or calculation error will be included.

      - Reference timing for NTA,UE-specific and NTA,common is the slot when UL transmission is supposed to arrive at the target satellite based on true satellite position.

- Recommended WF

  - The definition of common TA (NTA,adjcommon ) and UE specific TA (NTA,adjUE ) in TS 38.211 can be referred in TS38.133, and there is no need to further clarify in core requirements in TS38.133.

| Company | Comments |
| --- | --- |
| Qualcomm | Just to clarify the intent of our proposal. We are not proposing to modify the current definition of parameters in RAN1 spec. Our proposals are about how to set the values for those parameters in test cases so that UE UL transmission timing error can be accurately measured by TE without including any error due to incorrectly modeled timing relation between UE, satellite, and SRP (Synchronization Reference Point). In summary, we can skip this in Core part, and discuss it in Performance part. |
| CMCC | In 38.133 core requirement, the definition of NTA,adjcommon and NTA,adjUE shall follow the definition agreed in RAN1, we are ok with the recommended WF. In 38.133 test cases, we think the values of common TA (NTA,adjcommon ) and UE specific TA (NTA,adjUE ) should be further clarified in order to derive the reference timing for test. The value of NTA,adjcommon and NTA,adjUE used to determine the reference timing should be the value without estimation/calculation error. However, as QC’s comment, it can be discussed in performance part. |
| Ericsson | The recommended WF is fine. |
| Apple | Agree with recommended WF |
| Huawei | We agree with the recommended WF, and the symbols used in TS38.133 can be aligned with that used in TS38.211. |
| MTK | Fine with the recommended WF |
| LGE | Support recommended WF |
| CATT | Agree with recommended WF. |
| OPPO | Agree with recommended WF. |
| Xiaomi | Agree with recommended WF. |
| Nokia | The recommended WF is Ok. |

Issue 2-1-2: The clarification on downlink timing of the reference cell.

- Option 1: (Ericsson)

  - No need to add further clarification downlink timing of the reference cell.

- Option 2: (Huawei, Xiaomi)

  - The definition of downlink timing of reference cell used for legacy UE can be reused for NTN UE.

- Option 3: (Qualcomm)

  - In test cases, for UE transmit timing and timing advance error measurements, a time reference is defined as the downlink timing of the reference cell minus (N_TA + N_{TA,UE-specific} +N_{TA,common} + N_{TA,offset}) x T_c where

    - Reference timing of downlink is the DL slot corresponding to UL slot index where UE transmits the UL signal/channel.

    - Reference timing of N_{TA,UE-specific} is S3 + S4, where

      - for S3, the slot when the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model

      - for S4, the slot when the DL transmission corresponding to the reference timing of downlink is supposed to arrive at the target satellite based on actual received time of the slot and provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model

      - The reference propagator model shall be defined in RAN4 in such a way that those UEs using more accurate propagator model than the reference model are not penalized. The reference model can be determined based on companies’ input. And Eckstein Hechler based propagator model can be one of the candidate models.

    - Reference timing for N_{TA,common}, F3+F4, is derived according to N_{TA, common} related parameters broadcasted within a validity duration.

    - Note that downlink frame boundary should also be adjusted according to open-loop TA control related parameters provided by serving cell.

- Recommended WF

  - The definition of downlink timing of reference cell used for legacy UE can be reused for NTN UE.

| Company | Comments |
| --- | --- |
| Qualcomm | Same comment as Issue 2-1-1. |
| CMCC | We are ok with the recommended WF. |
| Ericsson | The recommended WF is fine. |
| Apple | Agree with recommended WF |
| Huawei | We agree with the recommended WF. |
| MTK | Fine with the recommended WF |
| LGE | Support the recommended WF |
| CATT | Agree with recommended WF. |
| OPPO | Agree with recommended WF. |
| Xiaomi | Agree with recommended WF. |
| Nokia | The recommended WF is Ok. |

Maintenance for TA adjustment requirements

Issue 2-2-1: NTN TA adjustment delay requirement..

- Option 1: (MTK)

  - UE shall adjust the timing of its uplink transmission timing at time uplink slot n+ k+1 for a timing advance command received in time slot n, and the value of k is defined in clause 4.2 in TS 38.213 [3].

- Option 2: (OPPO)

  - UE shall adjust the timing of its uplink transmission timing at time uplink slot n+ k+1+K_offset for a timing advance command received in time slot n, and the value of k is defined in clause 4.2 in TS 38.213 [3], K_offset is a parameter configured by higher layer as specified in [TS 38.213 clause 4.2].

- Option 3: (CMCC, Xiaomi)

  - UE shall adjust the timing of its uplink transmission timing at time slot n+ k +1 + 2^μ*K_offset for a timing advance command received in time slot n, and the value of k and K_offset is defined in clause 4.2 in TS 38.213 [3].

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Option 3. Whatever is agreed here in RAN4, that will be nothing but a duplication of RAN1 spec. |
| CMCC | Option 3. The description should be aligned with TS 38.213. |
| Ericsson | The way we understand he definition of K_offset, the unit is 1 ms. Then the map from K_offset in ms to slots becomes as stated in Option 3: (2^μ)*K_offset. |
| Apple | Option 3 based on TS38.213-h10 section 4.2. |
| Huawei | Option 3 is more aligned with the definition in TS38.213, which is defined as follow: |
| MTK | Fine with the Option 3, which is more accurate |
| LGE | We are fine with option 3. |
| CATT | Option 3 is aligned with latest RAN1 spec. |
| OPPO | Agree with option 3. |
| Xiaomi | Support option 3, which is specified in TS38.213. |

Issue 2-2-2: The requirements for the reliability of GNSS

- Option 1: (Nokia)

  - Capture the text proposal in TP #1

------------------------------------ TP#1 TS 38.133-----------------------------------------------

7.3C.1	Introduction

The timing advance is initiated from gNB to NTN UE in NR SA operation modes, with MAC message that implies the adjustment of the timing advance, as defined in clause 5.2 of TS 38.321 [7]. Operations over NTN constitutes an exception to this rule, whenever the UE is required to either apply  or  different of 0.  In this case the timing advance is initiated and maintained at the UE.

For NTN operations, the gNB may provide assistance to correct the timing advance initiated by the UE using MAC messages. For UE initiated operations, if UE considers GNSS is a reliable synchronization reference, the UE shall meet timing accuracy requirement as specified in this document and frequency accuracy requirement as specified in TS38.101-5 [xx].

-------------------------------- end of TP#1-------------------------------------------------------------------

- Option 2: ()

  - No need to introduce the requirements for the reliability of GNSS.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Option 2. |
| CMCC | We prefer Option2. If GNSS is not reliable, then UE shall fail the test cases. |
| Ericsson | Option 1, add the TP. |
| Apple | Option 2, we don’t think GNSS requirement shall be specified as long as UE can meet the corresponding RRM requirement, e.g., Te/Tp/Tq requirement. |
| MTK | Option 2. If GNSS is not reliable enough, then UE will fail current requirement. Don’t see the necessity to introduce this TP. |
| LGE | Option 2. |
| CATT | Option 2. Or we can add the text”If UE considers GNSS is a reliable synchronization reference, the UE shall meet RRM requirements” in general applicability for NTN in Chapter 3 of 38.133. |
| Xiaomi | Option 2, the observation is valid, RAN4 should assume GNSS is reliable if UE meets timing accuracy requirement and frequency accuracy requirement. But we do not think it is necessary to introduce the requirements for the reliability of GNSS. |
| Nokia | Support Option 1 because there is no such text exists in the specification. |

CRs comments collection for 1st round

| CR/TP number | Comments collection |
| --- | --- |
| R4-2208101 Xiaomi DraftCR on UE timer accuracy for NR_NTN | CATT: ok with this CR. |
|  | Company B |
| R4-2208361 OPPO Draft CR to the timing requirements for NR NTN | Ericsson: Unit of K_offset is 1*ms => +(2^µ) * K_offset |
|  | CATT: it is duplicated with R4-2208471 and should be update according to conclusion of Issue 2-2-1. |
| R4-2208471 MediaTek Introduction of Timing advance requirement for NTN | CATT: part of CR is duplicated with R4-2208361 and should be update according to conclusion of Issue 2-2-1 |
|  | Company B |
| R4-2208996 Huawei DraftCR on UE transmit timing requirements for NTN R17 | CATT: fine with update the parameters aligned with RAN1 spec. the same correction of parameters should be also applied in 7.1C.1 and texts below table in 7.1C.2 |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Issue 2-1-1: The clarification on NTA,UE-specific and NTA,common.

|  | Status summary |
| --- | --- |
| Issue 2-1-1 | 10 companies are fine with the recommendation WF and 1 company suggest to skip this discussion in core part and continue the discussion in performance part. Thus, it is proposed to capture the recommendation WF in WF for information. Tentative agreements: - The following clarification is captured in WF for information - The definition of common TA (NTA,adjcommon ) and UE specific TA (NTA,adjUE ) in TS 38.211 can be referred in TS38.133, and there is no need to further clarify in core requirements in TS38.133. Recommendations for 2nd round: - No further discussion in the 2nd round |

Issue 2-1-2: The clarification on downlink timing of the reference cell.

|  | Status summary |
| --- | --- |
| Issue 2-1-2 | 10 companies are fine with the recommendation WF and 1 company suggest to skip this discussion in core part and continue the discussion in performance part. Thus, it is proposed to capture the recommendation WF in WF for information. Tentative agreements: - The following clarification is captured in WF for information - The definition of downlink timing of reference cell used for legacy UE can be reused for NTN UE. Recommendations for 2nd round: - No further discussion in the 2nd round |

Issue 2-2-1: NTN TA adjustment delay requirement.

|  | Status summary |
| --- | --- |
| Issue 2-2-1 | All the companies support option 3. Tentative agreements: - UE shall adjust the timing of its uplink transmission timing at time slot n+ k +1 + 2^μ*K_offset for a timing advance command received in time slot n, and the value of k and K_offset is defined in clause 4.2 in TS 38.213 [3]. Recommendations for 2nd round: - No further discussion in the 2nd round |

Issue 2-2-2: The requirements for the reliability of GNSS.

|  | Status summary |
| --- | --- |
| Issue 2-2-2 | - Option 1: (Nokia, Ericsson) - Capture the text proposal in TP #1 - Option 2: (QC, CMCC, Apple, MTK, LGE, CATT, Xiaomi) - No need to introduce the requirements for the reliability of GNSS. Majority companies support option 2, and 2 companies support option 1. Recommendations for 2nd round: - Continue the discussion in the 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Issue 2-2-2: The requirements for the reliability of GNSS.

- Option 1: (Nokia, Ericsson)

  - Introduce the requirements for the reliability of GNSS

- Option 2: (QC, CMCC, Apple, MTK, LGE, CATT, Xiaomi)

  - No need to introduce the requirements for the reliability of GNSS.

| Company | Comments |
| --- | --- |
| Xiaomi | Support option 2, In V2X WI, since the GNSS is used as the synchronization resource, thus the requirement of GNSS reliability was introduced to guarantee the GNSS signal is reliable during the synchronization resource selection/reselection. However, in NTN, the GNSS is used to derive UE specific TA, the GNSS accuracy (50m) is assumed and is accounted in Te requirements. Thus, there is no need to introduce the GNSS reliability requirement as long as Te requirement is fulfilled. |
| Qualcomm | Support Option 2. Agree with Xiaomi. |
| Apple | Option 2. |
| CMCC | Support Option 2. Based on our understanding, if GNSS is not reliable, such as GNSS error larger than 50m, then it can’t pass the test. |
| OPPO | Support option 2. |
| Ericsson | We can support option 2. Based on discussion and input from other companies we agree that this is indirectly captured in Te requirements for NTN UE UL, to be tested. |
| Huawei | Support option 2. How to configure reliable GNSS can be considered for testing. |
| CATT | Support option 2. |
| LGE | Support option 2 |

Topic #3: Performance requirements for NR NTN RRM

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2207960 | Qualcomm Incorporated | Proposal 1: In test cases, for UE transmit timing and timing advance error measurements, a time reference is defined as the downlink timing of the reference cell minus (N_TA + N_{TA,UE-specific} +N_{TA,common} + N_{TA,offset}) x T_c where • Reference timing of downlink is the DL slot corresponding to UL slot index where UE transmits the UL signal/channel. • Reference timing of N_{TA,UE-specific} is S3 + S4, where o for S3, the slot when the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and Eckstein Hechler based propagator model o for S4, the slot when the DL transmission corresponding to the reference timing of downlink is supposed to arrive at the target satellite based on actual received time of the slot and provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and Eckstein Hechler based propagator model • Reference timing for N_{TA,common}, F3+F4, is derived according to N_{TA, common} related parameters broadcasted within a validity duration. • Note that downlink frame boundary should also be adjusted according to open-loop TA control related parameters provided by serving cell. |
| R4-2208103 | Xiaomi | Proposal 2: The existing definition of downlink timing defined in TS38.133 is reused in NTN scenario. TC index; TC; Purpose; Section ID. 1; NTN UE transmit timing test; To verify the UE can follow frame timing change of the connected gNB and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits.; A.14.x1.1. 2; NTN UE timing advance adjustment accuracy test; To verify the UE Timing Advance adjustment delay and accuracy requirement.; A.14.x2.2. |
| R4-2208423 | CMCC | Proposal 1: Only the NR-SA FR1 test cases for NTN timing requirement need to be defined. Proposal 2: Define the UE transmit timing test cases with stationary UE in GEO scenario. Observation 1: How to define NTA,UE-specific and NTA,common related parameters and how to reflect them in the test may need the feedback from TE vendor Proposal 3: For the test requirement, several updates should be included based on current test requirement: 1. The formula (NTA + NTA_offset) ×Tc ± Te should be updated to (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN, the parameter Te should be updated to Te_NTN. 2. The clarification of NTA,common and NTA,UE-specific are needed, which are: – The NTA,UE-specific and NTA,common are ideal value, no estimation or calculation error will be included. – Reference timing for NTA,UE-specific and NTA,common is the slot when UL transmission is supposed to arrive at the target satellite based on true satellite position. Proposal 4: Define the timing advance adjustment delay and accuracy test cases with stationary UE in GEO scenario. Proposal 5: The parameter Koffset should be defined and the delay requirements should be revisited accordingly based on legacy requirements. Such as: – The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. k+1+2μKoffset slots after the reception of the timing advance command, where k=5, Koffset=[TBD]. |
| R4-2209218 | Huawei, Hisilicon | Proposal 1: RAN4 to discuss the assumption on the frequency error for evaluating the accuracy performance. Inputs from the satellite system vendors are appreciated. Proposal 2: The RRM test for NTN WI should focus on LEO. At most one satellite is modelled in the tests besides the serving satellite. Proposal 3: Define the following TCs corresponding to existing TCs in clause A.6 for NTN - Cell reselection to intra- and inter-frequency neighbor cell  Including reselection measurement based on time and location trigger - Intra- and inter-frequency HO with known cell - Intra- and inter-frequency CHO  Including CHO with time or location condition - UE transmit timing - RLM and BFR - Intra-frequency measurement delay  Including 2 SMTCs per MO with and without collision  Including 2 satellites per SMTC - Inter-frequency measurement delay  Including 2 MGs - L1-RSRP measurement delay - RRM measurement accuracy |

Open issues summary and Companies views’ collection for 1st round

Measurement accuracy

Issue 3-1-1: Measurement accuracy.

- Option 1: (Huawei)

  - RAN4 to discuss the assumption on the time and frequency error for evaluating the accuracy performance. Inputs from the satellite system vendors are appreciated.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Support Option 1. At least for test cases with NGSO, time-varying propagation delay change, time/frequency error and sampling clock offset shall be modeled based on ephemeris information, feeder link propagation delay, relative elevation angles between UE-and-satellite and satellite-and-GW(assuming Synchronization Reference Position is located at GW). Without such a NTN specific time-varying parameter modeling, test cases would be not much different from those for the legacy TN, and RAN4 may end up defining test cases that are even easier to pass than HST. More importantly, a UE implementation that accurately deals with the time-varying characteristics based on the broadcasted NTN specific parameters would be unfairly penalized. As a first step, RAN4 would need information from satellite companies about orbit parameters for at least two satellites. And we should consider the satellites connected to different GWs. |
| Ericsson | We agree on Option1. |
| Apple | FFS on option 1. In our understanding, ephemeris information provided by network can be treated as ideal in the requirement design, and the only inaccuracy part could be from UE propagator model and UE measurement error. This measurement accuracy requirement can be treated in the same way as timing requirement (i.e., ephemeris itself is ideal but introduce margin for propagator model error). |
| Huawei | Option 1 is our proposal and we support it. The intention is to evaluate whether the TN measurement accuracy requirements can be re-used for NTN, and the main aspects to check is the time and frequency error, i.e. what is the typical time and frequency error when UE determines the measurement timing and frequency based on the assistance information. |
| CATT | Need further discussion on option1 in detail. In general, we support to discuss the assumption for evaluating the measurement accuracy requirements. But for the details of timing and frequency error, FFS The model of timing and frequency varies along with time |
| OPPO | We are fine to discuss time and frequency assumptions. |
| Nokia | Option 1. |

Test cases for NTN RRM requirements

Issue 3-2-1: Test coverage in term of scenarios for NTN RRM test cases.

- Option 1: (Huawei)

  - The RRM test for NTN WI should focus on LEO. At most one satellite is modelled in the tests besides the serving satellite.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Fine with Option 1 for the first part. Regarding “one satellite,” we prefer “two satellites” for mobility tests. If we can model one NGSO satellite in the test system, adding one more satellite would not be an issue. Is there any technical issue with that? |
| Ericsson | Time/location based idle mode mobility and connected mode mobility mainly aim at LEO. It’s necessary to prioritize LEO. But, we need GEO scenario to verify requirements defined differently, e.g. idle mode reselection relaxation on GEO . |
| Apple | General fine with option 1 and how to setup a LEO is FFS. |
| Huawei | Option 1 is our proposal and we support it. To QC, maybe we did not make the proposal clear enough, but our intention is that at most one neighbour satellite is modelled in the tests besides the serving satellite, so in the RRM measurement or mobility tests, there will be two satellites. |
| MTK | We also need test for GEO, at least for some tests, e.g. UE transmit timing. Because we need to consider some UE which supports GEO but may not support LEO. Similar to Issue 3-2-3, if the UE has passed LEO/NGSO test, then it can skip GEO test. |
| CATT | We support LEO for some requirements such as idle mode cell-reselection, mobility, connected mode and so on because GEO has the same requirements as TN system. But for timing requirements especially for UE timing advance adjustment accuracy, GEO is better for test. |
| OPPO | Fine with option 1. |
| Xiaomi | Fine with option 1. |
| Nokia | If the wording of Option 1 is improved as explained by Huawei, we can agree. |

Issue 3-2-2: NTN RRM test cases

- Option 1: (Huawei)

  - Define the following TCs corresponding to existing TCs in clause A.6 for NTN

    - Cell reselection to intra- and inter-frequency neighbor cell

      - Including reselection measurement based on time and location trigger

    - Intra- and inter-frequency HO with known cell

    - Intra- and inter-frequency CHO

      - Including CHO with time or location condition

    - UE transmit timing

    - RLM and BFR

    - Intra-frequency measurement delay

      - Including 2 SMTCs per MO with and without collision

      - Including 2 satellites per SMTC

    - Inter-frequency measurement delay

      - Including 2 MGs

    - L1-RSRP measurement delay

    - RRM measurement accuracy

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Okay with Option 1. UE transmit timing is also included in Issue 3-2-3. |
| Ericsson | Ok with Option1. |
| Apple | Generally fine with the list in option 1. But we are wondering why only considering known cell case for intra- and inter-frequency HO testing. |
| Huawei | Option 1 is our proposal and we support it. This is an initial list of TCs we are thinking of based on the core requirements, and we are open to other opinions. |
| MTK | Fine with Option 1 |
| CATT | Fine with option 1. |
| OPPO | Fine with option 1. |
| Xiaomi | Fine with option 1 |
| Nokia | Option 1 can be used as a starting point. |

Issue 3-2-3: Test case list for NTN UE timing requirements.

- Option 1: (Xiaomi)

  - The test cases list for NTN UE timing requirements are proposed in table as follows：

| TC index | TC | Purpose | Section ID |
| --- | --- | --- | --- |
| 1 | NTN UE transmit timing test | To verify the UE can follow frame timing change of the connected gNB and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. | A.14.x1.1 |
| 2 | NTN UE timing advance adjustment accuracy test | To verify the UE Timing Advance adjustment delay and accuracy requirement. | A.14.x2.2 |

- Option 2: (CMCC)

  - Only the NR-SA FR1 test cases for NTN timing requirement need to be defined.

  - Define the UE transmit timing test cases with stationary UE in GEO scenario.

  - Define the timing advance adjustment delay and accuracy test cases with stationary UE in GEO scenario.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Support Option 1, and there should be two different test cases for GSO and NGSO. UE supporting NGSO can skip GSO based test cases. UE mobility can be assumed stationary in the tests. |
| CMCC | Option 1 is a general guidance; we are ok with it. Option 2 provides more details. We are not against defining LEO scenario test cases, we just have some concern about test feasibility. If LEO is testable, we also support define two test cases for GEO and LEO scenarios. Before we conclude this issue, it is better to wait more feedback from satellite companies and TE vendors. |
| Ericsson | Option 1 is fine for both LEO and GEO tests. With regards to Option 2 we prefer not to limit test to GEO since many aspects of LEO orbits are more demanding like max Doppler frequency and the rate of change of Doppler frequency and rate of change of RTT. GEO is most demanding when it comes to maximum RTT. |
| Apple | Option 2. |
| Huawei | For option 1, we can agree the proposed test case list. For option 2, we agree with 2nd and 3rd bullets. For UE transmit timing requirements, the change of common TA and UE specific TA due to serving satellite movement are not considered the gradual timing adjustment requirements. For TA adjustment requirements, the change of common TA and UE specific TA is apart from current accuracy requirements. So, stationary satellite (i.e. GEO) can be selected for testing. |
| MTK | We can define TCs for both GEO and NGSO based on Option 1. For defining GEO TC, Option 2 is fine. |
| CATT | Fine with option 1. For option 2, for UE timing advance adjustment accuracy test, LEO is not suitable, GEO is better for testing. |
| Xiaomi | Support option 1. We think GEO and NGSO can be considered as one of the test configuration, like TDD/FDD, SCS. The UE is only required to be tested in one of the supported test configurations. |
| Nokia | Share the same view as Ericsson. |

Issue 3-2-4: Reference time for UE transmit timing and timing advance error test cases.

- Option 1: (Qualcomm)

  - In test cases, for UE transmit timing and timing advance error measurements, a time reference is defined as the downlink timing of the reference cell minus (N_TA + N_{TA,UE-specific} +N_{TA,common} + N_{TA,offset}) x T_c where

    - Reference timing of downlink is the DL slot corresponding to UL slot index where UE transmits the UL signal/channel.

    - Reference timing of N_{TA,UE-specific} is S3 + S4, where

      - for S3, the slot when the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model

      - for S4, the slot when the DL transmission corresponding to the reference timing of downlink is supposed to arrive at the target satellite based on actual received time of the slot and provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model

      - The reference propagator model shall be defined in RAN4 in such a way that those UEs using more accurate propagator model than the reference model are not penalized. The reference model can be determined based on companies’ input. And Eckstein Hechler based propagator model can be one of the candidate models.

    - Reference timing for N_{TA,common}, F3+F4, is derived according to N_{TA, common} related parameters broadcasted within a validity duration.

    - Note that downlink frame boundary should also be adjusted according to open-loop TA control related parameters provided by serving cell.

- Option 2: (CMCC)

  - For the test requirement, several updates should be included based on current test requirement:

    - The formula (NTA + NTA_offset) ×Tc ± Te should be updated to (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN, the parameter Te should be updated to Te_NTN.

    - The clarification of NTA,common and NTA,UE-specific are needed, which are:

      - The NTA,UE-specific and NTA,common are ideal value, no estimation or calculation error will be included.

      - Reference timing for NTA,UE-specific and NTA,common is the slot when UL transmission is supposed to arrive at the target satellite based on true satellite position.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Support Option 1. For Option 2: - Reference timing for NTA,UE-specific and NTA,common shall not be based on true satellite position unless any specific accuracy requirements on NW in terms of NTN specific timing related parameters (ephemeris and higher-layer Common TA parameters) are defined. - “NTA,UE-specific and NTA,common is the slot when UL transmission is supposed to arrive at the target satellite” is technically incorrect. To avoid this incorrect understanding, Option 1 is proposed. As we commented on Issue 2-1-1, Option 1 is not intended to modify/redefine the current definition of parameters in RAN1 spec. It is just to clarify how the values for those parameters should be set in test cases so that UE UL transmission timing error can be accurately measured by TE without including any error due to incorrectly modeled timing relation between UE, satellite, and SRP (Synchronization Reference Point) in Test System. To begin with, we try to illustrate how UL transmission time is determined in NTN with a high-level pseudo code. - Suppose that, at time t2, UE receives DL slot M (r.t. epoch time slot 0 at SRP) and needs to find the transmit time for UL slot N, UE needs to know - Arrival time of DL slot N (t3) - NTA,UE-specific (Tsu+Tsd) - NTA,Common (Tcu+Tcd) - UE calculates Tcd1 and then Tsd1. For Tcd1, since the satellite is moving, UE finds Tcd1=f(M*Ds+Tcd1), where f is the polynomial function of common TA, Ds is the slot duration. The above equation can be solved iteratively. - UE calculates Tcd and Tsd, finds t3=t2+(N-M)*Ds+Tcd+Tsd-Tcd1-Tsd1 - UE calculates Tcu and then Tsu. Again, as the satellite is moving, Tsu needs to be found iteratively (2-step). For Tcu, UE finds Tcu=f(N*Ds-Tcu). - Transmit time for UL slot N is t3-Tcd-Tcu-Tsd-Tsu. Fig 1. Timing Relation from SRP to UE With that, we provide further details about NTA,UE-specific and NTA,common. Please refer to Fig. 2 and 3 attached at the bottom as well. - For NTA,UE-specific and NTA,common calculations, a satellite position shouldn’t be based on a true position because any inaccuracy in broadcasted Ephemeris information and Epoch time should not account for in UE estimation accuracy. This should be a common understanding in the group. - For NTA,common, the parameter ‘t’ shall be set to time instances when signals from UE and a reference point are supposed to arrive at a target satellite. - As can be seen from Fig 2, an ideal UL transmission timing (T2) shall be a reference DL timing minus ‘round trip delay over service link and feeder link’. Here, it should be noted that round trip delay on service link consists of S3 and S4 which are not identical to each other, i.e. it is not “2 x one way propagation delay”. And the same goes to the round trip delay on feeder link. That is because satellite position keeps changing over time. Therefore, the ‘round trip delay over service link and feeder link’ should be S3 + F3 + S4 + F4 in Fig 2, and a satellite position to derive ‘S3 + F3’ shall be different from that for ‘S4 + F4’. The detailed relation is also presented in Fig 1. - For a reference satellite position that is used to derive the ideal TA value in UE UL timing requirements, since we don’t use a true satellite position, a specific orbit propagator model shall be defined which will be used only for error measurement purpose. Note that the model shall not be less accurate than typical models that can be considered for a real UE implementation. Otherwise, UE using more accurate model can be unfairly penalized. In the last meeting, ‘Eckstein Hechler’ was suggested by one company as the reference model. Based on our survey on orbit propagation model, the Eckstein-Hechler based propagator predicts better than the basic-Kepler model because it considers the effect of non-spherical Earth. Therefore, we agree that the Eckstein-Hechler based propagator model can be used to define the reference timing in UE UL timing requirements. - In summary, for UE UL timing error measurement purpose, RAN4 shall clarify define how to derive NTA,UE-specific and NTA,common. Fig 2. Reference system model of timing relation between UE and UL timing synchronization reference point in NTN Fig 3. Timing Relation between UE DL Reception and UE UL Transmission |
| CMCC | We support Option 2. In the test, it is important to give a reference time for TE, no error should be included in NTA,common and NTA,UE-specific, since the errors have already been captured in Te_NTN Regarding the reference timing for NTA,UE-specific and NTA,common, if our understanding is correct, QC thinks that the “true satellite position” will introduce the inaccuracy from broadcasted Ephemeris information and Epoch time. While in our understanding, the real-time “true satellite position” is known by TE, instead of calculate it by using Ephemeris information and Epoch time, therefore, no error will be introduced. This part may need double check with TE vendors. |
| Ericsson | In topic #2 we agree to recommended WF: The definition of common TA (NTA,adjcommon ) and UE specific TA (NTA,adjUE ) in TS 38.211 can be referred in TS38.133, and there is no need to further clarify in core requirements in TS38.133. The definition of downlink timing of reference cell used for legacy UE can be reused for NTN UE. For test requirements we can update along the lines of option 2, but NTA,UE-specific will include estimation error of GNSS position and NTA,common will not stay perfect for the duration of a test depending on accuracy of feeder link polynomial model and update frequency. |
| Apple | This issue could he discussed together with issue 3-2-3, if option 2 in issue 3-2-3 is agreeable, then we agree with option 2 in this issue since it could make the test requirement clear and simple. |
| Huawei | Compared with current test case setup for legacy UE, the configurations of common TA and UE specific TA should be introduced. So, RAN4 need to study how to configure the values of common TA and UE specific TA. As we commented in issue 3-2-3, GEO scenario is suggested to be used. The common TA can be indicated as a fixed value. UE specific TA is calculated based serving satellite position and UE position. Serving satellite position is derived from the configured ephemeris information. RAN4 need to investigate how to configure the ephemeris information of serving satellite for obtaining the desired UE specific TA. |
| OPPO | NTA,common is configured by NW, it may not be perfect due to estimation error at NW side. But such the error should not be considered as UE timing requirements. So when defining the ideal reference timing, the NTA,common value indicated by NW should be used, rather than ideal value. NTA,UE-specific is calculated by UE itself, the related estimation error should be verified. We agree with option 2 to use ideal value of NTA,UE-specific. |
| Xiaomi | RAN4 need to study how to configure the values of common TA and UE specific TA, and we also think the inputs from the satellite system vendors are appreciated. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Issue 3-1-1: Measurement accuracy.

|  | Status summary |
| --- | --- |
| Issue 3-1-1 | - Option 1: (Huawei, QC, Ericsson, OPPO, Nokia) - RAN4 to discuss the assumption on the time and frequency error for evaluating the accuracy performance. Inputs from the satellite system vendors are appreciated. - Option 2: (Apple, CATT) - FFS. - Introduce margin for propagator model error (Apple) 5 companies support option 1 and 2 company suggest FFS. Recommendations for 2nd round: - Continue the discussion in the 2nd round |

Issue 3-2-1: Test coverage in term of scenarios for NTN RRM test cases.

|  | Status summary |
| --- | --- |
| Issue 3-2-1 | - Option 1: (Huawei, QC, Apple, OPPO, Xiaomi, Nokia) - The RRM test for NTN WI should focus on LEO. At most one neighbour satellite is modelled in the tests besides the serving satellite. - Option 2: (Ericsson, MTK, CATT) - The RRM test should consider both LEO and GEO scenario, which can be determined by case-by-case basis. Recommendations for 2nd round: - Continue the discussion in the 2nd round |

Issue 3-2-2: NTN RRM test cases

|  | Status summary |
| --- | --- |
| Issue 3-2-2 | - Option 1: (Huawei, QC, Ericsson, Apple, MTK, CATT, OPPO, Xiaomi, Nokia) - Define the following TCs corresponding to existing TCs in clause A.6 for NTN - Cell reselection to intra- and inter-frequency neighbor cell - Including reselection measurement based on time and location trigger - Intra- and inter-frequency HO with known cell - Intra- and inter-frequency CHO - Including CHO with time or location condition - UE transmit timing - RLM and BFR - Intra-frequency measurement delay - Including 2 SMTCs per MO with and without collision - Including 2 satellites per SMTC - Inter-frequency measurement delay - Including 2 MGs - L1-RSRP measurement delay - RRM measurement accuracy All the companies are fine with option 1 in general, option 1 is used as baseline to define the RRM test case for NTN. Recommendations for 2nd round: - Continue the discussion on the test case list in the 2nd round. |

Issue 3-2-3: Test case list for NTN UE timing requirements.

|  | Status summary |
| --- | --- |
| Issue 3-2-3 | - Option 1: (Xiaomi, QC, CMCC, Ericsson, Huawei, MTK, CATT, Nokia) - The test cases list for NTN UE timing requirements are proposed in table as follows： TC index; TC; Purpose; Section ID. 1; NTN UE transmit timing test; To verify the UE can follow frame timing change of the connected gNB and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits.; A.14.x1.1. 2; NTN UE timing advance adjustment accuracy test; To verify the UE Timing Advance adjustment delay and accuracy requirement.; A.14.x2.2. - Option 2: (CMCC, Apple) - Only the NR-SA FR1 test cases for NTN timing requirement need to be defined. - Define the UE transmit timing test cases with stationary UE in GEO scenario. - Define the timing advance adjustment delay and accuracy test cases with stationary UE in GEO scenario. 8 companies are fine with option 1 in general, and QC, Ericsson and MTK propose to introduce test cases for both GSO and NGSO. 2 company support option 2. Recommendations for 2nd round: - Continue the discussion in the 2nd round based on the updated table in option 1. - FFS defining test cases for both GSO and NGSO. |

Issue 3-2-4: Reference time for UE transmit timing and timing advance error test cases.

|  | Status summary |
| --- | --- |
| Issue 3-2-4 | - Option 1: (Qualcomm) - In test cases, for UE transmit timing and timing advance error measurements, a time reference is defined as the downlink timing of the reference cell minus (N_TA + N_{TA,UE-specific} +N_{TA,common} + N_{TA,offset}) x T_c where - Reference timing of downlink is the DL slot corresponding to UL slot index where UE transmits the UL signal/channel. - Reference timing of N_{TA,UE-specific} is S3 + S4, where - for S3, the slot when the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model - for S4, the slot when the DL transmission corresponding to the reference timing of downlink is supposed to arrive at the target satellite based on actual received time of the slot and provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model - The reference propagator model shall be defined in RAN4 in such a way that those UEs using more accurate propagator model than the reference model are not penalized. The reference model can be determined based on companies’ input. And Eckstein Hechler based propagator model can be one of the candidate models. - Reference timing for N_{TA,common}, F3+F4, is derived according to N_{TA, common} related parameters broadcasted within a validity duration. - Note that downlink frame boundary should also be adjusted according to open-loop TA control related parameters provided by serving cell. - Option 2: (CMCC, Ericsson, Apple) - For the test requirement, several updates should be included based on current test requirement: - The formula (NTA + NTA_offset) ×Tc ± Te should be updated to (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN, the parameter Te should be updated to Te_NTN. - The clarification of NTA,common and NTA,UE-specific are needed, which are: - The NTA,UE-specific and NTA,common are ideal value, no estimation or calculation error will be included. - Reference timing for NTA,UE-specific and NTA,common is the slot when UL transmission is supposed to arrive at the target satellite based on true satellite position. 1 company support option 1. And 3 company support option 2 in general, some companies suggest RAN4 need to study how to configure the values of common TA and UE specific TA. Recommendations for 2nd round: - Continue the discussion in the 2nd round. - How to configure the values of common TA in test cases? - How to configure the values of UE specific TA in test cases? |

Discussion on 2nd round (if applicable)

Issue 3-1-1: Measurement accuracy.

- Option 1: (Huawei, QC, Ericsson, OPPO, Nokia)

  - RAN4 to discuss the assumption on the time and frequency error for evaluating the accuracy performance. Inputs from the satellite system vendors are appreciated.

- Option 2: (Apple, CATT)

  - FFS.

    - Introduce margin for propagator model error (Apple)

| Company | Comments |
| --- | --- |
| Xiaomi | Need more discussion, for option 1, should we assume additional time and frequency error based on the existing time (Te) and frequency error (0.1ppm)? |
| Qualcomm | Support Option 1. To be honest, however, we don’t see what is the significance of agreeing Option 1. If the “time and frequency error” is about time-varying time/frequency offsets, it should be accurately modeled in the test. And any additional error due to imperfect satellite time/frequency control should not be part of tests. Further clarification on Option 1 from original proponent of the option would be appreciated. |
| Apple | We also don’t fully understand the justification on option 1. Like what Qualcomm said, is that an offset defined in test or it’s an estimated error observed in field or lab? |
| OPPO | We understand the two options are not contradictory. Margin for accuracy measurement can be introduced after evaluating the impacts by time and frequency error in NTN. |
| Ericsson | Option 1. |
| Huawei | The original intention of option 1 is to clarify among companies whether we need to account for “time and frequency error” due to “imperfect satellite time/frequency control” in the accuracy requirements. If not, perhaps the accuracy of TN can be re-used. However, if the “time and frequency error” is large, the requirements we defined under ideal condition (no time/frequency error) may not be useful for the real deployment, i.e. the side condition of the requirements will never be met. This is why we are asking inputs from satellite system vendors about the “time and frequency error”. |
| CATT | In general, we support to further discuss the assumption for evaluating the measurement accuracy requirements. But it is still not clear for us of timing and frequency error in proposal 1, we are fine with FFS on the model of timing and frequency varies along with time. |

Issue 3-2-1: Test coverage in term of scenarios for NTN RRM test cases.

- Option 1: (Huawei, QC, Apple, OPPO, Xiaomi, Nokia)

  - The RRM test for NTN WI should focus on LEO. At most one neighbour satellite is modelled in the tests besides the serving satellite.

- Option 2: (Ericsson, MTK, CATT)

  - The RRM test should consider both LEO and GEO scenario, which can be determined by case-by-case basis.

| Company | Comments |
| --- | --- |
| Xiaomi | We would like to support a new option 3 which is combined with option 1 and 2. Option 3: - The RRM test should consider both LEO and GEO scenario, which can be determined by case-by-case basis. - One neighbour satellite is modelled in the tests besides the serving satellite |
| Qualcomm | Okay with Option 3 from Xiaomi. |
| Apple | We can agree on option 3. |
| OPPO | We can also support option 3 from Xiaomi. |
| Ericsson | We agree with option 1, that it is focus on LEO test, but some GEO tests should be included since GEO has other challenges, like RTT. |
| Huawei | We are also fine with option 3. |
| CATT | Fine with option 3. |
| LGE | Support option 3 |

Issue 3-2-2: NTN RRM test cases

- Option 1: (Huawei, QC, Ericsson, Apple, MTK, CATT, OPPO, Xiaomi, Nokia)

  - Define the following TCs corresponding to existing TCs in clause A.6 for NTN

    - Cell reselection to intra- and inter-frequency eighbour cell

      - Including reselection measurement based on time and location trigger

    - Intra- and inter-frequency HO with known cell

    - Intra- and inter-frequency CHO

      - Including CHO with time or location condition

    - UE transmit timing

    - RLM and BFR

    - Intra-frequency measurement delay

      - Including 2 SMTCs per MO with and without collision

      - Including 2 satellites per SMTC

    - Inter-frequency measurement delay

      - Including 2 MGs

    - L1-RSRP measurement delay

    - RRM measurement accuracy

- Recommended WF

  - Define test cases to verify the NTN RRM requirements captured in option 1.

| Company | Comments |
| --- | --- |
| Xiaomi | Support the recommended WF. |
| Qualcomm | Okay with recommended WF. |
| Apple | Fine with recommended WF. |
| Ericsson | The WF in fine. |
| Huawei | Fine with recommended WF. |
| CATT | Support the recommended WF |

Issue 3-2-3: Test case list for NTN UE timing requirements.

Tentative agreement:

| TC index | TC | Purpose | Section ID |
| --- | --- | --- | --- |
| 1 | NTN UE transmit timing test in GSO scenario | To verify the UE can follow frame timing change of the connected gNB and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits in GSO scenario. | A.14.x1.1 |
| 2 | NTN UE transmit timing test in NGSO scenario | To verify the UE can follow frame timing change of the connected gNB and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits in NGSO scenario. | A.14.x1.2 |
| 3 |  | NTN UE timing advance adjustment accuracy test in GSO scenario |  | To verify the UE Timing Advance adjustment delay and accuracy requirement in GSO scenario. |  | A.14.x2.3 |  |
| 4 | NTN UE timing advance adjustment accuracy test in NGSO scenario | To verify the UE Timing Advance adjustment delay and accuracy requirement in NGSO scenario. | A.14.x2.4 |

Note:

- For UE supporting NGSO, the GSO-based test cases can be skipped.

- For UE supporting GSO, the NGSO-based test cases can be skipped.

| Company | Comments |
| --- | --- |
| Xiaomi | Support the tentative agreement. |
| Qualcomm | Okay with tentative agreement, and the note should further say “UE supporting both NGSO and GSO, GSO-based test cases can be skipped if the UE passes NGSO-based test cases.” |
| Apple | Fine with tentative agreement. |
| CMCC | Ok with the tentative agreement and the note proposed by Qualcomm. |
| Ericsson | The test case list is fine. We understand that we need to consider total test load. However the notes need more work. How do we know that a UE supporting NGSO (ie LEO) can handle the large RTT of GSO (GEO)? |
| Huawei | Generally we are fine, but the testing principles can be updated as: - For UE supporting NGSO, the GSO-based test cases can be skipped. - For UE supporting GSO, the NGSO-based test cases can be skipped. - For UE supporting both GSO and NGSO, UE is only required to pass either the NGSO-based test cases or the GSO-based test cases. |
| CATT | Fine with the tentative agreement. For test 4, we have concern on how to do the test. How to distinguish open-loop TA error and close-loop TA error. |
| LGE | We are fine with the listed test cases with Huawei’s test principles. |

Issue 3-2-4A: How to configure the values of common TA in test cases?

Issue 3-2-4: Reference time for UE transmit timing and timing advance error test cases.

- Option 1: (Qualcomm)

  - In test cases, for UE transmit timing and timing advance error measurements, a time reference is defined as the downlink timing of the reference cell minus (N_TA + N_{TA,UE-specific} +N_{TA,common} + N_{TA,offset}) x T_c where

    - Reference timing of downlink is the DL slot corresponding to UL slot index where UE transmits the UL signal/channel.

    - Reference timing of N_{TA,UE-specific} is S3 + S4, where

      - for S3, the slot when the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model

      - for S4, the slot when the DL transmission corresponding to the reference timing of downlink is supposed to arrive at the target satellite based on actual received time of the slot and provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model

      - The reference propagator model shall be defined in RAN4 in such a way that those UEs using more accurate propagator model than the reference model are not penalized. The reference model can be determined based on companies’ input. And Eckstein Hechler based propagator model can be one of the candidate models.

    - Reference timing for N_{TA,common}, F3+F4, is derived according to N_{TA, common} related parameters broadcasted within a validity duration.

    - Note that downlink frame boundary should also be adjusted according to open-loop TA control related parameters provided by serving cell.

- Option 2: (CMCC, Ericsson, Apple)

  - For the test requirement, several updates should be included based on current test requirement:

    - The formula (NTA + NTA_offset) ×Tc ± Te should be updated to (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN, the parameter Te should be updated to Te_NTN.

    - The clarification of NTA,common and NTA,UE-specific are needed, which are:

      - The NTA,UE-specific and NTA,common are ideal value, no estimation or calculation error will be included.

      - Reference timing for NTA,UE-specific and NTA,common is the slot when UL transmission is supposed to arrive at the target satellite based on true satellite position.

- Recommended WF

  - Companies can continue provide comments based on option 1 and option 2.

  - According to companies’ comments after 1st round, it may be converged for common TA (NTA,common) in this meeting, companies can provide comments based on option 1A and option 2A.

    - How to configure the value of common TA (NTA,common) in test cases?

      - Option 1A

        - NTA,common is derived according to N_{TA, common} related parameters broadcasted within a validity duration.

      - Option 2A

        - NTA,common is configured as a fixed value without estimation or calculation error.

        - Reference timing for NTA,common is the slot when UL transmission is supposed to arrive at the target satellite based on true satellite position.

| Company | Comments |
| --- | --- |
| Xiaomi | For common TA, we are fine with option 2A |
| Qualcomm | We do not support Option 2A. - What does “fixed value” exactly mean? The feeder link can be time-varying. - For NTA,common, the parameter ‘t’ in Delay_common(t) shall be set to time instances when signals from UE and a reference point are supposed to arrive at a target satellite. A round trip propagation delay over feeder link is not two times one-way propagation delay, which is different from TN systems. Add Option 3A: - For NTA,common, there are two reference timing instances as below: - The time instance when UL transmission from UE is supposed to arrive at the target satellite based on true satellite position. - The time instance when DL transmission from a synchronization reference point (SRP) is supposed to arrive at the target satellite based on true satellite position. - The slot index of the DL transmission and the UL transmission above shall be the same. - Validity duration shall be further discussed and limited to a reasonable value so that satellite position projection error based on the broadcasted information shall not increase too much. - Note that satellite position estimation error that was accounted for in the Te_NTN requirement is only for service link. Thus, there can be a potential issue that the satellite position estimation error can also propagate to feeder link delay estimation error when UE replaces ‘t’ in Delaycommon(t) with the inaccurately estimated time instances due to the satellite position estimation error. For the propagation delays over service link, we can further discuss after agreeing to feeder link delay modeling. |
| Apple | We are fine with option 2A for simplicity. Common TA estimation is irrelevant with propagator model and that means the common TA estimation shall be aligned among UEs (no any implementation specific error inside) and shall be deterministic. Therefore, using fixed value for common TA, to some extends, could be equivalent to the case of common TA updating, as long as the common TA related information from network is valid |
| CMCC | After 1st round discussion, generally, we agree with OPPO’s observation that the NTA,common configured by NW may contains some error from NW side, and such the error should not be considered as UE timing requirements. If Option 1A is used, it will bring another issue that what is the reference timing for the parameter ‘t’ in Delay_common(t). Maybe the slot when UL transmission is supposed to arrive at the target satellite based on true satellite position can be used, while as comment by QC, it is not the perfect value. Regarding Option 2A, Although the error in NTA,common from NW side may be included in Te_NTN, if this error is so small that it can be ignored, we think Option 2A can also work. |
| OPPO | We are not clear about the meaning of “a fixed value without estimation or calculation error” in option 2. Which part of estimation or calculation error should be precluded here. The value of ideal common TA should be clearly specified in the test cases. If the “ideal common TA” is calculated based on the related parameters configured by NW and ideal model without any calculation error at UE side, then we can support option 2A. If the “ideal common TA” is propagation delay based on satellite position in practice, it may be too complicate to emulate the movement of satellite. |
| Ericsson | Option 1A, NTA,common is derived according to N_{TA, common} related parameters broadcasted within a validity duration. The reason for stating this is that if the reference point for timing is placed at gNB then the feeder link delay is time varying. It must be verified that a UE can handle this. |
| Huawei | How to configure common TA depends the target scenario. The configuration of common TA will be different between GEO (or GSO ) scenario and LEO(or NGSO ) scenario |
| CATT | For NTA, common, support option 1A. |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
| WF on UE timing requirements and NTN RRM performance requirements | Xiaomi |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2208101 | DraftCR on UE timer accuracy for NR_NTN | Xiaomi | Agreeable |  |
| R4-2208361 | Draft CR to the timing requirements for NR NTN | OPPO | Merged | To be merged in R4-2208471 |
| R4-2208471 | Introduction of Timing advance requirement for NTN | MediaTek | Revised | To capture the comments in 1st round |
| R4-2208996 | DraftCR on UE transmit timing requirements for NTN R17 | Huawei | Revised | To capture the comments in 1st round |
| R4-2208360 | Discussion on timing requirements for NR NTN | OPPO | Noted |  |
| R4-2208653 | Timing requirements | Ericsson | Noted |  |
| R4-2208995 | Discussion on remaining issues NTN timing related requirements | Huawei, Hisilicon | Noted |  |
| R4-2209639 | On GNSS-Related requirements for UE operation | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2209640 | On NTN timing requirements | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2207960 | Performance requirements | Qualcomm Incorporated | Noted |  |
| R4-2208103 | Discussion on the performance requirements for NTN UE timing | Xiaomi | Noted |  |
| R4-2208423 | Discussion on NTN timing test cases | CMCC | Noted |  |
| R4-2209218 | Discussion on measurement accuracy and TCs for NTN | Huawei, Hisilicon | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |
| R4-2210612 | WF on UE timing requirements and NTN RRM performance requirements | Xiaomi | Agreeable |  |
| R4-2211100 | Introduction of Timing advance requirement for NTN | MediaTek | Endorsed |  |
| R4-2211101 | DraftCR on UE transmit timing requirements for NTN R17 | Huawei | Endorsed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm | CH Park | chparkqc@qti.qualcomm.com |
| Apple | Jie Cui | Jie_cui@apple.com |
| MediaTek | Hsuanli Lin | Hsuanli.Lin@mediatek |
| Nokia, Nokia Shanghai Bell | Anthony Lo | Anthony.Lo@nokia.com |
| Ericsson | Magnus Larsson | Magnus.k.larsson@bahnhof.se |
| LGE | Jin-Yup Hwang | jinyup.hwang@lge.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)