3GPP TSG-RAN WG4 Meeting # 103-e												R4-2210578

Electronic Meeting, May 09 – 20, 2022

Agenda item:			9.15.3

Source:	Moderator (Qualcomm)

Title:	WF on 60 GHz UE Requirements

Document for:	Information

# Introduction

WF on issues discussed in R4-2210267

# Transmit

## Output RF spectrum emissions

### Spurious coexistence

Issue 1: Coex levels FR2-2 aggressor, FR2-1 victim

In FR2-1 spec n257, n261 and n262 are always protected at -5 dBm / 100 MHz, whereas n260 is protected at -2 dBm / 100 MHz. n258 and n259 are not listed as protected band.

Proposal 1: As starting point for discussion we suggest using these same values and also applying -5 dBm / 100 MHz towards n258 and n259.

Agreement: Use these same values and also applying -5 dBm / 100 MHz towards n258 and n259.

Issue 2: Coex levels FR2-1 aggressor, FR2-2 victim

Proposal 1: We also suggest as starting point for discussion that FR2-1 protect n263 at -5 dBm / 100 MHz.

Agreement: Use TBD values in the table.

## Transmit signal quality

### EVM

Agreement:

- EVM requirement:

  - Pi/2 BPSK: 30%

  - QPSK: 17.5%

- Scale the minimum EVM power level from 400 MHz for noise BW

  - FFS on the values.

Agreement: 16QAM and 64QAM EVM %age requirement the same as in FR2-1

- Configure PTRS for 16QAM and 64QAM EVM testing.

Issue 1: Phase noise mask assumption for PTRS

It would be helpful if we can agree on PN mask for PTRS study. Unfortunately we did not get to this in the GTW and I am hoping we can get some more comments here.

Proposal 1: Use PN mask from R4-2010176 Figure 2.2.4-2 (“On phase noise for 52.6-71 GHz”, Ericsson). Use the red curve which includes 5 dB design margin.

Recommended WF: Agree proposal 1

Issue 2: Scaling min EVM power level for BW > 400 MHz

Proposal 1: Scale minimum EVM power level based on BW for > 400 MHz

Agreement: agree proposal 1.

### Carrier leakage, image, and inband emissions

Agreement from GTW is: ‘PC1 further discuss output power levels for the -25 range and the -20 range’

Issue 1: Company comments on PC1 power levels for the -20/-25 ranges

Recommended WF: Experts have not provided any proposals.

## UE Maximum output power

### Power class upper limit and regions

Issue 1: Capturing EIRP and TRP values

Question for clarification, will any max EIRP and max TRP values be captured in TS 38.101-2 for FR2-2 power classes?

Recommended WF: TBD

### MPR

Issue 1: MPR methods and specs

- Proposals related to method

  - Proposal 1: Use MPR delta due to excess BW method as described in LGE (8647 proposal 3)

- Proposals PC3 MPR for 100MHz and 400 MHz

  - Proposal  2: Re-use FR2-1 PC3 MPR for FR2-2

- Proposals PC3 MPR for 800, 1600, 2000 MHz

  - Proposal  3: Consider 3dB(Y1), 4dB(Y2) and 4dB(Y3) as MPR delta (in Table 2.3) respectively. For Edge RB allocations, in case of Pi/2 BPSK and QPSK in DFT-s-OFDM, consider 4dB, 5dB and 5dB respectively

- Proposals PC2

  - Proposal 4: Apply FR2-2 PC3 MPR to PC2

Agreement: Further check the following bullets in this meeting

- For PC3 MPR for 100MHz and 400 MHz

  - Re-use FR2-1 PC3 MPR for FR2-2

- For PC3 MPR for 800, 1600, 2000 MHz

  - Use MPR delta due to excess BW method as described in LGE (8647 proposal 3)

    - FFS on the concrete values

- For PC2

  - Apply PC3 MPR to PC2

Issue 2: Image level assumption for EVM calculation

Background: EVM is one of the factors that determines MPR, for some waveforms. It is important to make an assumption about the image level because image sometimes lands on allocated RBs. From discussion and my understanding the options are

Option 1: Simulate EVM compliance for MPR without image

Option 2: Simulate EVM compliance with -35 dBc for 64QAM and -28 dBc for 16-QAM and lower.

Agreement: Simulate EVM compliance with -35 dBc for 64QAM and -28 dBc for 16-QAM and lower

- The numbers can be revisited in the next meeting.

### Configured transmitted power

Issue 1: PUmax tolerance

Proposal 1: The Pumax tolerance of band n263 could reuse existing requirements for FR2-1 operating band (Table 6.2.4-1).

Agreement: Use existing requirements for FR2-1 operating band (Table 6.2.4-1) as Pumax tolerance of band n263 and put the numbers in [ ] in the CR.

## Output power dynamics

### Minimum output power

Issue 1: Measurement bandwidth for minimum output power

Proposal 1: The measurement bandwidth is taken from the SU topic decision. Do any companies object?

Agreement: Align the numbers of measurement bandwidth with SU numbers

- MBW= SCS*(12*NRB+1)/1000

Issue 2: Proposals for minimum output power levels for PC1, PC2, PC3

Recommended WF: Experts have not been able to make any technical proposals

### TX off power

Issue 1: TX OFF power table

Proposal 1: Based on comments in the thread, agree this table. Any objections to proposal 1?

| Operating band | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257, n258, n259, n260, n261, n262 | -35 | -35 | -35 | -35 | N/A | N/A | N/A |
|  | 47.58MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz | N/A | N/A | N/A |
| n263 | N/A | -35 | N/A | -35 | -35 | -35 | -35 |
|  | N/A | TBD from SU | N/A | TBD from SU | TBD from SU | TBD from SU | TBD from SU |

Agreement: Align the numbers of measurement bandwidth with SU numbers

- MBW= SCS*(12*NRB+1)/1000

### Power control and tolerances

Issue 1: Tolerances

Proposal 1 : Use same as FR2-1 for absolute, relative, and aggregate power tolerances agreeing that more exceptions may be allowed for FR2-2

Recommended WF: TBD

### PRACH time mask

- Proposals

  - Option 1: Only specify the PRACH ON power measurement period requirements for 120kHz SCS.

  - Option 2: PRACH ON power measurement period table should be updated for 480 and 960 SCS as in (0166)

Issue 1: PRACH ON power measurement period

Option 1: Only specify the PRACH ON power measurement period requirements for 120kHz SCS.

Option 2: PRACH ON power measurement period table should be updated for 480 and 960 SCS as in (0166)

Recommended WF: TBD

### General ON/OFF time mask for n263

Issue 1: Which mask

Option 1: For unlicensed band n263, the transient period locates in both ON slot and OFF slots.

Option 2:  Define General ON/OFF time mask as in FR2-1

Recommended WF: TBD

Option 1

Option 2

## Features

### Multi-band relaxation

Issue 1: Multi-band relaxation factor handling

Option 1: UE multi-band relaxation factors (left as TBD in the draft CR) in Table 6.2.1.3-4

Option 2: For band n263 the multi-band relaxation is 1.0dB for both MBP,n (dB) and MBS,n (dB).

Agreement:

- MBR to be included in the requirements so this will be in draftCR

- Value of MBR is [1] dB

### CA

Issue 1: CA in rel 17

Option 1: Regarding CA within FR2-2 and UL MIMO, we have not found agreements on the related requirements.  Thus, in the interest of making progress, we recommend focusing the Rel-17 requirements for band n263 to cover just single carrier operation without UL MIMO. Include Apple notes on CA and UL MIMO from R4-2207696

Recommended WF: TBD

### UL MIMO

Issue 1: UL-MIMO in rel 17

Option 1: No Uplink MIMO in rel17.

Option 2: Uplink MIMO in rel17

Recommended WF: Option 1

### Beam correspondence

Issue 1: n263 beam correspondence

Proposal 1: All n263 UEs shall support beamCorrespondenceWithoutUL-BeamSweeping based on the round 1 comments.

Recommended WF: Perhaps we can agree on proposal 1 by majority.

Issue 2: n263 beam correspondence procedure

Proposal 1: If beam correspondence requirements for FR2-2 PC3 are introduced, then reuse the FR2-1 PC3 beam correspondence procedure for FR2-2 PC3. For beam correspondence tolerance, further discussion may be necessary.

Recommended WF: If I understand this correctly, this proposal would be handled in testability WI. Perhaps I am not understanding the proposal. If this is testability we don’t need to discuss in this thread.

## Time-related

### ON/ON transient periods for 480 and 960 SCS

Issue 1: Transient period

Option 1: Introduce 2 µS improved ON/ON transient period as optional UE capabilities for 480 and 960 kHz SCS.

Option 2: Same as FR2-1

Option 3: For optional ON-ON transient time, only one value among 1 us or 2 us is specified.

Recommended WF: TBD

### Beam direction switching time

Issue 1: Beam direction Switching time

Option 1: 200 nsec

Option 2: Use a UE beam direction switching time of 59 ns.

Option 3: 100 nsec (Ericsson proposed compromise).

Recommended WF: TBD

## FFT sizes (R4-2208226)

Discuss this topic in [311]. No further discussion in [132].

# Receive

## ACS

Issue 1: ACS

Proposal 1:Agree 100 and 400 MHz values in the table as they are common to both round 1 proposals. Do any companies object?

Proposal 2: Agree 800, 1600, 2000 MHz as the same 100, and 400 MHz. Do any companies object?

clarification for vivo on proposal 2: ‘the same’ means ACS is 21 dB for > 400 MHz CCBW.

| Company | Comment |
| --- | --- |
| Nokia | OK with the proposal |
| QCOM | OK with proposals 1 and 2 |
| Apple | We are ok with Proposal 1 only and further discuss ACS value for larger CBW > 400 MHz. |

Recommended WF: Agree proposal 1 and discuss CBW > 400 in next meeting.

|  | Case 1 |  | Case 2 |  |
| --- | --- | --- | --- | --- |
| ACS dB | Pwr | Pint | Pwr | Pint |
| 21 | REFSENS + 14 | REFSENS + 33.5 | -44.5 dBm | -25 dBm |

## IBB

Issue 1: IBB

Proposal 1: The IBB of band n263 is specified for all supported bandwidths.

Proposal 2: The IBB requirement of band n263 is the same as ACS requirement.

Proposal 3: Single CC Similar to FR2-1 with changed interference level (9508)

Proposal 4: intra contig DL CA similar to FR2-1 with interference level changed (9508)

Proposal 5: Define In-band Blocking requirement for band n263 as provided in Table 5. (R4-2209323)

| Company | Comment |
| --- | --- |
| Nokia | Nokia: We are ok with proposal 1, 2, 3 and 4. Proposal 5 has some issues, e.g. channel bandwidths include 200 MHz which is not defined for FR2-2 and requirements do not extend to wider than 400 MHz channel bandwidths. |
| Apple | Apple: Proposal 5, considering for further study CBW > 400 MHz |

Recommended WF: TBD

## RX maximum input level

Issue 1: Max input level specs

Proposal 1: For single CC same as FR2-1

Proposal 2: For intra contig DL CA same as FR2-1

| Company | Comment |
| --- | --- |
| Nokia | Support both proposals |
| QCOM | On proposal 1 we could accept the proposal for bandwidths <= 400 MHz. Wider bandwidths need study On proposal 2 CA is not decided so this hinges on that decision, as well as the effect of the CA BW being up to 2000 MHz. |
| Apple | Proposal 1 only |

Agreement: For single CC same as FR2-1

## RX spurious emissions

Issue 1: RX spurious emissions levels

Option 1:  -57 dBm/100 KHz; -47 dBm/1MHz

Option 2: -36 dBm/100kHz; -30 dBm MHz

| Company | Comment |
| --- | --- |
| Ericsson | Option 2 |
| QCOM | Option 2 |

Agreement: -36 dBm/100kHz; -30 dBm MHz

# Spec structure and suffix F for n263

Issue 1: Spec structure

Proposal 1:  Per the majority do not use suffix F for n263 per majority of comments in round 1. Do any companies object?

| Company | Comment |
| --- | --- |
| Intel | If there is not going to be a distinction between power classes in FR2 for sub-ranges FR2-1 and FR2-2, then we do not think it is necessary to have a new sub-clause with suffix F. This is unless we want to have a separate clause for unlicensed operation. |
| Huawei | To Intel: Yes the separate clause is for unlicensed operation. FR2-2 licensed bands would be kept in clauses without suffix F. |

Recommended WF: Proposal 1 per round 1 majority