3GPP TSG-RAN WG4 Meeting #103-e							        		       		R4-2210609

Electronic Meeting, May 09th – May 20th, 2022

Agenda item:			9.8.3

Source:	Samsung

Title:	WF on RRM performance requirement for FR2 HST

Document for:	Approval

# 1 Background

In RAN Plenary #89-e, the RAN4-led work item of NR support for high speed train scenario in FR2 has been approved [RP-210800].

Accordingly, the performance part of RRM requirement shall begin from RAN4#103-e, for the following objectives on RRM performance requirements captured in WID:

•	Specify the RRM performance requirements of measurement accuracy if identified.

•	Specify the RRM test cases related to new core requirements.

It is the first meeting to discuss RRM performance requirement for this work item.

# 2 Way Forward

### 2.1 Sub-Topic 1: Scope of RRM Performance Requirements

Issue 1-1: Necessity of Test Cases for Cell Re-selection Requirement

Way Forward:

- TC for Cell Re-selection Requirement:

  - New test case for A.7.1.1.X, Cell reselection to FR2 intra-frequency NR case for PC6 UE configured with highSpeedMeasFlagFR2-r17, DRX cycle=0.32s:

    - Test case definition and applicability:

      - Option-1: 1 test case defined from either unidirectional or bi-directional:

        - FFS unidirectional or bi-directional:

          - Consider bi-directional as starting point

      - Option-2: 2 test cases defined for unidirectional and bi-directional respectively:

        - Applicability rule is introduced: UE needs to be tested with one of the tests in either unidirectional or bidirectional.

    - SMTC period = 20 ms,

    - Propagation condition: AWGN (serving cell) and AWGN with [9722] Hz frequency offset (neighbour cell) for unidirectional or [19444] Hz frequency offset (neighbour cell) for bi-directional case.

Issue 1-2: Necessity of Test Cases for Re-establishment Delay Requirement

Way Forward:

- TC for Re-establishment Delay Requirement:

  - FFS new test case for A.7.3.2.1.X, Intra-frequency RRC Re-establishment in FR2 for PC6 UE configured with highSpeedMeasFlagFR2-r17.

    - Test case definition and applicability:

      - Option-1: 1 test case defined from either unidirectional or bi-directional:

        - FFS unidirectional or bi-directional:

          - Consider bi-directional as starting point

      - Option-2: 2 test cases defined for unidirectional and bi-directional respectively:

        - Applicability rule is introduced: UE needs to be tested with one of the tests in either unidirectional or bidirectional.

    - SMTC period = 20 ms,

    - Propagation condition: AWGN (serving cell) and AWGN with [9722] Hz frequency offset (neighbour cell) for unidirectional or [19444] Hz frequency offset (neighbour cell) for bi-directional case.

  - FFS the necessity of introducing TC for re-establishment delay requirement:

    - To be decided in next RAN4 meeting.

Issue 1-3: Necessity of Test Cases for Timing Related Requirement

Way Forward:

- TC for Timing Related Requirement:

  - TC for gradual timing adjustment requirement for FR2 PC6 UE:

    - Option 1: Need new test case (similar as A.7.4.1.1)

    - Option 2: Applying PC6 UE new requirement into A.7.4.1.1, and no need to define new test case.

    - Option 3: No new test case needed

  - TC for one shot large UL timing adjustment for FR2 PC6 UE

    - Define test case to verify one shot large UL timing adjustment requirement.

      - Option 1: New test defined in clause A.7.4.1

      - Option-2: one shot large UL timing adjustment test is combined into MAC-CE based TCI switch delay test. (The test shall verify the one shot large UL timing adjustment requirement and MAC-CE based TCI switch delay requirement.)

Issue 1-4: Necessity of Test Cases for SSB-based RLM and BFD

Way Forward:

- TC for SSB-based RLM and BFD:

  - No new SSB-based RLM/BFD test cases dedicated for FR2 PC6.

Issue 1-5: Necessity of Test Cases for MAC-CE based TCI State Switch Delay Requirement

Way Forward:

- TC for MAC-CE based TCI State Switch Delay Requirement:

  - New test case is needed for MAC CE based TCI state switching delay for FR2 PC6: A.7.5.8.X, MAC-CE based active TCI state switch for FR2 PC6 UE

    - Test case definition and applicability:

      - Option-1: 1 test case defined from either unidirectional or bi-directional:

        - FFS unidirectional or bi-directional:

          - Consider bi-directional as starting point

      - Option-2: 2 test cases defined for unidirectional and bi-directional respectively:

        - Applicability rule is introduced: UE needs to be tested with one of the tests in either unidirectional or bidirectional.

    - FFS DRX cycle = 40 ms or Non-DRX configured,

    - SMTC period = 20 ms,

    - Propagation condition: AWGN (serving cell) and AWGN with [9722] Hz frequency offset (neighbour cell) for unidirectional or [19444] Hz frequency offset (neighbour cell) for bi-directional case.

Issue 1-6: Necessity of Test Cases for L3 Measurement Requirement

Way Forward:

- TC for L3 Measurement Requirement:

  - New test case is needed for A.7.6.1.X, SA event triggered reporting test without gap for PC6 UE

    - SMTC period = 20 ms,

    - FFS DRX cycle = 40 ms or Non-DRX configured,

    - Propagation condition: AWGN (serving cell) and AWGN with 19444 Hz frequency offset (neighbour cell)

Issue 1-7: Necessity of Test Cases for L1-RSRP/SINR Measurement Requirement

Way Forward:

- TC for L1 Measurement Requirement:

  - New test case for A.7.6.3.2, SSB based L1-RSRP measurement for PC6 UE.

    - FFS DRX cycle = 40 ms or Non-DRX configured,

    - SMTC period = 20 ms,

    - Propagation condition: AWGN with 19444 Hz frequency offset

  - FFS new test case for A.7.6.6.2, SSB based L1-SINR measurement for PC6 UE.

    - FFS DRX cycle = 40 ms or Non-DRX configured,

    - SMTC period = 20 ms,

    - Propagation condition: AWGN with 19444 Hz frequency offset

  - FFS the necessity of introducing TC for SSB based L1-SINR measurement:

    - To be decided in next RAN4 meeting.

### 2.2 Sub-Topic 2: RRM Test Configuration and Applicability

Issue 2-1: Applicability of Set-1 and Set-2 of enhanced RRM requirements

Way Forward:

- Applicability of Set-1 and Set-2 of enhanced RRM requirements:

| TC category (if confirmed in Sub-Topic 1) | Applicability of set1 and set2 (either set1 with 2RX beams or set2 with 6RX beams, or both) |
| --- | --- |
| TC for Cell Re-selection Requirement | TBD |
| TC for Re-establishment Delay Requirement | TBD |
| TC for Timing Related Requirement | N/A (not differentiated by set1 and set2) |
| TC for MAC-CE based TCI State Switch Delay Requirement | TBD (for target TCI is unknown, SSB-based TCI switching to a RS with new QCL-TypeD reference) |
| TC for L3 measurement | TBD |
| TC for L1-RSRP/SINR Measurement Requirement | TBD |

Issue 2-2: Test configuration

Way Forward:

- Channel model for RRM test cases:

  - if AWGN is assumed, uni-directional should result in [9722] Hz frequency offset while bi-directional means [19444] Hz.

    - If Test setup is 1AoA based, the only difference between uni- and bi-directional should be the value of Doppler frequency offset.

### 2.3 Sub-Topic 3: Work Splitting for RRM Test Case CR Drafting

Way Forward:

- Work splitting is planned to be discussed based on:

| Test Case | Responsible company |
| --- | --- |
| TC for Cell Re-selection Requirement | TBA |
| TC for Re-establishment Delay Requirement (if introduced) | TBA |
| TC for gradual timing adjustment requirement for FR2 PC6 UE (if introduced) | TBA |
| TC for one shot large UL timing adjustment for FR2 PC6 UE (if introduced) | TBA |
| TC for MAC-CE based TCI State Switch Delay Requirement | TBA |
| TC for L3 measurement | TBA |
| TC for L1-RSRP Measurement Requirement | TBA |
| TC for L1-SINR Measurement Requirement (if introduced) | TBA |

### 2.4 Sub-Topic 4: Measurement Accuracy Requirement for FR2 HST UE

Issue 4-1: Measurement accuracy for FR2 HST UE

Way Forward:

- Measurement accuracy for L1 and L3 measurement:

  - FFS

  - Companies are encouraged to provide simulation results in next meeting.

Issue 4-3: Simulation assumption further alignment for L3 measurement

Way Forward:

- Confirm the simulation assumptions provided in R4-2209525, except:

  - AWGN with 19444 Hz offset between desired and interfering cells, for bi-directional case

  - use SINR = -6dB only, and no need to consider “FFS: -8dB, -7dB”.

Issue 4-4: Simulation assumption further alignment for L1 measurement

Way Forward:

- Confirm the simulation assumptions provided in R4-2209525, except:

  - AWGN with 19444 Hz offset for bi-directional case

# References

[1] R4-2210491, “Email discussion summary for [103-e][222] NR_HST_FR2_RRM_2”, Samsung.