3GPP TSG-RAN WG4 Meeting #103-e	R4-2210613

# Electronic Meeting, May 09 – May 20, 2022

Agenda item:			9.13

Source:	Moderator (MediaTek inc.)

Title:	WF on RLM/BFD relaxation for UE Power Saving enhancements

Document for:	Information

# Introduction

This document is to capture the all agreements in RAN4#103-e in email thread #225 on RLM/BFD relaxation for UE Power Saving enhancements.

Agreement in the 1st round

Agreements in the GTW

Agreement in the 2nd round

# WF on the Core Part

#### Issue 1-1: Clarification on multiple RLM-RS/BFD-RS

- For entering condition,

  - Option 1: good serving cell quality criterion is fulfilled when the radio link quality is better than the threshold (Qin + X dB) for at least one resource in the set of resources for RLM/BFD.

- For exiting condition,

  - Option 1: good serving cell quality criterion is not fulfilled when the radio link quality is worse than the threshold (Qin + X dB) for all resource in the set of resources for RLM/BFD.

Agreement: Option 1

- X with candidate values {2,4,6, 8} dB can be configured by NW with default value as 0 dB.

#### Issue 1-1A: Clarification on intra-band CA configured with SSB RLM and CSI-RS BFD (new)

Agreement: The agreement for the case of CSI-RS based RLM and CSI-RS based BFD will be applied for the case SSB based RLM and CSI-RS based BFD under intra-band CA configuration.

#### Issue 1-4: Evaluation period for UE fulfilling relaxed measurement criteria – Low mobility criterion

- Low mobility criterion should be viewed as fulfilled if low mobility condition specified in 5.7.13.1 of TS 38.331 has fulfilled for a period of TSearchDeltaP, connected

#### Issue 1-8: Clarifications for Low mobility criteria evaluation

For NR-DC, the following proposals can be discussed in R4#104-e:

  - Proposal 1-1: For NR-DC, UE may also evaluate low mobility criteria in PSCell if network configures, and apply the evaluated low mobility state in the corresponding cell group.

  - Proposal 1-2:

    - In NR-DC, the UE configured with low mobility criteria evaluates the low mobility criterion only on SpCell.

    - In NR-DC, if the relaxed measurement criterion (low mobility criteria) is met on a SpCell then the UE assumes that the relaxation criterion is met on all serving cells in the CG of that SpCell

  - Proposal 1-3: keep the previous agreement that evaluate low mobility criteria on PCell.

#### Issue 1-9: Introduce minimum requirement at transitions

FFS: Define the minimum requirement at transitions between relaxed and non-relaxed RLM/BFD measurements for OOS evaluation.

# WF on the Performance Part

#### Issue 2-1: Measurement accuracy requirements

- No additional accuracy requirement.

#### __Issue 2-2-1: Test case for exiting criteria

- Proposal 1 (revised): Define at least some of OOS TCs (for example RLM) to validate correct UE behavior when the UE needs to exit from the relaxation mode.

- FFS configure N310 = 2 to validate

#### Issue 2-2-2: Whether to define RLM INS tests

  - No need to introduce relaxed RLM test for verifying the in-sync evaluation requirements.

#### Issue 2-2-3: Whether to define RLM OOS tests

- Out-of-sync testing for RLM relaxation should be defined

#### __Issue 2-2-4: DRX period setting

- DRX periods of 40ms and 80ms are distributed in respective test cases, e,g. different values for FR1/FR2, EN-DC/SA, SSB/CSI-RS.

- The following proposals for the values of DRX cycle length will be discussed in R4#104-e:

  - Proposal 1: 40ms for TC1, 80ms for TC4, 40ms for TC5, 40ms for TC13, 40ms for TC16

  - Other proposals are not precluded.

#### Issue 2-2-5: Principle for test case reduction

Agreement: RAN4 agreed to introduce below test cases:

  - RLM SSB in FR1 EN-DC (TC1)

  - BFD CSI-RS in FR2 NR-SA (TC16)

  - BFD SSB in FR2 EN-DC (TC13)

  - RLM SSB in FR2 EN-DC (TC5)

  - RLM CSI-RS in FR1 NR-SA (TC4)

#### Issue 2-2-6: Test case list

- For RLM

| Case |  | Test case | Section | Volunteer | TC# |
| --- | --- | --- | --- | --- | --- |
| FR1 SSB | EN-DC | Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX=TBDms | A.4.5.1.X | MTK | TC1 |
| FR1 CSI-RS | NR-SA | Radio Link Monitoring Out-of-sync Test for FR1 Pcell configured with CSI-RS-based RLM in DRX= TBDms | A.6.5.1.X | CMCC | TC4 |
| FR2 SSB | EN-DC | Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX= TBDms | A.5.5.1.X | CATT | TC5 |

- For BFD

| Case |  | Test case |  | Volunteer | TC# |
| --- | --- | --- | --- | --- | --- |
| FR2 SSB | EN-DC | Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX= TBDms | A.5.5.5.X | ZTE | TC13 |
| FR2 CSI-RS | NR-SA | Beam Failure Detection and Link Recovery Test for FR2 Pcell configured with CSI-RS-based BFD and LR in DRX= TBDms | A.7.5.5.X | Nokia | TC16 |

#### __Issue 2-3-1: Test set up on relaxation criterion

- Option 1: only good serving cell quality criterion is configured

- Option 2: both low mobility criterion and good serving cell quality criterion are configured

- Option 4 (new): Test cases need to be for both combinations.

#### Issue 2-3-2: Test set up on good serving cell criterion

- For good serving cell criterion test case setup.

  - For RLM, offset = 0dB.

  - For BFD,

    - Option 1: offset = 0 dB

    - Option 2: offset = 4 dB

#### Issue 2-4-1: General test setting

- RAN4 to reuse the test configurations from the legacy tests as reference and only do the necessary modifications related to the relaxation.

#### __Issue 2-4-2: RLM OOS test setting – D1

FFS

- D1= TEvaluate_out_Relax  + p*TIndication_interval + 40 ms (the legacy margin)

  - where TEvaluate_out_Relax is  TEvaluate_out_SSB_Relax or TEvaluate_out_CSI-RS_Relax

  - FFS the value of p

    - Option 1: p = 1 (N310=2)

    - Option 2: p = 0

    - Other values of p are not precluded.

#### Issue 2-4-3: RLM OOS test setting – T1

- T1 is extended by 5s

#### __Issue 2-4-4: RLM OOS test – SNR setting

- Reuse the SNR variation setting configurations in legacy RLM out-of-sync tests for RLM measurement relaxation test cases.

- For SNR values,

| Test |  | SNR value of RLM-RS#1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | SNR1 | SNR2 | SNR3 |
| Relax RLM testing | FR1 | 1dB | Y11 dB | -15dB |
|  | FR2 | 2dB | Y12 dB | -15dB |

- where Y11/Y12 dB (SNR2) are FFS

  - Option 1: -4.5 dB

  - Option 2: set SNR2 as SNR1

  - Option 3: reuse the legacy values. (i.e., -7dB in FR1, -6dB in FR2)

- whether to configure RLM-RS#2 is FFS

#### __Issue 2-4-5A: BFD test setting – whether to include link recovery

- Proposal 1: No need to include link recovery. For BFD relaxation testcase design, only need to consider 3 time during, i.e. T1, T2, T3 where T3 is extended.

- Proposal 2: to include link recovery.

#### __Issue 2-4-5B: BFD test setting –T3

FFS

- T3= Evaluate_BFD _Relax  + p* TIndication_interval_BFD + 40 ms (the legacy margin)

  - where Evaluate_BFD _Relax is  Evaluate_BFD_SSB_Relax or Evaluate_BFD_CSI-RS_Relax

  - FFS the value of p

    - Option 1: p = 0 (beamFailureInstanceMaxCount = n1)

    - Other values of p are not precluded.

#### __Issue 2-4-6: BFD test – SNR setting

- For SNR values,

| Test |  | SNR value of BFD-RS#1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | SNR1 | SNR2 | SNR3 |
| Relax BFD testing | FR1 | 5dB | Y21 dB | -12dB |
|  | FR2 | 6dB | Y22 dB | -12dB |

- where Y21/Y22 dB (SNR2) are FFS

  - Option 1: -0.5 dB

  - Option 2: set SNR2 as SNR1

  - Option 3: reuse the legacy values. (i.e., -3dB in FR1, -2dB in FR2)

- whether to configure BFD-RS#2 is FFS