3GPP TSG-RAN WG4 Meeting #103-e 														R4-2210655

Electronic Meeting, May 09– May 20 2022

Agenda item:			9.8.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	WF on BS demodulation requirement for FR2 HST

Document for:	Approval

# Background

- The following WFs were approved previously

  - R4-2106102, WF for FR2 HST Demodulation, RAN4#98-bis, Samsung

  - R4-2017828, WF on NR support for HST in FR2”, Samsung. RAN4#97-e meeting

  - R4-2103240, WF on Deployment Scenario and UE RF Requirement for FR2 HST”, Samsung. RAN4#98-e meeting

  - R4-2106100, WF on FR2 HST Deployment scenario Analysis, Samsung, RAN4#98b-e meeting

  - R4-2106101, WF on Channel Modelling for FR2 HST, Nokia, RAN4#98b-e meeting

  - R4-2106102, WF on Demodulation requirement for FR2 HST”, Samsung, RAN4#98b-e meeting

  - R4-2108860, WF on FR2 HST Deployment Scenario Analysis, Samsung, RAN4#99-e meeting

  - R4-2108637, WF for FR2 HST Demodulation, RAN4#99, Samsung

  - R4-2115726, WF on FR2 HST demodulation, RAN4#100, Samsung

  - R4-2120703, WF on BS demodulation requirement for FR2 HST, RAN4#101, Nokia, Nokia Shanghai Bell, Samsung.

  - R4-2203006, WF on BS demodulation requirement for FR2 HST, RAN4#101b, Nokia, Nokia Shanghai Bell,

  - R4-2207235, WF on BS demodulation requirement for FR2 HST, RAN4#102, Nokia, Nokia Shanghai Bell

- Corresponding Email summary in RAN4#103-e

  - R4-2210523	Email discussion summary for [103-e][320] NR_HST_FR2_Demod_Part2.

# WF on topic#2: PUSCH

## Requirements selection

Issue 2-1-1: Requirement selection procedure

Agreements:

Ideal threshold = 2.5dB. Impaired threshold = 4dB.

## Manufacturer’s declarations and applicability rules

Issue 2-2-1: Clarify Rel-16 manufacturer’s declarations and applicability rules to explicitly apply to FR1

Agreements:

Yes, for applicability rules.

Existing manufacture declaration D.109 can be reused for FR2 HST.
Modify this declaration to also be applicable to BS type 2-O, i.e., n/a is changed to x for BS type 2-O for D.109.

Issue 2-2-2: Implement Issue 2-2-1 in NR_HST or NR_HST_FR2

Agreements:

Use NR_HST_FR2 to implement any required FR1/FR2 distinctions starting from Rel-17.

## Other

Issue 2-3-1: Terminology to use for FR1 and FR2 distinction

Agreements:

Use either BS type 1-O or type 2-O to distinguish FR1 and FR2 in applicability rules and requirements.

# WF on topic#3: PUSCH with UL timing adjustment requirements

## SRS configuration

Issue 3-1-2: SRS Transmission periodicity

Agreements

The intention of the prior agreements was to have SRS sent once per radio frame, in the last symbol of the first special slot of the radio frame.
The slot-based value of TSRS in NOTE1 of the “Test parameters for testing UL timing adjustment” table shall be scaled accordingly with the SCS, i.e., 15 kHz: TSRS =10, 30 kHz: TSRS =20, 60 kHz: TSRS =40, 120 kHz: TSRS =80.

It is recommended to also add this correction in Rel-16 maintenance.

## Other

Issue 3-2-1: Clarification of UL TA applicability rule

Agreements:

“Applicability of UL timing adjustment requirements for different scenarios” (TS 38.141-2, 8.1.2.1.6) shall be clarified to take HST FR2 (i.e., for BS type 2-O with HST FR2 declared) into account.

# CR split (informative)

| Section number | Section title | Responsible company |
| --- | --- | --- |
| TS 38.104 |  |  |
|  | Big CR | Samsung |
| 11 | Radiated performance requirements |  |
| 11.2 | Performance requirements for PUSCH |  |
| 11.2.2 | Requirements for BS type 2-O |  |
| 11.2.2.x | Requirements for PUSCH for high speed train | Intel |
| 11.2.2.y | Requirements for UL timing adjustment | CATT |
| 11.4 | Performance requirements for PRACH |  |
| 11.4.2 | Requirements for BS type 2-O |  |
| 11.4.2.2 | PRACH detection requirements |  |
| 11.4.2.2.x | Minimum requirements for high speed train | Huawei |
| Annex A | Reference measurement channels | Intel |
| Annex G.3 | High speed train condition | Nokia |
| Annex G.4 | Moving propagation conditions | CATT |
| TS 38.141-2 |  |  |
|  | Big CR | Nokia |
| 4.6 | Manufacturer's declarations | Samsung, Nokia |
| 8 | Radiated performance requirements |  |
| 8.1.2 | Applicability rule |  |
| 8.1.2.4 | Applicability of PUSCH for high speed train performance requirements | Huawei |
| 8.2 | OTA performance requirements for PUSCH |  |
| 8.2.4 | Performance requirements for PUSCH for high speed train | Ericsson, Samsung |
| 8.2.5 | Performance requirements for UL timing adjustment | CATT |
| 8.4 | OTA performance requirements for PRACH |  |
| 8.4.1 | PRACH false alarm probability and missed detection |  |
| 8.4.1.6 | Test requirement for high speed train | Huawei |
| Annex A | Reference measurement channels | Intel |
| Annex E | OTA measurement system set-up | Ericsson |
| Annex J.3 | High speed train condition | Nokia |
| Annex J.4 | Moving propagation conditions | CATT |