3GPP TSG-RAN WG4 Meeting #103-e 													     R4-2210667

Electronic Meeting, May 9-20, 2022

Agenda item:			9.17.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	WF on PUCCH demodulation performance of Rel-17 NR coverage enhancement

Document for:	Approval

# Background

- The following WFs were approved previously:

  - R4-2203031 WF on PUCCH demodulation performance of Rel-17 NR coverage enh, RAN4#101-bis-e.

  - R4-2207211 WF on PUCCH demodulation performance of Rel-17 NR coverage enh, RAN4#102-e.

- Corresponding Email summary in RAN4#103-e

  - R4-2210530	Email discussion summary for [103-e][327] NR_cov_enh_Demod.

# WF on topic#2: PUCCH Enhancements of Rel-17 NR Coverage Enhancement

## General

Issue 2-1-1: Configured TDW length for JCE in BS PUCCH demod requirements

Agreement: – Agree to follow PUSCH conclusion for configured TDW length for JCE in BS PUCCH demod requirements

Issue 2-1-2: Number of repetitions for BS PUCCH demodulation requirements with JCE

Agreement: – Agree to follow PUSCH conclusion for the number of repetitions for BS PUCCH demodulation requirements with JCE

Issue 2-1-3: Frequency hopping for BS PUCCH demodulation requirements with JCE

Agreement: – Agree to disable Inter-slot frequency hopping for BS PUCCH demodulation requirements with JCE.

Issue 2-1-4: Antenna configuration for BS PUCCH demodulation requirements with JCE

Agreement: – Agree to use 2 Rx only for BS PUCCH demodulation requirements with JCE.

Issue 2-1-5: Frequency range coverage for BS PUCCH demodulation requirements with JCE

Agreement: – Agree to use the same approach as PUSCH JCE for frequency range coverage for BS PUCCH demodulation requirements with JCE

Issue 2-1-6: DMRS configuration for PUCCH format 3

  - Option 1: only cover without additional DMRS

  - Option 2: Cover both with and without additional DMRS

  - Make decision on this issue in the next meeting.

  - Encourage companies to bring simulation results for both DMRS1+1 and DMRS1+0 for the next meeting

Issue 2-1-7: Other parameters for BS PUCCH demodulation requirements with JCE

Agreement: Agree to apply the existing test parameters specified in Rel-15 for PUCCH requirement with JCE as starting point

# For information:

At least baseline configuration has been decided, we encourage companies to provide simulation results for alignment in next meeting.

For convenience, the agreed parameters are copied below:

| Test metric | - PUCCH format 3: UCI block error probability - PUCCH format 1: NACK to ACK detection probability and ACK missed detection probability |
| --- | --- |
| aTDW | - Use the same aTDW agreed for PUSCH JCE |
| cTDW | - Follow PUSCH conclusion |
| Number of repetitions | - Follow PUSCH conclusion |
| Frequency hopping | - Inter-slot frequency hopping disabled - Intra-slot frequency hopping disabled |
| TDD UL-DL pattern | - Use the same TDD UL-DL pattern as in PUSCH JCE. |
| Phase and power offset modelling | - Align both phase offset model and power offset model with the JCE test modelling for PUSCH |
| Bit payload | - PUCCH format 1: 2 bits payload - PUCCH format 3: 16 bits payload |
| Resource allocation | - Agree to use 14 symbols and 1 PRB for both PUCCH format 1 and format 3 |
| Frequency range coverage | - Use the same approach as PUSCH JCE |
| Antenna configuration | - 2 Rx only |
| DMRS configuration for PUCCH F3 | - DMRS1+1 and DMRS1+0 Note: Not agreed yet, the decision will be made in the next meeting. |