3GPP TSG-RAN WG4 Meeting #103-e	R4-2210671

# Electronic Meeting, 9th – 20th May, 2022

Title:	WF on demodulation requirement for Enhancement on Multi-TRP

Source:	Huawei, HiSilicon

Agenda item:			9.18.4.2.2

Document for:	Approval

Introduction

This WF capture all agreements and open issues for the following topics in [103-e][328] NR_FeMIMO_Demod.

- Topic #1: Demodulation requirement for Multi-TRP enhancement

  - Sub-topic #1-1 Test Scope

  - Sub-topic #1-2 Test setup for PDCCH requirement for Enhancement on Multi-TRP if introduced

  - Sub-topic #1-3 Test setup for PDSCH requirement for Enhancement on Multi-TRP if introduced

The agreed WFs on demodulation requirement for Enhancement on Multi-TRP in previous meeting is listed as following.

- R4-2203092, RAN4#101bis-e

- R4-2207207, RAN4#102-e

Topic #1: Demodulation requirement for Multi-TRP enhancement

Sub-topic #1-1 Test Scope

Issue 1-1-1: Whether to define PDCCH requirement for multi-TRP repetition transmission schemes

Agreements:

- Introduce PDCCH requirements for multi-TRP repetition transmission

  - Option 1: With UE soft-combining

  - Option 2: Without UE soft-combining

Issue 1-1-2: Whether to define PDSCH requirement for Multi-TRP inter-cell operation

Agreements:

- No PDSCH requirement for multi-TRP inter-cell operation for Rel-17 FeMIMO WI

Sub-topic #1-2 Test setup for PDCCH requirement for Enhancement on Multi-TRP if introduced

Issue 1-2-1: PDCCH requirement setup

Agreements:

- Aggregation level

  - Option 1: AL=2

  - Option 2: AL=4

  - Decide aggregation level base on simulation results in next meeting.

- Operating SNR condition for selection AL

  - Option 1: SNR > [-4] dB

  - Other options are not precluded

- SNR setting for each TRP

  - Option 1: Balance SNR (no blocking considered)

    - The SNRs for TRP #1 and TRP #2 are assumed to be balanced with a scaling factor of 1/sqrt(2) for the transmitted signal from each TRP

  - Option 2: Blocked TRP testing

    - Blocked TRP testing for Option 2 in Issue 1-1-1 (without UE soft-combining): Probability of blocking p=10% with X=10 dB blocking as a starting point

  - Other options are not precluded

- Time offset/Frequency offset for each TRP

  - Same value as single-DCI PDSCH with FDM scheme as starting point

    - 15KHz SCS: timing offset =-0.5us, frequency offset=200Hz

    - 30KHz SCS: timing offset =-0.25us, frequency offset=300Hz

Reference

- R4-2210531, Email discussion summary for [103-e][328] NR_FeMIMO_Demod, RAN4#103-e, Samsung

- R4-2203092, WF on demodulation requirement for Enhancement on Multi-TRP, RAN4#101bis-e, Huawei, HiSilicon

- R4-2207207, WF on demodulation requirement for Enhancement on Multi-TRP, RAN4#102-e, Huawei, HiSilicon