3GPP RAN WG4 Meeting #103-e		R4-2210798

Electronic meeting, May 09 – May 20, 2022	(revision of R4-2210343)

Agenda item:	11.2.2.1

Source:	Apple

Title:	Further input on performance when using the next larger channel

WI/SI:	FS_NR_eff_BW_util

Release:	Rel-17

Document for:	Approval

# 1	Introduction

During Rel-16 and Rel-17 discussions, several operators expressed an interest in enabling more efficient utilization of "non-standard" channel bandwidths, i.e. the ones which are not present now in TS 38.101 specifications. Referring to the corresponding operator requests, the following channel bandwidths were suggested by operators: 6, 7, 11, 12, 13, 33, 35, 45. As an outcome a new SI was agreed at the RAN#89 meeting aiming to study further which existing solutions can be used and whether new mechanism should be devised [1].

RAN WG4 has been considering several solutions, latest technical descriptions of which are captured in TR 38.844. One of the considered solutions is using next larger standard channel. In this TP we provide further technical information on the anticipated performance when the next larger channel is configured to the UE.

# 2	Further inputs on UE channel filters

## 2.1	Comments from the previous meeting RAN4#102

During the previous RAN4#102 meeting several questions and comments were expressed, which unfortunately were not addressed properly. In this section we answer some of those questions with the corresponding clarifications in the TP marked with highlights.

Q1:	From Qualcomm. How exactly is this measurement done? what device was used?

A1:	The measurements were performed using the same testbed and setup that is used for device conformance testing. The exact device is not relevant here because we do not establish any worst- or the best-case baseline, but rather show how the resulting performance degrades.

Q2:	From Nokia. Is our understanding correct in that the larger CBW method has significant degradation for 6MHz CBW? More clarification is needed when this method can be used. It cannot be left to deployment matter, as no guideline is presented.

A2:	As the difference between the configured and the irregular channel becomes larger, yes, performance starts degrading (provided that a UE does not change/adapt its digital filter). Whether it is "significant" or not is a matter of the blocker signal strength that can be suppressed. However, it is indeed the case that neither 6MHz nor even 9MHz irregular channel will be able to suppress completely the 32dB higher blocker, whereupon the 6MHz channel will naturally experience the biggest negative impact from the neighbour blocker.

Q3:	From Nokia. Since we’re looking at SINR in the 10MHz BW only and there’s no throughput measurements of the 46 and 29 RBs respectively, it is difficult to conclude whether the SINR degradation of the 10MHz is also reducing the UE throughput / sensitivity by 32dB.

A3:	SINR degradation will naturally lead to the (peak) throughput degradation, but additional measurements are needed to show the exact relationship between them.

Q4:	From Nokia. Is the blocker, in terms of the frequency offsets in TS 38.101-1 subclause 7.5, an adjacent channel interferer?

A4:	Yes, the blocker is the 3GPP standard blocker of the 5MHz size which is set so that it is always right next to the irregular channel.

Q5:	From Nokia. The SINR is mentioned once, but not used later. The Tdoc first writes about a measured SNR and an estimated SINR, but later, it writes about an estimated SNR. This is confusing, and the bandwidth to which the SNR refers it is not completely clear. If the estimated SNR includes the interference in blanked PRBs, it is meaningless

A5:	For the sake of consistency, all instances are changed to just SNR. As for the "estimated/measured" SNR, it is obviously an estimation based on the actual measurements as you cannot measure SNR. And as mentioned in the TP, the SNR is estimated based on the measured samples taken from the 10MHz channel irrespective of the irregular channel bandwidth size. It was done intentionally to estimate the worst-case scenario of the performance as perceived by the UE side.

## 2.2	Comments during RAN4#103

Q1:	From Nokia. Section 2.1, A4: in TS 38.101-1 subclause 7.6.2 about in-band blocking, the centre of the 5 MHz wide blocker has at least an offset of 7.5 MHz from the wanted signal's channel edge, i.e. it is not right next to the irregular channel, contrary to an adjacent channel interferer according to subclause 7.5. In subclause 7.5, the term 'blocker' is not used. Also the wanted signal level fits better to subclause 7.5 than 7.6. Could Apple please double-check the consistency of the frequency offset and the term 'blocker'?

A1:	We checked TS 38.101-1 and our understanding is that the blocker location is always specified as center-to-center offset, i.e. from center of the wanted signal to the center of the blocker. And this is also how it is assumed in our measurements. Nevertheless, to avoid any misunderstanding, we clarified it explicitly in the TP. As an example, the blocker offset is 7MHz for the 9MHz irregular channel meaning that the blocker is right next to the edge of the 9MHz allocation (2.5+4.5MHz).

Q2:	From Nokia. The effect of the huge degradation of the estimated SNR at an irregular BW of 6 MHz is not completely clear – is this relevant in the first place for the link quality reporting, or similarly relevant for the ACS.

A2:	Since the blocker "enters" the area of the 10MHz filter, it naturally creates additional interference causing worse channel performance.

Q3:	From MediaTek. I need to check further, but was this based on testing a real UE or was it based on a simulation using purely the ACS according to 3GPP specs as reference with no assumption on the actual filter?

A3:	The results are based on the actual measurements performed with a device, in which the both the analogue and digital filter were set to 10MHz to estimate the worst-case scenario.

Q4:	From Intel. We do not agree to the current text introducing and explaining the simulation results.  The presented figure is a worst case scenario with a narrowband (5MHz) blocker located directly against the edge of the desired CBW and NW unaware and not utilizing the UE channel filter as in Figure 6.1.3-1.  The simulation is also made worse by selecting a 5MHz blocker versus a wider BW.  The text needs to make it clear that this is a worst case scenario and not a typical scenario.   Also it would be better if the results could be presented as REFSENSE degradation instead of SNR since SNR degradation is somewhat subjective.

A4:	The 5MHz blocker is the standard 3GPP blocker used for lower frequencies (and the centre frequency is 1850MHz in our measurements). And our intention was indeed to measure the worst-case scenario.

## 2.2	Text proposal

-------------------- BEGIN --------------------

### 6.1.3	UE channel filters

A typical UE architecture utilises a number of filters of two major types – analogue and digital – and it is generally up to the UE implementation how they are combined. Nevertheless, it is often the case that a UE uses first the wideband analogue filter which typically covers a whole band. In addition to that, a UE may use another NR channel bandwidth specific analogue filter, premise function on which is to filter our non-adjacent blockers. However, since even the NR channel bandwidth specific analogue filter cannot ensure "brick-wall" like filtering, a UE also applies digital filter after ADC to eliminate adjacent  interferer. Depending on the UE implementation, the digital filter is a combination of the hardware and software components that allow the UE to apply the corresponding filter coefficients to support a wide range of standard channels, e.g. from 5MHz to 100MHz in case of FR1.

As an example, consider Figure 6.1.3-1. In this example, RBs are scheduled close to the NR band edge.  The potential adjacent interferer below the band edge will be removed by the CBW filter with no degradation of ACS at the lower side of the irregular BW since the filters have the same capability as the regular CBW operation.

Figure 6.1.3-1: Example of the NW utilizing the lower RBs near the NR band edge with BWP

However, for the scenario in the Figure 6.1.3-2 the wider channel filter cannot protect against adjacent interferer(s) when the irregular spectrum block is narrower than the channel filter and/or there are adjacent interferers on both sides.

NOTE: for the text above multi-operator scenario can be considered further in this example.

Current specifications do not define how a UE configures its digital filter within the configured channel bandwidth. So, in the provided example below, a UE implementation may configure the digital filter in accordance with the carrier bandwidth "ignoring" the actual smaller bandwidth part size. This is illustrated further in Figure 6.1.3-2. The wanted signal is smaller than 10MHz, but the UE filter is always set to 10MHz as signalled by the network. As can be seen, if there is an adjacent interferer, then it can "leak" into the wanted signal region.

Figure 6.1.3-2: Possible scenarios for the 10MHz channel filter.

To estimate anticipated performance described in Figure 6.1.3-2 when a UE configures its digital filter according to the channel bandwidth potentially allowing the adjacent interferer to "leak" into the wanted signal area, a series of measurements were conducted using the same testbed and setup as used for the commercial device conformance testing. The adjacent interfering signal is always set in such a way that it is right next to the wanted irregular channel bandwidth signal. Table 6.1.3-1 below summarises adjacent interfering channel parameters used in measurements, where adjacent interferer offset is the offset from the wanted signal center to the adjacent interfering signal center.

Table 6.1.3-1: Summary of adjacent interfering signal parameters.

| Irregular channel (MHz) |  | Effective bandwidth (MHz) |  | Adjacent interferer offset (MHz) |  | Adjacent interferer bandwidth (MHz) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 8.28 | 7 | 5 |
| 8 | 7.2 | 6.5 |  |
| 7 | 6.3 | 6 |  |
| 6 | 5.22 | 5.5 |  |

The following common parameters were applied:

-	Center frequency: 1850MHz

-	UE channel: 10MHz

-	Wanted signal level at UE antenna: -82dBm

-	Adjacent interferer level at UE antenna: from -120dBm to -50dBm in step of 1dB

-	Adjacent interferer: 5MHz, 16QAM, SCS 15kHz (3GPP standard ACS test)

Intermediate irregular channel bandwidths are considered – from 9MHz to 6MHz – and the adjacent interferer offset is set accordingly. As the main performance indicator, the resulting SNR is estimated over the whole 10MHz region irrespective of the actual irregular channel size. This approach provides the worst-case estimation of the resulting SNR because in the real life a UE will most likely estimate SNR over the configured bandwidth part corresponding to the irregular channel size or its sub-part.

As can be seen from Figure 6.1.3-3 below, when the adjacent interferer level is low then the estimated SNR is around 39dB for all irregular channels (if the wanted signal is -82dBm and the adjacent interferer  is -120dBm, then the theoretical SNR would be around 38dB, but in our case the adjacent interferer  is not even fully inside the wanted signal bandwidth). The 9MHz irregular channel bandwidth has same SNR of 39dB even when the adjacent interferer level is same as the wanted signal. SNR degradation starts at the adjacent interferer level of approximately -72dBm, i.e. 10dB higher than the wanted signal. When the adjacent interferer level is 32dB higher than the wanted signal, i.e. -50dBm, the estimated SNR is around 27dB for the 9MHz irregular channel.

As for the opposite extreme case of the 6MHz irregular channel, SNR degradation can be already observed at the adjacent interferer level of around -100dBm, i.e. 18dB lower than the wanted signal. When the adjacent interferer level becomes as high as the wanted signal, the estimated SNR drops down to 27dB. Finally, when the adjacent interferer is 32dB higher than the wanted signal, the SNR becomes -5dB.

These results show the relative degradation in SNR that can occur due to adjacent channel interferers leaking through the next larger BW filter, and the actual performance degradation will depend on a particular scenario, especially ACI strength and the UE channel filter overlap with the interfering signal. Typical 3GPP ACS requirements are conducted so that a UE should be able to meet 95% of the target throughput of the reference QPSK level when the adjacent interferer is 32dB higher than the wanted signal (and which corresponds to approximately -1.5dB SNR). Referring to the right-most point on the X-axis of Figure 6.1.3-3, it can be seen that 9MHz and 8MHz irregular channel SNR is above -1.5dB, while the 6MHz channel SNR is clearly below -1.5dB.

NOTE: 	The corresponding throughput measurements should be also provided to assess throughput degradation, if any, in the QPSK reference channel.

Figure 6.1.3-3: Estimated SNR depending on the adjacent interferer level.

To further reduce the risk of ACS reduction, as shown in this second example in Figure 6.1.3-2 and Figure 6.1.3-3, it is possible for new UEs to align their digital filter to the actual smaller bandwidth part within the wider analogue CBW filter bandwidth.

NOTE:	It is FFS whether it will require new UE capability and/or configuration signalling for the NW to indicate the adjacent interferer location, left, right of the BWP.

-------------------- END --------------------

# 3	Conclusions

In this discussion paper we have presented TP for TR 38.844 providing further technical input on the anticipated performance for a solution when the next larger channel is configured at the UE side.

# 4	References

RP-202103, "New SID: Study on Efficient utilization of licensed 	spectrum that is not aligned with existing NR channel bandwidths", T-Mobile USA, Ericsson