3GPP TSG-RAN WG4 Meeting #103-e						                                     R4-2210873

Electronic Meeting, 9th of May – 20th of May, 2022						 (revision of R4-2210216)

Agenda item:			9.12.3.3

Source:	Inmarsat

Title:	TP for TS 38.108, 6.6.4 Operating band unwanted emissions

Document for:	Approval

# Introduction

During RAN WG4 Meeting RAN4#102-e, it was agreed that further discussion would be required in order to address  Section 6 of TS 38.108, specifically in respect to sub-section 6.6.4 Operating band unwanted emissions.

This document provides a new skeleton TP for TS38.108 Section 6.6.4 Operating band unwanted emissions, in revision of the previous TP [3], as noted in the chairman’s notes [4]:

| R4-2207375 | Draft TP for TS 38.108 Section 6.6.4 Operating band unwanted emissions | Inmarsat | Postponed | UEM is not concluded. |
| --- | --- | --- | --- | --- |

The skeleton proposal was thus provided in line with the agreements made in approved Way Forward on SAN SEM and spurious emission made during RAN4#102-e [5]

This TP was further revised during RAN4#103-e following agreed WF during GTW discussion on May 11th [6] and merged with draft TP from Thales [7].

# Reference

[1] TS 38.108, “NR; Satellite Node radio transmission and reception”;

[2]	TR 38.863, “Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects”;

[3] R4-2207375, Draft TP for TS 38.108 Section 6.6.4 Operating band unwanted emissions, Inmarsat

[4] RAN4_102-e_BSRF_Demod_Test_ Session_report_EOMv2_AfterPostMeetingApproval.docx

[5] R4-2207456

[6] RAN4_103-e_BS_Demod_Test_Session_RRM_Session[219-234]_report_09_May17_AfterFirstRoundv2

[7] R4-2210000_v03_LN_THALES

# Text proposals for TS 38.108

------------------------------------------------<Start of TP>-----------------------------------------------

### 6.6.4	Operating band unwanted emissions

#### 6.6.4.1	General

Unless otherwise stated, the operating band unwanted emission (OBUE) limits for SAN in FR1 are defined from channel edge up to frequencies separated from the channel edge by 200% of the necessary bandwidth.

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer’s specification.

Basic limits are specified in the tables below, where:

-	f is the separation between the channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

-	PSDchannel represents the Power Spectral Density of the channel for a given channel bandwidth

-	BWChannel is the considered NR channel bandwidth or SAN total RF bandwidth for a given operating band.

-	ΔSat_ClassdBis the SAN class parameter in dB identified to characterize different SAN classes.

For a multi-carrier single-band connector the definitions above apply to the lower edge of the carrier transmitted at the lowest carrier frequency and the upper edge of the carrier transmitted at the highest carrier frequency within a specified frequency band.

-	The operating band unwanted emission basic limits of the band where there are carriers transmitted, as defined in the tables of the present clause for the largest frequency offset (fmax), shall apply from channel edge up to frequencies separated from the channel edge by 200% of the necessary bandwidth.

#### 6.6.4.2	Minimum requirements for SAN type 1-H

For SAN operating in Bands n256, n255, basic limits are specified in table 6.6.4.2 1 for GEO and LEO class respectively, in line with Annex 5 of ITU recommendation SM.1541-6 [x].

The SAN Operating Band Unwanted Emissions (OBUE) requirements for GEO and LEO classes are therefore defined as described in Table 6.6.4.2-1 below.

Table 6.6.4.2-1: SAN LEO and GEO Classes OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 2* BWChannel | 0.002 MHz f_offset < 2* BWChannel + 0.002 MHz | maxSE limit, PSDchannel – ΔSat_ClassdBX-40×log10 f_offset-0.002BWChannel×*2+1dBm | 4kHz |
| NOTE 1: PSDchannel = Prated,c – 10log10(BWChannel) – 24, unit dBm/4kHz. NOTE 2: SE limit is spurious emission limit specified in spurious emission clause [6.6.5]. NOTE 3: PSD attenuation as in ITU-R SM.1541-6 [x], Annex 5 OoB domain emission limits for space services. NOTE 4: ΔSat_ClassdB=0 dB for GEO class and ΔSat_ClassdB=3 dB for LEO class. |  |  |  |

------------------------------------------------<End of TP>-------------------------------------------------