3GPP TSG-RAN4 Meeting #103-e	R4-2211052

# Electronic meeting, May 9 – May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2303 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on PRS-RSRP measurement period without gaps |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - CSSF for PRS-RSRP measurement is still FFS. - Reduced number of samples and reduced Rx beam sweeping factor can also be applied for the PRS measurement without gap. - The PRS resources for calculating TPRS,i need to be clarified based on the agreements in last meeting. - The measurement period requirements when PPW is configured need to be added to capture the agreements in last meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update CSSF for PRS-RSRP measurement. - Update measurement requirements for reduced number of samples and reduced Rx beam sweeping factor. - Clarify the PRS resources used for calculating TPRS,i. - Clarify the measurement period requirements when PPW is configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement period requirements for the PRS-RSRP measurement without gap are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2208212 |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.9.3.6	Measurement Period Requirements without Measurement Gaps

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.3.3) PRS-RSRP measurements as defined in TS 38.215 [4] without measurement gap, on configured positioning frequency layers, within TPRS-RSRP,total ms.

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i

where

i is the index of positioning frequency layer,

L is the total number of positioning frequency layers,

Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

TPRS-RSRP,i=CSSFi*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

where

CSSFiCSSFi= FFS

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1. If positioning frequency layer i is in FR2, NRxBeam,i=  the Rx beam sweeping factor indicated in the high layer parameter numberOfRxBeamSweepingFactor if UE support lowerRxBeamSweepingThan8-FR2, otherwise, NRxBeam,i= [8],

Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resource instances that meet the applicability conditions and fully or partially overlapped with PRS processing window are considered.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot  in clause 6.4.3 of TS 37.355 [34],

Nsample is the number of PRS-RSRP measurement samples andNsample

if UE supports supportedDL-PRS-ProcessingSamples and is indicated by LMF to perform measurement with reduced sample number, Nsample= 1 if the following conditions are met; [Nsample= 2 otherwise].

- PRS bandwidth is within the active BWP, and

- Difference between the serving cell SS-RSRP and neighbor cell/TRP PRS-RSRP is within [6] dB.

if UE does not support supportedDL-PRS-ProcessingSamples, or UE is not indicated by LMF to perform measurement with reduced sample number, Nsample= 4;

Tlast = Ti +Tavailable_PRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time,

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,PPWRPi is the least common multiple between TPRS,i and PPWRPi,

TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

PPWRPi is the PRS processing window repetition period in positioning frequency layer i.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of  TperPRS with muting among the DL PRS resource sets is used to derive TPRS,i, where:

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. If bitmap b1  for higher-layer parameter DL-Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note:	For the purpose of calculating TPRS,i, only the PRS resources that meet the applicability conditions and fully or partially covered by the PRS processing window are considered.

When PRS-RSRP measurements are configured for DL-AoD, the time TPRS-RSRP,total starts from the first PRS processing window instance aligned with DL PRS resources in the assistance data after both the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

When the PRS-RSRP measurement is configured together with RSTD measurement then the PRS-RSRP measurement shall meet the RSTD measurement requirements defined in clause 9.9.2.

When the PRS-RSRP measurement is configured together with UE Rx-Tx time difference measurement then the PRS-RSRP measurement shall meet the UE Rx-Tx time difference measurement requirements defined in clause 9.9.4.

If CSSF changes during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource:

if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in this section apply provided UE indicate that PRS is higher priority than other signals within PRS processing window.

The requirements in clause 9.9.3 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AoD-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while PRS-RSRP measurements are being performed then the UE shall complete the ongoing PRS-RSRP measurements session. The PRS-RSRP measurement period can be longer. The UE shall meet the PRS-RSRP measurement accuracy requirements in clause 10.1.24.

If during the measurement period of one or more positioning frequency layers, the PPW is reconfigured or reactivated, the PRS-RSRP measurement period can be longer.

<End of Change 1>