3GPP TSG-RAN WG4 Meeting # 103-e 	R4-2211114

# Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2382 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on L1-RSRP measurement requirements for inter-cell BM in R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. Redundant clarification of known condition. Prefer to capture the applicability of known cell condition in 9.13.2; 2. The beamManagementSSB-CSI-RS should count both serving cell RS and SSB from different PCI; 3. The inter-cell beam measurements should be performed on PCell, PSCell or SCell. 4. Restriction on Nmax =1 should be capture in applicability section. 5. The sharing factor of L1-RSRP measurement requirements between SC and CDP needs to be further checked. 6. Some other editorial change are needed, such as removing unnecessary notes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. capture the applicability of known cell condition in 9.13.2, and related descritpion in 9.13.1 is removed. 2. Clarify the capability beamManagementSSB-CSI-RS in 9.13.1 and 9.5.1. 3. Clarify that inter-cell beam measurements requirements are applicable for PCell, PSCell and SCell. 4. Capture restriction on Nmax = 1 in applicabiliy section. 5. Put the sharing factor for both SC and CDP in square brackets. 6. Editorial revisions, such as removing unnecessary editor’s notes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreements are not correctly implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.1, 9.5.4.1, 9.13.1, 9.13.2, 9.13.3, 9.13.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2209503 |  |  |  |  |  |  |  |  |

<Start of Change #1>

9.5.1	Introduction

When configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP. The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements within the active BWP.

The UE shall be able to measure all CSI-RS resources and/or SSB resources of the nzp-CSI-RS-ResourceSet and/or csi-SSB-ResourceSet within the CSI-ResourceConfig settings configured for L1-RSRP for the active BWP, provided that the number of resources, including the number of SSB resources of the cell with PCI different from serving cell configured for L1-RSRP measurements in 9.13, does not exceed the UE capability indicated by beamManagementSSB-CSI-RS.

<End of Change #1>

<Start of Change #2>

9.5.4.1	SSB based L1-RSRP Reporting

The UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

The value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-1 for FR1. The value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-2 for FR2 when [highSpeedMeasFlagFR2-r17] is not configured, and defined in Table 9.5.4.1-3 for FR2 power class 6 UE when [highSpeedMeasFlagFR2-r17] is configured, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8 in Table 9.5.4.1-2.

For a UE that supports either concurrent measurement gaps, pre-MG gaps or NCSG, measurement gaps in this section includes any configured and active gap.

Editor’s note: the term “[measurement gap]” in this clause could be either the legacy gap or the VIL or NCSG. RAN4 to further study whether a better terminology can be used to replace [measurement gap].

For FR1, for a UE not supporting [concurrent gaps] or when the UE is not configured with concurrent gaps,

-	P=11-TSSBxGP, when in the monitored cell there are [measurement gaps] configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no [measurement gaps]  overlapping with any occasion of the SSB.

For FR2, for a UE not supporting [concurrent gaps] or when the UE is not configured with concurrent gaps,

-	P= PSC1-TSSBTSMTCperiod, when S SSB is partially overlapped with measurement gap (TSSB <MGRP) and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P is Psharing factor, when SSB is not overlapped with measurement gap and SSB is fully overlapped with SMTC period (TSSB = TSMTCperiod).

-	P=PSC1-TSSBxRP-TSSBTSMTCperiod, when SSB is partially overlapped with [measurement gap] and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with [measurement gap] and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TSSB < 0.5*TSMTCperiod

-	P is Psharing factor1-TSSBxRP, when SSB is partially overlapped with [measurement gap] and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with [measurement gap] and TSMTCperiod = xRP and TSSB = 0.5*TSMTCperiod

-	P= PSC1-TSSBTSMTCperiod, when SSB is partially overlapped with [measurement gap] (TSSB < xRP) and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with [measurement gap].

-	P is  Psharing factor1-TSSBxGP, when SSB is partially overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with [measurement gap] (TSMTCperiod < xRP)

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

When concurrent gaps are configured,

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target SSB.

Where:

-	TSSB = ssb-periodicityServingCell of the serving cell

-	TSMTCperiod = the configured SMTC period

[-	PSC = [2] if the SSB measurement occasions of the cell with PCI different from serving cell are fully overlapped with SSB measurement occasions of the serving cell, and TSSB = TSSB_CDP < TSMTCperiod

-	PSC = 11-TSSBTSSB_CDP if the SSB measurement occasions of the cell with PCI different from serving cell are fully overlapped with SSB measurement occasions of the serving cell, and TSSB < TSSB_CDP < TSMTCperiod

-	PSC = 1 if the SSB measurement occasions of the cell with PCI different from serving cell are partially overlapped with SSB measurement occasions of the serving cell, and TSSB_CDP < TSSB, and SSB measurement occasions of the serving cell are either fully overlapped with SMTC, or partially overlapped with SMTC (TSSB ≤ TSMTC).

-	TSSB_CDP = SSB periodicity of the cell with PCI different from serving cell]

[Editor’s Note: FFS PSC at least for the case when considering SMTC and measurement gaps, the remaining L1 measurement occasions are fully overlapped between serving cell and cell with PCI different from serving cell.]

[Editor’s Note: FFS PSC = 1 for HST scenario]

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	TSSB = ssb-periodicityServingCell

-	TSMTCperiod = the configured SMTC period

-	If the UE is configured with Pre-MG, an SSB or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When a measurement gap is configured,

-	an SSB or an SMTC occasion is considered to be as overlapped with the [measurement gap] if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	When NCSG is configured,

-	an SSB or an SMTC occasion is considered to be as overlapped with the [measurement gap] if

it overlaps the VIL1 or VIL2 of NCSG, or

it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

and

xRP = VIRP

When concurrent gaps are configured, an SSB or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to [9.1.2B].

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and [measurement gap] configurations does not meet pervious conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 [or highSpeedMeasFlag-r17] are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or [HSTmeasurementEnhancement-r17] |  |

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 9.5.4.1-3: Measurement period TL1-RSRP_Measurement_Period_SSB configured with [highSpeedMeasFlagFR2-r17] for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N1Note2)*TSSB) |
| DRX cycle ≤ 80ms | max(TReport, ceil(M*P*N1Note2*M2)*max(TDRX,TSSB)) |
| 80ms< DRX ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: N1 = 2 when [highSpeedMeasFlagFR2-r17] = [set1]; N1 = 6 when [highSpeedMeasFlagFR2-r17] = [set2]. Note 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

<End of Change #2>

<Start of Change #3>

### 9.13.1	Introduction

When configured by the network, the UE shall be able to perform L1-RSRP measurements of configured measurement resources from a cell with different PCI in addition to serving cell (PCI indicated in additionalPCI-r17), with the measurement resources configured as SSBs of the cell with different PCI.

The UE shall be able to measure all SSB resources of the cell with different PCI in csi-SSB-ResourceSet within the CSI-ResourceConfig settings for the active BWP, while the additionalPCI-r17 of the SSB resources are different from serving cell PCI. The number of resources, including the number of resources configured for serving cell L1-RSRP measurement in 9.5, does not exceed the UE capability indicated by beamManagementSSB-CSI-RS.

The UE shall report the measurement quantity (reportQuantity) and send periodic, semi-persistent or aperiodic reports, according to the higher layer parameter reportConfigType of each reporting setting CSI-ReportConfig for the active BWP.

[In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.]

9.13.2	Requirements Applicability

The requirements in clause 9.13 apply, provided the SSB from cell with PCI different from serving cell configured for L1-RSRP if the following conditions are met:

- the number of cells with PCI different from seving cells Nmax = 1 for FR2 and FFS for FR1

- The cell with different PCI from serving cell is known

- The SSB resources configured for L1-RSRP measurements are measurable

An SSB resource configured for L1-RSRP for cell with different PCI from serving cell shall be considered measurable when for each relevant SSB the following conditions are met:

-  L1-RSRP related side conditions given in clauses 10.1.19.1 and 10.1.20.1 for FR1 and FR2, respectively, for a corresponding band,

-  SSB_RP and SSB Ês/Iot according to Annex B.2.4.1 for a corresponding band.

The cell with different PCI from serving cell is considered as known if the following conditions are met in this requirement:

- The SSB from the cell with different PCI completely contained in the active BWP or associated with initial downlink BWP of the UE

- The SSB of the cell with different PCI from serving cell has the same SCS, sfn-SSB-Offset and center frequency as the SSB of the serving cell

- The timing difference of arrival at UE between the SSBs of serving cell and cell with different PCI is less than CP length of the corresponding SCS

- The UE has sent a valid L3 measurement report during the last 5 seconds, and

- The SSB from the cell with different PCI remains detectable according to the cell identification requirements specified in clause 9.2.

Otherwise, the cell is unknown.

9.13.3	Measurement Reporting Requirements

The UE shall send L1-RSRP reports only for report configurations configured for the active BWP[for cell with different PCI from serving cell under the known cell condition. Otherwise, the UE shall not send L1-RSRP reports.]

The UE shall report the L1-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1dB step size according to clause 10.1.19 for FR1 and 10.1.20 for FR2 if nrofReportedRS is configured to one. If nrofReportedRS is configured to be larger than one, or if groupBasedBeamReporting-r17 is enabled, the UE shall use differential L1-RSRP based reporting as defined in clause 10.1.19 for FR1 and 10.1.20 for FR2. The differential L1-RSRP is quantized to a 4-bit value with 2dB step size. The mapping between the reported L1-RSRP value and the measured quantity is described in 10.1.6.

In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

9.13.3.1	Periodic Reporting

Reported L1-RSRP measurements contained in periodic L1-RSRP measurement reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively.

The UE shall only send periodic L1-RSRP measurement reports for an active BWP.

The UE shall transmit the periodic L1-RSRP reporting on PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

9.13.3.2	Semi-Persistent Reporting

Reported L1-RSRP measurements contained in a Semi-Persistent L1-RSRP measurement report shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively. This requirement applies for semi-persistent L1-RSRP reports send on PUSCH or PUCCH.

The UE shall only send semi-persistent L1-RSRP measurement reports on PUSCH, if a DCI request has been received.

The UE shall only send semi-persistent L1-RSRP measurement reports on PUCCH, if an activation command [7] has been received.

The UE shall transmit the semi-persistent L1-RSRP reporting on PUSCH or PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

9.13.3.3	Aperiodic Reporting

Reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively.

The UE shall only send aperiodic L1-RSRP measurement report if a DCI trigger has been received.

After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

9.13.4	L1-RSRP measurement requirements

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting

If a cell with PCI different from serving cell is known according 9.13.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_CDP.

The value of TL1-RSRP_Measurement_Period_SSB_CDP is defined in Table 9.13.4.1-1 for FR1, The value of TL1-RSRP_Measurement_Period_SSB_CDP is defined in Table 9.13.4.1-2 for FR2 when [highSpeedMeasFlagFR2-r17] is not configured, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8.

For FR1,

-	P=11-TSSB_CDPMRGP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

For FR2,

-	P=PCDP1-TSSB_CDPTSMTCperiod, when SSB is not overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB_CDP < TSMTCperiod).

-	P= PCDP1-TSSB_CDPMGRP-TSSB_CDPTSMTCperiod, when SSB is partially overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB_CDP < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB_CDP < 0.5*TSMTCperiod

-	P= PCDP1-TSSB_CDPTSMTCperiod,when SSB is partially overlapped with measurement gap (TSSB_CDP <MGRP) and SSB is partially overlapped with SMTC occasion (TSSB_CDP < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

Where:

[-	TSSB_CDP = SSB periodicity of the cell with PCI different from serving cell

-	TSMTCperiod = the configured SMTC period

-	PCDP = [2] if the SSB measurement occasions of the cell with PCI different from serving cell are fully overlapped with SSB measurement occasions of the serving cell, and TSSB_SC = TSSB_CDP < TSMTCperiod

-	PCDP = 1 if the SSB measurement occasions of the cell with PCI different from serving cell are fully overlapped with SSB measurement occasions of the serving cell, and TSSB_SC < TSSB_CDP < TSMTCperiod

-	PCDP = 1 if the SSB measurement occasions of the cell with PCI different from serving cell are partially overlapped with SSB measurement occasions of the serving cell, and TSSB_CDP < TSSB_SC = TSMTCperiod, and SSB measurement occasions of the serving cell are fully overlapped with SMTC.

-	PCDP = 11-TSSB_CDPTSSB_SC, if the SSB measurement occasions of the cell with PCI different from serving cell are partially overlapped with SSB measurement occasions of the serving cell, and TSSB_CDP < TSSB_SC, and SSB measurement occasions of the serving cell are partially overlapped with SMTC (TSSB_SC < TSMTC)

-	TSSB_SC = ssb-periodicityServingCell of the serving cell]

[Editor’s Note: FFS PCDP at least for the case when considering SMTC and measurement gaps, the remaining L1 measurement occasions are fully overlapped between serving cell and cell with PCI different from serving cell.]

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For either an FR1 or FR2 cell with PCI different from serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 cell with PCI different from serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.13.4.1-1: Inter-cell L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_CDP for known cells with different PCIs in FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB_CDP (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_CDP) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_CDP)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB_CDP is the periodicity of the SSB-Index configured for inter-cell L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: [K = 1 when TSSB_CDP ≤ 40 ms and highSpeedMeasFlag-r16 are configured; otherwise] K = 1.5. [Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16]] [Editor’s Note: Whether inter-cell L1-RSRP measurement requirements are applicable in HST scenario] |  |

Table 9.13.4.1-2: Inter-cell L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_CDP for known cells with different PCIs in FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB_CDP (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB_CDP) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB_CDP)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB_CDP is the periodicity of the SSB-Index configured for inter-cell L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

<End of Change #3>