3GPP TSG-RAN WG4 Meeting #103	R4-2211150

Electronic Meeting, 9th ‒ 20th May, 2022

Source: 	Huawei

Title: 	WF on NR FR1/FR2 repeater MU and TT

Agenda Item:	9.5.5

Document for:	Approval

# Introduction

This WF captures the agreements and open issues around the FR1 conducted and FR2 OTA MU and TT values.

# Way forward

The agreements and the open issues have been captured below:

## FR1 MU and TT

It is tentatively agreed to take the value sin R4-2210014 as a baseline and discuss further, the following potential issue which may require adjusting the MU values have been raised:

- Input signal uncertainty and it effect on the output power.

- Unlicensed bands can be removed MU only needed to 5GHz

- ALCR BW’s need clarification on which BW is intended.

- Confirm UL and DL values are the same

The following table is used as the starting point for further discussion in next meeting,

| Subclause | Maximum Test System Uncertainty and Range over which Test System Uncertainty applies |
| --- | --- |
| 6.2 Output power | ±0.7 dB, f ≤ 3 GHz ±1.0 dB, 3 GHz < f ≤ 6 GHz (Note) |
| 6.3 Frequency stability | ±12 Hz Measurement results of 500 Hz |
| 6.4 Out of band gain | ±0,5 dB, f ≤ 3.0 GHz ±0,8 dB, 3.0 GHz < f ≤ 6 GHz Calibration of test set-up shall be made without DUT in order to achieve the accuracy |
| 6.5.2 ACLR | ACLR/ CACLR BW ≤ 20MHz: ±0.8 dB BW > 20MHz: ±1.2 dB Absolute power ±2.0 dB, f ≤ 3 GHz Absolute power ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) CACLR BW ≤ 20MHz: ±0.8 dB BW > 20MHz: ±1.2 dB CACLR absolute power ±2.0 dB , f ≤ 3 GHz CACLR absolute power ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) |
| 6.5.3 Operating band unwanted emission (except 9.1.5.4) | ±1.5 dB, f ≤ 3 GHz ±1.8 dB, 3 GHz < f ≤ 6 GHz (Note) and The interference from the signal generator ACLR shall be minimum 10 dB below that of a Base Station according toTS36.141-1 [11] |
| 6.5.4.2.1 General Spurious emissions | 9 kHz < f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 19 GHz: ±4.0 dB 19 GHz < f ≤ 26 GHz: ±4.5 dB |
| 6.5.4.2.2 Additional spurious emissions requirements | ±2.0 dB for > -60 dBm, f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 4.2 GHz ±3.0 dB, 4.2 GHz < f ≤ 6 GHz ±3.0 dB for ≤ -60 dBm, f ≤ 3 GHz ±3.5 dB, 3 GHz < f ≤ 4.2 GHz ±4.0 dB, 4.2 GHz < f ≤ 6 GHz |
| 6.5.4.2.3 Co-location with base stations and repeater type 1-C Nodes | ±3.0 dB |
| 6.6 Error vector magnitude | 1% signal analyser, 2% stimulus signal Requirement limit shifted by RSS requirement and stimulus signal EVM. Analyser error added to requirement limit. |
| 6.7 Input intermodulation | ±1.3 dB, f ≤ 3 GHz ±1.4 dB, 3 GHz < f ≤ 6 GHz |
| 6.8 Output intermodulation | The value below applies only to the interfering signal and is unrelated to the measurement uncertainty of the tests in 6.6.3 (ACLR), 6.6.4 (OBUE) and 6.6.5 (spurious emissions) which have to be carried out in the presence of the interferer. ±1.0 dB |
| 6.9 Adjacent channel rejection ratio | 0,7 dB |
| 6.10 Transmit ON/OF power | Transmit OFF power ±2.0 dB , f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) |

## FR2 MU and TT

It is tentatively agreed to take the value sin R4-2210015 as a baseline starting point and discuss further, the following potential issue which may require adjusting the MU values have been raised:

- Input signal uncertainty and it effect on the output power.

  - Potentially test at +10dB ALC condition to avoid variation

- Effect of the OTA input signal feed on TRP accuracy measurements

- OTA out of band gain, OTA input intermodulation, OTA ACRR require more analysis

The following table is used as the starting point for further discussion in next meeting,

| Subclause | Maximum Test System Uncertainty and Range over which Test System Uncertainty applies |  |
| --- | --- | --- |
| 7.2 OTA Output power | ±1.7 dB (24.25 – 29.5 GHz) ±2.0 dB (37 – 43.5 GHz) ±2.2 dB (43.5 GHz < f ≤ 48.2 GHz) | The BS value is used for FR1, same approach can be used for FR2 |
| 7.3 OTA Frequency stability | ±12 Hz Measurement results of TBD Hz |  |
| 6.4 OTA Out of band gain | FFS | This is not a BS requirement so no available value. Also it includes the input and output uncertainties |
| 7.5.2 OTA ACLR | Relative ACLR: ±2.3 dB (24.25 – 29.5 GHz) ±2.6 dB (37 – 43.5 GHz) ±2.8 dB (43.5 GHz < f ≤ 48.2 GHz) Absolute ACLR: ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 7.5.3 OTA Operating band unwanted emission | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 7.5.4 OTA Spurious emissions | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz |  |
| 7.6 OTA Error vector magnitude | 1% signal analyser, 2% stimulus signal Requirement limit shifted by RSS requirement and stimulus signal EVM. Analyser error added to requirement limit. | Same as FR1 |
| 7.7 OTA Input intermodulation | FFS | NO equivalent BS requirement but can be calculated using same approach as FR1 |
| 7.8 OTA Adjacent channel rejection ratio | FFS | No equivalent in BS spec. Relative measurement of 2 powers. |
| 7.9 OTA Transmit ON/OF power | Transmit OFF power ±2.9 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.6 dB (43.5 GHz < f ≤ 48.2 GHz) |  |