3GPP TSG-RAN WG4 Meeting #103-e	R4-2210672

Electronic Meeting, 9 May ‒ 20 May 2022

Title: 	WF on RedCap demodulation and CQI reporting requirements

Agenda Item:	9.19.5

Source: 	Ericsson

Document for:	Approval

# Topic #1: General topics for UE demodulation and CSI reporting requirements

## Sub-topic 1-1 Specification structure

Agreement: Spec structure of UE demodulation and CSI reporting requirements for RedCap in TS38.101

- Option 1: Use new Suffix “I”

- Option 2: Option 1: Use new Suffix “I”

Agreement: UL/DL pattern used for FDD tests for 1Rx UE

- Introduce demodulation/CSI requirements covering both FD-FDD and HD-FDD.

  - DDDSU applied for HD-FDD

  - Existing pattern applied for FD-FDD

  - Same demodulation requirement applied for FD-FDD and HD-FDD with different FRCs

  - For CSI requirements,

    - CQI reporting test in static/fading condition (periodic CSI reporting)

      - CSI-RS periodicity and offset: 5/1

      - CSI-Report periodicity and offset: 5/4

        - Comment: slot offset is changed from 0 to 4 because of the pattern of DDDSU)

    - CQI/RI/PMI delay: 8 ms

      - PMI reporting tests (aperiodic CSI reporting)

      - CSI request: 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0.

        - Comment: To keep the aperiodic report slot offset to 4, and UL slot is every 5 slots, CSI request should be changed from mod(i, 5)=1 to mod(i, 5)=0).

      - Aperiodic Report Slot Offset: 4 slots

      - CQI/RI/PMI delay: 6 ms

- Test applicability rule:

  - If RedCap UE support only HD-FDD in a FDD band, this UE is tested with HD-FDD mode otherwise UE is tested with FD-FDD mode

# Topic #2: UE demodulation requirements

## Sub-topic 2-1 PDSCH demodulation requirements

Agreement: Additional PDSCH demodulation requirements

- Not define any additional PDSCH demodulation requirements other than those agreed in last RAN4 meeting (moderator: RAN4#102-e) in Rel-17 for RedCap

Agreement: Define 256QAM demodulation requirements for FR1

- Introduce 256QAM requirement for 2Rx

Way forward/Agreements: Define 256QAM demodulation requirements for FR1

- For 256QAM requirements for 1Rx,

  - Option 1: Define 256QAM demodulation requirements for 1Rx in FR1

    - Option 1a: MCS20

    - Option 1b: Same MCS as 2Rx test case, that is, MCS24

  - Option 2: Not to define 256QAM demodulation requirements for 1Rx in FR1

## Sub-topic 2-2 SDR requirements

Agreement: Test setup for SDR requirements

- Reuse the existing test procedure and test setup with removal unfeasible test parameters from Rel-15 SDR test.

- For FR1, define SDR requirements with configuration 1T1R for UE supporting 1 layer and 2T2R for UE supporting 2 layers. For FR2, define SDR requirements for 2 layers only with configuration 2T2R

## Sub-topic 2-3 PDCCH demodulation requirements

Agreement: Test setup for PDCCH FR1 TDD 30kHz

- Set CORESET duration to 1 for FR1 TDD PDCCH demodulation requirements

- Set REG bundle size to 2 for FR1 TDD PDCCH demodulation requirements

Agreement/Way forward: Aggregation level(s)

- 1Rx UE:

  - FR1 FDD 15kHz: Define AL8 only

  - FR1 TDD 30kHz: Define AL4 only

- 2Rx UE:

  - FR1 FDD 15kHz: Define both AL4 and AL8

  - FR1 TDD 30kHz: Define both AL4 and AL8

  - FR2 TDD 120kHz: Define both AL4 and AL8

# Topic #3: CSI reporting requirements

## Sub-topic 3-1 CQI reporting requirements in static condition

Agreement: Test setup for CQI reporting test in static condition

- Define the codebookSubsetRestriction for 2T1R to avoid the equivalent channel matrix is 0.

- Set codebookSubsetRestriction to 000001 if H = [1 1] is used

Agreement: Test points for CQI reporting test in static condition for 2Rx

- Set SNR=8/9dB for the lower test point.

- Set SNR=14/15dB for the higher test point.

Agreement/Way forward: Test points for CQI reporting test in static condition for 1Rx

- SNR test points X dB lower than 2Rx test case

  - For Lower SNR test point

    - Option 1: X = 3dB

    - Option 2: X = 2dB

  - For Higher SNR test points

    - X = 3dB

## Sub-topic 3-2 CQI reporting requirements in fading condition

Agreement: Test metric and pass/fail criteria for CQI reporting test in fading condition for 1Rx/2Rx UE

- Reuse the same test metrics and pass/fail criteria as existing WB CQI reporting tests in fading channel for RedCap UE:

  - A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time

  - The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ.

  - When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

  - Set α=20%, γ=1.05

Agreement/Way forward: Test points for CQI reporting test in fading condition for 2Rx

- Lower SNR test points

  - Option 1: SNR=4/5dB

  - Option 2: SNR=6/7dB

- Higher SNR test points:

  - SNR=12/13dB

Agreement: Test points for CQI reporting test in fading condition for 1Rx

- Set SNR test points 3 dB higher than 2Rx case

## Sub-topic 3-3 PMI reporting requirements

Agreement: Test metric and pass/fail criteria for PMI reporting test for 1Rx/2Rx UE

- Reuse the same test metrics and pass/fail criteria as existing PMI reporting tests for RedCap UE

  - The minimum requirements are specified in terms of the ratio γ=tue/trnd, where tue is 90% of the maximum throughput observed at SNRUE using the precoders configured according to UE reports, and trnd is the throughput measures at SNRUE with random precoding.

  - Set γ=1.3

## Sub-topic 3-4 RI reporting requirements for 2Rx UE

Way forward: Whether to define RI reporting requirements for RedCap 2Rx UEs

- Option 1: Define RI reporting requirements

- Option 2: Not define RI reporting requirements

# Simulation assumption

Way forward: Collect simulation results

- Interested companies are encouraged to provide the simulation results with ‘New’. Interested companies can also provide the simulation results with ‘Reuse’ for reference

## PDSCH demodulation requirements

FR1 FDD 15kHz

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | New |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | New |
| 1Rx UE | 10MHz / 15kHz | 256QAM MCS20 or MCS24 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-3 | New |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | Reuse |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | Reuse |
| 2Rx UE | 10MHz / 15kHz | 256QAM 0.82 Rank 1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-3 | Reuse |

FR1 TDD 30kHz

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New |
| 1Rx UE | 20MHz / 30kHz | 256QAM MCS20 or MCS24 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-3 | New |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New |
| 2Rx UE | 20MHz / 30kHz | 256QAM 0.82 Rank 1 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-3 | New |

FR2 TDD 120kHz

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-3 Test 1-1 | Reuse |
| 2Rx UE | 100MHz / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-1 | TDLA30-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-2 | Reuse |
| 2Rx UE | 100MHz / 120kHz | 64QAM 0.46 Rank 2 | FR2.120-2 | TDLA30-75 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-6 | Reuse |

## PDCCH demodulation requirements

Candidate options for FR1 FDD 15kHz

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x1 | 1% | 5.3.2.1.1-1 Test 3 (R.PDCCH.1-2.4 FDD) | New |
| 1Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x1 low | 1% | 5.3.2.1.2-1 Test 3 (R.PDCCH.1-1.3 FDD) | New |
| 2Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 low | 1% | 5.3.2.1.1-1 Test 3 (R.PDCCH.1-2.4 FDD) | Reuse |
| 2Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 low | 1% | 5.3.2.1.2-1 Test 3 (R.PDCCH.1-1.3 FDD) | Reuse |

Test setup for PDCCH FR1 TDD 30kHz

The following parameter setup for PDCCH demodulation requirements.

| Parameter | Unit | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |  |
| CCE to REG mapping type |  | interleaved |  | interleaved |  |
| Interleaver size |  | 3 |  |  |  |
| REG bundle size |  | 2 | 2 |  |  |
| Shift Index |  | 0 |  |  |  |

Candidate options for FR1 TDD 30kHz

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 2 (R.PDCCH.2-1.2 TDD) | New |
| 1Rx UE | 20 | 48 | 1 | 8 | TDLC300-100 | 2x1 low | 1% | 5.3.2.2.2-1 Test 1 (R.PDCCH.2-1.3 TDD) | New |
| 2Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 2 (R.PDCCH.2-1.2 TDD) | New |
| 2Rx UE | 20 | 48 | 1 | 8 | TDLC300-100 | 2x2 low | 1% | 5.3.2.2.2-1 Test 1 (R.PDCCH.2-1.3 TDD) | New |

Candidate options for FR2 TDD 120kHz

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100 | 60 | 1 | 4 | TDLA30-300 | 1x2 low | 1% | 7.3.2.2.1-1 Test 1-2 (R.PDCCH. 5-1.2 TDD) | Reuse |
| 2Rx UE | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 low | 1% | 7.3.2.2.2-1 Test 2-1 (R.PDCCH.5-1.3 TDD) | Reuse |

## PBCH demodulation requirements

| 1Rx UE or 2Rx UE | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCK block index | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Unknown | 5.4.2.1-2 Test 1 | New |
| 1Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Unknown | 5.4.2.2-2 Test 1 | New |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Unknown | 5.4.2.1-2 Test 1 | Reuse |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Known | 5.4.2.1-3 Test 1 | Reuse |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Unknown | 5.4.2.2-2 Test 1 | Reuse (*1) |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Known | 5.4.2.2-3 Test 1 | Reuse (*1) |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 1 | Reuse |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 1 | Reuse |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 2 | Reuse |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 2 | Reuse |
| Note 1: RAN4 can keep the same SNR requirements but need to change the channel bandwidth configuration. |  |  |  |  |  |  |  |

## CQI reporting requirements in static condition

FR1 FDD 15kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 1 | 2 x 1 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 2Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 2 | 2 x 2 with static channel specified in Annex B.1 in TS 38.101-4 | New |

FR1 TDD 30kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 1 | 2 x 1 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 2Rx UE | Table 6.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 2 | 2 x 2 with static channel specified in Annex B.1 in TS 38.101-4 | New |

FR2 TDD 120kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | Table 8.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 100 / 120 | Table 1 | 2 | 2 x 2 ULA high | Reuse |

## CQI reporting requirements in fading condition

FR1 FDD 15kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 1 ULA high | New |
| 2Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 2 ULA high | New |

FR1 TDD 30kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.2.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 1 ULA high | New |
| 2Rx UE | Table 6.2.2.2.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 2 ULA high | New |

FR2 TDD 120kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | Table 8.2.2.2.2.1-1 Tests 1/2 | Wideband CQI reporting test | 100 / 120 | Table 1 | 1 | 2 x 2 ULA high | Reuse |

## PMI reporting requirements

FR1 FDD 15kHz

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 6.3.2.1.1-1 Test 1 | Single PMI, Type I, rank 1 | 10 / 15 | 4 | 4 x 1 ULA high | New |
| 2Rx | Table 6.3.2.1.1-1 Test 1 | Single PMI, Type I, rank 1 | 10 / 15 | 4 | 4 x 2 XP high | Reuse |

FR1 TDD 30kHz

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 6.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 20 / 30 | 4 | 4 x 1 ULA high | New |
| 2Rx | Table 6.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 20 / 30 | 4 | 4 x 2 XP high | New |

FR2 TDD 120kHz

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 2Rx | Table 8.3.2.2.1-1 Test 2 (TDD FR2.120-1) | Single PMI, Type I, rank 1 | 100 / 120 | 2 | 2 x 2 ULA low | Reuse |

## RI reporting requirements (If agreed)

FR1 FDD 15kHz

| 1Rx UE or 2Rx UE | Reference | CBW / SCS | CQI table | SNR test point | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 2Rx | Table 6.4.2.1-1 Tests 1 | 10MHz / 15KHz | Table 1 | 0dB | 2x2 ULA Low | New |
| 2Rx | Table 6.4.2.1-1 Tests 2 | 10MHz / 15KHz | Table 1 | 20dB | 2x2 ULA Low | New |
| 2Rx | Table 6.4.2.1-1 Tests 3 | 10MHz / 15KHz | Table 1 | 20dB | 2x2 ULA High | New |

FR1 TDD 30kHz

| 1Rx UE or 2Rx UE | Reference | CBW / SCS | CQI table | SNR test point | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 2Rx | Table 6.4.2.2-1 Tests 1 | 20MHz / 30KHz | Table 1 | 0dB | 2x2 ULA Low | New |
| 2Rx | Table 6.4.2.2-1 Tests 2 | 20MHz / 30KHz | Table 1 | 20dB | 2x2 ULA Low | New |
| 2Rx | Table 6.4.2.2-1 Tests 3 | 20MHz / 30KHz | Table 1 | 20dB | 2x2 ULA High | New |

FR2 TDD 120kHz

| 1Rx UE or 2Rx UE | Reference | CBW / SCS | CQI table | SNR test point | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 2Rx | Table 8.4.2.2-1 Tests 1 | 100MHz / 120KHz | Table 1 | 0dB | 2x2 ULA Low | Reuse |
| 2Rx | Table 8.4.2.2-1 Tests 2 | 100MHz / 120KHz | Table 1 | 16dB | 2x2 ULA Low | Reuse |
| 2Rx | Table 8.4.2.2-1 Tests 3 | 100MHz / 120KHz | Table 1 | 16dB | 2x2 XP High | Reuse |

# Appendix: Spec structure for RedCap UE demodulation and CSI reporting requirements

Table 1	Specification structure for RedCap UE Demodulation performance requirements (FR1 - Conducted requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 5.1 | General |  |
| 5.1.1 | Applicability of requirements |  |
| 5.1.1.x | Applicability of requirements for RedCap UEs | New section |
| 5.2 | PDSCH demodulation requirements |  |
| 5.2.1 | 1RX requirements |  |
| 5.2.1.1 | FDD |  |
| 5.2.1.1.1 | Minimum requirements for RedCap UEs | New section |
| 5.2.1.2 | TDD |  |
| 5.2.1.2.1 | Minimum requirements for RedCap UEs | New section |
| 5.2.2 | 2RX requirements |  |
| 5.2.2.1 | FDD |  |
| 5.2.2.1.X | Minimum requirements for RedCap UEs | New section |
| 5.2.2.2 | TDD |  |
| 5.2.2.2.X | Minimum requirements for RedCap UEs | New section |
| 5.3 | PDCCH demodulation requirements |  |
| 5.3.1 | 1RX requirements |  |
| 5.3.1.1 | FDD |  |
| 5.3.1.1.1 | Minimum requirements for RedCap UEs | New section |
| 5.3.1.2 | TDD |  |
| 5.3.1.2.1 | Minimum requirements for RedCap UEs | New section |
| 5.3.2 | 2RX requirements |  |
| 5.3.2.1 | FDD |  |
| 5.3.2.1.X | Minimum requirements for RedCap UEs | New section |
| 5.3.2.2 | TDD |  |
| 5.3.2.2.X | Minimum requirements for RedCap UEs | New section |
| 5.4 | PBCH demodulation requirements |  |
| 5.4.1 | 1RX requirements |  |
| 5.4.1.1 | FDD | New section |
| 5.4.1.2 | TDD | New section |
| 5.4.2 | 2RX requirements |  |
| 5.4.2.1 | FDD | Add minimum requirement tables for RedCap UEs |
| 5.4.2.2 | TDD | Add minimum requirement tables for RedCap UEs |
| 5.5 | Sustained downlink data rate provided by lower layers |  |
| 5.5.2 | FR1 single carrier requirements for RedCap | New section |

Table 2	Specification structure for RedCap UE CSI reporting requirements (FR1 - Conducted requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 6.1 | General |  |
| 6.1.1 | Applicability of requirements |  |
| 6.1.1.x | Applicability of requirements for RedCap UEs | New section |
| 6.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 6.2.1 | 1RX requirements |  |
| 6.2.1.1 | FDD |  |
| 6.2.1.1.1 | CQI reporting definition under AWGN conditions for RedCap UEs |  |
| 6.2.1.1.1.1 | Minimum requirement for periodic CQI reporting for RedCap | New section |
| 6.2.1.1.2 | CQI reporting under fading conditions for RedCap UEs |  |
| 6.2.1.1.2.1 | Minimum requirement for wideband CQI reporting for RedCap | New section |
| 6.2.1.2 | TDD |  |
| 6.2.1.2.1 | CQI reporting definition under AWGN conditions for RedCap UEs |  |
| 6.2.1.2.1.1 | Minimum requirement for periodic CQI reporting for RedCap | New section |
| 6.2.1.2.2 | CQI reporting under fading conditions for RedCap UEs |  |
| 6.2.1.2.2.1 | Minimum requirement for wideband CQI reporting for RedCap | New section |
| 6.2.2 | 2RX requirements |  |
| 6.2.2.1 | FDD |  |
| 6.2.2.1.1 | CQI reporting definition under AWGN conditions for RedCap UEs |  |
| 6.2.1.1.1.X | Minimum requirement for periodic CQI reporting for RedCap | New section |
| 6.2.2.1.2 | CQI reporting under fading conditions for RedCap UEs |  |
| 6.2.2.1.2.X | Minimum requirement for wideband CQI reporting for RedCap | New section |
| 6.2.2.2 | TDD |  |
| 6.2.2.2.1 | CQI reporting definition under AWGN conditions for RedCap UEs |  |
| 6.2.2.2.1.X | Minimum requirement for periodic CQI reporting for RedCap | New section |
| 6.2.2.2.2 | CQI reporting under fading conditions for RedCap UEs |  |
| 6.2.2.2.2.X | Minimum requirement for wideband CQI reporting for RedCap | New section |
| 6.3 | Reporting of Precoding Matrix Indicator (PMI) |  |
| 6.3.1 | 1RX requirements |  |
| 6.3.1.1 | FDD |  |
| 6.3.1.1.1 | Single PMI with 4 TX TypeI-SinglePanel Codebook for RedCap UEs | New section |
| 6.3.1.2 | TDD |  |
| 6.3.1.2.1 | Single PMI with 4 TX TypeI-SinglePanel Codebook for RedCap UEs | New section |
| 6.3.2 | 2RX requirements |  |
| 6.3.2.1 | FDD |  |
| 6.3.2.1.X | Single PMI with 4 TX TypeI-SinglePanel Codebook for RedCap UEs | New section |
| 6.3.2.2 | TDD |  |
| 6.3.2.2.X | Single PMI with 4 TX TypeI-SinglePanel Codebook for RedCap UEs | New section |
| 6.4 | Reporting of Rank Indicator (RI) |  |
| 6.4.2 | 2RX requirements |  |
| 6.4.2.1 | FDD | FFS whether to specify requirements |
| 6.4.2.2 | TDD | FFS whether to specify requirements |

Table 3	Specification structure for RedCap UE Demodulation performance requirements (FR2 - Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.1 | General |  |
| 7.1.1 | Applicability of requirements |  |
| 7.1.1.x | Applicability of requirements for RedCap UEs | New section |

Table 4	Specification structure for RedCap UE CSI reporting requirements (FR2 - Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 8.1 | General |  |
| 8.1.1 | Applicability of requirements |  |
| 8.1.1.x | Applicability of requirements for RedCap UEs | New section |