# 3GPP TSG-RAN WG4 Meeting #103-e                                                              R4-2210435

# Electronic Meeting, May 09 – May 20, 2022

Title:	LS on UE capability and network assistant signalling for CRS interference mitigation in the scenario with overlapping spectrum for LTE and NR with 30kHz SCS

Release:	Rel-17

Work Item:	NR_demod_enh2-Perf

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact Person:

Name:	Shiyuan Wang

E-mail Address:	wangshiyuan@chinamobile.com

Attachments:

1. Overall Description:

In the past RAN4#102-e meeting, RAN4 has agreed the UE capability and network assistant signalling for CRS interference mitigation (CRS-IM) in the Scenario1 and Scenario 2 with serving cell 15kHz SCS, and the corresponding LS R4-2207238 was sent to RAN2.

In this RAN4#103-e meeting, the UE capability and network assistant signalling for CRS interference mitigation (CRS-IM) in Scenario 2 with serving cell 30kHz SCS has been discussed and agreed to be introduced. Following agreements are reached by RAN4:

- From the RAN4 perspective, the following new NR UE capabilities are agreed to be defined for CRS-IM:

- Capability #4: NR UE capable of performing CRS-IM in scenario 2 with 30kHz SCS without Rel-17 new network assistant signalling on LTE channel bandwidth.

- Capability #5: NR UE capable of performing CRS-IM in scenario 2 with 30kHz SCS with Rel-17 new network assistant signalling on LTE channel bandwidth.

- The granularity of the above capabilities is Per Feature Set per CC.

- The above capabilities are applicable for FR1 only without FDD/TDD difference.

- The above capabilities are optional for UE to report.

- With the default network configuration assumptions in LS R4-2207238 which was approved in RAN4#102-e meeting, RAN4 has agreed that for UE supporting Capability #4, the UE can perform CRS-IM without Rel-17 new RRC network assistant signalling in scenario 2 with 30 kHz SCS when MeasObjectEUTRA IE is configured and the configured measurement gaps overlap with neighbour LTE cell PBCH position.

- Meanwhile, new RRC based network assistant signalling is agreed to be introduced to assist CRS-IM in Rel-17 for Scenario 2 with serving cell 30kHz SCS, and the Rel-17 new RRC signalling are optionally to be indicated to UE supporting Capability #4.

- Regarding the content of the Rel-17 new RRC network assistant signalling, for each neighbour LTE cell, RAN4 reach following agreements for Scenario 2 with serving cell 30kHz SCS:

- Channel bandwidth

  - For scenario 2 with 30kHz SCS, the LTE channel bandwidth can be optionally signalled to UE supporting Capability #4, and should be signalled to UE supporting Capability #5 but not supporting Capability #4.

- For the other contents including downlink centre frequency, CRS port number, Cell ID, v-shift, CRS-muting, and MBSFN configuration. RAN4 agreed to reuse the same agreements for Scenario 2 with serving cell 15kHz SCS captured in last meeting LS R4-2207238.

- The above information is signalled under each serving cell with flexibility to support per UE level configuration with up to 8 interference cell information.

2. To RAN WG2 group.

ACTION: RAN4 respectfully request RAN2 to take the above information into account and design the corresponding UE capability and network assistance signalling for CRS-IM in scenario with overlapping spectrum for LTE and NR with 30kHz SCS.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #104-e 	15 Aug - 26 Aug 2022		        	        Online

TSG-RAN WG4 Meeting #104-bis-e 	10 Oct - 19 Oct 2022	                                  Online