3GPP TSG-RAN4 Meeting #104-e	R4-2211880

# Electronic Meeting, Aug. 15 – 26, 2022

Agenda item:	11.13.2

Source:	Apple

Title:   	On UE-UE CLI modeling

Document for:	Discussion

# 1	Introduction

The SI of “Study on evolution of NR duplex operation” was recently approved [1], in which inter-gNB and inter-UE CLI handling and solution are studied. RAN1 started their work and sent an LS to RAN4 asking for the CLI modeling in the following scenarios [2]:

- self-interference modelling for system level simulation

- gNB-gNB and UE-UE co-channel inter-subband CLI modelling for system level simulation

- gNB-gNB and UE-UE adjacent-channel CLI modelling for system level simulation

In this contribution, we share our views on UE-UE co-channel inter-subband CLI modelling and UE-UE adjacent-channel CLI modelling.

# 2	Discussion

## 2.1 UE-UE co-channel inter-subband CLI modelling

## In this section, we discuss the model of both TX and RX interference for the co-channel case.

## 2.1.1 TX unwanted emission

For the co-channel case, since RAN4 has specified the in-band emission requirement to consider effects of TX non-linearity, carrier leakage, IQ imbalance, TX noise, as shown in the figure below for illustration purpose only

Fig. 1: Illustration of in-band TX impairments

The in-band emission is defined as the average emission across 12 sub-carriers and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB. The requirement for FR1 is copied below:

Fig. 2: In-band emission requirement in 38.101-1

With the in-band emission requirement, the interference experienced by a victim RB can be derived. Therefore, we propose:

Proposal 1: For UE-UE co-channel inter-subband CLI modeling of TX unwanted emission, use the in-band emission requirement in 38.101-1 (FR1) and 38.101-2 (FR2).

## 2.1.2 RX selectivity/blocking

To model RX selectivity/blocking, we consider the following two factors:

- RX saturation: as we understand, if the input signal power is high, the RX is saturated. In RAN4, we have the requirement of maximum input level, above which the RX could be saturated. Therefore, any inter-suband interference higher than the maximum input level will lead to receiver degradation. It is assumed that the RX will not correctly decode the data.

- Second-order and third-order distortion (second/third-order nonlinearity of RF chain, i.e. LNA, mixer).

It is also noted that UE RX does not have any filtering to mitigate such in-channel blocker.

Therefore, we propose the following modeling in proposal 2:

Proposal 2: For UE-UE co-channel inter-subband CLI modeling of RX selectivity/blocking, use the current maximum input level specified in RAN4 as a threshold:

- If inter-subband interference is higher than the threshold, it is assumed it will result large receiver degradation and hence the RX will not correctly decode the data

- For inter-subband interference that is smaller than the threshold, treat the blocker as interference, i.e. consider a dB-to-dB increase of interference due to blocker power

## 2.2 UE-UE adjacent-channel CLI modelling

## In this section, we discuss the model of both TX and RX interference for the adjacent-channel case.

## 2.2.1 TX unwanted emission

To TX unwanted emission into adjacent channel, there are existing SEM/ACLR requirements to limit emission due to TX non-linearity, carrier leakage, IQ imbalance, TX noise. Since SEM provides the absolute emission level while ACLR provides a relative emission with respect to TX power, ACLR is more suitable for the modeling. Since ACLR is specified with the assumption of adjacent channel bandwidth being equal to the assigned channel, it needs some adaptation to make it apply to UE transmission that does not occupy the entire channel. We use the following figure to show our proposed model.

Fig. 3: Model of TX unwanted emission in adjacent channel

In Fig. 3, it is proposed to have two ACLR levels, ACLR_level_1 is used for the RB allocation (of the same number of RBs as for interfering RB allocation) in the victim channel next to the interfering RB allocation, and ACLR_level_2 is used for the rest of the victim channel for each RB allocation of the same number of RBs as for interfering RB allocation. Given the ACLR for PC3 is 30dB, the values of ACLR_level_1 and ACLR_level_2 are discussed and agreed. A starting point is 28dB and 33dB, respectively.

If the victim RB allocation has a different number of RBs than that for the interfering RB allocation, the emission power can be scale up or down accordingly with the use of ACLR_level_1 and ACLR_level_2.

Proposal 3: For UE-UE adjacent-channel CLI modeling of TX unwanted emission, use the two ACLR level model shown in Fig. 3.

## 2.2.2 RX selectivity/blocking

There are quite a few factors that affect RX SNR due to RX non-linearity in the presence of adjacent channel blocker.

- DC or near DC frequency components due to RX second-order non-linearity

- Cross modulation of the blocker due to third-order non-linearity

- RX reciprocal mixing

- BB filtering

In addition, the receiver SNR may suffer further degradation due to receiver gain backoff in the presence of a blocker in the adjacent channel.

Current RAN4 core requirements only covers blockers with either 5MHz bandwidth or channel bandwidth of the wanted signal. As such, it is hard to develop an selectivity/blocking model relative to frequency offset between victim RBs and the blocker. It is also noted that in the core requirement, the highest power of the blocker is -25dBm.

As such, we have the following model:

Proposal 4: For UE-UE adjacent-channel inter-subband CLI modeling of RX selectivity/blocking, use the following model:

- If the blocker is higher than -25dBm, it is assumed it will result large receiver degradation and hence the RX will not correctly decode the data

- For the blocker that is smaller than -25dBm, use the ACS values to calculate the resulting interference

- In addition, consider a 5dB SNR degradation due to receiver gain backoff

# 3	Conclusions

In this contribution, we make the following proposals on providing the CLI modeling:

Proposal 1: For UE-UE co-channel inter-subband CLI modeling of TX unwanted emission, use the in-band emission requirement in 38.101-1 (FR1) and 38.101-2 (FR2).

Proposal 2: For UE-UE co-channel inter-subband CLI modeling of RX selectivity/blocking, use the current maximum input level specified in RAN4 as a threshold:

- If inter-subband interference is higher than the threshold, it is assumed it will result large receiver degradation and hence the RX will not correctly decode the data

- For inter-subband interference that is smaller than the threshold, treat the blocker as interference, i.e. consider a dB-to-dB increase of interference due to blocker power

Proposal 3: For UE-UE adjacent-channel CLI modeling of TX unwanted emission, use the two ACLR level model shown in Fig. 3.

Proposal 4: For UE-UE adjacent-channel inter-subband CLI modeling of RX selectivity/blocking, use the following model:

- If the blocker is higher than -25dBm, it is assumed it will result large receiver degradation and hence the RX will not correctly decode the data

- For the blocker that is smaller than -25dBm, use the ACS values to calculate the resulting interference

- In addition, consider a 5dB SNR degradation due to receiver gain backoff

# 4	References

RP-220974, "Revised SID: Study on evolution of NR duplex operation", CMCC

R1-2205543, "LS on interference modelling for duplex evolution ", CMCC