3GPP TSG-RAN WG4 Meeting # 104-e                                  R4-2211918

Electronic Meeting, August 15 – August 26, 2022

Source: 	Apple

Title: 	Overview on RRM requirements for ATG

Agenda Item:	11.12.4

Document for:	Discussion

# Introduction

An Rel-18 WI on ATG was approved in [1]. The following scopes are relevant for development of RRM requirements.

- Specify features to core specifications for ATG BS and UE [RAN4]

  - Scenario:

    - BS on the ground, and the CPE type of UE mounted in the aircraft

    - A direct radio link between BS on the ground and CPE type of UE mounted in the aircraft

    - Note: The deployment characteristics described in the justification section shall be taken as a basis for the technical discussion.

  - Specify core requirements for coexistence between ATG and IMT terrestrial network

    - Specify RRM core requirements for ATG UE

      - Taking into account identified differences between ATG and fully ground based systems

      - Considering the different nature of ATG UEs and their view of the network, increased cell sizes and other relevant aspects

    - Specify new UE/BS type(s) for ATG network if necessary

This contribution will present an overview on the requirement structure and initial analysis on the RRM requirements for ATG.

# Discussion

| Sub-clause | Requirement | Comments |
| --- | --- | --- |
| 3 | Definitions, symbols and abbreviations |  |
| 3.5 | Frequency band grouping | Depending on potential different REFSENS requirement for ATG UE, new grouping might be needed. |
| 3.6 | Applicability of requirements |  |
| 4 | RRC_IDLE state mobility |  |
| 4.1 | Cell Selection | No impact observed |
| 4.2 | Cell Res-election | Take the current HST requirement as the starting point and check what need to be further enhanced. |
| 4.3 | Minimization of Drive tests (MDT) | SDT feature is not as meaningful for ATG UE as for the TN UE. Not applicable in this release. |
| 4.4 | Idle Mode CA/DC measurement | Not applicable in this release. Focus on single CC only. |
| 5 | RRC_INACTIVE state mobility |  |
| 5.1 | Cell Re-selection |  |
| 5.3 | Minimizaiton of Drive Test (MDT) | SDT feature is not as meaningful for ATG UE as for the TN UE. Not applicable in this release. |
| 5.4 | Idle Mode CA/DC Measurement | Not applicable in this release. Only focused on single CC. |
| 5.5 | Configured Grant based small date transitions | Not applicable in this release. For future study if needed. |
| 5.6 | NR measurements for positioning | Not applicable in this release. For future study if needed. |
| 6 | RRC_CONNECTED state mobility |  |
| 6.1 | Handover | - Only intra-frequency HO and [inter-frequency HO] need to be defined. - There is no need to define Handover to other RATs. - FFS whether to include DAPS and NR conditional handover in this release. - No need to discuss NR handover with PSCell in this release. - CCA related requirement is not relevant for this WI. |
| 6.2 | RRC Connection Mobility Control | - RRC Re-establishment delay need to be considered? - Radom access requirement is needed. - RRC connection release with redirection is needed |
| 7 | Timing |  |
| 7.1 | UE transmit timing | - FFS if UE specific TA shall be considered in the Te requirement design, like in NTN - The Tp/Tq needs to be revisited considering the high UE moving speed as well as the timing drifting at UE |
| 7.2 | UE timer accuracy | - No impact on the timer accuracy is observed |
| 7.3 | Timing advanced | - FFS on the necessity of considering the open loop TA (UE specific TA if needed) and close loop (TAC based adjustment) for the TA adjustment requirement, like in NTN. |
| 7.4 | Cell phase synchronization | This requirement is necessary, and the legacy TN requirement can be reused or tightened |
| 7.5 | Maximum Transmission Timing Difference | This requirement is not needed in this release. Only consider single CC operation. |
| 7.6 | Maximum Receiving Time Difference | This requirement is not needed in this release. Only consider single CC operation. |
| 7.7 | deriveSSB-IndexFromCell tolerance | The time misalignment tolerance for ‘deriveSSB-IndexFromCell= true’ shall be revisited due to the extreme large radius of ATG cell. |
| 7.9 | deriveSSB-IndexFromCell tolerance | This section was introduced in MG enhancement WI which is not needed for ATG UE |
| 8 | Signaling characteristics |  |
| 8.1 | Radio link monitoring | FFS: if the existing RLM evaluation and L1 interval is applicable to ATG UE. |
| 8.2 | Interruption | No interruption needs to be considered due to single CC operation in this release. |
| 8.3 | SCell Activation and Deactivation Delay | No interruption needs to be considered due to single CC operation in this release. |
| 8.4 | UE UL carrier RRC reconfiguration delay |  |
| 8.5 | Link Recovery Procedure | FFS: if the existing BFD/CBD evaluation and BFD L1 interval is applicable to ATG UE. |
| 8.6 | Active BWP switching delay | No impact observed. The current requirement can be reused. |
| 8.8 | EN-DC: U-UTRAN PSCell Addition and Release | Not applicable due to single CC operation in this release. |
| 8.9 | NR-DC: PSCell Addition and Release Delay | Not applicable due to single CC operation in this release. |
| 8.10 | Active TCI state switching delay | FFS: if the existing TCI switching requirement is applicable to ATG UE. |
| 8.11 | PSCell Change | Not applicable due to single CC operation in this release. |
| 8.12 | Uplink spacial relation switch delay | NA |
| 8.13 | UE-spcific CBW change | No impact observed |
| 8.14 | Pathloss reference signal switching delay | FFS: if the existing PL-RS switching requirement is applicable to ATG UE. |
| 8.15 | Active downlink TCI state switch | No impacted identified. The current requirement could be reused. |
| 8.16 | Active uplink TCI state switching delay for unified TCI | NA. Since this is from R16 eMIMO |
| 8.17 | SCG Activation and Deactivation Delay | Not applicable due to single CC operation in this release. |
| 8.18 | TRP specific Link Recovery Procedures | NA. Since this is from R16 eMIMO |
| 8.19 | Pre-configured measurement GAP activation/deactivation | GAP enhancement contents are not considered for ATG UE in Rel-18. |
| 9 | Measurement procedure |  |
| 9.1 | General measurement requirement | GAP design and related capability/scaling needs to be reconsidered. |
| 9.2 | NR intra-frequency measurement | This requirement needs to be defined for ATG UE. FFS on the details |
| 9.3 | NR inter-frequency measurement | This requirement needs to be defined for ATG UE. FFS on the details. |
| 9.4 | Inter-RAT measurement | This requirement is not needed. Only single 5G NR mode is in the scope. |
| 9.5 | L1-RSRP measurement for reporting | Similar requirement is applicable for ATG UE |
| 9.6 | NE-DC: measurement | Not applicable due to single CC operation in this release. |
| 9.7 | Cross Link Interference measurement | To avoid interference, the network should be based on synchronized operation. Hence, this requirement is not necessary for ATG-UE. |
| 9.8 | L1-SINR measurement for reporting | Similar requirement is needed for ATG UE. |
| 9.9 | NR measurement for positioning | Not applicable in this release. FFS for future release. |
| 9.13 | L1-RSRP measurement for a cell with different PCI from serving cell |  |
| 10 | Measurement performance |  |
| 10.1 | NR measurement | The following measurement performance are needed. - Intra-frequency RSRP accuracy requirement for FR1 - Inter-frequency RSRP accuracy requirement for FR1 - Intra-frequency RSRQ accuracy requirement for FR1 - Inter-frequency RSRQ accuracy requirement for FR1 - Intra-frequency SINR accuracy requirement for FR1 - Inter-frequency SINR accuracy requirement for FR1 - Power headroom - Pcmax,c,c - L1-RSRP accuracy requirements for FR1 - SFTD accuracy requirements - CLI measurement accuracy requirement?? - Measurements for E-UTRAN and UTRAN FDD are not needed. |

Based on the discussion above, the following observations and proposals are concluded.

Observation: ATG UE is a special UE in terms of operating scenarios and potential different UE behaviours.

Proposal 1: It is proposed to define a basic RRM requirement for single CC operation in Rel-18. E.g. CA/DC/enhanced features like MDT are not considered.

Proposal 2: It is proposed to define RRM requirement for ATG UE in separate subclause

Proposal 4: It is proposed that ATG UE in R18 is GNSS capable.

# Summary

This contribution presented an overview on ATG UE RRM requirements and the following observations and proposals are concluded.

Observation: ATG UE is a special UE in terms of operating scenarios and potential different UE behaviours.

Proposal 1: It is proposed to define a basic RRM requirement for single CC operation in Rel-18. E.g. CA/DC/enhanced features like MDT, MG enhancement are not considered.

Proposal 2: It is proposed to define RRM requirement for ATG UE in separate subclause

Proposal 4: It is proposed that ATG UE in R18 is GNSS capable.

# References

[1] RP-220962, Air-to-ground network for NR, CMCC