3GPP TSG-RAN WG4 Meeting #104-e	R4-2211994

# Electronic Meeting, August 15 – August 26, 2022

Title:	Handling of humidity inconsistency among specifications

Source:	Samsung

Agenda item:	13.3.2

Document for:	Approval

1.	Introduction

RAN5 identified inconsistency among specifications regarding humidity limit for normal condition, i.e., there is lower humidity limit for NTC in some specifications, but no lower limit in some other specifications. And RAN5 sends RAN4 LS in [1] asking RAN4 why lower humidity limit differs and detailed consideration to keep lower humidity limit in core specifications:

To RAN4 group.

ACTION:

- RAN WG5 respectfully requests RAN WG4 to provide the reason why the lower humidity limit in normal temperature test environment differs in GERAN/UTRAN/E-UTRAN/NR. Are there any detail considerations to keep the lower humidity limit for normal test environment?

In this contribution we provide our views on humidity limit for normal condition together with a reply LS.

2. 	Discussion

Besides the humidity inconsistency cases mentioned in the RAN5 LS [1], more inconsistency cases have been identified, as summarized in our previous contribution [2] captured in below table:

Table 1. Summary of humidity inconsistency among specifications

| Humidity requirement | Related specifications |
| --- | --- |
| With humidity range 0% ~ 75% | TS 51.010-1, TS 36.508 v16.7.0, ETSI EN 301 908-13, etc. |
| With humidity range 25% ~ 75% | TS 36.101, TS 38.101-1, TS 38.101-2, etc. |
| Without humidity range (room temperature) | TS 37.144, TS 373.154, TS 38.161, etc. |

In last RAN4 meeting, companies expressed divergent views on how to handling the specification inconsistency issue, including following options [3]:

- Option 1: replace the mention of 25% - 75% with “(under room humidity conditions unless otherwise stated)” - Option 2: RAN4 will not update the definition of test environment. RAN5 is encouraged to continue discussing and deciding whether the same approach could apply to 5G NR in the context of conformance testing. - Option 3: Remove the detail description of relative humidity in normal test conditions. Add a note to indicate that the relative humidity condition will be declared by manufacturers and recorded in the test report. - Option 4: Keep the current description of relative humidity in normal test conditions as it is now in RAN4 specs with the range of “25% ~ 75%”. - Option 5: For normal conditions, with relative humidity within the range 25% to 75% or 0% to 75%

Though companies’ views are divergent, it seems that most companies consider it necessary to resolve the specification inconsistency issue. Indeed it is not good for 3GPP to have inconsistent requirements among specifications. No matter what detailed solution it would be, we propose to establish a consensus to resolve the specification inconsistency issue first of all.

Proposal 1:	it is agreed in RAN4 that the humidity inconsistency issue among specifications should be resolved.

As discussed previously, there is no much performance dependency with humidity in normal room test except for some special cases e.g. ESD.  So it is not necessary to specify exact humidity range for so many requirements which are not sensitive to humidity.

In some specifications (TS 37.144, TS 373.154, TS 38.161, etc.), the temperature is only required with “room temperature” with and example temperature recommended but not mandated:

A.2.2 Temperature All the OTA requirements are applicable in room temperature e.g. 25C.

Following similar rule, it is enough to just require with “room humidity” as a side condition for normal temperature. So we propose to remove the explicit humidity range, i.e., to remove the both lower limit and upper limit. Instead, normal temperature test is required to be performed under room humidity condition unless otherwise stated.

Proposal 2:	It is proposed to remove the explicit humidity range and normal temperature test is required to be performed under room humidity condition unless otherwise stated.

| +15C to +35C | for normal conditions (under room humidity conditions unless otherwise stated) |
| --- | --- |
| -10C to +55C | for extreme conditions (see IEC publications 68-2-1 and 68-2-2) |

A draft reply LS is provided in Annex.

3. 	Conclusion

Proposal 1:	it is agreed in RAN4 that the humidity inconsistency issue among specifications should be resolved.

Proposal 2:	It is proposed to remove the explicit humidity range and normal temperature test is required to be performed under room humidity condition unless otherwise stated.

| +15C to +35C | for normal conditions (under room humidity conditions unless otherwise stated) |
| --- | --- |
| -10C to +55C | for extreme conditions (see IEC publications 68-2-1 and 68-2-2) |

A draft reply LS is provided in Annex.

4. 	References

- R5-221604 (R4-2207623)    “LS on lower humidity limit in normal temperature test environment”, Source: RAN5, To: RAN4

- R4-2208487    “Handling of humidity inconsistency among specifications”, Samsung

- R4-2210467    “Email discussion summary for [103-e][135] NR_reply_LS_UE_RF_Canadan77”, Moderator (Apple)

5. 	Annex: draft Reply LS

Title:	[Draft] Reply LS on lower humidity limit in normal temperature test environment

Response to:	R5-221604 / R4-2207623

Release:	Release 15

Work Item:	5GS_NR_LTE-UEConTest

Source:	RAN4

To:	RAN5

Cc:	N/A

Contact Person:

Name: 	Bozhi Li

E-mail Address: 	bozhi.li@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

RAN4 would like to thank RAN5 for the LS (R5-221604 / R4-2207623) on lower humidity limit in normal temperature test environment.

According to discussion in RAN4, the issue of humidity inconsistency among specifications does exist. Besides the cases mentioned in R5-221604, RAN4 identifys other cases of inconcistency. Refer to following for the summary of humidity inconsistency among specifications.

Table 1. Summary of humidity inconsistency among specifications

| Humidity requirement | Related specifications |
| --- | --- |
| With humidity range 0% ~ 75% | TS 51.010-1, TS 36.508 v16.7.0, ETSI EN 301 908-13, etc. |
| With humidity range 25% ~ 75% | TS 36.101, TS 38.101-1, TS 38.101-2, etc. |
| Without humidity range (room temperature) | TS 37.144, TS 373.154, TS 38.161, etc. |

RAN4 agrees that it is necessary to align the humidity condition among specifications. It is RAN4 understanding that performance dependency due to humidity is negligable for RF core requirements. There may be some dependency for minority specification e.g. ESD in EMC specification. After discussion, it is concluded in RAN4 that RAN4 will replace the inconsistent humidity range with room humidity unless otherwise stated in core specifications.

2. Actions:

To RAN5

ACTION: 	RAN4 respectfully asks RAN5 to take above RAN4 answers into consideration for the future work.

3. Date of Next TSG-RAN WG4 Meetings:

3GPP RAN4#104-bis-e	10 – 19 Oct 2022	Online