3GPP TSG RAN WG4 Meeting #104		R4-2212343

Electronic meeting, 15 – 26th August 2022

Agenda item:	11.17.2

Source:	Apple

Title:	Potential RF related issues for the MUSIM enhancements

WI/SI:	NR_DualTXRX_MUSIM

Release:	Rel-18

Document for:	Decision

# 1	Introduction

Multi-SIM operation (hence MUSIM) is a new feature that was initially standardised by 3GPP in Rel-17 [1] allowing more consistent UE behaviour when the latter must handle two active SIM cards. The Rel-17 MUSIM enhancements main goal was the Dual-Rx/Single-Tx scenario when a UE listens to two SIM cards in DL, but the UL is anyway limited to a single transmission. In Rel-18 a new WI was agreed [2], which aims at adding further enhancements to the MUSIM operation. More precisely, simultaneous transmission over two UL connections will be enabled ensuring fewer interruptions while communicating to both networks.

In this discussion paper we raise several potential RF related issues for the MUSIM operation presenting the corresponding technical background and design challenges.

# 2	Potential RF related issues

## 2.1	Conflicting bands and band combinations for MUSIM

Purely from the UE RF design perspective, MUSIM operation with two active UL connections for both SIM cards can be logically perceived as some form of the (un-synchronised) UL carrier-aggregation, whereupon depending on the exact location of operators’ frequencies it can be either inter-band, intra-band non-contiguous or intra-band contiguous. As per existing framework defined by RAN WG4, it is not possible to assume that a UE will be able to handle all possible band/carrier combinations, which a UE may end up to while operating two active SIM cards. In case of the legacy multi-carrier feature, the network is aware of the UE capabilities and thus can choose the corresponding band/carrier combinations if the simultaneous operation over several bands/carrier is needed. However, in case of the MUSIM case, there are two networks which are not even aware of each other’s presence. Thus, it is not possible to assume a UE supporting two UL active connections over two SIM cards will be able to utilise them in all possible band/carrier combinations where a UE can operate in single network mode.

This is not a completely new issue for the MUSIM feature and a similar problem was already raised back in Rel-17 even in the context of the Dual-Rx/Single-Tx scenario. As explained in the corresponding contribution [3], even when a UE has a single UL transmission, there still can be issue with Tx from the first SIM falling into the Rx of the second SIM. Such a problem exists in the multi-carrier scenario as well, whereupon the network is aware of such a limitation and can either avoid configuring such a combination or avoid scheduling simultaneous Rx and Tx in both bands. However, as mentioned earlier, there are two independent networks where one network is not aware of another network configuration. As already discussed in Rel-17, there is a need for the mechanism to resolve MUSIM band conflicts.

Proposal 1:	Address the MUSIM related RF issue when for the uninterrupted operation a UE should use particular band/carrier combinations for two SIM cards.

.

## 2.2	Power back-off for MUSIM

Another potential issue for the Dual-Rx/Dual-Tx MUSIM operation is the required power back-off that a UE may apply while operating two UL active connections for two SIM cards. In fact, there are several core reasons, which are quickly summarised below, why a UE may resort for applying a large(-er) Tx power back-off:

-	Total Tx power. A UE design has a limit on the total maximum Tx power that the device can output. Irrespective of the actual device power class, e.g. +23dBm or +26dBm, there can be a situation when a UE ends up to the cell edge as perceived by both networks, where a UE has registered with two SIM cards. In this case every network may instruct the UE to operate at its maximum Tx power, but a UE will not be able to do that because the Tx power will have to be shared between two UL transmissions. In fact, it is enough to be on the cell edge of just one network, which most likely will be instructing a UE to use the maximum UL power. However, from an individual network perspective a UE will be applying power back-off that goes beyond the limits of what existing A-MPR margins allow.

-	Inter-modulation.  Another potential reason for the extra power back-off is the intermodulation issues between two UL transmissions. RAN WG4 framework already accounts for the inter-modulation issues in certain UL CA  configurations and allows the UE to apply extra power back-off in addition to the A-MPR margin. However, in the MUSIM case one network is not aware of the second network or even the second UL transmission. Thus, from an individual network perspective a UE will be applying power back-off, which is larger than what the existing specifications allow for the single UL case.

As can be seen, irrespective of the root cause, there are several reasons why the MUSIM with two UL active connections may apply power back-off, which from an individual network perspective will be larger than existing MPR and/or A-MPR limits.

Proposal 2:	Address the MUSIM related RF issue when for the uninterrupted operation a UE should apply power back-off larger than existing MPR/A-MPR limits.

# 3	Conclusions

In this discussion paper we have presented our initial considerations for two potential RF related issues for the MUSIM Dual-Rx/Dual-Tx enhancements. As presented in the paper, the MUSIM UE operating two SIM cards with two independent networks not aware of each other configuration may easily end up with a band/carrier configuration when two UL active connections cannot be supported. Furthermore, even if a UE can potentially support two UL active connections, there is a probability that a UE will have to apply large(-er) Tx power back-off that will exceed A-MPR margins as perceived by each network.

Proposal 1:	Address the MUSIM related RF issue when for the uninterrupted operation a UE should use particular band/carrier combinations for two SIM cards.

Proposal 2:	Address the MUSIM related RF issue when for the uninterrupted operation a UE should apply power back-off larger than existing MPR/A-MPR limits.

# 4	References

RP-213679, "Revised WID: Core part: Support for Multi-SIM devices for LTE/NR", vivo, China Telecom, China Unicom

RP-220955, "Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR", vivo

R2-2202518, "Additional issues related to MUSIM - Aspects of MUSIM RRC Band Conflict, Processing Delay and Caller ID retrieval requirements", Apple