3GPP TSG-RAN WG4 Meeting # 104-e                	R4-2212675

Electronic Meeting, August 15 – 26, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PUSCH simulation results for the extension to 71 GHz

Agenda item: 	9.14.8.3.1

Document for: 	Discussion

Introduction

This paper presents PUSCH simulation results for the definition of demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Simulation assumptions

Simulation assumptions are shown below and are derived considering the agreements on the WF [1].

Table 1 Common simulation parameters

| Parameter | value |
| --- | --- |
| Transform precoding | Disabled and enabled |
| CP | Normal CP |
| Carrier frequency | 70 GHz |
| DMRS symbols | [1 9] |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| SCS | 120, 480, and 960 kHz |
| TDD pattern | 120 kHz: 3D1S1U, S=10D:2G:2U 480 kHz: 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U 960 kHz: 29D3S8U, S1=10D:4G:0U, S2=0D:12G:2U, S3=0D:0G:14U |
| DMRS type | Type 1 with single-symbol DM-RS (l0=2) |
| Number of DMRS symbols | 1+1 |
| Number of DM-RS CDM groups with data | 2 |
| Symbol length | 10 |
| Start symbol index | 0 |
| PRTS | On |
| HARQ | ON |
| Time domain resource allocation type | type B |
| Frequency domain resource without transform precoding | For 120 kHz SCS and - CHW= 100 MHz: 66 - CHW= 400 MHz: 264 For 480 kHz SCS and - CHW= 400 MHz: 66 - CHW= 1600 MHz: 248 For 960 kHz SCS and - CHW= 400 MHz: 33 - CHW= 1600 MHz: 124 - CHW= 2000 MHz: 148 |
| Frequency domain resource without transform precoding | For 120 kHz SCS and - CHW= 100 MHz: 64 - CHW= 400 MHz: 256 For 480 kHz SCS and - CHW= 400 MHz:64 - CHW= 1600 MHz: 240 For 960 kHz SCS and - CHW= 400 MHz: 32 - CHW= 1600 MHz: 120 - CHW= 2000 MHz: 144 |
| MCS index | MCS 4, 16, 20 |
| Propagation condition | TDLA10-200 TDLA10-650 TDLD10-200 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz; CBW = 400 MHz SCS = 480 kHz; CBW = 400 MHz and 1600 MHz SCS = 960 kHz; CBW = 400 MHz |
| Phase noise model | 38808 Set2 |
| Number of HARQ transmissions | 4 |
| RV sequence | {0.2.3.1} |
| Testing metric | SNR at 70% of the maximum throughput |
| Phase noise compensation | CPE and ICI compensation CPE only in some test cases |

Simulation results

Table 2 Simulation results with transform precoding disabled and 120 kHz SCS

| SCS | CBW | MCS | Channel model | PN compensation | SNR @ 70% TPUT |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA10-650 | CPE+ICI | -3.27 |
|  |  | 16 | TDLA10-200 | CPE+ICI | 8.94 |
|  |  |  | TDLA10-650 | CPE+ICI | 8.19 |
|  |  |  | TDLA10-200 | CPE only | 8.94 |
|  |  | 20 | TDLD10-200 | CPE+ICI | 11.78 |
|  |  |  | TDLD10-200 | CPE only | 13.57 |
|  | 400 | 4 | TDLA10-650 | CPE+ICI | -3.05 |
|  |  | 16 | TDLA10-200 | CPE+ICI | 9.36 |
|  |  |  | TDLA10-650 | CPE+ICI | 8.49 |
|  |  |  | TDLA10-200 | CPE only | 9.36 |
|  |  | 20 | TDLD10-200 | CPE+ICI | 12.16 |
|  |  |  | TDLD10-200 | CPE only | 14.30 |

Table 3 Simulation results with transform precoding disabled and 480 kHz SCS

| SCS | CBW | MCS | Channel model | PN compensation | SNR @ 70% TPUT |
| --- | --- | --- | --- | --- | --- |
| 480 | 400 | 4 | TDLA10-650 | CPE+ICI | -3.43 |
|  |  | 16 | TDLA10-200 | CPE+ICI | 8.50 |
|  |  |  | TDLA10-650 | CPE+ICI | 7.88 |
|  |  |  | TDLA10-200 | CPE only | 8.50 |
|  |  | 20 | TDLD10-200 | CPE+ICI | 10.98 |
|  |  |  | TDLD10-200 | CPE only | 11.74 |
|  | 1600 | 4 | TDLA10-650 | CPE+ICI | -3.36 |
|  |  | 16 | TDLA10-200 | CPE+ICI | 8.61 |
|  |  |  | TDLA10-650 | CPE+ICI | 8.08 |
|  |  |  | TDLA10-200 | CPE only | 8.61 |
|  |  | 20 | TDLD10-200 | CPE+ICI | 11.78 |
|  |  |  | TDLD10-200 | CPE only | 12.30 |

Table 4 Simulation results with transform precoding disabled and 960 kHz SCS

| SCS | CBW | MCS | Channel model | PN compensation | SNR @ 70% TPUT |
| --- | --- | --- | --- | --- | --- |
| 960 | 400 | 4 | TDLA10-650 | CPE+ICI | -3.01 |
|  |  | 16 | TDLA10-200 | CPE+ICI | 8.02 |
|  |  |  | TDLA10-650 | CPE+ICI | 7.82 |
|  |  |  | TDLA10-200 | CPE only | 8.02 |
|  |  | 20 | TDLA10-650 | CPE+ICI | 11.13 |
|  |  |  | TDLA10-650 | CPE only | 11.40 |
|  | 1600 | 16 | TDLD10-200 | CPE+ICI | 8.19 |
|  |  |  | TDLD10-200 | CPE only | 8.40 |
|  |  | 20 | TDLD10-200 | CPE+ICI | 12.05 |
|  |  |  | TDLD10-200 | CPE only | 12.24 |
|  | 2000 | 4 | TDLA10-650 | CPE+ICI | -3.44 |
|  |  | 16 | TDLA10-200 | CPE+ICI | 8.40 |
|  |  |  | TDLA10-650 | CPE+ICI | 8.51 |
|  |  |  | TDLA10-200 | CPE only | 8.71 |
|  |  | 20 | TDLD10-200 | CPE+ICI | 11.99 |
|  |  |  | TDLD10-200 | CPE only | 12.19 |

Table 5 Simulation results with transform precoding enabled

| SCS | CBW | MCS | Channel model | PN compensation | SNR @ 70% TPUT |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA10-650 | CPE+ICI | -2.78 |
|  | 400 | 4 | TDLA10-650 | CPE+ICI | -2.01 |
| 480 | 400 | 4 | TDLA10-650 | CPE+ICI | -3.26 |
|  | 1600 | 4 | TDLA10-650 | CPE+ICI | -2.58 |
| 960 | 400 | 4 | TDLA10-650 | CPE+ICI | -2.89 |
|  | 1600 | 4 | TDLA10-650 | CPE+ICI | -2.68 |
|  | 2000 | 4 | TDLA10-650 | CPE+ICI | -2.76 |

References

- R4-2210664, WF on general and BS aspects for FR2-2 demodulation requirements, Intel Corporation

- R4-2207223, WF on demodulation performance requirements definition for 52.6 - 71 GHz, Intel Corporation