3GPP TSG-RAN WG4 Meeting #104-e	R4-2212894

Electronic Meeting, 15 August – 26 August 2022

Agenda Item:	9.18.5.2.1

Source:	Ericsson

Title:	CQI reporting requirements for RedCap

Document for:	Discussion

# 1	Introduction

RAN4#103-e agreed with WF on RedCap UE demodulation and CQI reporting requirements [1]. This contribution provides our simulation results and discusses the open issues on CQI reporting requirements for RedCap.

# 2	Test points of CQI reporting in fading condition

Agreement: Test metric and pass/fail criteria for CQI reporting test in fading condition for 1Rx/2Rx UE - Reuse the same test metrics and pass/fail criteria as existing WB CQI reporting tests in fading channel for RedCap UE: - A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time - The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ. - When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to 0.02. - Set α=20%, γ=1.05 Agreement: Test points for CQI reporting test in fading condition for 2Rx - Lower SNR test points - Option 1: SNR=5/6 dB - Option 2: SNR=6/7 dB - Higher SNR test points: - SNR=12/13dB Agreement: Test points for CQI reporting test in fading condition for 1Rx - Set SNR test points 3 dB higher than 2Rx case

One of the remaining open issues for the CQI reporting test is the lower SNR test point for fading condition.  Table 2 shows our simulation result of the CQI reporting in fading condition according to the simulation parameters in Table 1, where the applied CQI-MCS mapping table is shown in Table 4.

Table 1	Simulation parameters for CQI reporting test in fading condition.

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | FDD 15KHz Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 2 ULA high | New |

Table 2	Simulation results of CQI reporting test in fading condition for 2Rx FDD SCS=15kHz.

|  | Median CQI | Percentage of reported CQI Index not in {medCQI-1, medCQI+1} (α) | BLER with following CQI | TP ratio with following CQI and Median CQI (γ) |
| --- | --- | --- | --- | --- |
| 0 | 6 / QPSK | 0.48 | 0.11 | 1.48 |
| 1 | 6 / QPSK | 0.49 | 0.11 | 1.49 |
| 2 | 7 / 16QAM | 0.51 | 0.12 | 1.46 |
| 3 | 7 / 16QAM | 0.51 | 0.11 | 1.47 |
| 4 | 8 / 16QAM | 0.50 | 0.11 | 1.42 |
| 5 | 9 / 16QAM | 0.53 | 0.12 | 1.46 |
| 6 | 9 / 16QAM | 0.53 | 0.13 | 1.37 |
| 7 | 10 / 64QAM | 0.52 | 0.14 | 1.52 |
| 8 | 10 / 64QAM | 0.52 | 0.15 | 1.43 |
| 9 | 11 / 64QAM | 0.50 | 0.16 | 1.44 |
| 10 | 11 / 64QAM | 0.51 | 0.16 | 1.34 |
| 11 | 12 / 64QAM | 0.48 | 0.16 | 1.45 |
| 12 | 12 / 64QAM | 0.48 | 0.17 | 1.32 |
| 13 | 13 / 64QAM | 0.40 | 0.16 | 1.50 |
| 14 | 13 / 64QAM | 0.37 | 0.14 | 1.37 |
| 15 | 14 / 64QAM | 0.30 | 0.15 | 1.58 |
| 16 | 14 / 64QAM | 0.23 | 0.13 | 1.43 |

Option 2 means RAN4 sets the same SNR test point as the existing CQI reporting test defined in Rel-15 with CQI table 2. On the other hand, we think Option 1 adjusts the SNR test point considering the CQI table 1. Observing our simulation results, both the options satisfy the requirements (α=20%, γ=1.05, BLER ≥ 2%) with enough margin.

Although the configured CQI table is different between RedCap and Rel-15 CQI reporting tests, we slight prefer to set the same SNR test points as Rel-15 (Option 2). However we can also accept Option 1.

Proposal 1: For the wideband CQI reporting test with CQI table 1 in fading condition, set SNR=6/7dB for lower test points for RedCap 2Rx UE (both FDD 15kHz and TDD 30kHz).

# 3	UL/DL pattern used for FDD tests for 1Rx UE

## 3.1	CQI reporting tests

Introduce demodulation/CSI requirements covering both FD-FDD and HD-FDD. - DDDSU applied for HD-FDD - Existing pattern applied for FD-FDD - Same demodulation requirement applied for FD-FDD and HD-FDD with different FRCs - For CSI requirements, configure the following CSI feedback scheduling pattern applicable for both FD-FDD and HD-FDD - CQI reporting test in static/fading condition (periodic CSI reporting) - CSI-RS periodicity and offset: [10/1] - CSI-Report periodicity and offset: [10/9] - CQI/RI/PMI delay: - Option 1: 10ms - Option 2: 14ms

RAN4 has agreed to use DL/UL scheduling pattern DDDSU for HD-FDD 15kHz by reusing the existing TDD 15kHz pattern. RAN4 also discussed the DL CSI-RS scheduling and UL CSI report scheduling, and the possible option is to reuse the existing TDD test, i.e., CSI-RS periodicity/offset=10/1 and CSI-report periodicity/offset=10/9. Figure 1 illustrates the existing CSI-RS/CSI-report scheduling used for TDD 30kHz CQI reporting tests. Note in this figure two slots correspond to 1ms because of SCS=30kHz. In this configuration, UE is requested to report CSI in slot 9 according to the CSI-RS resource for CSI acquisition in slot 1. If we consider the minimum delay of 4ms, gNB applies the new CSI report in slot 17. However since slot 17 is special slot with 7D1S2U and CQI test does not use the special slots for PDSCH BLER measurement, the new CQI is applied from slot 20. With this configuration, the CQI delay is 19 slots (=9.5ms).

Figure 1	CSI-RS/CSI-report scheduling used for TDD 30kHz CQI reporting tests.

Figure 2 illustrates CSI-RS/CSI-reporting scheduling for HD-FDD 15kHz CQI reporting tests according to the way forward. In this configuration, UE is requested to report CSI in slot 9 according to CSI-RS in slot 1. If we consider the minimum delay of 4ms, gNB applies the new CSI report in slot 13. Since RAN4 test will configure DDDSU, the slot 13 is supposed to be ‘switching slot’ and therefore this slot is not used for PDSCH BLER measurements. If we use the next available DL slot, i.e., slot 15, the CQI delay becomes 14ms. Please note that one slot corresponds to 1ms because of SCS=15kHz.

Figure 2	CSI-RS/CSI-report scheduling option 1 for HD-FDD 15kHz CQI reporting tests.

Alternative option is to configure DL CSI-RS is configured as periodicity/offset=10/5, as illustrated in Figure 3, then the CQI delay can be reduced to 10ms.

Figure 3	CSI-RS/CSI-report scheduling option 2 for HD-FDD 15kHz CQI reporting tests.

Table 3 compares Option 1 (CQI delay=14ms) and Option 2 (CQI delay=10ms). Figure 4 shows the throughput gain of follow CQI over fixed median CQI, where we also put the simulation results with 2Rx FDD in Table 2 for reference. From the simulation results, it is observed the throughput gain with option 1 is lower than option 2 because of larger CQI delay. Although the gains are still above the agreed threshold (γ=1.05), we prefer option 2 as it results in the similar throughput gain with 2Rx FDD results.

Table 3	Comparison between CSI-RS/CSI-report scheduling option 1 and option 2.

|  | Option 1 (CQI delay=14ms) |  |  | Option 2 (CQI delay=10ms) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of reported CQI Index not in {medCQI-1, medCQI+1} | BLER with following CQI | TP ratio with following CQI and Median CQI | Percentage of reported CQI Index not in {medCQI-1, medCQI+1} | BLER with following CQI | TP ratio with following CQI and Median CQI |
| 0 | 0.33 | 0.18 | 1.22 | 0.34 | 0.14 | 1.31 |
| 2 | 0.40 | 0.15 | 1.28 | 0.41 | 0.10 | 1.37 |
| 4 | 0.51 | 0.14 | 1.32 | 0.50 | 0.10 | 1.42 |
| 6 | 0.56 | 0.17 | 1.30 | 0.54 | 0.11 | 1.42 |
| 8 | 0.57 | 0.19 | 1.24 | 0.55 | 0.12 | 1.37 |
| 10 | 0.57 | 0.20 | 1.21 | 0.58 | 0.13 | 1.33 |
| 12 | 0.57 | 0.20 | 1.30 | 0.56 | 0.16 | 1.39 |
| 14 | 0.55 | 0.22 | 1.21 | 0.54 | 0.16 | 1.32 |
| 16 | 0.52 | 0.22 | 1.20 | 0.53 | 0.17 | 1.31 |
| 18 | 0.41 | 0.21 | 1.24 | 0.42 | 0.16 | 1.32 |
| 20 | 0.25 | 0.19 | 1.29 | 0.26 | 0.15 | 1.37 |

Figure 4	Throughput gain of follow CQI over fixed median CQI.

Proposal 2: For CQI reporting tests for HD-FDD SCS=15kHz in static/fading condition, configure the following CSI feedback scheduling pattern.

- CSI-RS periodicity and offset: 10/5

- CSI-Report periodicity and offset: 10/9

- CQI/RI/PMI delay: 10ms

# 4	Summary

Proposal 1: For the wideband CQI reporting test with CQI table 1 in fading condition, set SNR=6/7dB for lower test points for RedCap 2Rx UE (both FDD 15kHz and TDD 30kHz).

Proposal 2: For CQI reporting tests for HD-FDD SCS=15kHz in static/fading condition, configure the following CSI feedback scheduling pattern.

- CSI-RS periodicity and offset: 10/5

- CSI-Report periodicity and offset: 10/9

- CQI/RI/PMI delay: 10ms

# References

- R4-2210672 “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.

# Appendix

Table 4	Mapping of CQI Index to Information Bit payload (CQI table 1)

| CBW/SCS |  |  |  | 10MHz / 15kHz | 10MHz / 15kHz | 20MHz / 30kHz | 20MHz / 30kHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 52 | 51 | 51 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 | 1 | 2 |
| Number of DMRS REs |  |  |  | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 |
| Available REs |  |  |  | 6240 | 6240 | 6120 | 6120 |
| CQI index | Spectrum efficiency | MCS index | Modulation |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A |
| 1 | 0.2344 | 0 | 2 | 1480 | 2976 | 1480 | 2856 |
| 2 | 0.2344 | 0 | 2 | 1480 | 2976 | 1480 | 2856 |
| 3 | 0.3770 | 2 | 2 | 2408 | 4744 | 2408 | 4616 |
| 4 | 0.6016 | 4 | 2 | 3752 | 7424 | 3752 | 7296 |
| 5 | 0.8770 | 6 | 2 | 5504 | 11016 | 5376 | 10760 |
| 6 | 1.1758 | 8 | 2 | 7296 | 14600 | 7168 | 14344 |
| 7 | 1.4766 | 11 | 4 | 9224 | 18432 | 8968 | 17928 |
| 8 | 1.9141 | 13 | 4 | 12040 | 24072 | 11784 | 23568 |
| 9 | 2.4062 | 15 | 4 | 15112 | 30216 | 14600 | 29192 |
| 10 | 2.7305 | 18 | 6 | 16896 | 33816 | 16896 | 33816 |
| 11 | 3.3223 | 20 | 6 | 20496 | 40976 | 20496 | 40976 |
| 12 | 3.9023 | 22 | 6 | 24576 | 49176 | 24072 | 48168 |
| 13 | 4.5234 | 24 | 6 | 28168 | 56368 | 27656 | 55304 |
| 14 | 5.1152 | 26 | 6 | 31752 | 63528 | 31240 | 62504 |
| 15 | 5.5547 | 28 | 6 | 31752 | 69672 | 33816 | 67584 |