3GPP TSG-RAN WG4 Meeting # 104-e 		R4-2213068

# Electronic Meeting, August 15 – 26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title: 	Simulation results for Redcap PDCCH

Agenda item:	9.18.5.1.2

Document for:	Discussion

Introduction

The last eMeeting’s discussion and WF for UE demodulation and CSI requirements for RedCap [1] are captured in the summary [2] and WF [3].

In this contribution, we present a summary of our simulation results for PDCCH based on the agreed configurations in [3].

PDCCH simulation results summary

The agreed configurations for PDSCH demodulation performance were revised in the last meeting and are depicted in [3]. Results are presented both for alignment and for specification of requirements.

Furthermore, the following configurations are used, where appropriate:

| Parameter | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- |
| TDD UL-DL pattern | FR1.30-1 |  |  |  |
| CCE to REG mapping type | interleaved |  | interleaved |  |
| Interleaver size | 3 |  |  |  |
| REG bundle size | 2 | 2 |  |  |
| Shift Index | 0 |  |  |  |

PDCCH simulation results for alignment

Simulation results for alignment are reported in this section for agreed simulation assumptions contained in the WF [3].

FDD 15kHz FR1

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x1 | 1% | 5.3.2.1.1-1 Test 3 | New | 14.6 |
| 1Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x1 low | 1% | 5.3.2.1.2-1 Test 3 | New | 4 |
| 2Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 low | 1% | 5.3.2.1.1-1 Test 3 | Reuse | 5.5 |
| 2Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 low | 1% | 5.3.2.1.2-1 Test 3 | Reuse | -0.9 |

TDD 30kHz FR1

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 2 | New | 2.7 |
| 1Rx UE | 20 | 48 | 1 | 8 | TDLC300-100 | 2x1 low | 1% | 5.3.2.2.2-1 Test 1 | New | 2.6 |
| 2Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 2 | New | -0.7 |
| 2Rx UE | 20 | 48 | 1 | 8 | TDLC300-100 | 2x2 low | 1% | 5.3.2.2.2-1 Test 1 | New | -3.3 |

TDD 120kHz FR2

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100 | 60 | 1 | 4 | TDLA30-300 | 1x2 low | 1% | 7.3.2.2.1-1 Test 1-2 | Reuse | -0.2 |
| 2Rx UE | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 low | 1% | 7.3.2.2.2-1 Test 2-1 | Reuse | -2.8 |

PDCCH simulation results with impairments

Simulation results including impairments are reported in this section for agreed simulation assumptions contained in the WF [3].

FDD 15kHz FR1

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x1 | 1% | 5.3.2.1.1-1 Test 3 | New | 17.1 |
| 1Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x1 low | 1% | 5.3.2.1.2-1 Test 3 | New | 6.5 |
| 2Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 low | 1% | 5.3.2.1.1-1 Test 3 | Reuse | 8.0 |
| 2Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 low | 1% | 5.3.2.1.2-1 Test 3 | Reuse | 1.6 |

TDD 30kHz FR1

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 2 | New | 5.2 |
| 1Rx UE | 20 | 48 | 1 | 8 | TDLC300-100 | 2x1 low | 1% | 5.3.2.2.2-1 Test 1 | New | 5.1 |
| 2Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 2 | New | 1.8 |
| 2Rx UE | 20 | 48 | 1 | 8 | TDLC300-100 | 2x2 low | 1% | 5.3.2.2.2-1 Test 1 | New | -0.8 |

TDD 120kHz FR2

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100 | 60 | 1 | 4 | TDLA30-300 | 1x2 low | 1% | 7.3.2.2.1-1 Test 1-2 | Reuse | 2.9 |
| 2Rx UE | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 low | 1% | 7.3.2.2.2-1 Test 2-1 | Reuse | 0.3 |

Conclusion

In this contribution we have provided a summary of our PDCCH simulation results for agreed configurations in the WF [3] both for alignment and for requirements specification.

The following proposal is made.

Include PDCCH results for alignment and for requirements specification into the simulation summary.

- References

- R4-220966, "Revised WID on support of reduced capability NR devices", Ericsson, RAN #95e

- R4-2210532,"Email discussion summary for [103-e][329] NR_RedCap_Demod", Moderator (Ericsson)

- R4-2210672, "WF on RedCap demodulation and CQI reporting requirements", Ericsson