3GPP TSG-RAN WG4 Meeting #104-e	R4-2213485

Electronic Meeting, August 15 – 26, 2022

Title: 	Discussion on remaining issues for R17 TRP specific BFR

Source: 	Huawei, HiSilicon

Agenda item:	9.17.2.3

Document for:	Discussion

Introduction

In this contribution, we provide discussion on remaining issues of TRP-specific BFR requirements for NR FeMIMO in R17.

Discussion

In TS38.213, the followings are defined for link recovery procedure:

A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes by failureDetectionSet1 and failureDetectionSet2 that can be activated by a MAC CE [11 TS 38.321] and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList1 and candidateBeamRSList2, respectively, for radio link quality measurements on the BWP of the serving cell. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the set q1,1. …. SS/PBCH block indexes associated with a physical cell identity other than the one provided by physCellId in ServingCellConfigCommon can be provided in either q1,0 or q1,1 set and the corresponding q0,0 or q0,1 set is associated with the physical cell identity. … The physical layer in the UE assesses the radio link quality according to the set q0, q0,0, or q0,1, of resource configurations against the threshold Qout,LR. For the set q0, the UE assesses the radio link quality only according to SS/PBCH blocks on the PCell or the PSCell or periodic CSI-RS resource configurations that are quasi co-located, as described in [6, TS 38.214], with the DM-RS of PDCCH receptions by the UE. The UE applies the Qin,LR threshold to the L1-RSRP measurement obtained from a SS/PBCH block. The UE applies the Qin,LR threshold to the L1-RSRP measurement obtained for a CSI-RS resource after scaling a respective CSI-RS reception power with a value provided by powerControlOffsetSS.

Cell specific link recovery is configured when UE is provided a set q0 and a set q1 for a serving cell. TRP specific link recovery is configured when UE is provided respective two sets q0,0 and q0,1 and corresponding two sets q1,0 and q1,1 for a serving cell. For TRP specific link recovery, both two sets q1,0 and q1,1  are indicated for radio link quality measurements on the serving cell. UE . The two sets q1,0 and q1,1  are indicated by failureDetectionSet1 and failureDetectionSet2, which are defined in TS38.331 as follows:

failureDetectionSet1, failureDetectionSet2 Configures parameters for beamfailure detection towards beam failure detection resources configured in the set. If additionalPCIList is configured for the serving cell, each RS in one set can be associted only to one PCI. The failureDetectionSet1 and failureDetectionSet2 are always configured together, and not more than two reference signals are configured in one set for a UE that does not support the MAC CE based BFD-RS activation.

Based on the definition in RAN1 and RAN2 specifications, it can be observed that two sets q1,0 and q1,1 are configured for radio link quality measurements on the BWP of the serving cell, even though SSBs in either q1,0 or q1,1 set can be associated with a PCI different with serving cell.

Observation 1: For TRP specific link recovery, both two BFD resource sets are configured for radio link quality measurements on a serving cell, even though SSB resources in either one resource set can be associated with a PCI different from the serving cell.

For TRP specific link recovery requirements in TS38.133, the wordings “downlink radio link quality of a serving cell and cell with different PCI” and “sets … configured for a serving cell and cell with different PCI” are not properly used. The UE only assess the downlink radio link quality of serving cell. BFD/CBD resource sets are only configured for a serving cell.

Proposal 1: For TRP specific link recovery, it is suggested to use the wording “a serving cell” instead of “a serving cell and cell with different PCI”, where the serving cell can be either configured with additionalPCIList or not.

According to T38.331, the SSB(s) in the failureDetectionSet2 can be indicated to be associated with an additional PCI, and the SSB(s) in the candidateBeamRSList2 shall be associated with the same additional PCI.

Proposal 2: For TRP specific link recovery, it is suggested to clarify that the SSBs in set and can be indicated to be associated with an additional PCI.

In FR2, the sharing factor PTRP=2 is introduced for TRP specific BFD/CBD measurements when BFD/CBD RS resources between two sets are overlapped, which means that BFD/CBD measurements on RS resources from two sets are equally shared. Hence, there is no need to define measurement restrictions between BFD/CBD RS resources from different sets.

Proposal 3: For TRP specific BFD/CBD measurements in FR2, it is suggested that there is no measurement restrictions between BFD/CBD RS resources from different sets.

Conclusions

This contribution provides our analysis on remaining issues of TRP-specific BFR requirements. The followings are provided:

Observation 1: For TRP specific link recovery, both two BFD resource sets are configured for radio link quality measurements on a serving cell, even though SSB resources in either one resource set can be associated with a PCI different from the serving cell.

Proposal 1: For TRP specific link recovery, it is suggested to use the wording “a serving cell” instead of “a serving cell and cell with different PCI”, where the serving cell can be either configured with additionalPCIList or not.

Proposal 2: For TRP specific link recovery, it is suggested to clarify that the SSBs in set and can be indicated to be associated with an additional PCI.

Proposal 3: For TRP specific BFD/CBD measurements in FR2, it is suggested that there is no measurement restrictions between BFD/CBD RS resources from different sets.

Reference

- R4-2207123, “Big CR: RRM requirements for Rel-17 NR feMIMO”, Samsung