3GPP TSG-RAN WG4 Meeting#104-e      	R4-2213760

E-meeting, 15 – 26 Aug, 2022

Title:	Discussion on support of intra-band non-collocated EN-DC/NR-CA deployment

Source:	Nokia

Agenda item:	11.11.2

Document for:	Discussion

# 1	Introduction

Previous RAN agreed a new WID on support of intra-band non-collocated EN-DC/NR-CA deployment with following objectives

- Phase I:

  - Study the feasibility to support non-co-located scenario for FR1 intra-band non-contiguous EN-DC/NR-CA except for 2-layer case of EN-DC with supporting the UE capability of interBandMRDC-WithOverlapDL-Bands-r16 already specified in Rel-16 and Rel-17.

    - Investigate the tolerable power imbalance between carriers

    - Investigate the required arrival time difference between CCs

    - Investigate the additional impacts of contiguous case, if time units are available.

    - Evaluate the UE performance under the power imbalance and arrival time difference

      - Discuss and decide reference UE architecture considering the UE capability of interBandMRDC-WithOverlapDL-Bands-r16 for 2-layer MIMO case for NR-CA, and 4-layer MIMO case for both EN-DC/NR-CA

        - NOTE 1: For UE capable of supporting 2-layer MIMO, only assuming power imbalance 25dB

        - NOTE 2: For UE capable of supporting 4-layer MIMO, 2-layer MIMO power imbalance assumption can be considered as a base line but another value or the same power imbalance with different throughput performance requirement is not precluded.

        - NOTE 3: RAN4 is recommended to start the work on 2-layer first and after that start 4-layer work based on the conclusion of 2-layer work.

    - Work is limited to CA/EN-DC for EN-DC/NR-CA for bands 42, n77/n78

    - Investigate whether the power imbalance should be explicitly (e.g. as an RF requirement) or implicitly specified (e.g. through a demodulation performance test). Specify the power imbalance based on the outcome of the investigation.

    - If any change in RAN1 or RAN2 spec is needed, it will be triggered by RAN4 LS

- Phase II:

  - Phase II work will get started after the feasibility in phase I is confirmed

  - Specify MRTD and MTTD requirements in non-collocated deployment

  - Discuss and decide if the different requirements will be specified based on UE capability of interBandMRDC-WIthOverlapDL-Bands-r16.

Currently non-collocated deployment possibility is only defined for two DC combinations DC_42_n77 and DC_42_n78 and the purpose of this WI is to extend non-collocated deployment possibilities to FR1 intra-band non-contiguous EN-DC/NR-CA with up to 4-layer MIMO

# 2	Discussion

## 2.1	Background

As was stated in previous paragraph currently non-collocated deployment is defined only for two EN-DC band combinations listed in table below from [2] with associated notes

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_42A_n77A3,4,9,11 DC_42A_n77C3,4,9,11 DC_42C_n77A3,4,9,11 DC_42C_n77C3,4,9,11 DC_42D_n77A3,4,9,11 DC_42D_n77C DC_42E_n77A3,4,9,11 DC_42E_n77C | N/A |
| DC_42A_n77(2A)3,4,9,11 DC_42C_n77(2A)3,4,9,11 | N/A |
| DC_42A_n78A3,4,9,11 DC_42A_n78C3,4,9,11 DC_42C_n78A3,4,9,11 DC_42C_n78C3,4,9,11 DC_42D_n78A3,4,9,11 DC_42D_n78C3,4,9,11 DC_42E_n78A3,4,9,11 DC_42E_n78C3,4,9,11 | N/A |
| NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 9: The combination is not used alone as fall-back mode of other band combinations in which UL in Band 42 or Band 48 is not used. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For UEs indicating interBandMRDC-WithOverlapDL-Bands-r16, the power imbalance requirement defined in clause 7.6B.2.6 apply. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |

UE capabilities in notes are described in [3]

interBandMRDC-WithOverlapDL-Bands-r16

Indicates the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.2/7.6.5 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE). If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with (NG)EN-DC/NE-DC MRTD<3us according to clause 7.6.3 in 38.133 [5] and intra-band RF requirements (i.e. Type 1 UE).

interBandContiguousMRDC

Indicates for an inter-band (NG)EN-DC/NE-DC combination, where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [4]), that the UE supports intra-band contiguous (NG)EN-DC/NE-DC requirements (see TS 38.101-3 [4]). If the field is absent for such an inter-band (NG)EN-DC/NE-DC combination, the UE supports intra-band non-contiguous (NG)EN-DC/NE-DC requirements.

As we can see from notes and explanation from [3] even though the combinations are interband configurations intraband EN-DC requirements are applied as intraband UE architecture is assumed (Type 1 UE). Latest REL17 progress to enable non-collocated operation was that Type 2 UE requirements were developed for in-band blocking, from [2] clause 7.6B.2.6

Power imbalance requirement only for band combination of type 2 UE which is indicated by capability interBandMRDC-WithOverlapDL-Bands-r16 is a measure of a receiver’s ability to receive LTE and NR signals with 25dB power imbalance.

Hence type 2 UEs can tolerate much larger power imbalance than type 1 UEs.

## 2.2 	REL18 work

As stated in WID during phase 1 RAN4 should discuss and decide reference UE architecture considering the UE capability of interBandMRDC-WithOverlapDL-Bands-r16 for 2-layer MIMO case for NR-CA, and 4-layer MIMO case for both EN-DC/NR-CA. Work is limited to CA/EN-DC for EN-DC/NR-CA for bands 42, n77/n78.

RAN4 needs to discuss the achievable power difference. For UEs supporting 2-layer MIMO REL17 power imbalance 25dB is assumed. For UE supporting 4-layer MIMO, 2-layer MIMO REL17 power imbalance assumption can be considered as a base line but another value or the same power imbalance with different throughput performance requirement is not precluded.

Although for 2-layer MIMO the REL17 EN-DC power imbalance is assumed to be reused the REL18 use case is NR-CA therefore there is a need to introduce UE type 2 requirements for 38.101-1 in addition to 38.101-3. It would be attractive to reuse EN-DC type 2 UE requirement structure as defined in clause 7.6B.2.6 of [2]. If RAN4 agrees to re-use type 2 UE requirement structure from [2] into NR-CA then work on 4-layer MIMO can start.

************************** Type 2 UE requirements in 38.101-3 ********************************

#### 7.6B.2.6	Power imbalance for type 2 UE of inter-band EN-DC with overlapping DL bands

Power imbalance requirement only for band combination of type 2 UE which is indicated by capability interBandMRDC-WithOverlapDL-Bands-r16 is a measure of a receiver’s ability to receive LTE and NR signals with 25dB power imbalance. For these test parameters in table 7.5B.6-1, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.6B.2.6-1: Test parameters for FDD-FDD or TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping DL bands

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Frequency relationship (Center of BWanother Relative to edge of BWwanted) |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz) |
|  | Another carrier with overlapping DL bands | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier | REFSENS + 1 | BWwanted > BWanother |  |
|  | Another carrier with overlapping DL bands | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier | REFSENS + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Another carrier with overlapping DL bands | Power of wanted carrier + 25 |  |  |

************************** Type 2 UE requirements in 38.101-3 ********************************

# 3	Conclusion

Although for 2-layer MIMO the REL17 EN-DC power imbalance is assumed to be reused the REL18 use case is NR-CA therefore there is a need to introduce UE type 2 requirements for 38.101-1 in addition to 38.101-3. It would be attractive to reuse EN-DC type 2 UE requirement structure as defined in clause 7.6B.2.6 of [2]. If RAN4 agrees to re-use type 2 UE requirement structure from [2] into NR-CA then work on 4-layer MIMO can start.

# 4	References

[1] RP-221809, New WID: Support of intra-band non-collocated EN-DC/NR-CA deployment, RAN#96, KDDI

[2] TS 38.101-3

[3] TS 38.306