3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213799

# Electronic Meeting, Aug.15th -26 th ,2022

Title: 	Simulation results and discussions on CQI reporting requirements for Rel-17 NB-IOT

Source: 	Huawei, HiSilicon

Agenda item:       9.24.6.1.3

Document for:	Discussion

Background

As per [1], RAN4 has agreed the following for CQI test:

CQI measurement resources Remove the square bracket on the measurement duration and specify at least [4] subframes with NRS for CQI estimation with the following wording, captured the agreements in CR revised R4-2208953: • The reported candidateRep value shall be derived from the channel quality measured from the time UE finishes the decoding of Downlink Channel Quality report MAC CE to the end of NPDCCH period which carries the uplink grant of channel quality report for measurement of DL channel quality of the configured carrier. During the measurement period, UE expects that NRS is transmitted in at least 4 consecutive subframes. The number of subframes with NRS for CQI estimation between 1 NRS port and 2 NRS port scenario - At least 4 subframes with NRS for both 1 NRS port and 2 NRS ports for CQI measurement The number of subframes for CQI measurement in the test setup: - 4 subframes Test metric for CQI test - The reported candidateRep value shall be in the range of +/- 1 of the reported median more than 90% of the time. - If the NPDSCH BLER using the transport format indicated by median candidateRep value is less than or equal to 0.1, the BLER using the transport format indicated by the (median candidateRep value + 1) shall be greater than 0.1. - If the NPDSCH BLER using the transport format indicated by the median candidateRep value is greater than 0.1, the BLER using transport format indicated by (median candidateRep value – 1) shall be less than or equal to 0.1. SNR value: - Not consider to add impairment margin for CQI definition test. Ideal SNR test point for CQI reporting test - 10/11dB

In this paper, we provide our corresponding simulations results and views on simulation assumptions.

Discussions and simulation results

The simulation results for CQI test are captured in Table 2-1, it is noted that the simulation results are applicable for both FDD and TDD mode.

Table 2-1: Simulation results for NB-IOT CQI test

| SNR(dB) | Median report value | Prob (Median report value-1<CQI< Median report value+1) | BLER when CQI equals to median report value | BLER when CQI equals to median report value+1 |
| --- | --- | --- | --- | --- |
| 10 | candidateRep-L | 97.0% | 0 | 100% |
| 11 | candidateRep-M | 97.5% | 0 | 100% |

For TDD, the parameters uplink downlink subframe configuration and special subframe configuration are lost. As proposed in NPDSCH demodulation part, we propose to use uplink downlink configuration 4 and special subframe configuration 4 for TDD  CQI test.

Proposal 1: Use uplink downlink configuration 4 and special subframe configuration 4 for TDD CQI test.

Another issue is scheduling in special subframe, Rel-15 NR CQI test always ensure the constant actual code rate through the test which means special subframe is not used for scheduling PDSCH. Following the same procedure, we suggest to design the TDD CQI pattern satisfying that continuous 4 scheduled NPDSCH are not overlapped with special slot. FDD CQI pattern introduced in [2] should be updated based on the agreement that 4 sub-frames are used for CQI calculation.

Proposal 2: RAN4 should design the TDD CQI pattern satisfying that continuous 4 scheduled NPDSCH are not overlapped with special slot.

Proposal 3: FDD CQI pattern introduced in [2] should be updated based on the agreement that 4 sub-frames are used for CQI calculation.

Conclusion

In this paper, we provide our simulation results for Rel-17 NB-IOT CQI testing. The proposals are:

Proposal 1: Use uplink downlink configuration 4 and special subframe configuration 4 for TDD CQI test.

Proposal 2: RAN4 should design the TDD CQI pattern satisfying that continuous 4 scheduled NPDSCH are not overlapped with special slot.

Proposal 3: FDD CQI pattern introduced in [2] should be updated based on the agreement that 4 sub-frames are used for CQI calculation.

Reference

[1]   R4-2210674 Way forward for performance requirements of Rel-17 NB-IOT and eMTC.

[2]   R4-2201432 View on Rel-17 NB-IoT UE demodulation requirements

Appendix

Table 5-2: Summary of simulation assumption for CQI

| Parameter | Value |
| --- | --- |
| Duplex mode | HD-FDD and TDD |
| Operation mode | Standalone |
| Carrier type | Non-anchor |
| Number of antennas | 1T1R |
| Number of NRS port | 1 |
| Propagation condition | AWGN |
| SCS | 15KHz |
| UE category | Cat-NB2 UE capable of 16QAM |
| Number of NPDCCH repetition | 1 (no repetition) |
| Number of NPDSCH repetition | 1 (no repetition) |
| TB Scheduling | single-TB scheduling |
| Number of scheduled subframes with NRS for CQI measurements | 4 subframes |
| Noise Estimation | Practical |
| Channel Estimation | Practical |
| Frequency offset | 0 |
| Time offset | 0 |
| Max number of HARQ transmission | 1 |
| The ratio of EPRE of NPDSCH in symbols with and without NRS to NRS | 0dB |
| Test metric | Refer to section 2.2 |
| SNR | SNR for reported value corresponds to 16QAM, i.e, candidateRep-K to candidateRep-O |