3GPP TSG-RAN WG4 Meeting #104-e	R4-2213860

# Electronic Meeting, 15th – 26th Aug, 2022

Title: 	Discussion on UE NTN demod PDSCH

Source: 	Huawei, HiSilicon

Agenda item:	9.11.7.3.1

Document for:	Discussion

Background

During RAN#103-e meeting, WF [1] on NTN demodulation requirements – general and PDSCH was approved. In this contribution, we share our views about NTN UE PDSCH demodulation requirements.

Discussion

PDSCH requirements for GEO and LEO

- Proposals - Option 1: Define the requirements for LEO only - Option 2: Define the requirements for LEO and GEO separately if time-varying propagation delay is assumed - Agreements: - Further discuss in next meeting.

In last meeting, it is agreed that UE compensation prior to the baseband processing is not be verified in demodulation test, under the assumption UE compensation functionality will be covered by other requirements. So from demodulation point of view, the only difference between LEO and GEO is the k_offset value since there is negligible difference between two scenarios after UE frequency and timing pre-compensation. Therefore, we think only one LEO-600 case is enough.

Define the requirements for LEO-600 only.

Applicability rules for LEO requirements

- Agreements: - If define the requirements for LEO only, the following applicability rules are aggregable. UE-NR-Capability-v1700; Applicability. nonTerrestrialNetwork-r17; ntn-ScenarioSupport-r17. Supported; GSO only; FFS. NGSO only; UE needs to pass the additional LEO test in TS38.101-5 and TS38.101-4 requirements. N/A Note: N/A means UE supports both GSO and NGSO; UE needs to pass the additional LEO test in TS38.101-5 and TS38.101-4 requirements. Not supported; N/A; UE needs to pass TS38.101-4 requirements only. - Further discuss the applicability of GSO only in next meeting.

As per discussion in clause 2.1, if only LEO cases are defined, we think UE that only support GEO scenario should also pass the additional LEO cases. We don’t see too much difference for the GEO and LEO scenario from demodulation point of view after UE frequency and timing pre-compensation.

If only LEO cases are defined, all UE supports nonTerrestrialNetwork-r17 needs to pass the additional LEO test in TS38.101-5 and TS38.101-4 requirements.

K_offset value

- Proposals - Option 1: To discuss the specific K_offset values based on the elevation angle - Option 2: Define the requirements for LEO and GEO separately if time-varying propagation delay is assumed - Agreements: - Select the K_offset value equal to or greater than twice the satellite-UE one-way delay - Further discuss the specific K_offset values based on the elevation angle.

Here we derived the satellite-UE RTT delay for different tilt angle (i.e. α in Figure 2.3-1 derived from TR 38.811) and different scenario as showing in Table 2.3-1.

Table 2.3-1 Satellite-UE RTT delay

| Tilt angle/° |  | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Delay/ms | LEO-600 | 12.89 | 9.29 | 7.17 | 5.89 | 5.08 | 4.56 | 4.24 | 4.06 | 4.00 |
|  | LEO-1200 | 20.89 | 16.38 | 13.34 | 11.29 | 9.92 | 9.01 | 8.43 | 8.11 | 8.01 |
|  | GEO | 270.74 | 263.87 | 257.58 | 252.04 | 247.37 | 243.64 | 240.94 | 239.30 | 238.75 |

Figure 2.3-1 Illustration of the tilt angle α

The K_offset can be calculate based on the function of ceil (Satellite-UE RTT delay) as following Table 2.3.2. We slightly prefer to consider α = 10° as the starting point.

Table 2.3-2 K_offset value in slot

| Tilt angle/° | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LEO-600 | 13 | 10 | 8 | 6 | 6 | 5 | 5 | 5 | 4 |
| LEO-1200 | 21 | 17 | 14 | 12 | 10 | 10 | 9 | 9 | 9 |
| GEO | 271 | 264 | 258 | 253 | 248 | 244 | 241 | 240 | 239 |

Select the K_offset value as per Table 2.3.2 based on detailed channel model. We slightly prefer to consider α = 10° as the starting point.

  - Modulation order

- Proposals - Option 1: Further consider 64QAM - Option 2: Do not consider 64QAM - Agreement: - Further consider 64QAM as 2nd priority

In RAN4#101-e meeting, agreement was achieved from the RF side that

- Include 64QAM as optional with manufacture declaration basis for SAN.

- Include 64QAM (DL and UL ) for NTN satellite UE as optional feature with granularity [per UE]

From our understanding, 64QAM is an important feature to provide higher throughput data transmission. Based on the link budget evaluation as following Table 2.4-1 and Table 2.4-2, 64QAM is feasible at least for rank1 and MCS17. Also considering RF side has introduced 64QAM feature, we propose to define PDSCH performance requirements for 64QAM.

Table 2.4-1 Link budget analysis for NTN PDSCH

|  | LEO600-rural | LEO600-urban | LEO1200-rural | LEO1200-urban | GEO-rural | GEO-urban | LEO600-rural | LEO600-urban | LEO1200-rural | LEO1200-urban | GEO-rural | GEO-urban |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sattellite Max Tx power (dBm) based on TR38.863 | 46.81 | 46.81 | 52.81 | 52.81 | 50.81 | 50.81 | 46.64 | 46.64 | 52.64 | 52.64 | 50.64 | 50.64 |
| Coupling loss 50%-tile based on R4-2115628 (dB) | 125.65 | 125.6 | 131.48 | 131.43 | 140.08 | 139.98 | 125.65 | 125.6 | 131.48 | 131.43 | 140.08 | 139.98 |
| Received power level at UE antenna connector (dBm) | -78.84 | -78.79 | -78.67 | -78.62 | -89.27 | -89.17 | -79.01 | -78.96 | -78.84 | -78.79 | -89.44 | -89.34 |
| Noise figure for NTN UE (dB) | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 |
| DL RB allocation (RB) | 106 | 106 | 106 | 106 | 106 | 106 | 51 | 51 | 51 | 51 | 51 | 51 |
| SCS (kHz) | 15 | 15 | 15 | 15 | 15 | 15 | 30 | 30 | 30 | 30 | 30 | 30 |
| 10log10 (kT) for dBm/Hz | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| Thermal Noise | -92.19 | -92.19 | -92.19 | -92.19 | -92.19 | -92.19 | -92.36 | -92.36 | -92.36 | -92.36 | -92.36 | -92.36 |
| SNR | 13.35 | 13.40 | 13.52 | 13.57 | 2.92 | 3.02 | 13.35 | 13.40 | 13.52 | 13.57 | 2.92 | 3.02 |

Table 2.4-2 Link level simulation results for NTN PDSCH (Rank1, MCS17)

| Case Number | SCS (kHz) / CBW (MHz) | Propagation condition | Antenna configuration | SNR (dB) @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Disabled HARQ | HARQ process 32 |
| 1 | 15 / 5 | NTN-TDLA100-200 | 2x2 | 10.59 | 10.08 |
| 2 | 15 / 5 | NTN-TDLC100-200 | 2x2 | 11.05 | 10.34 |
| 3 | 30 / 10 | NTN-TDLA100-200 | 2x2 | 8.85 | 8.30 |
| 4 | 30 / 10 | NTN-TDLC100-200 | 2x2 | 9.18 | 8.47 |

Define PDSCH performance requirements for 64QAM.

  - SCS/CBW set

- Proposals - Option 1: Further consider 20MHz CBW for 30kHz SCS - Option 2: Do not consider new case for 20MHz CBW for 30kHz SCS - Agreement: - FFS whether 30kHz need to be considered

As per RF agreements, two new NTN satellite bands n256/n255 is introduced with {5, 10, 15, 20}MHz bandwidth configuration for 15kHz SCS and {10, 15, 20}MHz bandwidth configuration for 30kHz/60kHz SCS. For the normal UE performance requirements, only 10MHz and 20MHz bandwidth configuration case is defined for 15kHz SCS and 30kHz SCS respectively. For NTN UE performance requirements definition, we propose to use same typical bandwidth and SCS configuration as normal TN UE performance requirements, i.e. 10MHz bandwidth for 15kHz SCS and 20MHz bandwidth for 30kHz SCS. Also considering that 30kHz SCS has been introduced in satellite performance requirements, it is reasonable to define 30kHz SCS performance requirements for NTN UE.

For NTN UE performance requirements, select both 10MHz bandwidth for 15kHz SCS and 20MHz bandwidth for 30kHz SCS.

Simulation assumption

Overall, for disabled HARQ and HARQ process 32 requirements for NTN PDSCH, we propose to use following simulation assumptions as Table 2.6-1 and Table 2.6-2 respectively.

Table 2.6-1 Simulation assumption for PDSCH performance requirements with disabled HARQ

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| Number of HARQ Processes |  |  | 16 (8 with HARQ enabled, 8 with HARQ disabled) |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2+K_offset for enabled HARQ process, otherwise N/A |
| Maximum number of HARQ transmission |  |  | 4 for enabled HARQ process, 1 for disabled HARQ process |

Table 2.6-2 Simulation assumption for PDSCH performance requirements with HARQ Processes 32

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| Number of HARQ Processes |  |  | 32 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2+K_offset |
| Maximum number of HARQ transmission |  |  | 4 |

For disabled HARQ and HARQ process 32 requirements for NTN PDSCH, use simulation assumptions as captured in Table 2.6-1 and Table 2.6-2 respectively.

Proposals

In this contribution, we discuss on UE PDSCH demodulation requirements for NTN. Our observations and proposals are:

- Define the requirements for LEO-600 only.

If only LEO cases are defined, all UE supports nonTerrestrialNetwork-r17 needs to pass the additional LEO test in TS38.101-5 and TS38.101-4 requirements.

Select the K_offset value as per Table 2.3.2 based on detailed channel model. We slightly prefer to consider α = 10° as the starting point.

| Tilt angle/° | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LEO-600 | 13 | 10 | 8 | 6 | 6 | 5 | 5 | 5 | 4 |
| LEO-1200 | 21 | 17 | 14 | 12 | 10 | 10 | 9 | 9 | 9 |
| GEO | 271 | 264 | 258 | 253 | 248 | 244 | 241 | 240 | 239 |

Define PDSCH performance requirements for 64QAM.

For NTN UE performance requirements, select both 10MHz bandwidth for 15kHz SCS and 20MHz bandwidth for 30kHz SCS.

For disabled HARQ and HARQ process 32 requirements for NTN PDSCH, use following simulation assumptions respectively.

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| Number of HARQ Processes |  |  | 16 (8 with HARQ enabled, 8 with HARQ disabled) |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2+K_offset for enabled HARQ process, otherwise N/A |
| Maximum number of HARQ transmission |  |  | 4 for enabled HARQ process, 1 for disabled HARQ process |

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| Number of HARQ Processes |  |  | 32 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2+K_offset |
| Maximum number of HARQ transmission |  |  | 4 |

Reference

R4-2210661, WF on NTN demodulation requirements – general and PDSCH, RAN4#103-e, Qualcomm Incorporated