3GPP TSG-WG RAN4 Meeting #104-e	R4-2213914

# Online, 15th – 26th Auguest, 2022

Source: 				Nokia, Nokia Shanghai Bell

Title: 				A-MPR for NS_05

Agenda Item: 	5.1.2

Document for:	Approval

# Introduction

In RAN4#103e, A-MPR for NS_05 was discussed and the WF [1] agreed the following.

RAN4 will study the NS_05 A-MPR requirements by exploring the following options:

	Option 1: Applicability of the PC2 A-MPR requirements for NS_05, considering the regulations in Japan;

	Option 2: Revisit the large A-MPR values (e.g. ≥10 dB) for NS_05 in both PC2 and PC3 tables; Revise the A-MPR values if found necessary;

	System impact should be considered where lowering PC3 and/or PC2 A-MPR values is also a possibility

	Option 3: Other options are not precluded.

Interested companies are encourage to submit contributions to the next RAN4 meeting.

This contribution presents our view on the WF.

# Discussion

As already discussed, a newly proposed A-MPR value makes PC2 power lower than PC3 for A1 region. This is not an acceptable behaver from the system performance perspective.

Observation 1: PC2 power lower than PC3 is not an acceptable behaviour from system performance perspective.

PC2 is not yet specified for band 1/n1 in Japanese regulations by MIC (Ministry of Internal Affairs and Communications). Currently, there is no use case of PC2 A-MPR for NS_05.

Observation 2: Currently, there is no use case of PC2 A-MPR for NS_05.

Revisiting protection requirement of PHS band has been recently discussed in MIC’s new generation mobile communication subcommittee since public PHS service termination in March 2023 as sXGP system (band 39 TD-LTE based) for a possible modern replacement. There are still private PHS systems (campus network and cordless phones) remaining, however, their deployment is quite limited and may be migrated to new systems as well.

The band is unlicensed and shared among PHS, DECT and sXGP systems. Only PHS system has been protected from band 1/n1 as a legacy system with a long history. This protection was implemented as NS_05 in 3GPP RAN4 specifications for Japan specific requirement.

There is a trend to introduce HPUE in Japanese regulations. Band 41/n41 is already supporting it and introduction of HPUE to n77/n78/79 is being worked in the same regulatory group. FDD bands are not yet in the scope but may be so someday.

At least, revising coexistence requirement related to NS_05 is studied in the Japanese regulatory group. It is recommended to wait for its outcome no to waste work effort. If protection requirement is not required at all, NS_05 can be made obsolete. If protection requirement is modified, then modified A-MPR or new NS may be needed, which triggers technical discussion in RAN4.

Proposal 1: Keep the current specification as it is until regulatory requirement is concluded.

# Conclusion

Observation 1: PC2 power lower than PC3 is not an acceptable behaviour from system performance perspective.

Observation 2: Currently, there is no use case of PC2 A-MPR for NS_05.

Proposal 1: Keep the current specification as it is until regulatory requirement is concluded.

# Reference

[1] R4-2211172 WF on A-MPR for NS_05, Huawei, HiSilicon, RAN4#103-e

[2] R4-2209181 Consideration on practical PA implementation for NS_05 A-MPR, Huawei, HiSilicon, RAN4#103-e

[3] R4-2210240 Email discussion summary for [103-e][104] NR_PC2_UE_FDD_maintenance, Moderator (China Unicom), RAN4#103-e