3GPP TSG-RAN WG4 Meeting # 104-e	R4-2214033

# Electronic Meeting, August 15 – August 26, 2022

Source: 	Huawei

Title: 	Inputs to M.2070 update for IMT.2020: NR BS receiver spurious emissions (3.5, 4.5)

Agenda Item:		6.1

Document for:	Discussion

# Introduction

During the RAN4#103-e meeting discussion on the Generic unwanted emission for the purpose of the LS reply to the ITU-R WP5D request asking for IMT-2020 updates to the M.2070 was captured in thread [314] in [1]. Based on that discussion, a WF on LS response to ITU-R on Generic unwanted emission (IMT-2020) was agreed in [2] with the agreements as listed below:

The following worksplit was agreed beforehand:

| 1 | Operating bands (& frequency ranges) | Ericsson |
| --- | --- | --- |
| 2.1 - 2.3 | Definitions, symbols and abbreviations | All (as needed) |
| 2.4 | Conducted and radiated requirement reference points, BS types | Huawei |
| 3.1 | Conducted Operating band unwanted emissions | Ericsson |
| 3.2 | Conducted Adjacent channel leakage ratio (ACLR) | ZTE |
| 3.3 | Conducted Cumulative adjacent channel leakage ratio (CACLR) | Nokia |
| 3.4 | Conducted Transmitter spurious emissions | QC |
| 3.5 | Conducted Receiver spurious emissions | Huawei |
| 4.1 | OTA Operating band unwanted emissions | Ericsson |
| 4.2 | OTA Adjacent channel leakage ratio (ACLR) | ZTE |
| 4.3 | OTA Cumulative adjacent channel leakage ratio (CACLR) | Nokia |
| 4.4 | OTA Transmitter spurious emissions | QC |
| 4.5 | OTA Receiver spurious emissions | Huawei |
| Att. 1 | Definition of test tolerance | (Copied from M.2070) |

In this contribution we provide IMT-2020 updates to the M.2070 recommendation, covering receiver spurious emissions as per work-split agreed beforehand.

# Discussion

During the RAN4#103-e meeting, initial inputs for the receiver spurious emissions were provided in [3], listing related sections and technical content to be reflected in the M.2070 updates for IMT-2020. Based on this, content of Annex A was generated covering both conducted and radiated received spurious emissions of NR BS.

# Conclusions

In this contribution we have provided inputs to the IMT-2020 update of M.2070 generic unwanted emissions, covering both conducted and radiated receiver spurious emissions, as per the pre-agreed work-split.

Proposal 1: It is proposed to consider Rx spur related inputs provided in Annex A into the final version of the M.2070 update for IMT-2020.

# References

[1]		R4- 2210517, “Email discussion summary for [103-e][314] LS_Response_ITU-R”, RAN4#103-e

[2]		R4-2210648, “WF on LS response to WP5D on Generic unwanted emission (IMT-2020)”, RAN4#103-e, Ericsson, Nokia, Qualcomm, ZTE, Huawei

[3]		R4-2209645, “Inputs to the ITU-R LS response on generic unwanted emission for (IMT-2020): Receiver spurious emissions”, RAN4#103-e, Huawei

# Annex A: Inputs to M.2070

------------------------------ Modified sections ------------------------------

## 3.5	Conducted receiver spurious emissions

### 3.5.1	Conducted receiver spurious emissions BS type 1-C

The receiver spurious emissions for BS type 1-C for each antenna connector shall be below the applicable basic limits defined in clause 3.5.3.

### 3.5.2	Conducted receiver spurious emissions BS type 1-H

The receiver spurious emissions for BS type 1-H are that for each TAB connector TX min cell group and each applicable basic limit in clause 3.5.3, the power summation emissions at the TAB connectors of the TAB connector TX min cell group shall not exceed a BS limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell), unless stated differently in regional regulation.

### 3.5.3	Basic limits for receiver spurious emissions

Basic limits for receiver spurious emissions are specified in table 7.6.5.1-1 of TS 38.141-1 [1].

NOTE:	Conformance to the BS type 1-H emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the emissions power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)	The unwanted emissions power at each TAB connector shall be less than or equal to the BS type 1-H limit as defined in this clause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

------------------------------ Next modified section ------------------------------

## 4.5	OTA receiver spurious emissions

### 4.5.1	OTA receiver spurious emissions BS type 1-O

The receiver spurious emissions for BS type 1-O at RIB interface shall be below the applicable limits defined in table 7.7.5.1-1 of TS 38.141-2 [2].

### 4.5.2	OTA receiver spurious emissions BS type 2-O

The receiver spurious emissions for BS type 2-O at RIB interface shall be below the applicable limits defined in table 7.7.5.2-1 of TS 38.141-2 [2], with the step frequencies defined in table 7.7.5.2-2 of TS 38.141-2 [2].

In addition, the requirement to protect Earth Exploration Satellite Service (EESS) shall apply for BS operating in the frequency range 24.25 – 27.5 GHz, as defined in table 7.7.5.2-3 of TS 38.141-2 [2].

------------------------------ End of modified section ------------------------------