3GPP TSG-RAN WG4 Meeting # 104-e														R4-2214189

Electronic Meeting, 15 – 26 Aug 2022

Agenda item:			9.21.4

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	Email discussion summary for [104-e][329] NR_IIOT_URLLC_enh_Demod

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Moderator (Nokia, Nokia Shanghai Bell) | Axel Mueller | axel.mueller@nokia-bell-labs.com |
| Samsung | Yunchuan Yang | yc0301.yang@samsung.com |
| Nokia | Qiping Zhu | qiping.zhu@nokia.com |
| Ericsson | Nicholas Pu | Nicholas.pu@ericsson.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

## Scope

This T-doc will be used to guide and summarize the email discussion for the topic of Rel-17 Enhanced IIoT and URLLC support demodulation requirements (AI 9.21.3), with the email thread identifier “[104-e][329] NR_IIOT_URLLC_enh_Demod”.

The scope of this email discussion are Rel-17 Enhanced IIoT and URLLC support demodulation requirements, and in particular the agenda items:

9.21.3	Demodulation performance and CSI requirements

9.21.3.1	PUCCH requirements

Priority topics are marked directly in the open issues’ summaries.

## Notes on email discussions

From the meeting arrangement:

- Delegates are strongly encouraged to provide comments/concerns asap - Silence within a reasonable timeframe means no objection - It is strongly encouraged that each company/delegate consolidate their comments/views and send them out in one email for each email thread - Length of file names shall be reduced, e.g. - At the beginning of first round, moderators share / ftp / tsg_ran / WG4_Radio / TSGR4_98_e / Inbox / Drafts / [98e][101] NR_NewRAT_SysParameters\Summary_101_1st round_v01.docx - After update by company A: Summary_101_1st round_v02_companyA - After update by company B: Summary_101_1st round_v03_companyA_companyB - After update by company C: Summary_101_1st round_v04_companyB_companyC

# Topic #1: PUCCH requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-20xxxxx | Company A | Proposal 1: Observation 1: |
| R4-2212248 | Ericsson | Title: Discussion on sub-slot PUCCH repetition demodulation requirements Proposal 1: Define new requirement for sub-slot PUCCH repetition HARQ-ACK. Proposal 2: Take above parameter configurations for sub-slot PUCCH format 0 repetition demodulation requirements if it is agreed to be introduced. Parameter; Test. Number of sub-slots; 2. Number of UCI information bits; 1. Number of PRBs; 1. Number of symbols in a sub-slot; 2. First PRB prior to frequency hopping; 0. Intra-sub-slot frequency hopping; N/A. Inter-sub-slot frequency hopping; Enabled. First PRB after frequency hopping; The largest PRB index – (Number of PRBs – 1). Group and sequence hopping; neither. Hopping ID; 0. Initial cyclic shift; 0. First symbol; 5, 12. Channel model; TDLC-300-100 Low. SCS and CBW; 5MHz 15kHz SCS and 10MHz 30kHz SCS. Antenna configuration; 1Tx2Rx. |
| R4-2213670 | Samsung | Title: View on PUCCH demodulation requirement for Enhanced IIOT and URLLC support SubslotLengthfor PUCCH Proposal 1: RAN4 defines PUCCH sub-slot repetition requirement with subslotlenght as 7 Number of repetitions Proposal 2: RAN4 defines PUCCH sub-slot repetition requirement with sub slot repetition as 2 Inter sub-slot hopping and First symbol Proposal 3: RAN4 defines PUCCH sub-slot repetition requirement with inter-slot hopping enable - 5 as the startingsymbol index for first sub-slot repetition - 12 as the startingsymbol index for second sub-slot repetition Other simulation assumption Proposal 4: Reuse existing Rel-15 PUCCH format 0 requirement with 2 symbols for PUCCH requirement with sub-slot repetition. |
| R4-2213773 | Nokia | Title: Simulation for Multiple Sub-Slots PUCCH Repetition Proposal 1: RAN4 to define PUCCH demodulation performance requirements for PF0, with sub slot repetition and inter sub-slot hopping, but condition introduction in the specification on the availability of at least 2 simulation inputs. Proposed Configurations Proposal 2: Enable inter sub-slot frequency hopping. Proposal 3: Take the above table of test parameters for multi-subslot PUCCH format 0, as the baseline of the discussion in RAN4. Parameter; Test. Number of information bits; 2. Number of PRBs; 1. Number of PUCCH symbols; 2. Number of PUCCH repetitions (nrofSlots); 4. Number of Sub-slot symbols (subslotLengthForPUCCH-r16); 7. Number of slots; 2. First symbol of sub-slot (startingSymbolIndex); 5. First PRB prior to frequency hopping; 0. Intra-subslot frequency hopping (intraSlotFrequencyHopping); disabled. Inter-subslot frequency hopping (interslotFrequencyHopping); enabled. First PRB after frequency hopping; The largest PRB index – (nrofPRBs – 1). Group and sequence hopping; neither. Hopping ID; 0. Initial cyclic shift; 0. Proposal 4: Use DTX->Ack, miss-ACK and NACK-ACK as performance indicators for multiple sub-slot PUCCH with format0. |
| R4-2213820 | Huawei,HiSilicon | Title: Discussion on Rel-17 URLLC PF0 subslot repetition requirements Whether to introduce enhanced PF0 requirements Proposal 1: Introduce PF0 performance requirements with sub slot repetition. Simulation assumptions Proposal 2: Use test assumptions in Table 2-1: Parameter; Test. SCS(kHz)/Bandwidth(MHz); 30/20. Propagation conditions; TDLC300-100 Low. Antenna condition; 1T2R. Number of UCI information bits; 1. Number of PRBs; 1. Number of symbols; 2. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; Disabled. Inter-slot frequency hopping; Enabled. First PRB after frequency hopping; The largest PRB index – (Number of PRBs – 1). Group and sequence hopping; neither. Hopping ID; 0. Initial cyclic shift; 0. First symbol; 12. Sub slot length; 2 symbols. Repetition number; 2. Test metric; Prob(ACK miss)<1% Prob(DTX to ACK)<1%. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Interested companies are expected to add their views directly under the respective issues in a dialogue-like form, i.e., identical to how the chair would record views during a f2f meeting.

Please add further table rows as required and do not change previous comments of your company or other companies. Answering to questions from other companies is encouraged.

### Sub-topic 1-1: Introduction of sub-slot PUCCH repetition requirements

Sub-topic description:

In the last meeting it was left open, if sub-slot PUCCH repetition requirements are to be introduced, to allow for further checking. The following agreements were captured in the WF [R4-2210673]:

Issue 1-2-1: Introduce PUCCH requirements Agreements: Option 2a as starting point and further confirm by RAN4#104-e meeting. - Option 2a: Develop PUCCH requirements for PF0, with sub slot repetition and inter sub-slot hopping, but condition introduction in the specification on the availability of at least 2 simulation inputs

Open issues and candidate options before e-meeting:

Issue 1-1-1: Introduction of sub-slot PUCCH repetition requirements

- Proposals

  - Option 1 (Ericsson, Samsung): Define new requirement for sub-slot PUCCH repetition HARQ-ACK.

  - Option 2 (Nokia): RAN4 to define PUCCH demodulation performance requirements for PF0, with sub slot repetition and inter sub-slot hopping, but condition introduction in the specification on the availability of at least 2 simulation inputs.

  - Option 3 (Huawei): Introduce PF0 performance requirements with sub slot repetition.

  - Option 4 (Moderator): Introduce performance requirements with sub slot repetition.

- Recommended WF

  - All contributors agree that requirements should be introduced and there is active participation from 4 contributors, which makes the last meeting’s condition on inputs redundant.
All contributors seem also aligned on using PF0, which is recommended WF in the next subsection.
As such the moderator recommends agreeing the following here:

  - Option 4 seems agreeable.

| Company | Comments |
| --- | --- |
| Samsung | We are fine to introduce PUCCH performance requirement for PF0 with sub slot repetition |
| Nokia | In principle, we support option 2/3/4. And we don’t think the test requirement should restrict the payload to HARQ-ACK. |
| Huawei | Option 2/3/4 |
| Ericsson | Support Option 3. |
| Moderator | GTW on August 16th, for information: Agreement: option 3 agreed |

### Sub-topic 1-2: Requirement parameters

Sub-topic description:

In this sub-topic we discuss all parameters and configurations that are needed to define the minimum requirements.

Open issues and candidate options before e-meeting:

Issue 1-2-1: PUCCH format

- Proposals

  - Option 1 (Ericsson, Samsung, Nokia, Huawei): Introduce only PF0 performance requirements.

- Recommended WF

  - Option 1 seems agreeable.

| Company | Comments |
| --- | --- |
| Samsung | OK with option 1 and recommended WF |
| Nokia | Support the WF |
| Huawei | Support the WF |
| Ericsson | Support the WF |
| Moderator | As a consequence of GtW agreement on issue 1-1-1, this issue is decided as PF0. |

Issue 1-2-2: KPIs

- Proposals

  - Option 1 (Nokia): Use DTX->Ack (1%), miss-ACK (1%), and NACK-ACK (0.1%).

  - Option 2 (Huawei): Prob(ACK miss)<1% and Prob(DTX to ACK)<1%.

- Recommended WF

  - Discuss in first round.

  - The inclusion of NACK to ACK likely also depends on the agreement for UCI information bits.

| Company | Comments |
| --- | --- |
| Samsung | We support option 2, which is same test metric for PF0 without repetition defined in Rel-15. NACK to ACK is not bottleneck for performance of PUCCH format 0, since only 1 bit is considered for requirement. |
| Nokia | We prefer to keep the DTX->Ack, miss-ACK , and NACK-ACK open and further check which KPI is the bottleneck for this feature. |
| Huawei | We prefer to keep three metric open and further discuss in the next meeting based on the simulation results |
| Ericsson | We are fine with the Nokia and Huawei’s proposal that companies can check the necessary of NACK-ACK performance by simulation. |

Issue 1-2-3: Antenna Configuration

- Proposals

  - Option 1 (Ericsson, Samsung, Nokia, Huawei): 1T2R

- Recommended WF

  - Agree option 1.

| Company | Comments |
| --- | --- |
| Samsung | OK with option 1 and recommended WF |
| Nokia | Support the WF |
| Huawei | Support the WF |
| Ericsson | We support WF |

Issue 1-2-4: SCS/CBW

- Proposals

  - Option 1 (Ericsson): 15kHz/5MHz and 30kHz/10MHz.

  - Option 2 (Huawei): 30kHz/20MHz.

  - Other options not precluded.

- Recommended WF

  - Collect further input during first round.

| Company | Comments |
| --- | --- |
| Samsung | We prefer to define the practical SCS/BW as 15 KHz/10MHz, and 30KHz/40MHz, we also ok the minimum CBW per each SCS |
| Nokia | We prefer the configuration we implemented - 30KHz/40MHz, which is a usual testing configuration. We are also fine with using the minimum BW for SCS 15kHz and 30kHz. |
| Huawei | Considering PF0 only occupy 1 RB, CBW has no impact on performance. We can comprise to use 30kHz/40MHz which is typical configuration. Furthermore, we propose to not consider 15kHz SCS to reduce simulation work. It is noted that in PF1 multi-slot repetition requirements definition also only 30kHz is considered. |
| Ericsson | We support the agreement in GTW that only 30kHz/10MHz is considered for the requirement. |
| Moderator | GTW on August 16th, for information: Agreement: 30kHz/10MHz |

Issue 1-2-5: Channel model

- Proposals

  - Option 1 (Ericsson, Huawei): TDLC-300-100 Low

- Recommended WF

  - Agree option 1.

| Company | Comments |
| --- | --- |
| Samsung | Ok with option 1 |
| Nokia | Support the WF. |
| Huawei | Support the WF |
| Ericsson | Support the WF |

Issue 1-2-6: Number of UCI information bits

- Proposals

  - Option 1 (Ericsson, Huawei): 1 bit.

  - Option 2 (Nokia): 2 bits.

- Recommended WF

  - Note: It is the moderator’s understanding that the type of UCI payload (CSI/SR or N/ACK) influences the selectable values for “startingSymbolIndex”, so please comment on payload type, if this is a concern. See notes of issue 1-2-9 for more background.

| Company | Comments |
| --- | --- |
| Samsung | We support option 1 with only 1 bit, similar as PUCCH format 0 without sub slot repetition defined in Rel-15 |
| Nokia | Support Option 2. We understand the information bit is related to the payload. It is ok to have 1 bit for N/ACK while it is not that meaningful to have 1 bit for CSI. |
| Huawei | Based on our understanding, PF0/2 sub-slot is introduced for ACK/NACK transmission. Hence 1 bit is enough which also aligned with Rel-15 PF0 asssumptions |
| Ericsson | We support the agreement in GTW that use Option 1. |
| Moderator | GTW on August 16th, for information: Agreement: option 1 agreed Please consider to still comment on on the payload type of the 1 bit UCI, so that the CRs and KPIs can be written accordingly. In PF0 Rel-15 1 bit, the payload type was HARQ-ACK, which the moderator would currently propose as following in the WF of the round one summary. |

Issue 1-2-7: Number of PRBs

- Proposals

  - Option 1 (Ericsson, Samsung, Nokia, Huawei): 1

- Recommended WF

  - Moderator: If PF0 is agreed, this value is necessarily 1.

  - Option 1 is agreeable.

| Company | Comments |
| --- | --- |
| Samsung | Ok with option 1 |
| Nokia | Support option 1 |
| Huawei | Support option 1 |
| Ericsson | Support Option 1. |

Issue 1-2-8: PUCCH repetitions across “14-symbol slot” boundaries

- Proposals

  - Option 1 (Ericsson, Samsung): No.

  - Option 2 (Nokia, Huawei): Yes.

- Recommended WF

  - Since the sub-slot PUCCH allocation is quite different between companies, the moderator tries to establish the goals of the configurations before moving to specific parameter configurations.

  - Please discuss and align here on the need for having sub-slot repetitions crossing the “14-symbol slot” boundary/ies.

| Company | Comments |
| --- | --- |
| Samsung | We would like to clarify the meaning of across 14 symbol slots, whether it is same as PUSCH repetition type B, where one repetition transmission can across the slot boundary? In our understanding, it can be allocated serval slots pending on configured repetition number for PUCCH slot repetition transmission, while from slot combination processing perspective, we think 2 repetitions can meet the test purpose, the PUCCH slot repetition transmission can be done within one slot, since PUCCH is not the bottleneck for UL, there is no need to configure large number of repetitions |
| Moderator | 14 symbol slots are intended to mean normal, i.e., non-sub-slot, slots. In the moderator’s understanding, in this feature a repetition cannot be split by a “14 symbol slot” boundary. But repetitions/PUCCH allocations can be located in sub-slots of different slots. |
| Nokia | Option 2 We understand that Option 1 – not crossing the regular slot boundary (i.e. 2 sub-slot repetitions over one regular slot) is sufficient to test the feature, but Option -2 provides a more diverse case for testing the cross slot boundary configuration as well as testing the buffer length over multiple regular slots. |
| Huawei | Option 1 We share the same views with Samsung, in our contribution, we proposed to use repetition 2 which means no across slot boundary repetition transmission. We don’t think it is necessary to consider large repetition and across slot repetition transmission |
| Ericsson | We support Option 1. |
| Moderator | GTW on August 16th, for information: Agreement: option 1 agreed |

Issue 1-2-9: Symbol level PUCCH resource configuration [Number of PUCCH symbols per sub-slot]

- Moderator’s comments:

  - The moderator has tried to add a unified graphical representation to each proposed option for check, if the moderator has correctly understood the intent behind each option.
Please comment, if this not acceptable and it will be removed for all or specific options.

  - In the moderator’s understanding, the configurable range of startingSymbolIndex and its handling differs between CSI/SR and HARQ payloads. 
For CSI/SR payloads, the startingSymbolIndex is with respect to the start of the “14-symbol slot” that contains the PUCCH allocation, i.e., the startingSymbolIndex is up to 14 symobls.
For HARQ payload, the K1 value would point to the first sub-slot containing a PUCCH allocation, i.e., the startingSymbolIndex range is limited within the sub-slot length and a K1 configuration is needed in the test to point to the correct sub-slot.

- Proposals

  - Option 1 (Ericsson, Samsung):

    - Symbols per sub-slot / PUCCH length = 2
Sub-slot repetitions (nrofSlots) = 2
Sub-slot length (subslotLengthForPUCCH-r16) = 7
First symbol of PUCCH (startingSymbolIndex) = 5

  - Option 2 (Nokia):

    - Symbols per sub-slot / PUCCH length = 2
Sub-slot repetitions (nrofSlots) = 4
Sub-slot length (subslotLengthForPUCCH-r16) = 7
First symbol of PUCCH (startingSymbolIndex) = 5

  - Option 3 (Huawei):

    - Symbols per sub-slot / PUCCH length = 2
Sub-slot repetitions (nrofSlots) = 2
Sub-slot length (subslotLengthForPUCCH-r16) = 2
First symbol of PUCCH (startingSymbolIndex) = 12

- Recommended WF

  - Symbols per sub-slot / PUCCH length = 2 seems agreeable.

  - In case agreement is requested to a setup, where the PUCCH allocation is not in the first sub-slot of a “14-symbol” slot, and HARQ payload is used, please comment on how to handle the test configuration. See moderator’s notes for background.

  - Discuss in first round and try to compromise if there are no strong technical concerns.

| Company | Comments |
| --- | --- |
| Samsung | Support option 1, as mentioned in issue 1-2-11, from slot combination processing perspective, we think 2 repetitions can meet the test purpose, the PUCCH slot repetition transmission can be done within one slot, since PUCCH is not the bottleneck for UL, there is no need to configure large number of repetitions |
| Nokia | Option 2 For Option 1, although it is sufficient to test part of the feature, Option -2 provides a more diverse case for testing the cross slot boundary configuration as well as the buffer length over multiple regular slots. For Option 3 starting the PUCCH at symbol 12 with subslot length 2, we understand that for HARQ-ACK such configuration requires an indication of k (dl-DataToUL-ACK), which means the configuration needs to define k, and both DL, and UL channels to enable the n+k. |
| Huawei | Option 3 We think sub-slot 2 has more stringent requirements on processing delay which needs to be verified. As for repetition number, we think 2 is sufficient. |
| Ericsson | We support Option 1. |
| Moderator | GTW on August 16th, for information: Agreement: option 1 agreed |

Issue 1-2-10: Inter-(sub)Slot frequency hopping (interSlotFrequencyHopping)

- Background:

  - It is the moderator’s understanding that setting interSlotFrequencyHopping=enabled, is the intended way to enable FH between each sub-slot.

- Proposals

  - Option 1 (Ericsson, Samsung, Nokia, Huawei): Hop between each sub-slot, i.e., interSlotFrequencyHopping=enabled.

- Recommended WF

  - Discuss in first round.

| Company | Comments |
| --- | --- |
| Samsung | OK with option 1 |
| Nokia | Support option 1 |
| Huawei | Support option 1 |
| Ericsson | Support option 1. |

Issue 1-2-11: Intra-(sub)Slot frequency hopping (intraslotFrequencyHopping)

- Moderator’s notes:

  - It is the moderator’s understanding that setting intraslotFrequencyHopping =enabled, is the intended way to enable FH within each PUCCH allocation (e.g., hop after 2 of 4 PUCCH symbols within a sub-slot).
It is not possible to enable both interslotFrequencyHopping and intraslotFrequencyHopping at the same time.

- Proposals

  - Option 1 (Nokia, Huawei, Samsung): Do not hop within each PUCCH allocation, i.e., intraslotFrequencyHopping=disabled.

  - Option 2 (Ericsson): N/A.

- Recommended WF

  - Configuration intraslotFrequencyHopping=disabled seems agreeable.

| Company | Comments |
| --- | --- |
| Samsung | OK with option 1 |
| Nokia | Support option 1 |
| Huawei | Support option 1 |
| Ericsson | Support Option 1. |

Issue 1-2-12: Frequency hopping PRBs

- Proposals

  - Option 1 (Ericsson, Nokia, Huawei, Samsung [according to R4-2213670 Table 1]):

    - First PRB prior to frequency hopping = 0.

    - First PRB after frequency hopping = The largest PRB index – (Number of PRBs – 1).

- Recommended WF

  - Option 1 seems agreeable.

| Company | Comments |
| --- | --- |
| Samsung | OK with option 1 and recommended WF |
| Nokia | Support the WF |
| Huawei | Support the WF |
| Ericsson | Support the WF |

Issue 1-2-13: Other legacy parameters

- Proposals

  - Option 1 (Ericsson, Nokia, Huawei, Samsung [according to R4-2213670 Table 1]):

    - Group and sequence hopping = neither

    - Hopping ID =0

    - Initial cyclic shift = 0

- Recommended WF

  - Option 1 seems agreeable.

| Company | Comments |
| --- | --- |
| Samsung | OK with option 1 and recommended WF |
| Nokia | Support the WF |
| Huawei | Support the WF |
| Ericsson | Support the WF |

### Sub-topic 1-3: Other

Sub-topic description:

In this sub-topic companies are invited to bring issues to the attention of the group, which have not been captured in the previous sub-topics.

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Title, Source |
|  | Company A |
|  | Company B |
|  | None |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Sub-topic 1-1 | Sub-topic 1-1: Introduction of sub-slot PUCCH repetition requirements Issue 1-1-1: Introduction of sub-slot PUCCH repetition requirements GtW agreements: Introduce PF0 performance requirements with sub slot repetition. Candidate options: None. Recommendations for 2nd round: No open issues or options. |
| Sub-topic 1-2 | Sub-topic 1-2: Requirement parameters Issue 1-2-1: PUCCH format GtW agreements: Following issue 1-1-1 agreements in GtW, this issue has already been decided above. Issue 1-2-2: KPIs Tentative agreements: Use Prob(DTX to ACK)<1%. FFS: Prob(ACK miss)<1%. FFS: Prob(PUCCH NACK to ACK bits) < 0.1%. Candidate options: None. Recommendations for 2nd round: Keep Prob(ACK miss)<1% and NACK-ACK (0.1%) open until simulation results are available to decide bottleneck. Issue 1-2-3: Antenna Configuration Tentative agreements: 1T2R. Candidate options: None. Recommendations for 2nd round: Tentative agreements are agreable. Issue 1-2-4: SCS/CBW GtW agreements: 30kHz/10MHz. Candidate options: None. Recommendations for 2nd round: No open issues or options. Issue 1-2-5: Channel Model Tentative agreements: TDLC-300-100 Low. Candidate options: None. Recommendations for 2nd round: Tentative agreements are agreable. Issue 1-2-6: Number of UCI information bits Tentative and GtW agreements: 1bit. HARQ-ACK payload. Candidate options: None. Recommendations for 2nd round: Tentative agreements are agreable. Issue 1-2-7: Number of PRBs Following agreement to PF0, this is necessarily 1 PRB. Issue 1-2-8: PUCCH repetitions across “14-symbol slot” boundaries GtW agreements: No. Candidate options: None. Recommendations for 2nd round: No open issues or options. Issue 1-2-9: Symbol level PUCCH resource configuration [Number of PUCCH symbols per sub-slot] GtW agreements: - Symbols per sub-slot / PUCCH length = 2 - Sub-slot repetitions (nrofSlots) = 2 - Sub-slot length (subslotLengthForPUCCH-r16) = 7 - First symbol of PUCCH (startingSymbolIndex) = 5. Candidate options: None. Recommendations for 2nd round: No open issues or options. Issue 1-2-10: Inter-(sub)Slot frequency hopping (interSlotFrequencyHopping) Tentative agreements: Hop between each sub-slot, i.e., interSlotFrequencyHopping=enabled. Candidate options: None. Recommendations for 2nd round: Tentative agreements are agreable. Issue 1-2-11: Intra-(sub)Slot frequency hopping (intraslotFrequencyHopping) Tentative/GtW agreements: Do not hop within each PUCCH allocation, i.e., intraslotFrequencyHopping=disabled. Candidate options: None. Recommendations for 2nd round: Tentative agreements are agreable. Issue 1-2-12: Frequency hopping PRBs Tentative agreements: - First PRB prior to frequency hopping = 0. - First PRB after frequency hopping = The largest PRB index – (Number of PRBs – 1). Candidate options: None. Recommendations for 2nd round: Tentative agreements are agreable. Issue 1-2-13: Other legacy parameters Tentative agreements: - Group and sequence hopping = neither - Hopping ID =0 - Initial cyclic shift = 0. Candidate options: None. Recommendations for 2nd round: Tentative agreements are agreable. |
| Sub-topic 1-3 | Sub-topic 1-3: Other No issues raised by the group. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on enhanced IIoT and URLLC support demodulation and CSI requirements | Nokia, Nokia Shanghai Bell |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
|  | None |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on … | YYY |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
|  | WF on enhanced IIoT and URLLC support demodulation and CSI requirements | Nokia, Nokia Shanghai Bell |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| NONE |  |  |  |  |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents