3GPP TSG-RAN WG4 Meeting #104-e	R4-2214372

Electronic Meeting, 15 ‒ 26 Aug, 2022

Title: 	WF on NTN UE RF requirement maintenance

Agenda Item:	     9.11.4

Source: 	ZTE

Document for:	Approval

Topic #1 NTN UE RF requirement maintenance

Sub-topic 1-1 frequency error for NTN UE conformance testing

### 1.1.1 Issue 1-1: Frequency error assumption for UE RF requirements other than frequency error testing

Agreements:

  - Option 1: Doppler is set to zero for other than frequency error UE RF requirement

Sub-topic 2-1 OOBB requirements for n256

Agreements:

  - Option 1:

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Void NOTE 3: Void NOTE 4: Void |  |  |  |  |  |