3GPP TSG-RAN WG4 Meeting #104-e							        		       		R4-2214470

Electronic Meeting, August 15th – 26th, 2022

Agenda item:			9.7.3

Source:	Samsung

Title:	WF on RRM performance requirement for FR2 HST

Document for:	Approval

# 1 Background

In RAN Plenary #89-e, the RAN4-led work item of NR support for high speed train scenario in FR2 has been approved [RP-210800]. In RAN4#103-e, the work on performance part of RRM requirement has been started with the WF approved [R4-2210609].

# 2 Way Forward

### 2.1 Sub-Topic 1: General Configuration and Applicability

#### Issue 1-1-1: Applicability of Set-1 and Set-2 of enhanced RRM requirements

Agreement:

- Applicability of Set-1 and Set-2 of enhanced RRM requirements:

  - General principle:

    - Use NW RRC configuration highSpeedMeasFlagFR2-r17 = set1 or set2 as TC condition. No need to test both set1 and set2 for one TC.

  - Detailed applicability for relevant tests:

    - TC for Cell Re-selection Requirement

      - Set1 only

    - TC for Re-establishment Delay Requirement

      - Set2 only (if TC for re-establishment delay confirmed)

    - TC for MAC-CE based TCI State Switch Delay Requirement

      - Set2 only

    - TC for L3 measurement

      - Option 2: Set2 only

    - TC for L1-RSRP and L1-SINR (if introduced) Measurement Requirement

      - Option 1: Set1 only

### 2.2 Sub-topic 2: RRM performance Scope

#### Issue 1-2-1: Necessity of Test Cases for Re-establishment Delay Requirement

Agreement:

- Necessity of TC for re-establishment delay requirement:

  - TCs for Re-establishment Delay Requirement can be skipped

#### Issue 1-2-2: Necessity of Test Cases for gradual timing adjustment

During GTW on 17th Aug, the following agreement is achieved:

Agreement:

- Applying PC6 UE new requirement into A.7.4.1.1, and no need to define new test case

#### Issue 1-2-3: Necessity of TC for one shot large UL timing adjustment for FR2 PC6 UE

Agreements:

- Necessity of TC for one shot large UL timing adjustment for FR2 PC6 UE

  - One shot large UL timing adjustment test is combined into MAC-CE based TCI switch delay test

#### Issue 1-2-4: Necessity of TC for L1-SINR Measurement

Conclusion:

- Necessity of TC for L1-SINR Measurement:

  - Wait core requirements decision to define a new test case if needed.

### 2.3 Sub-topic 3: RRM TC Configuration

#### Issue 1-3-1: Test Cases for L1-RSRP Measurement

Agreement:

- Test Cases for L1-RSRP Measurement and L1-SINR Measurement (if introduced)

  - DRX mode, DRX cycle = 40ms

#### Issue 1-3-2: Test Cases for Cell Re-selection Requirement

Agreement:

- Test Cases for Cell Re-selection Requirement:

  - Apply AWGN with 19444Hz frequency offset

#### Issue 1-3-3: Test Cases for MAC-CE based TCI State Switch Delay Requirement

Agreement:

- Test Cases for MAC-CE based TCI State Switch Delay Requirement

  - Channel model and timing offset:

    - Uni-directional with the timing difference is set properly as proposed: Timing offset between SSB 0 and SSB 1 is 2.33us

  - DRX cycle:

    - non-DRX.

During GTW on 25th Aug, the following agreement is achieved:

Agreement:

- AOA

Configuration set2 with 2-AOA setup is applied for this test case.

#### Issue 1-3-4: Test Cases for L3 Measurement Requirement

Agreement:

- Test Cases for L3 Measurement

  - Non-DRX mode

- Note: tentative agreement is based on DRX for L1 measurement, and non-DRX for L3-measurement.

### 2.4 Sub-topic 4: Measurement accuracy for FR2 HST UE

#### Issue 1-4-1: L1-measurement simulation result and alignment

Agreement:

- L1-RSRP measurement:

  - The legacy accuracy of FR2 L1-RSRP measurement can be reused for L1-RSRP measurement in FR2 HST.

- L1-SINR measurement:

  - For the below three categories of L1-SINR measurement (introduced in Rel-16 eMIMO WI):

| CMR | IMR | L1-SINR accuracy requirements |
| --- | --- | --- |
| CSI-RS | N/A | Type 1: CSI-RS based CMR and no dedicated IMR configured |
| SSB | NZP-IMR | Type 2: SSB based CMR and dedicated IMR configured |
|  | CSI-IM |  |
| CSI-RS | NZP-IMR | Type 3: CSI-RS based CMR and dedicated IMR configured |
|  | CSI-IM |  |

    - For Type-2 and Type-3 L1-SINR measurement:

      - The legacy accuracy of FR2 L1-SINR measurement can be reused for L1-SINR measurement in FR2 HST.

        - An upper bound SNR = FFS is defined.

          - FFS FR1 HST side condition can be reused or not

    - For Type-1 L1-SINR measurement:

      - The legacy accuracy of FR2 L1-SINR measurement can be reused for L1-SINR measurement in FR2 HST;

        - An upper bound SNR = FFS is defined.

          - FFS FR1 HST side condition can be reused or not

#### Issue 1-4-2: L3-measurement simulation result and alignment

Agreement:

- For SS-RSRP and SS-RSRQ measurement:

  - The legacy accuracy requirement for Rel-15 FR2 SS-RSRP and SS-RSRQ measurement can be reused.

- For SS-SINR measurement:

  - FFS whether or not Rel-15 FR2 SS-SINR measurement can be reused, by further study:

    - Impact of uni- and bi-directional deployment scenario;

    - SNR upper bound.

      - FFS FR1 HST side condition can be reused or not

# References

[1] R4-2214127, “Email discussion summary for [104-e][207] NR_HST_FR2-Perf”, Samsung.