3GPP TSG-RAN WG4 Meeting # 104-e 										R4-2214483

# Electronic Meeting, Aug. 15 – Aug. 26, 2022

Agenda item:			9.17.5

Source:	Samsung

Title:	WF on FeMIMO RRM Performance Requirement and Test Case

Document for:	Approval

Introduction

This contribution is to capture the agreements and way forward on the discussion for Rel-17 FeMIMO RRM performance requirements in RAN4 #104-e meeting email thread [104-e][222] NR_feMIMO_RRM_2.

Agreements

Agreements in GTW session

Pathloss RS in UL (and joint TCI) test case

- For Pathloss RS in UL (and joint TCI) test case, use CSI-RS as the Pathloss RS in the test case, and the CSI-RS is QCLed Type-D with the corresponding SSB.

Introduce new unified TCI state configurations

- Introduce separate DL, UL and joint TCI state configuration in unified TCI configuration section

Data transmission on cell with different PCI in the test case

- For DL TCI state switching test case(s), data transmission is based on TCI state which is QCLed with the SSB from cells with different PCIs.

Whether configure L1-RSRP on the serving cell in the test case

- Configure L1-RSRP measurement on both serving cell and cell with different PCI

- For test case, two cells are both configured for L1-RSRP, but we only test the cell with different PCI.

Agreements during email discussion

Unified TCI state switching

Test requirement for R17 unified TCI state switching test case

- For DL TCI test case, follow the same logic of R16 downlink TCI state switching test case

- For UL TCI test case, follow the same logic of R16 uplink spatial test case

- For Joint TCI test case, test requirement is MAX(DL TCI state switch delay, UL TCI state switch delay)

Test procedure for R17 unified TCI state switching test case

- T1 = 0.2s where only SSB#0 is transmitted and UE is configured for data transmission which QCLed with SSB#0

- T2 = 2s where UE is configured L1-RSRP measurement on SSB#1, UE reports measurement results and then TCI state switching MAC-CE is sent

- At the beginning of T2, the SSB#1 corresponding to TCI state 1 starts transmitting

Configuration of pathloss RS in UL (and joint TCI) test case

- Reuse legacy CSI-RS for tracking configuration in TS38.133 for pathloss RS configured in the test case

Subsections for unified TCI state switching test case

- Three subsections, 1 Test Purpose and Environment, 2 Test parameters and 3 Test Requirements

L1-RSRP measurement on cell with different PCI

SSB configuration of L1-RSRP measurement in test case

- The same period for SSBs from serving cell and cell with different PCI, clarify which PCI the SSB is associated to in the test.

- Sharing factor equals 2 is applied for 2 SSBs in FR2 test case.

AOA setup for tested FR2 cell in test case

- AoA setup of FR2 cell to be test in the test case is Setup 3

- One AoA is for FR2 serving cell, and another is for cell with different PCI

CP length configuration in the test case

- Specify the CP length configuration in the test: Normal CP is used

- Clarify that timing offset between SSB of SC and SSB of CDP is within CP

TRP specific BFR

AOA setup in FR2 test

- Two AoAs are configured in the test, each of which is for one of two TRPs

BFD/CBD RS for TRP specific link recovery tests

- BFD-RS of two TRPs are overlapped, though SSB index may be different

- SSB could be used as BFD-RS for PCell and PSCell in the test

- Configure SSB/CSI-RS for the test case as table below

| Resource set | SSB based link recovery test | CSI-RS based link recovery test |
| --- | --- | --- |
| BFD-RS set (q0,0) | SSB#0 | CSI-RS#0 |
| CBD-RS set (q1,0) | SSB#1 | CSI-RS#1 |
| BFD-RS set (q0,1) | SSB#2 | CSI-RS#2 |
| CBD-RS set (q1,1) | SSB#3 | CSI-RS#3 |
| RLM-RS set | SSB#0, SSB#1, SSB#2, SSB#3 | CSI-RS#0, CSI-RS#1, CSI-RS#2, CSI-RS#3 |

Test procedures for TRP specific BFR test case

- Beam failure occurs on one BFD-RS resource set. SNR level of another TRP will always keep at high level

Remaining Open Issues

Unified TCI state switching

Pathloss RS configuration in joint TCI test case

- Option 1: Pathloss RS is maintained

- Option 2: Do not configure pathloss RS in joint TCI case

TRS configuration for cell with different PCI in the test case

- Configure a TRS for serving cell that is QCLed type-D with the SSB from cell with different PCI in the test

  - Option 1: Introduce new TRS, TCI and SSB configuration for the cell with different PCI

  - Option 2: No need to introduce a new TRS configuration or new TCI configuration

The rate of correct events for unified TCI state switching test case

- Option 1: Specify “The rate of correct events observed during repeated tests shall be at least [90]%” in the test case

- Option 2: No need to specify “90%” threshold for the test case as there is few time variation in performing switching TCI

How to configure maintained PL-RS / NOT maintained PL-RS in the test case

- Proposal 1: If an original PL-RS is configured, it is a maintained PL-RS; If a new RS is configured as PL-RS, it is a “not maintained PL-RS”

- Proposal 2: Whether to maintain PL-RS is up to UE implementation. The value of delay in test requirement can be calculated based on the worst case.

- Proposal 3: Before sending the TCI switch command, UE is expected to measure on the active TCI list and maintain the PL-RS.

- Proposal 4: For non-maintain PL-RS, PL-RS is configured in TCI state and TCI state is unknown.

TRP specific BFR

The time period of each procedure in the test case

- For TRP specific beam failure detection and link recovery test case, T2~T5 duration time may be extended due to scaling factor

- Take the table below as a starting point and further consider each value

| FR | RS | DRX | T1(s) | T2(s) | T3(s) | T4(s) | T5(s) | D1(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | SSB | Non-DRX | [1] | [2.61] | [3.24] | [0] | [1.97] | [1.93] |
| FR2 | CSI-RS | DRX | [1] | [10.81] | [10.28] | [0] | [0.57] | [0.53] |

Beam recovery method configured in the test case

- Option 1: Test only one configuration for test cases

  - For BFR on SPCells, CFRA based BFR is configured for all test cases

  - For BFR on SCells, dedicated BFR resource is configured for all test cases

- Option 2: Follow the legacy configuration as Rel-16 test cases

  - For BFR on SPCells, CFRA and CBRA based BFR is configured for different test cases

  - For BFR on SCells, dedicated BFR resource is configured and not configured for different test cases

- Option 3: Other configurations are not precluded

BFD-RS configured for TRP specific BFR test case

- Since SSBs with different index are always non-overlapped according to the pre-defined SSB pattern within half-frame for intra-cell TRPs, SSBs as BFD-RS cannot be overlapped in the test case.

  - Option 1: In the test case, if SSBs is configured as BFD-RS, they are not overlapped and the duration time will not be extended.

  - Option 2: In the test case, if SSBs is configured as BFD-RS, they comes from cells with different PCI.

  - Option 3: Other solution are not precluded.