3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215351

Electronic Meeting, 10 Oct. – 19 Oct. 2022

Agenda Item:	5.28.3

Source:	Charter Communications, Inc.

Title:	Discussion on UE RF requirements for SP and LPI

Document for:	Approval

# Introduction

In last RAN4 meeting, a WF for introducing Power class 3 (PC3) in NR-unlicensed spectrum was approved [1]. In this paper we are focusing on A-MPR simulations for Network signaling value NS_54. The paper will recap the requirements, simulations assumptions and present results for NS_54.

# Discussion

The networking signaling value NS_54 is used when operating in band n96. The following additional requirements are set when a UE signal NS_54. The UE is not allowed to exceed the density limits of 17dBm/MHz of in-band emissions for NS_54, as seen in Table 6.2F.1-2 in TS 38.101-1 [2]. Some of Table .2F.1-2 is shared below.

Table 6.2F.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n96 | NS_53 | 20, 40, 60, 80 | 5925 – 7125 | -1 |
|  | NS_54 | 20, 40, 60, 80 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |
|  | NS_59 | 20, 40, 60, 80 | 5925 – 7125 | 5 |
|  | NS_60 | 20, 40, 60, 80 | 5925 – 7125 | 2 |
|  | NS_61 | 20, 40, 60, 80 | 5925 - 6425 | 1 |
| n102 | NS_58 | 20, 40, 60, 80 | 5945 – 6425 | 10 |

Additional requirements for the spurious emission limit for operations using the NS_54 the TS 38.101-1 [2] says; “When "NS_53" or "NS_54" or "NS_60" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5F.3.3.5-1.”

Table 6.5F.3.3.5-1: Additional requirements

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

Based on the defined NR-ARFCN number for band n96 (Table 5.4.2.3-3) , the lowest number for each bandwidth using the formula

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |

This gives center frequencies:

- 20MHz Bandwidth: 5955MHz

- 40MHz Bandwidth: 5965.02MHz

- 60MHz Bandwidth: 5974.98MHz

- 80MHz Bandwidth: 5985MHz

- 100MHz Bandwidth: 5995.02MHz

Hence, this generates the lower channel bandwidth edge at 5945MHz for all channel bandwidths.

Table 5.4.2.3-3: Allowed NREF (NR-ARFCN) for operation in Band n96

| Channel Bandwidth | Allowed NREF |
| --- | --- |
| 20 MHz | 797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668, 829000, 830332, 831668, 833000, 834332, 835668, 837000, 838332, 839668, 841000, 842332, 843668, 845000, 846332, 847668, 849000, 850332, 851668, 853000, 854332, 855668, 857000, 858332, 859668, 861000, 862332, 863668, 865000, 866332, 867668, 869000, 870332, 871668, 873000, 874332 |
| 40 MHz | 797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000, 829668, 832332, 835000, 837668, 840332, 843000, 845668, 848332, 851000, 853668, 856332, 859000, 861668, 864332, 867000, 869668, 872332 |
| 60 MHz | 798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332, 830332, 831668, 835668, 837000, 841000, 842332, 846332, 847668, 851668, 853000, 857000, 858332, 862332, 863668, 867668, 869000, 873000 |
| 80 MHz | 799000, 804332, 809668, 815000, 820332, 825668, 831000, 836332, 841668, 847000, 852332, 857668, 863000, 868332 |
| 100 MHz | 799668, 803668, 810332, 814332, 821000, 825000, 831668, 835668, 842332, 846332, 853000, 857000, 863668, 867668, 869000, 870332, 871668 |

In other words, the additional requirements are offset from the channel bandwidth edge by 20MHz. Also, there is only the first and perhaps also the second edge channels of each bandwidth that will be affected by the additional band specific requirements. All other channels will follow the MPR backoff limits.

## Simulation agreements

From the Way Forward [1], simulation agreements were

### Way Forward on PC3 A-MPR for 1Tx:

- PC3 calibration point: 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB0 waveform at 30dB ACLR

- Post PA losses 4dB

Impairment assumptions

- CIM3: 60dBc

- CIM5: 70dBc

- IQ image: 28 dB

- Carrier leakage: 28dBc

## Results

Charter’s simulation results for NS_54 with Power Class 3, see Table 1.

Table 1: A-MPR for NS_54 power class 3

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-OFDM | QPSK |  | ≤ [3.0] | ≤ [5.0] |
|  | 16 QAM |  | ≤ [3.5] | ≤ [5.0] |
|  | 64 QAM |  | ≤ [3.5] | ≤ [5.0] |
|  | 256 QAM |  | ≤ [5.0] | ≤ [6.0] |
| CP-OFDM | QPSK |  | ≤ [4.5] | ≤ [6.0] |
|  | 16 QAM |  | ≤ [5.0] | ≤ [6.0] |
|  | 64 QAM |  | ≤ [5.5] | ≤ [6.0] |
|  | 256 QAM |  | ≤ [6.5] | ≤ [7.0] |
| NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz], and 100 MHz channels centered at the nearest NR-ARFCN corresponding to [5995 MHz]. |  |  |  |  |

For NOTE 2, all the other valid channels and bandwidths other than those enumerated in NOTE 3 will follow the MPR table for Power class 3 for shared spectrum.

Proposal 1: Agree on the A-MPR PC3 simulation results presented in Table 1.

# Conclusion

In this paper, Charter proposes the following:

Proposal 1: Agree on the A-MPR PC3 simulation results presented in Table 1.

# Reference

[1] R4-2214437, WF on NR-U PC3 requirements, Charter Communications Inc.

[2] TS 38.101-1: User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V17.6.0 (2022-06)