3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215379

10th Oct – 19th Oct, 2022

Source:	Qualcomm Incorporated

Title:	Investigation of band combinations for MSD reduction

Agenda item:	6.6.4.1

Document for:	Approval

# 1.	Introduction

In RAN4#104-e [1] it was decided that several band combinations having different impairments could be studied by companies to determine whether MSD reduction is possible. In this paper we present our analysis for lower MSD on some of these combinations.

# 2. 	Discussion

In WF [1] it was agreed that the following band combinations would be studied for the possibility of lowering MSD.

## 1.1 Candidate example band combinations

Use the following example band combinations to study the feasibility of MSD improvement for different MSD types:

- CA_n28-n40 (harmonic mixing)

- CA_n41-n77 (cross band isolation)

- CA_n1-n3-n78 and fallback combinations (IMD on the 3rd band, cross band isolation on CA_n1-n3 using 50MHz channel bandwidth, IMD2/4 and 2nd harmonic and harmonic mixing on CA_n3-n78)

Note 1: All supported power classes for the above example band combinations can be analyzed

Note 2: Band combinations with two bands are in the first priority

## 1.2 power class to be considered for the band combinations in lower MSD study

Both PC2 and PC3 band combinations could be considered as example band combination. Other power class is not excluded.

For the band combination CA_n1-n3-n78 it is seen that CA_n3-n78 has IMD2, IMD4 and HD2 impairments.

Based on typical numbers table 1 gives an analysis for IMD2 for CA-n3_n78 where the IMD2 product of n78 Tx and n3 Tx fall into n3 Rx.

| Ant-Ant Isolation | 10 | dB |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PCB Isolation | 70 | dB |  |  |  |
| Power class | PC2 |  |  |  |  |
| n77 PA output power | 30 | dBm |  |  |  |
| IMD2 at n78 PA output | -71 | dBm |  |  |  |
| Diplexer attenuation | 15 | dB |  |  |  |
| Total IMD2 @n78 ANT | -86.0 | dBm |  |  |  |
| Total IMD2 @n3 ANT | -96.0 | dBm |  |  |  |
| n77 jammer @ n3 PA input | -40.0 | dBm |  |  |  |
| IMD2 at n3 PA output | -65.0 | dBm |  |  |  |
| Post PA IL | 2.0 | dB |  |  |  |
| Total IMD2 @n3 ANT | -63.0 | dBm |  |  |  |
| PRX n3 path |  |  |  | DRX n3 path |  |
| Mixer IIP2 | 72.0 | dB |  | 72.0 | dB |
| LNA IIP2 | 30.0 | dBm |  | 30.0 | dBm |
| Noise from mixer IIP2 distortion | -96.0 | dBm |  | -96.0 | dBm |
| Rx IMD @ ANT | -94.0 | dBm |  | -94.0 | dBm |
| Total distortion @ n3 ANT | -63.0 | dBm |  | -73.0 | dBm |
| Reference sensitivity | -98.0 | dBm |  | -98.0 | dBm |
| Thermal noise | -94.0 | dBm |  | -94.0 | dBm |
| total noise @ n3 ANT | -63.0 |  |  | -72.9 | dBm |
| MRC Sensitivity | -72.4 |  |  |  | dBm |
| MSD | 25.6 |  |  |  | dB |

Table 1 – CA-n3_n78 budget for IMD2

Based on the above numbers it is seen that for improvement in PCB and antenna isolation the MSD due to IMD2 behaves as indicated in table 2.

| Ant ISO (dB) | PCB ISO (dB) | MSD (dB) |
| --- | --- | --- |
| 10 | 70 | 25.6 |
| 10 | 80 | 17.3 |
| 10 | 90 | 13.0 |
| 20 | 70 | 24.6 |
| 20 | 80 | 14.4 |
| 20 | 90 | 8.1 |

Table 2 – CA-n3_n78 MSD for IMD2 as a function of PCB and antenna isolation

From the results in table 2 it can be clearly seen that the MSD for IMD2 can be reduced by increasing both the PCB and antenna isolations.

Observation 1: For CA-n3_n78 the MSD due to IMD2 where n78 Tx and n3 Tx mix and fall into n3 Rx shows the following behaviour with PCB and antenna isolation:

| Ant ISO (dB) | PCB ISO (dB) | MSD (dB) |
| --- | --- | --- |
| 10 | 70 | 25.6 |
| 10 | 80 | 17.3 |
| 10 | 90 | 13.0 |
| 20 | 70 | 25.6 |
| 20 | 80 | 14.4 |
| 20 | 90 | 8.1 |

Similar analysis for CA-n3_n78 for IMD4 where the IMD4 of n78 Tx and 3*n3 Tx falls into n3 Rx shows the following behaviour of MSD with PCB and antenna isolation given in table 3.

| Ant ISO (dB) | PCB ISO (dB) | MSD (dB) |
| --- | --- | --- |
| 10 | 70 | 9.5 |
| 10 | 80 | 5.5 |
| 10 | 90 | 4.0 |
| 20 | 70 | 4.8 |
| 20 | 80 | 3.9 |
| 20 | 90 | 2.7 |

Table 3 – CA-n3_n78 MSD for IMD4 as a function of PCB and antenna isolations

From the results in table 3 it is seen that the MSD for IMD4 can be reduced by increasing both the PCB and antenna isolations.

Observation 2: For CA-n3_n78 the MSD due to IMD4 where n78 Tx and 3*n3 Tx mix and fall into n3 Rx shows the following behaviour with PCB and antenna isolation:

| Ant ISO (dB) | PCB ISO (dB) | MSD (dB) |
| --- | --- | --- |
| 10 | 70 | 9.5 |
| 10 | 80 | 5.5 |
| 10 | 90 | 4.0 |
| 20 | 70 | 4.8 |
| 20 | 80 | 3.9 |
| 20 | 90 | 2.7 |

For CA-n3_n78 for the 2nd harmonic of n3 Tx falling into n78 Rx we have the following typical budget

| Band | n3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| HD order | 2 |  |  |  |  |
| Rx BW | 100 | MHz |  |  |  |
| PC | 2 |  |  |  |  |
| Ant-Ant Isolation | 10 | dB |  |  |  |
| PCB Isolation | 70 | dB |  |  |  |
| n3 PA HD2 | -6 | dBm |  |  |  |
| HD2 filtering in Tx chain | 48 | dB |  |  |  |
| Triplexer attenuation | 25 | dB |  |  |  |
| n3 HD2 @Ant | -79 | dB |  |  |  |
| PRX n78 path |  |  |  | DRX n78 path |  |
| Rx FE loss | 3 | dB |  | 3 | dB |
| Total HD pwr at Ant input | -72.9 | dBm |  | -72.9 | dBm |
| Thermal noise @RX Ant | -81.5 | dBm |  | -81.5 | dBm |
| total noise @ n3 ANT | -72.3 | dBm |  | -72.3 | dBm |
| Reference sensitivity | -85.5 | dBm |  | -85.5 | dBm |
| MRC Sensitivity | -73.6 |  |  |  | dBm |
| MSD | 11.9 |  |  |  | dB |

Table 4 – CA-n3_n78 budget for HD2

It is seen that for improvement in PCB and antenna isolation the MSD due to HD2 behaves as indicated in table 5.

| PCB ISO (dB) | Ant ISO (dB) | MSD (dB) |
| --- | --- | --- |
| 70 | 10 | 11.9 |
| 80 | 10 | 4.4 |
| 90 | 10 | 1.7 |
| 70 | 20 | 11.8 |
| 80 | 20 | 3.9 |
| 90 | 20 | 0.7 |

Table 5 – CA-n3_n78 MSD for HD2 as a function of PCB and antenna isolation

Observation 3: For CA-n3_n78 the MSD due to HD2 where the second harmonic of n3 falls into n78 Rx shows the following behaviour with PCB and antenna isolation:

| PCB ISO (dB) | Ant ISO (dB) | MSD (dB) |
| --- | --- | --- |
| 70 | 10 | 11.9 |
| 80 | 10 | 4.4 |
| 90 | 10 | 1.7 |
| 70 | 20 | 11.8 |
| 80 | 20 | 3.9 |
| 90 | 20 | 0.7 |

For CA-n41_n77 where cross band isolation is an issue with Tx noise from n41 falling into n77 and vis-a-versa. The below table shows the budget for n77 Tx noise falling into n41.

| Aggressor band | n77 | PC3 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Victim band | n41 |  |  |  |  |
| Transceiver noise at PA output | -154.5 | dBm/Hz |  |  |  |
| PA output noise | -150 | dBm/Hz |  |  |  |
| Total noise at PA output | -148.7 | dBm/Hz |  |  |  |
| Antenna iso | 15 | dB |  |  |  |
| Tx noise @Rx | -163.7 | dBm/Hz |  |  |  |
| PRX path for n41 |  |  |  | DRX path for n41 |  |
| DL BW | 10.0 | MHz |  | 10 | MHz |
| Rx thermal noise @ LNA input | -91.1 | dBm |  | -91.1 | dBm |
| DAC images at Tx output | -91.7 | dBm |  | -91.7 | dBm |
| DAC images at RX input | -106.7 | dBm |  | -106.7 | dBm |
| Total interference @ Rx | -93.5 | dBm |  | -93.5 | dBm |
| Total noise @ Rx | -89.1 | dBm |  | -89.1 | dBm |
| n41 Reference sensitivity | -95.1 | dBm |  |  |  |
| MRC sensitivity | -91.8 | dBm |  |  |  |
| MSD | 3.3 | dBm |  |  |  |

Table 6 – CA-n41_n77 budget for cross band isolation with n41 victim

The MSD in the current specifications for this combination is only 4.5dB and currently with 15 dB of antenna isolation the MSD is 3.3 dB. Therefore, even with large increases in antenna isolation the maximum MSD reduction that can be achieved is only a further 3.3 dB. The below table gives the variation in MSD with antenna isolation. As the lower MSD feature is meant to address band combinations with large MSDs we think that the low MSD of this band combination does not make it a candidate for this feature.

| Ant ISO (dB) | MSD (dB) |
| --- | --- |
| 15 | 3.3 |
| 20 | 1.3 |
| 25 | 0.5 |

Table 7 – CA-n41_n77 variation of MSD with antenna isolation for n41 victim

Observation 4: For CA-n41_n77 the MSD due to cross band isolation where n77 Tx noise falls into n41 Rx the following behaviour is observed with antenna isolation:

| Ant ISO (dB) | MSD (dB) |
| --- | --- |
| 15 | 3.3 |
| 20 | 1.3 |
| 25 | 0.5 |

In our opinion the low MSD for this band combination does not make it a candidate for the low MSD feature

For CA-n28_n40 for the 3rd RX LO harmonic of n28 mixing with n40 Tx and falling in the n28 Rx band we can have the following typical budget.

| n40 Tx output power at ANT | 23 | dBm |  |  |
| --- | --- | --- | --- | --- |
| n40 Tx FE loss | 3 | dB |  |  |
| Antenna isolation | 10 | dB |  |  |
| n28 Duplexer attenuation for n40 Tx | 15 | dB |  |  |
| n28 FE attenuation for n40 Tx | 50 | dB |  |  |
| n28 FE gain | 15 | dB |  |  |
| PCB isolation From n40 PA output to n28 transceiver input | 70 | dB |  |  |
| PRX |  |  | DRX |  |
| n28 RX BW | 10 | MHz | 10 | MHz |
| Total interferer at n28 RX input | -43.4 | dBm | -43.4 | dBm |
| 3LO harmonic rejection | 10 | dB | 10 | dB |
| Total interference at ANT | -68.4 | dBm | -68.4 | dBm |
| Thermal noise | -91.5 | dBm | -91.5 | dBm |
| Total noise | -68.3 | dBm | -68.3 | dBm |
| Sensitivity | -95.5 | dBm |  |  |
| MRC sensitivity | -69.4 |  |  |  |
| MSD | 26.1 | dB |  |  |

Table 8– CA-n28_n40 budget for Rx LO harmonic mixing with n40 Tx falling into n28 Rx

It is seen that for improvement in PCB and antenna isolation the MSD behaves as indicated in table 8.

| PCB iso (dB) | Ant iso(dB) | MSD (dB) |
| --- | --- | --- |
| 70 | 10 | 26.1 |
| 80 | 10 | 19.7 |
| 90 | 10 | 17.8 |
| 70 | 20 | 25.6 |
| 80 | 20 | 16.2 |
| 90 | 20 | 10.1 |

Table 9 – CA-n28_n40 MSD for Rx LO harmonic falling into n28 Rx as a function of PCB and antenna isolation

From the above table it is observed that the MSD is dominated by the PCB isolation and that the antenna isolation has very little impact.

Observation 5: For CA-n28_n40 the MSD due to 3rd RX LO harmonic of n28 mixing with n40 Tx and falling into 28 Rx the following behaviour is observed with PCB and antenna isolation:

| PCB iso (dB) | Ant iso(dB) | MSD (dB) |
| --- | --- | --- |
| 70 | 10 | 26.1 |
| 80 | 10 | 19.7 |
| 90 | 10 | 17.8 |
| 70 | 20 | 25.6 |
| 80 | 20 | 16.2 |
| 90 | 20 | 10.1 |

For most band combinations that have large MSDs an increase in the PCB and/or antenna isolation will enable a reduction in MSD.

Observation 6: The band combinations analysed in this study shows that MSD can be lowered by increasing PCB and or antenna isolations

# Conclusion

Several band combinations from those suggested in WF [1] were analysed and the following observations are made:

Observation 1: For CA-n3_n78 the MSD due to IMD2 where n78 Tx and n3 Tx mix and fall into n3 Rx shows the following behaviour with PCB and antenna isolation:

| Ant ISO (dB) | PCB ISO (dB) | MSD (dB) |
| --- | --- | --- |
| 10 | 70 | 25.6 |
| 10 | 80 | 17.3 |
| 10 | 90 | 13.0 |
| 20 | 70 | 25.6 |
| 20 | 80 | 14.4 |
| 20 | 90 | 8.1 |

Observation 2: For CA-n3_n78 the MSD due to IMD4 where n78 Tx and 3*n3 Tx mix and fall into n3 Rx shows the following behaviour with PCB and antenna isolation:

| Ant ISO (dB) | PCB ISO (dB) | MSD (dB) |
| --- | --- | --- |
| 10 | 70 | 9.5 |
| 10 | 80 | 5.5 |
| 10 | 90 | 4.0 |
| 20 | 70 | 4.8 |
| 20 | 80 | 3.9 |
| 20 | 90 | 2.7 |

Observation 3: For CA-n3_n78 the MSD due to HD2 where the second harmonic of n3 falls into n78 Rx shows the following behaviour with PCB and antenna isolation:

| PCB ISO (dB) | Ant ISO (dB) | MSD (dB) |
| --- | --- | --- |
| 70 | 10 | 11.9 |
| 80 | 10 | 4.4 |
| 90 | 10 | 1.7 |
| 70 | 20 | 11.8 |
| 80 | 20 | 3.9 |
| 90 | 20 | 0.7 |

Observation 4: For CA-n41_n77 the MSD due to cross band isolation where n77 Tx noise falls into n41 Rx the following behaviour is observed with antenna isolation:

| Ant ISO (dB) | MSD (dB) |
| --- | --- |
| 15 | 3.3 |
| 20 | 1.3 |
| 25 | 0.5 |

In our opinion the low MSD for this band combination does not make it a candidate for the low MSD feature

Observation 5: For CA-n28_n40 the MSD due to 3rd RX LO harmonic of n28 mixing with n40 Tx and falling into 28 Rx the following behaviour is observed with PCB and antenna isolation:

| PCB ISO (dB) | Ant ISO (dB) | MSD (dB) |
| --- | --- | --- |
| 70 | 10 | 26.1 |
| 80 | 10 | 19.7 |
| 90 | 10 | 17.8 |
| 70 | 20 | 25.6 |
| 80 | 20 | 16.2 |
| 90 | 20 | 10.1 |

Observation 6: The band combinations analysed in this study shows that MSD can be lowered by increasing PCB and or antenna isolations

# References

[1] R4-2214452, WF on study for lower MSD, RAN4#104-e, Aug 15-26, 2022