3GPP TSG-RAN WG4 Meeting #104-bis-e                                                                                  R4-2215411

Electronic Meeting, October 10 – October 19, 2022

Source: 	CATT

Title: 	Discussion on conformance testing for NTN SAN

Agenda item:	4.2.3.1.3

Document for:	Approval

- Introduction

In RAN4 #104-e meeting, A WF [1] on NTN SAN RF conformance was agreed. Some remaining issues in WF need further discussion in next meeting.

In this contribution, we provide our views on remaining issues for conformance testing for NTN SAN.

- Discussion

Declaration identifier for conducted testing and radiated testing

As per the WF [1], the remaining issues concerning manufacturer declaration are shown as below:

- Sub-topic 1-2: Manufacturer declarations Option 1: Manufacturer declarations will be in 2 tables, one for Tx and one for Rx. The following naming shall be used - Using declaration identifier D.x for conducted testing. - Using declaration identifier DE.x for radiated testing. Option 2: other proposals not precluded

In last RAN4#104-e meeting, we propose the option 1, there are two tables for manufacturer declarations, one table is for conducted testing and the other table is for radiated testing. Other companies suggest that there can be one table covering all manufacturer declarations for conducted testing and radiated testing. We can also accept that there can be one table covering all manufacturer declarations for conducted testing and radiated testing.

Proposal 1: One table covering all manufacturer declarations for conducted testing and radiated testing, using unified declaration identifier D.x for conducted testing and radiated testing is acceptable.

Test model naming

In last RAN4#104-e meeting, other company raised test model naming issue. To maintain NTN specification independence, full content and description of test models for NTN in 38.181 is needed, and referring to NR BS specification is not good choice. To avoid naming confusion, the NR-SAN-FR1-TM can be used for NTN test model naming.

Proposal 2: Using NR-SAN-FR1-TM for NTN test model naming.

Beam peak direction

In RAN4#104-e meeting, some companies were concerned about beam peak direction as below:

First we need to have more discussion is consideration of declarations such as “beam peak direction” and other spatial declarations make sense for NTN scenario.

Referring to TR 37.842, the beam peak direction and beam centre direction are defined in Fiugre F-3 copied as below.

There are two types of beams, one type of beam is beam with ripple(left), and other type of beam is beam with no ripple(right). The definition of beam direction pair in TN supports two types of beams. For NTN beam, it is high gain and narrow beamwidth beam, which is beam with no ripple. So, the beam peak direction and beam centre direction always align. And NTN beam is one of two types of beams defined in TN. So we think beam direction pair in TN are applicable for NTN.

Proposal 3: Beam direction pair in TN is applicable for NTN

Figure F-3: Example of position of beam centre direction and beam peak direction for a beam with ripple (left) and a beam with no ripple (right)

Conformance test directions for manufacturer declaration

In RAN4#104-e meeting, some companies were concerned about conformance test directions for manufacturer declaration as below:

Conformance test directions: more analysis is needed to verify if the reuse from TN is reasonable for NTN scenario.

From our understanding, for NTN scenario, one beam is used for one NTN cell, which can be considered as fixed beam. From Transmitter side, the adaptive beamforming is not supported. Reference beam centre direction is one point in Θ-Φ plot shown in Figure 2-1, and four OTA peak directions set maximum steering direction(s) coincide with the reference beam centre direction. Currently the declaration “OTA peak directions set maximum steering direction(s)” from TN supports “The maximum steering direction(s) may coincide with the reference beam centre direction”.

Figure 2-1 examples for declaration “OTA peak directions set maximum steering direction(s)” for one NTN beam

From Receiver side, the redirection of receiver target is not supported. In this case, receiver target redirection range coincides with sensitivity RoAoA, it also can be considered as “OSDD does not include a receiver target redirection range”. For this case, sensitivity RoAoA is 2D beamwidth contour in Θ-Φ plot shown in Figure 2-2, and four maximum direction(s) inside sensitivity RoAoA and receiver target reference direction need to be declared. Currently the declaration “Conformance test directions” for receiver from TN supports “OSDD does not include a receiver target redirection range”.

Figure 2-2 examples for declaration “Conformance test directions” for receiver

For OTA REFSENS conformance test directions, the OTA REFSENS RoAoA is sensitivity RoAoA in Figure2-2. For this case, OTA REFSENS RoAoA is 2D beamwidth contour in Θ-Φ plot shown in Figure 2-2, and four maximum direction(s) inside OTA REFSENS RoAoA and OTA REFSENS receiver target reference direction need to be declared. Currently the declaration “OTA REFSENS conformance test directions” from TN supports above five directions declaration.

Based on above analysis, we think that the conformance test directions declaration including OTA peak directions set maximum steering direction(s), conformance test directions for OTA sensitivity, and OTA REFSENS conformance test directions from TN are applicable for NTN.

Proposal 4: The conformance test directions declaration including OTA peak directions set maximum steering direction(s), conformance test directions for OTA sensitivity, and OTA REFSENS conformance test directions from TN are applicable for NTN.

Requirements shall be met for any transmitter setting

In RAN4#104-e meeting, some companies had concern about “Requirements shall be met for any transmitter setting.” as below:

We also need to understand what does it mean This appears in other BS specs as well 😊 Are we referring to any supported RF configuration of the Tx? (e.g. nr of carriers, contiguous-non contiguous operation, modulation, power level, etc) or we refer to setting the transmitter on or off? And if it is ON does it mean it transmits data or not? Maybe need to review a bit that bulleted list.

For clarification, from our understanding, any transmitter setting in “Requirements shall be met for any transmitter setting.” may include the following:

1) The applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8.

2) The corresponding test models or set of physical channels in clause 4.9.2.

3) For OTA test, beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

4) for conducted test, set the connector under test to transmit at manufacturers declared rated carrier output power (Prated,c,AC or Prated,c,TABC, D.21).

How to set those setting is described in detail in procedure of each receiver requirement. So “Requirements shall be met for any transmitter setting.” from TN can be reused for NTN.

Proposal 5: “Requirements shall be met for any transmitter setting.” from TN can be reused for NTN.

Co-location and CLTA related declaration, MU, and requirement

The “Protection of the BS receiver of own or different BS” in TS 38.108 is defined as a colocation requirement shown in the following table.

This requirement shall be applied for NR FDD operation in order to prevent degradation of own receivers by emissions from a type 1-O SAN. This requirement is a co-location requirement as defined in clause 4.9, the power levels are specified at the co-location reference antenna output. The total power of any spurious emission from both polarizations of the co-location reference antenna connector output shall not exceed the limits in clause 6.6.5.2.2.

Referring to TS 38.108, the co-location requirement as defined in clause 4.9 in TS 38.108 is missing. At same time, the 30dB isolation between co-location reference antenna and BS isn’t considered. The total power of any spurious emission from both polarizations of the co-location reference antenna connector output shall not exceed the basic limits in clause 6.6.5.2.2 + X dB, where X = -30 dB.

Some companies suggest that justification to keep such co-location requirement for SAN may require being re-visited. Because the spurious emissions basic limits for protection of the SAN receiver is -96dBm/100kHz,  whether such lower power can be tested by TRP need more discussion. If -96dBm/100kHz can be tested by TRP, we support this protection of the BS receiver of own or different BS requirement as TRP requirement.

Proposal 6: If -96dBm/100kHz can be tested by TRP, we support protection of the BS receiver of own or different BS requirement as TRP requirement.

Proposal 7:  If keeping BS receiver of own or different BS requirement as co-location requirement. The following works are needed.

1)  Add co-location requirement in clause 4.9 in TS 38.108.

2)  In TS 38.108, the total power of any spurious emission from both polarizations of the co-location reference antenna connector output shall not exceed the basic limits in clause 6.6.5.2.2 + X dB, where X = -30 dB.

3) Co-location and CLTA related declaration, MU, and requirement need to be added in TS 38.181.

OTA measurement setup

As per the WF [1], the remaining issues OTA measurement setup are shown as below:

Issue 3-1-2: Measurement set-up It is recommended to continue discussions and receive more input from TE vendors about how such a task can be performed.

Figure 2-4: Radiated reference points for BS type 1-O (left), SAN type 1-O(right)

Figure 2-5: Measurement set up for radiated transmit power, OTA output power dynamics, OTA transmitted signal quality, and OTA occupied bandwidth.

From our consideration, comparing the radiated reference points for BS type 1-O and SAN type 1-O in Figure 2-4, we can find that the NTN payload RF is similar to NR BS. We think the OTA measurement setup for TN in annex of TS 38.141-2 can be reused for NTN, and NR BS in measurement setup figure should be replaced by NTN payload RF. For example, the Figure 2-5 is measurement setup for radiated transmit power, OTA output power dynamics, OTA transmitted signal quality, and OTA occupied bandwidth for NTN.

Proposal 8: OTA measurement setup for TN in annex of TS 38.141-2 can be reused for NTN, and NR BS in measurement setup figure should be replaced by NTN payload RF.

Non-contiguous spectrum operation

Referring to TS 38.108, the non-contiguous spectrum operation is not applicable currently. So, non-contiguous spectrum operation related is not needed in TS 38.181.

Proposal 9: Non-contiguous spectrum operation related is not needed in TS 38.181.

Multi-band operation

Referring to TS 38.108, the multi-band operation is not applicable currently. So, multi-band operation related is not needed in TS 38.181.

Proposal 10: Multi-band operation related is not needed in TS 38.181.

- Conclusion

This contribution provides our general consideration on RF conformance testing for NTN SAN. The following proposals are concluded as follows:

Proposal 1: One table covering all manufacturer declarations for conducted testing and radiated testing, using unified declaration identifier D.x for conducted testing and radiated testing is acceptable.

Proposal 2: Using NR-SAN-FR1-TM for NTN test model naming.

Proposal 3: Beam direction pair in TN is applicable for NTN

Proposal 4: The conformance test directions declaration including OTA peak directions set maximum steering direction(s), conformance test directions for OTA sensitivity, and OTA REFSENS conformance test directions from TN are applicable for NTN.

Proposal 5: “Requirements shall be met for any transmitter setting.” from TN can be reused for NTN.

Proposal 6: If -96dBm/100kHz can be tested by TRP, we support protection of the BS receiver of own or different BS requirement as TRP requirement.

Proposal 7:  If keeping BS receiver of own or different BS requirement as co-location requirement. The following works are needed.

1)  Add co-location requirement in clause 4.9 in TS 38.108.

2)  In TS 38.108, the total power of any spurious emission from both polarizations of the co-location reference antenna connector output shall not exceed the basic limits in clause 6.6.5.2.2 + X dB, where X = -30 dB.

3) Co-location and CLTA related declaration, MU, and requirement need to be added in TS 38.181.

Proposal 8: OTA measurement setup for TN in annex of TS 38.141-2 can be reused for NTN, and NR BS in measurement setup figure should be replaced by NTN payload RF.

Proposal 9: Non-contiguous spectrum operation related is not needed in TS 38.181.

Proposal 10: Multi-band operation related is not needed in TS 38.181.

- References

- R4- 2214371, WF on NTN SAN RF conformance, Ericsson, RAN4#104-e

- R4-2210634, WF on NTN SAN RF conformance, Ericsson, RAN4#103-e