3GPP TSG RAN WG4 Meeting #104-bis-e                   R4- 2215416

Electronic Meeting, October 10th - October 19th, 2022

Agenda item:			4.3.5

Source:	CATT

Title:	Discussion of remaining issues on RRM core requirements for extension to 71GHz

Document for:	Discussion

- Introduction

In the last RAN4 meeting, WF [1] on RRM requirements from 52.6GHz to 71GHz was approved, however we still haven't reached consensus on some issues.

In this contribution, the remaining issues on RRM core requirements for extension to 71GHz are further discussed.

- Discussion

## General requirements

In the RAN4#104-e meeting, the following agreement has been reached for cell detection relaxation in general requirements [1]:

Issue 1-3-1: Cell detection relaxation FFS - Option 1: Introduce the factor KFR to account for the cell-detection relaxation agreed in RAN4 #103, where: For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. - Option 2: Introduce UE capability on relaxed requirements for cell detection.

For the above two options, we prefer to introduce UE capability on relaxed requirements for cell detection. We understand the intention of option 1, but we think that it is too rigid to directly select a suitable value for different SCS, and we need to comprehensively weigh various factors. For 480 kHz and 960 kHz, the recommended KFR value will cause a long delay, the delay will be even longer than 120 kHz for some senior UEs, which is unreasonable.

Therefore, we prefer a more flexible method, such as option 2, which makes it easier for UEs of different levels to flexibly support various mobility cases without causing long cell detection delays for all UEs. Compared with option 1, it is a more compromise method.

In addition, some companies think that we only need to define the minimum RAN4 requirements, and UE can always do better [2], but we think that we should consider common situations as much as possible instead of always defining the minimum requirements, and the final implementation is basically based on the defined requirements.

Proposal 1: Introducing UE capability on relaxed requirements is a more compromise method to make it easier for UEs of different levels to flexibly support various mobility cases without causing long cell detection delays for all UEs.

## Timing requirements

In the last RAN4 meeting, there is still no final consensus on the timing advantage adjustment accuracy, and the agreement made during the last meeting is duplicated as below [1]:

Issue 2-1-1: Timing Advance adjustment accuracy FFS: - Option 1（Apple）: UL Sub Carrier Spacing(kHz); 15; 30; 60; 120; 480; 960. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc; [±12 Tc]; [±8 Tc]. - Option 2: UL Sub Carrier Spacing(kHz); 15; 30; 60; 120; 480; 960. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc; [±8 Tc]; [±4 Tc].

The discussion on the timing advantage adjustment accuracy has lasted for many meetings. As shown above, only one company believed that we need to further consider the implementation issues of UL Sub Carrier Spacing with 480 kHz and 960 kHz, and take option 1 as a compromise solution.

However, we believe that option 2, as a conclusion that has been formed before, is more reasonable to calculate with linear scale, because we cannot ignore the UL uncertainty budget, and more companies still support this option. Proponents of option 1 need to further provide more specific requirements for implementation.

Proposal 2: Since we cannot ignore the UL uncertainty budget, option 2, as a conclusion formed before, is more reasonable to calculate with linear scale.

Proposal 3: It is suggested that the proponents of Option 1 further provide more specific implementation requirements.

## LBT impact on RRM requirements

In the last RAN4 meeting, there is still no final consensus on the maximum separation between two consecutive measurements, and the two options are duplicated as below [1]:

Issue 3-1-1: maximum separation between two consecutive measurements FFS: - Option 1: The requirement apply provided any two measurement shall not be separated in time by more than the maximum time requirement for the cell to remain known. - Option 2: The requirement only applies when - Within the set of measurements any two measurements shall not be separated in time by more than 2 seconds when no DRX is configured, and - Within the set of measurements any two measurements shall not be separated in time by more than max(1 DRX occasion group duration, 2 seconds) when DRX is configured.

In our opinion, the difference between option 1 and option 2 is whether only a fixed maximum time is used to separate samples. In addition to the fixed maximum separation time, Option 2 also takes DRX occasion group duration into account when configuring DRX, which we think is reasonable.

The SMTC occasion group or DRX group duration depends on the SMTC and DRX configurations of each UE and the RX beam sweeping factor, while the SMTC and DRX configurations depend on the mobility scenarios. In FR2-2, various channel conditions will also change more significantly, we need to consider different mobility scenarios as one of the factors. Therefore, for maximum separation between two consecutive measurements, it is necessary to consider DRX occasion group duration.

In addition, the RX beam sweeping factor in FR2-2 has also changed, SMTC occasion group or DRX group duration is also different from that before. It is meaningful to separate samples by using the maximum value between a given number of DRX occasion groups and the maximum period between samples.

Observation 1: The SMTC occasion group or DRX group duration depends on the SMTC and DRX configurations of each UE and the RX beam sweeping factor, while the SMTC and DRX configurations depend on the mobility scenarios.

Observation 2: The RX beam sweeping factor in FR2-2 has also changed, SMTC occasion group or DRX group duration is also different from that before.

Proposal 4: In addition to the fixed maximum separation time, option 2 also takes DRX occasion group duration into account when configuring DRX is more reasonable.

- Summary

In this paper, we provide our views on general RRM impact for the extension to 71 GHz. From this discussion we have derived the following observations and proposals:

Proposal 1: Introducing UE capability on relaxed requirements is a more compromise method to make it easier for UEs of different levels to flexibly support various mobility cases without causing long cell detection delays for all UEs.

Proposal 2: Since we cannot ignore the UL uncertainty budget, option 2, as a conclusion formed before, is more reasonable to calculate with linear scale.

Proposal 3: It is suggested that the proponents of Option 1 further provide more specific implementation requirements.

Observation 1: The SMTC occasion group or DRX group duration depends on the SMTC and DRX configurations of each UE and the RX beam sweeping factor, while the SMTC and DRX configurations depend on the mobility scenarios.

Observation 2: The RX beam sweeping factor in FR2-2 has also changed, SMTC occasion group or DRX group duration is also different from that before.

Proposal 4: In addition to the fixed maximum separation time, option 2 also takes DRX occasion group duration into account when configuring DRX is more reasonable.

- References

- R4-2214476 WF on NR extension to 71 GHz RRM requirements (Part 1), Huawei, HiSilicon

- R4-2214270 Email discussion summary for [104-e][218] NR_ext_to_71GHz_RRM_1, Moderator (Huawei)