3GPP TSG-RAN WG4 Meeting # 104-bis-e      	R4-2215501

Electronic Meeting, Oct 10 – Oct 19, 2022

Agenda Item:	4.2.6.1

Source: 	CMCC

Title: 			 Discussion on RRM test cases for NTN

Document for:	Discussion

- Introduction

In RAN4#104-e meeting, the NTN RRM test cases were discussed, a way forward was agreed in [1]. In this contribution, we further provide our views on open issues.

- Test case coverage, design and configuration

Issue 2-3: Test coverage regarding numerology and duplex mode.

Agreement:

  - Test configurations for TDD and numerology higher than 15kHz are not considered.

  - FFS on whether to define UE timing TC for 30KHz SCS.

    - Test configuration includes both 15KHz SCS and 30KHz SCS cases in the same TC, no separate TC is defined for 30KHz SCS case.

Among current TN timing test cases, there is no test configuration for FDD 30kHz SCS. In NTN, we also mainly focus on FDD 15kHz SCS scenario. Therefore, don’t define UE timing test configuration for FDD 30kHz SCS is fine for us.

Proposal 1: No need to define UE timing test configuration for FDD 30kHz SCS scenario.

- Test case design for Cell reselection

Issue 3-1: General test design for intra/inter-frequency cell reselection.

Agreement:

  - RAN4 needs to study proper values of signal quality/power parameters to model the satellite communication condition.

  - To verify time based cell reselection measurement (time based), the signal level of active cells should be larger than Srxlev or Squal threshold in the test design.

  - RAN4 not include the trigger cases: “t-service+10” and “T1+10” in TCs.

    - FFS whether to include “t-service” configuration in TCs.

  - FFS on whether and how to include “K_multi” parameter in the tests

First, we support to model only one cell under one satellite. To avoid too complicated test scenario, we suggest that the total number of cells (satellites) is not larger than 2 in cell reselection TC.

The cases with K_multi_SMTC larger than 1 are as follows given the basic assumption above.

- Case 1: NGSO, one SMTC configured, 2 satellites measured on 1 SMTC. UE don’t support of simultaneously measurements on more than 1 target cells belonging to different NGSO satellites within a SMTC

- Case 2: NGSO, two SMTC configured, SMTC partially overlap with each other, 2 satellites measured on 2 SMTC separately.

- Case 3: GSO, two SMTC configured, SMTC partially overlap with each other, 2 satellites measured on 2 SMTC separately.

For the three cases above, the configuration in Case 2 is more typical in real network compared with Case 1 and Case 3. Therefore, we propose to at least introduce Case 2 with K_multi_SMTC = 2. Regarding to the test parameters of Case 2, the length of T2 and T3 and the cell re-selection delay requirements should be multiplied by K_multi_SMTC.

In addition, we also support to introduce Case 1 as test configuration. Regarding to the test parameters of Case 3, the length of T2 and T3 should be multiplied by K_multi_SMTC. The test requirement should be defined according to UE capability. For example, for UE support of simultaneously measurements on more than 1 target cells belonging to different NGSO satellites within a SMTC, the legacy test requirement can be reused. For UE don’t support of simultaneously measurements on more than 1 target cells belonging to different NGSO satellites within a SMTC, the cell re-selection delay to a newly detectable cell and an already detected cell should be multiplied by K_multi_SMTC.

Proposal 2: At least to introduce the test case of ‘NGSO, two SMTC configured, SMTC partially overlap with each other, 2 satellites measured on 2 SMTC separately’ with scaling factor K_multi_SMTC

- the length of T2 and T3 and the cell re-selection delay requirements should be multiplied by K_multi_SMTC = 2

Proposal 3: The test case of ‘NGSO, one SMTC configured, 2 satellites measured on 1 SMTC’ could be also introduced with scaling factor K_multi_SMTC

- the length of T2 and T3 should be multiplied by K_multi_SMTC = 2

- The test requirement should be defined according to UE capability. For UE don’t support parallel measurements on more than 1 NGSO satellites within a SMTC, the cell re-selection delay to a newly detectable cell and an already detected cell should be multiplied by K_multi_SMTC. Otherwise, the current test requirement could be reused.

- Test case design for handover

Issue 4-2: Test case for CHO with time-based or location-based condition.

Agreement:

  - Regarding test cases for CHO with time condition, set the time instant fulfilling t1-Threshold-r17 to be at T2, and set the time instant fulfilling duration-r17 to be at (T2+ Tmeasure).

  - Regarding test cases for CHO with location condition, set the time instant fulfilling t1- condEvent D1 to be at T2.

Open issues:

  - Regarding test cases for CHO, T2 is the instant when the last TTI containing the RRC message implying conditional handover command is sent to UE, others shall follow A.6.3.1.2.

  - RAN4 shall check the necessity of adding test cases in which settings don’t fulfill power based events and time/location based events simultaneously, to examine UE’s behavior in this type of scenario.

In the real network, it is very typical that power based events and time/location based events could not be satisfied simultaneously. In order to verify UE correct behavior, it is necessary to add a test case under such scenario. Regarding the test configuration, the time instant fulfilling t1-Threshold-r17 can be set at (T2+2*Tmeasure), the time instant fulfilling duration-r17 can be set at (T2+ 3*Tmeasure). The test requirement should be 2*Tmeasure + Tinterrupt + TCHO_execution from the start of T2, others shall follow A.6.3.1.2

Proposal 4: Add a test case in which test setting don’t fulfill power based events and time/location based events simultaneously.

- Set the time instant fulfilling t1-Threshold-r17 at (T2+2*Tmeasure), and set the time instant fulfilling duration-r17 at (T2+ 3*Tmeasure ).

- Test requirement should be 2*Tmeasure + Tinterrupt + TCHO_execution from the start of T2, others shall follow A.6.3.1.2

- Test case design for UE timing requirements

Issue 5-2: GNSS signal for NTN UE timing related test cases.

Open issues:

  - Option 1: the test parameter for GNSS signal power levels defined in B.4.1 is reused in NTN test cases.

  - Option 2: use AT command approach to acquire UE location

    - AT command approach: Use existing defined AT command: “Update UE Location Information”, defined in TS 38.509 to provide the UE with location coordinates, as used in some existing 3GPP test cases today.

As we known, GNSS positioning error has been captured in Te_NTN requirement. It is quite important to test whether the total error including GNSS error could fulfil the Te_NTN requirement or not, and one key point to implement such test is UE generate GNSS signal by itself. Apparently, Option 2 “Update UE Location Information” method which the SS send positioning information to the UE couldn’t reflect actual GNSS error of tested UE. Therefore, we support Option 1.

Proposal 5: The test parameter for GNSS signal power levels defined in B.4.1 is reused in NTN test cases.

Issue 5-3: Reference time for NTN UE timing related test cases.

Agreement:

  - For the test requirement, the reference time should be (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN

    - FFS NTA,UE-specific and NTA,common used by test system do not include UE GNSS estimation error and satellite positioning error from UE calculation.

  - In test cases for NGSO, a test equipment shall adjust the downlink transmission frame boundary/Doppler shift and UL reception timing according to open-loop TA control related parameters defined in the NTN specific system information and the satellite constellation with respect to UE position.

    - FFS the following aspects:

      - For the transmission timing/Doppler shift adjustment and UL reception timing, RAN4 defines a reference propagator model.

      - The reference propagator model shall be defined in such a way that those UEs using more accurate propagator model than the reference model are not penalized.

      - The reference model can be determined based on companies’ input. Eckstein Hechler based propagator model can be one of the candidate models.

We copy the draft Te_NTN test requirement as follows

Following will be the test sequence for this test 1) Set up PCell according to parameters given in Table A.X1.Y1.1.1.1-1. 2) After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±Te_NTN of the first detected path of DL SSB.

Before discussion, in order to avoid confusion, we define the parameter NTA,adjcommon and NTA,adjUE used by test system as NTA,adjcommon_TE and NTA,adjUE_TE, the parameter NTA,adjcommon and NTA,adjUE derived by UE as NTA,adjcommon_UE and NTA,adjUE_UE.

In the above test requirement, we need to give the description for parameter NTA,adjcommon and NTA,adjUE. In our view, the NTA,adjcommon and NTA,adjUE above are NTA,adjcommon_TE and NTA,adjUE_TE, which are used by test system. From another perspective, if the NTA,adjcommon and NTA,adjUE above is NTA,adjcommon_UE and NTA,adjUE_UE, the GNSS error and satellite positioning error from UE calculation will not be involved in Te_NTN, which will be inconsistent with RAN4 previous conclusions.

The description for NTA,adjcommon_TE and NTA,adjUE_TE should at least contain the clarification that UE GNSS estimation error and satellite positioning error from UE calculation are not involved.

Proposal 6: For the test requirement, the reference time should be NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±Te_NTN

- For the NTA,adjcommon and NTA,adjUE in the test requirement, the description should at least contain the clarification that UE GNSS estimation error and satellite positioning error from UE calculation are not involved.

- Conclusion

In this contribution, we discuss the NTN timing test cases and provide our proposals. The proposals are:

Proposal 1: No need to define UE timing test configuration for FDD 30kHz SCS scenario.

Proposal 2: At least to introduce the test case of ‘NGSO, two SMTC configured, SMTC partially overlap with each other, 2 satellites measured on 2 SMTC separately’ with scaling factor K_multi_SMTC

- the length of T2 and T3 and the cell re-selection delay requirements should be multiplied by K_multi_SMTC = 2

Proposal 3: The test case of ‘NGSO, one SMTC configured, 2 satellites measured on 1 SMTC’ could be also introduced with scaling factor K_multi_SMTC

- the length of T2 and T3 should be multiplied by K_multi_SMTC = 2

- The test requirement should be defined according to UE capability. For UE don’t support parallel measurements on more than 1 NGSO satellites within a SMTC, the cell re-selection delay to a newly detectable cell and an already detected cell should be multiplied by K_multi_SMTC. Otherwise, the current test requirement could be reused.

Proposal 4: Add a test case in which test setting don’t fulfill power based events and time/location based events simultaneously.

- Set the time instant fulfilling t1-Threshold-r17 at (T2+2*Tmeasure), and set the time instant fulfilling duration-r17 at (T2+ 3*Tmeasure ).

- Test requirement should be 2*Tmeasure + Tinterrupt + TCHO_execution from the start of T2, others shall follow A.6.3.1.2

Proposal 5: The test parameter for GNSS signal power levels defined in B.4.1 is reused in NTN test cases.

Proposal 6: For the test requirement, the reference time should be NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±Te_NTN

- For the NTA,adjcommon and NTA,adjUE in the test requirement, the description should at least contain the clarification that UE GNSS estimation error and satellite positioning error from UE calculation are not involved.

- Reference

[1] R4-2214473, WF on performance part for NR NTN, Xiaomi

[2] R4-2210612, WF on timing requirements and performance requirements for NR NTN, Xiaomi