3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215574

Electronic Meeting, 10 – 19 October 2022

Agenda Item:	4.3.4.3

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposal on measurement uncertainty of BS OTA receiver requirements for extending current NR operation to 71 GHz

Document for:	Approval

# 1.	Introduction

The revised work item for extending current NR operation to 71 GHz was approved at TSG RAN#97-e [1]. One of the objectives of this work item is to define BS conformance testing requirements for the band(s) in the frequency range between 52.6GHz and 71GHz. This topic was discussed at TSG RAN4#103-e and the WF was agreed [2-3].

This contribution proposes an approach to decide the measurement uncertainty of BS OTA receiver requirements for extending current NR operation to 71 GHz based on the agreed WF in [2].

# 2.	Discussion

The following points relating to measurement uncertainty and calibration were agreed in [2]:

  - The measurement capabilities resulting from different test equipment implementations should be studied and provided by test equipment vendors to decide the achievable and satisfactory measurement performance for BS type 2-O testing in the frequency range between 52.6 GHz and 71 GHz.

  - Measurement uncertainty and test tolerance analysis needs to be properly captured and therefore update to WID is needed to include TR 37.941.

  - Use of LNA if applicable should be in MU budget for FR2-2 except TX OFF power measurement. It is FFS whether mixer can be considered for some of test cases if applicable.

  - MU values for network analyser

The following information have been provided:

    - UE conformance testing, the MU of 1 sigma of 0.85 dB is assumed for the frequency range 40.8 GHz to 80 GHz.

    - AAS BS conformance testing (TR 37.941), the MU of 1 sigma of 0.30 dB is assumed for the frequency range up to 60 GHz.

Further analysis is required regarding what value to assume for FR2-2 MU evaluation.

  - As general principle, if a component is needed, it should be taken into account in MU budget.

  - The following aspect are FFS for FR2-2:

    - Add intermediate calibration stage of spectrum analyser absolute power accuracy for DL EIRP measurement, with the intension to break the trend with very large measurement uncertainties for high frequencies.

    - For test system measurement uncertainty, extend measurement procedures to enable for test setups to break the trend where measurement uncertainty grows rapidly as function of frequency.

    - If required introduce external mixer stage test setup and corresponding calibration procedures for receiver requirements and out-of-band requirements to improve measurement uncertainty.

    - Use of power sensor/meter for signal leveling should not be mandated for test requirements and related test setups.

It can be seen from there are numerous aspects to be studied for measurement uncertainty and calibration in the agreed WF in [2]. As the next TSG RAN4#105 meeting is scheduled to be the last one before the completion of the conformance testing part of the work item, to arrive on a timely agreement for the measurement uncertainty in the frequency range between 52.6GHz and 71GHz (currently denoted as FR2-2), it is proposed to adopt the approach that was used to obtain the measurement uncertainty for receiver requirements in FR2-1 (formerly denoted as FR2), which is recorded in the WF agreed at TSG RAN4#88 [4].

The measurement uncertainty for receiver sensitivity in FR2-1 in the agreed WF in [4] was decided from proposals from different companies, where the major difference came from the uncertainty value for the RF signal generator. For BS conformance testing requirements in FR2-2, the uncertainty value of 0.85 dB for the network analyzer was included as an option for FFS in the agreed WF in [2]. Hence this component would become another major factor for the difference between the measurement uncertainty for OTA receiver requirements in FR2-1 and FR2-2. Therefore, the first step to decide the measurement uncertainty for OTA receiver requirements in FR2-2 is to identify the uncertainty values for the existing components in FR2-1 (current candidates include RF signal generator and network analyzer) that need to be updated notably in FR2-2, assuming the uncertainty values for the other components in FR2-1 in clause 10.2 of TR 37.941 [5] can be reused in FR2-2.

Moreover, additional components like mixer and power sensor/meter were included as options for FFS in the agreed WF in [2]. The measurement uncertainty values for these additional components would need to be decided, along with the additional calibration stages which should aim to reduce the uncertainty values for these additional components. Therefore, the second step to decide the measurement uncertainty for OTA receiver requirements in FR2-2 is to decide the additional components (current candidates include mixer and power sensor/meter) and the corresponding uncertainty values.

After the components and the corresponding uncertainty values are decided in the two steps above, the third step is to calculate the measurement uncertainty for receiver sensitivity in FR2-2 from the measurement uncertainty for receiver sensitivity in FR2-1 by updating the uncertainty values of the components decided in step 1 as well as adding the uncertainty values of the components decided in step 2 in the corresponding tables in clause 10.2 of TR 37.941, Spreadsheet 4 in TR 37.941 can be used as a tool for the calculation.

Finally, the fourth step is to calculate the measurement uncertainty for other receiver requirements in FR2-2 from the measurement uncertainty for receiver sensitivity in FR2-2 using the corresponding formula in clauses 10 and 12 of TR 37.941.

# 3.	Conclusion

This contribution has proposed an approach to decide the measurement uncertainty of BS OTA receiver requirements for extending current NR operation to 71 GHz based on the agreed WF, the steps are listed as follows.

Step 1) Decide the measurement uncertainty for OTA receiver requirements in FR2-2 is to identify the uncertainty values for the existing components in FR2-1 (current candidates include RF signal generator and network analyzer) that need to be updated notably in FR2-2, assuming the uncertainty values for the other components in FR2-1 in clause 10.2 of TR 37.941 can be reused in FR2-2.

Step 2) Decide the measurement uncertainty for OTA receiver requirements in FR2-2 is to decide the additional components (current candidates include mixer and power sensor/meter) and the corresponding uncertainty values.

Step 3) Calculate the measurement uncertainty for receiver sensitivity in FR2-2 from the measurement uncertainty for receiver sensitivity in FR2-1 by updating the uncertainty values of the components decided in step 1 as well as adding the uncertainty values of the components decided in step 2 in the corresponding tables in clause 10.2 of TR 37.941.

Step 4) Calculate the measurement uncertainty for other receiver requirements in FR2-2 from the measurement uncertainty for receiver sensitivity in FR2-2 using the corresponding formula in clauses 10 and 12 of TR 37.941.

# References

[1]	RP-222656, “Revised WID: Extending current NR operation to 71 GHz”, Qualcomm.

[2]	R4-2214373, “WF on FR2-2 BS test environments”, Ericsson.

[3]	R4-2214374, “WF on FR2-2 BS conformance testing”, Nokia, Nokia Shanghai Bell.

[4]	R4-1811846, “WF on MU and TT for Rx requirements for FR2”, Nokia, Nokia Shanghai Bell.

[5]	3GPP TR 37.941 v17.0.0: “Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements”.