3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215575

Electronic Meeting, 10 – 19 October 2022

Agenda Item:	6.4.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on definition of FR2 multi-band BS

Document for:	Approval

# 1.	Introduction

The revised SID on NR BS RF requirement evolution was approved at TSG RAN#97-e [1]. The objective of this study item is to study certain aspects for FR2 multi-band BS, including the following aspects relating to FR1 multi-band BS:

    - Investigate if FR1 multi-band methods are re-usable for FR2, and (if so) agree on the appropriate inter-RF BW gaps

    - Investigate if FR1 exceptions are acceptable for FR2

This topic was discussed at TSG RAN4#104-e and the WF was agreed [2]. This contribution provides investigation on current FR1 definition of multi-band RIB and proposals on definition of FR2 multi-band BS according to the agreed WF.

# 2.	Discussion

It was agreed in [2]:

## Definition of FR2 multi-band BS

- To investigate if the FR1 definition of multi-band RIB can be re-used for FR2

- To revisit the definition of multi-band BS for FR2, and clarify whether the following scenarios should be considered as multi-band BS

1) Multi-band transmitter and/or receiver with common active RF components

2) Single-band transmitter and receiver

3) Configurable BS for different bands with the same hardware

4) BS covers full-band or sub-band of band A and band B

5) BS covers consecutive spectrums with different band number, for example, n258+n261

6) BS covers overlapping spectrums with different band number, for example, n258+n257

- FFS whether need to introduce the definition of multi-band BS for FR2 into specification

The FR1 definition of multi-band RIB in clause 3.1 of TS 38.104 [3] is:

multi-band RIB: operating band specific RIB associated with a transmitter or receiver that is characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

It is clear from the current definition that scenario 1) above should be considered as multi-band BS, while scenario 2) above should not.

Moreover, whether scenario 3) above should be considered as multi-band BS depends on whether carriers in different bands can be processed in common active RF components simultaneously. In case only one band can be configured to operate at any time (i.e., more than one bands cannot be configured to operate simultaneously, then this configurable BS should not be considered as multi-band BS from the current definition. Note that the requirements on single-band operation of multi-band BS are clarified by CR agreed at RAN4#68 [4]:

-	 If the BS is configured for single-band operation, single-band requirements shall apply to the antenna connector configured for single-band operation and no exclusions or provisions for multi-band capable BS are applicable. Single-band requirements are tested separately at the antenna connector configured for single-band operation, with all other antenna connectors terminated.

On the other hand, the following manufacturer’s declaration is specified in clause 4.8 of TS 38.141-2 [5]:

| D.18 | Maximum Radio Bandwidth of the operating band with multi-band dependencies | Largest Radio Bandwidth that can be supported by the operating bands with multi-band dependencies. Declared for each supported operating band which has multi-band dependencies (D.16). | c | x | n/a |
| --- | --- | --- | --- | --- | --- |

Therefore, the current definition allows multi-band BS to cover full-band or sub-band of the operating bands with multi-band dependencies, and thus scenario 4) above should be considered as multi-band BS from the current definition.

The original definition of multi-band BS in clause 3.1 of TR 37.812 [6] is:

MB-MSR Base Station: MSR Base Station characterized by the ability of its transmitter and/or receiver to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s).

Note that the condition ‘and where this different operating band is not a sub-band or superseding-band of another supported operating band‘ was not included in the original definition. The condition ‘non-overlapping’ was added by CR agreed at RAN4#66 [7]:

MB-MSR Base Station: MSR Base Station characterized by the ability of its transmitter and/or receiver to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different non-overlapping operating band than the other carrier(s).

The condition was then updated to allow a BS supports no overlapping frequency range in the two bands in common active RF components to be counted as Multi-band base station by CR agreed at RAN4#79 [8]:

MB-MSR Base Station: MSR Base Station characterized by the ability of its transmitter and/or receiver to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not a sub-band or superseding-band of another supported operating band) than the other carrier(s).

Therefore, scenarios 5) and 6) above have already been considered in the current definition. It is clear that scenario 5) above should be considered as multi-band BS from the current definition, while whether scenario 6) above should be considered as multi-band BS depends on whether BS supports no overlapping frequency range in the two bands in common active RF components.

To summarize, all scenarios 1) to 6) above have been considered in the current FR1 definition of multi-band RIB. Therefore, it is proposed to reuse the current FR1 definition of multi-band RIB for FR2.

Moreover, it is specified in clause 4.8 of TS 38.104 [3]:

BS type 1-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

-	All RIBs are multi-band RIBs.

-	A combination of single-band RIBs and multi-band RIBs provides support of the BS type 1-O capability of operation in multiple operating bands.

It can be seen that FR1 BS capable of supporting operation in multiple operating bands with different implementation at the single-band and/or multi-band RIB have already be covered by the current definition. Therefore, there is no need to introduce the definition of multi-band BS for FR2 into specification.

# 3.	Conclusion

This contribution has provided investigation on current FR1 definition of multi-band RIB and proposals on definition of FR2 multi-band BS according to the agreed WF. The proposals are summarized as follows:

Proposal 1: To reuse the current FR1 definition of multi-band RIB for FR2.

Proposal 2: No need to introduce the definition of multi-band BS for FR2 into specification.

# References

[1]	RP-222501, “SID revision: NR BS RF requirement evolution”, Huawei, HiSilicon.

[2]	R4-2214375, “WF on investigation of mmWave multi-band BS”, Huawei.

[3]	3GPP TS 38.104 v17.6.0: “NR; Base Station (BS) radio transmission and reception”.

[4]	R4-134422, “Clarification on single-band operation for MB-MSR”, Huawei.

[5]	3GPP TS 38.141-2 v17.6.0: “NR; Base Station (BS) conformance testing; Part 2: Radiated conformance testing”.

[6]	3GPP TR 37.812 v11.3.0: “Radio Frequency (RF) requirements for Multi-band and Multi-standard radio (MB-MSR) Base Station (BS)”.

[7]	R4-130333, “Some clarifications on MB-MSR requirement in TS 37.104”, Huawei.

[8]	R4-164666, “Corrections on definition of multi-band definition and blocking”, Huawei, Nokia, Ericsson.