3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215577

Electronic Meeting, 10 – 19 October 2022

Agenda Item:	6.7.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Link level simulation results for FR2-1 UL 256QAM

Document for:	Information

# 1.	Introduction

The revised work item on NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 was approved at TSG RAN#96 [1]. One of the objectives of this work item is to investigate and enable UL 256QAM for FR2-1.

One of the main tasks to investigate and enable UL 256QAM for FR2-1 is to study the gain, operating SNR, phase noise model and implementation aspects. This topic was discussed at TSG RAN4#104-e and the link level simulation assumptions were included in the agreed WF [2]. This contribution provides link level simulation results for FR2-1 UL 256QAM according to the agreed simulation assumptions.

# 2.	Simulator assumptions

The simulation results in this contribution are obtained with a subset of the agreed simulation cases in [2], because of the time constraint to obtain the simulation results with the whole set of cases. The selected subset of cases is yellow-highlighted in table 1, which is a copy of the simulation assumptions agreed in [2].

Table 1. Simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257), 39 GHz (n260) and 48GHz (n262) |
| CBW | 50 MHz, 100MHz |
| SCS | 120 kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM: CP-OFDM: MCS 23, 24 in TS 38.214 Table 5.1.3.1-1, other MCSs are not precluded. DFT-s-OFDM: MCS 22, 23 in TS 38.214 Table 6.1.4.1-1, other MCSs are not precluded. 256QAM: CP-OFDM/DFT-s-OFDM: MCS 21, 23 in TS 38.214 Table 5.1.3.1-2, other MCSs are not precluded. Baseline: fixed MCSs |
| Symbol type | CP-OFDM; DFT-s-OFDM |
| HARQ | 8, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | CP-OFDM: KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) DFT-s-OFDM: (,)=(4, 4), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): example1 (UE) + example1(BS) Option b): example2 (UE) + example2(BS) Option d): example1 (UE) + example2(BS) |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: 3%, 3.5%, 4%, rxEVM: 3%, 3.5%, 4% Option 1: txEVM >= rxEVM; |
| Other parameters | follow assumptions in TS38.104 Section 11.2.2 . |

# 3.	Simulation results and observations

The link level simulation results at 29 GHz (n257) and 39 GHz (n260) using the highlighted parameters in table 1 are shown in figures 1 and 2, respectively.

Figure 1. Simulation results at 29 GHz (n257)

Figure 2. Simulation results at 39 GHz (n260)

It can be observed from figure 1 that at 29 GHz (n257):

-	If MCS index is 21, 256QAM goes over 64QAM in around 24-25 dB for all simulated EVMs.

-	If MCS index is 23, 256QAM goes over 64QAM in around 28 dB for (transmit and receive) EVM of 3%, around 32 dB for (transmit and receive) EVM of 3.5%, and loses for (transmit and receive) EVM of 4%.

On the other hand, it can be observed from figure 2 that at 39 GHz (n260):

-	Only with (transmit and receive) EVM of 0%, MCS21 for 256QAM seems to win 64QAM in around 36 dB.

Note that the PN is challenging case at 39 GHz (n260), and due to small allocation, the number of PTRS symbols is quite low so even higher complexity ICI compensation would not really help here.

# 4.	Conclusion

This contribution has provided link level simulation results for FR2-1 UL 256QAM according to the agreed simulation assumptions, with the observations summarized as follows:

Observation 1: For the simulation results at 29 GHz (n257):

-	If MCS index is 21, 256QAM goes over 64QAM in around 24-25 dB for all simulated EVMs.

-	If MCS index is 23, 256QAM goes over 64QAM in around 28 dB for (transmit and receive) EVM of 3%, around 32 dB for (transmit and receive) EVM of 3.5%, and loses for (transmit and receive) EVM of 4%.

Observation 2: For the simulation results at 39 GHz (n260):

-	Only with (transmit and receive) EVM of 0%, MCS21 for 256QAM seems to win 64QAM in around 36 dB.

# References

[1]	RP-221862, “Revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi.

[2]	R4-2214453, “WF on UL 256QAM”, Xiaomi.