3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215588

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	4.3.7.2.1

Source:	Apple

Title:	On PDCCH and PBCH demod requirements for 52.6 - 71 GHz

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#104-e UE demodulation requirements for 52.6 - 71 GHz was discussed and way forward [1] was agreed. In this contribution we present simulation results for alignment for PDCCH and PBCH demodulation requirements.

# 2. PDCCH Demod

In [1] we made several agreements on PDCCH demodulation requirements. The simulation assumptions for PDCCH demod requirements were agreed. Based on the agreed simulation parameters, we present results for PDCCH performance in FR2-2.

Table 1: Initial Simulation results for PDCCH demod in FR2-2

| Test number | CBW/SCS | CORESET duration | CORESET RB | Intl. size | REG bundle size | Agg level | Prop. Condition | DCI format | Payload size | Ant. Config and corr Mtx | 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 1 | 60 | 3 | 2 | 2 | TDLA30-200 | 1_0 | 40 | 1x2 Low | 5.6 |
| 2 | 100/120 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low | 1.2 |
| 3 | 100/120 | 1 | 60 | 3 | 2 | 8 | TDLA10-200 | 1_1 | 56 | 2x2 Low | -1.1 |
| 4 | 100/120 | 2 | 60 | 3 | 2 | 16 | TDLA30-200 | 1_0 | 40 | 2x2 Low | -4.4 |
| 5 | 400/480 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low | 1.2 |
| 6 | 400/480 | 1 | 60 | 2 | 6 | 8 | TDLA10-200 | 1_1 | 56 | 1x2 Low | -1.6 |
| 7 | 400/480 | 2 | 60 | 3 | 2 | 16 | TDLA10-200 | 1_0 | 40 | 2x2 Low | -4.2 |

# 2. PBCH Demod

In [1] we had the following agreements for PBCH performance requirements:

Issue 2-2-1: SSB index assumption - Agreement: Only with not known SSB index Issue 2-2-2: Test setup for PBCH requirements - Define requirements for the tests below. Propagation condition to be further discussed; Test number; SCS/CBW; Antenna configuration; Propagation condition; PBCH SNR (dB) @ 1% Pm-bch; SS/PBCH block index. 1-1; 120kHz/100MHz; 1x2 Low; [TDLA10-200]; New; unknown. 1-2; 480kHz/400MHz; [TDLA10-200]; New; unknown.

In [1] there was an agreement on the channel models to be used based on channel bandwidth.

Based on this we defined the PDSCH and PDCCH demod requirements with the applicable channel model for the SCS and CBW combination.  We propose to use the same for PBCH requirements as well and define PBCH demod requirements with TDLA30-200 for 120 KHz/100MHz and TDLA10-200 for 480KHz/400MHz.

Proposal #1: Align the channel model for SCS/CBW with PDSCH/PDCCH requirements for PBCH demod requirements - TDLA30-200 for 120KHz/100MHz and TDLA10-200 for 480KHz/400MHz.

Based on the agreed simulation parameters and proposed channel model above, we provide simulation results for PBCH for FR2-2.

| Test number | SCS/CBW | Antenna configuration | Propagation condition | SS/PBCH block index | PBCH SNR (dB) @ 1% Pm-bch |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120kHz/100MHz | 1x2 Low | TDLA30-200 | unknown | -7.5 |
| 1-2 | 480kHz/400MHz |  | TDLA10-200 | unknown | -7.5 |

# 3. Conclusion

In this paper we present simulation results for PDCCH and PBCH demodulation requirements for 52.6-71GHz. Our observations and proposals are captured below:

Proposal #1: Align the channel model for SCS/CBW with PDSCH/PDCCH requirements for PBCH demod requirements - TDLA30-200 for 120KHz/100MHz and TDLA10-200 for 480KHz/400MHz.

# Reference

- R4-2214390, “WF on FR2-2 UE demodulation requirements”, Qualcomm Incorporated.