3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215595

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	4.5.3.1.1

Source:	Apple

Title:	On demod requirements for HST Scheme-A

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#104-e demodulation and CSI requirements FeMIMO was discussed and way forward [1] was agreed. In this contribution we present our views on the open issues, and simulation results for requirements with HST Scheme-A

# 2. Discussion

Based on the agreed simulation assumptions captured in Table 1, we present simulation results for PDSCH demod for HST Scheme-A.

Table 1: Simulation Assumptions for HST Scheme-A

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | FDD | TDD |  |
| SCS/CBW | 15 kHz /10 MHz | 30 kHz /40 MHz |  |
| Antenna configuration | 2x2; 2x4 |  |  |
| Max Doppler Shift | 870 Hz |  | 1667 Hz |
| Rank | 2 |  |  |
| MCS | 13, 17 |  |  |
| DMRS type | Type 1 |  |  |
| Number of DMRS symbols | 1+1+1 |  |  |
| TDD pattern |  | 7D1S2U, S: 6D 4G 4U |  |
| TRS configuration | 10ms, 2 slot pattern |  |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |  |
| Ds and Dmin | Ds =700m; Dmin=150m |  |  |

Table 2: Simulation results for HST Scheme-A

|  | Ant Config | MCS13 |  | MCS17 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR @ 70% Max TP | Is Max TP reached | SNR @ 70% Max TP | Is Max TP reached |
| FDD | 2x2 | 10.1 | Yes | 14.6 | No (85%) |
|  | 2x4 | 8.3 | Yes | 13.0 | No (83%) |
| TDD | 2x2 | 11.1 | Yes | 15.1 | No (80%) |
|  | 2x4 | 9.4 | Yes | 13.6 | No (78%) |

Based on the simulation results we observe that with MCS 17 max TP is not achieved with HST Scheme-A for FDD or TDD. We don’t see that its feasible to define requirements with MCS17 given the performance limitation. Hence, we propose to define requirements with MCS13 for HST Scheme-A.

Observation #1: With MCS17 in HST Scheme-A max throughput is not achieved for FDD and TDD.

Proposal #1: Define requirements with MCS13 for HST Scheme-A.

# 3. Conclusion

In this paper, we provide simulation results for requirements for PDSCH with HST Scheme-A. Our observations and proposals are captured below:

Observation #1: With MCS17 in HST Scheme-A max throughput is not achieved for FDD and TDD.

Proposal #1: Define requirements with MCS13 for HST Scheme-A.

# Reference

- R4-2214397, “WF on UE demodulation and CSI requirements for FeMIMO”, Samsung.