3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215625

# Electronic Meeting, October 10 – October 22, 2022

Agenda item:	5.6.5.1.1

Source:	Apple Inc.

Title:	On PDSCH/SDR Demodulation Requirements for Reduced Capability Devices in NR

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#104-e RedCap UE demodulation and CSI reporting requirements were discussed and a way forward [1] was agreed. In this contribution we present our views on open issues.

# 2. Discussion

During RAN4#104-e it was discussed

Agreement: Introduce UE demodulation and CSI reporting for 2Rx RedCap UE supporting HD-FDD in FDD bands as well as 1Rx UE

In the WF [1] , it was requested for interested companies to evaluate if the same requirements can be applied for both 2Rx HD-FDD and 2Rx FD-FDD.

Proposal #1: The requirements with HD-FDD are the same as the corresponding (full-duplex) FDD requirements - can be applied for both 2Rx HD-FDD and 2Rx FD-FDD

Agreement: Define 256QAM demodulation requirements for 1Rx RedCap UE in FR1 or not

- Option 1: Define 256QAM demodulation requirements for 1Rx in FR1

  - Option 1a: MCS20

  - Option 1b: MCS is based on the operating SNR

- Option 2: Not to define 256QAM demodulation requirements for 1Rx in FR1

Most companies supported the idea of defining this 256QAM demodulation requirement, and also supported to use MCS20 as the evaluation point. This requirement was conceded to network operators for them to be able to evaluate the actual achievable cell spectral efficiency with 256QAM RedCap UEs. The straightforward selection of MCS20 since it corresponds to the lowest coderate in the 256QAM MCS table.

Simulation Results

For the agreed simulation parameters stated in [1], we contribute with our performance results for PDSCH demodulation. For clarity of exposition, we present the full table requested in [1] but omitting the results of those test cases that are reused from Rel.15.

FR1 FDD 15kHz

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | SNR[dB] | Impair Res. [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | 256QAM 0.67 Rank 1 | TDLA30-10 | 2x1 ULA low | 70% of peak rate | 23.8 | 25.8 |

FR1 TDD 30kHz

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD pattern | Propagation condition | Antenna configuration | Metric | SNR[dB] | Impair Res. [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz/ 30kHz | 256QAM 0.67 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 ULA low | 70% of peak rate | 24.2 | 26.2 |

# 3. Conclusion

In this contribution we provide our position on the remaining open issue and we provide the requested simulation results for PDSCH demodulation requirements for RedCap UE. Our observations and proposals are captured below:

Proposal #1: The requirements with HD-FDD are the same as the corresponding (full-duplex) FDD requirements - can be applied for both 2Rx HD-FDD and 2Rx FD-FDD

# Reference

- R4-2214394, “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.

- R4-2214317, “Email discussion summary for [104-e][328] NR_RedCap_Demod”, Ericsson