3GPP TSG-RAN WG4 Meeting #104bis-e 	R4-2215678

Electronic meeting, 10th – 19th October, 2022

Source:	Ericsson

Title:	Simulation results for NR SAN PUCCH demodulation requirements

Agenda item:	4.2.7.2.2

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the SAN PUCCH demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for further discussion.

# Simulation result

Table 2-1 Simulation parameters for NTN PUCCH

| Parameter | Format 0 | Format 1 | Format 2 |  | Format 3 | Format 4 |
| --- | --- | --- | --- | --- | --- | --- |
| number of UCI information bits | 1 | 2 | 4 | 22 | 16 | 22 |
| Number of PRBs | 1 | 1 | 4 | 9 | 1 | 1 |
| Number of symbols | 1 | 14 | 1 | 2 | 14 | 14 |
| First PRB prior to frequency hopping | 0 | 0 | 0 | 0 | 0 | 0 |
| Intra-slot frequency hopping | N/A | Enabled | N/A | Enabled | Enabled | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |  |  |  |  |
| Group and sequence hopping | neither | neither | Neither | Neither | Neither | Neither |
| Hopping ID | 0 | 0 | 0 | 0 | 0 | 0 |
| Initial cyclic shift | 0 | 0 | 0 | 0 | 0 |  |
| First symbol | 13 | 0 | 13 | 12 | 0 | 0 |
| Index of orthogonal cover code (timeDomainOCC) | N/A | 0 | N/A | N/A | N/A | n0 |
| Length of the orthogonal cover code | N/A | N/A | N/A | N/A | N/A | N2 |
| Test Metric | D2A error <1% and ACK miss <1% | NACK -> ACK <0.1% ACK miss <1% | ACK miss <1% | UCI BLER<1% | UCI BLER<1% | UCI BLER<1% |

Table 2-2.1 Simulation results for NTN PUCCH PF0 with 15kHz SCS, 5MHz CBW,

| Number | Number of | Propagation conditions and |  |
| --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 15.3 |
|  |  | NTN-TDLC5-200 | 4.6 |
| 1 | 2 | NTN-TDLA100-200 | 6.9 |
|  |  | NTN-TDLC5-200 | 1.5 |

Table 2-2.2 Simulation results for NTN PUCCH PF0 with 30kHz SCS, 10MHz CBW

| Number | Number of | Propagation conditions and |  |
| --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 16 |
|  |  | NTN-TDLC5-200 | 4.7 |
| 1 | 2 | NTN-TDLA100-200 | 6.9 |
|  |  | NTN-TDLC5-200 | 1.4 |

Table 2-3.1 Simulation results for NTN PUCCH PF1 with 15kHz SCS, 5MHz CBW, NACK->ACK <0.1%

| Number | Number of | Propagation conditions and |  |
| --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | -2.3 |
|  |  | NTN-TDLC5-200 | -4.8 |
| 1 | 2 | NTN-TDLA100-200 | -7.4 |
|  |  | NTN-TDLC5-200 | -9 |

Table 2-3.2 Simulation results for NTN PUCCH PF1 with 30kHz SCS, 10MHz CBW, NACK->ACK <0.1%

| Number | Number of | Propagation conditions and |  |
| --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 0.6 |
|  |  | NTN-TDLC5-200 | -4.9 |
| 1 | 2 | NTN-TDLA100-200 | -6.2 |
|  |  | NTN-TDLC5-200 | -9 |

Table 2-4.1 Simulation results for NTN PUCCH PF1 with 15kHz SCS, 5MHz CBW, ACK miss <1%

| Number | Number of | Propagation conditions and |  |
| --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 0.2 |
|  |  | NTN-TDLC5-200 | -3.8 |
| 1 | 2 | NTN-TDLA100-200 | -6.2 |
|  |  | NTN-TDLC5-200 | -8.3 |

Table 2-4.2 Simulation results for NTN PUCCH PF1 with 30kHz SCS, 10MHz CBW, ACK miss <1%

| Number | Number of | Propagation conditions and |  |
| --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 2.7 |
|  |  | NTN-TDLC5-200 | -3.8 |
| 1 | 2 | NTN-TDLA100-200 | -4.1 |
|  |  | NTN-TDLC5-200 | -8.2 |

Table 2-5.1 Simulation results for NTN PUCCH PF2 with 15kHz SCS, 5MHz CBW, ACK miss <1%

| Number | Number of | Propagation conditions and |  |
| --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 12.7 |
|  |  | NTN-TDLC5-200 | 3 |
| 1 | 2 | NTN-TDLA100-200 | 3 |
|  |  | NTN-TDLC5-200 | -2.5 |

Table 2-5.2 Simulation results for NTN PUCCH PF2 with 30kHz SCS, 10MHz CBW,  ACK miss <1%

| Number | Number of | Propagation conditions and |  |
| --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 11 |
|  |  | NTN-TDLC5-200 | 2.8 |
| 1 | 2 | NTN-TDLA100-200 | 3.1 |
|  |  | NTN-TDLC5-200 | -2.4 |

Table 2-6.1 Simulation results for NTN PUCCH PF2 with 15kHz SCS, 5MHz CBW,  UCI BLER <1%

| Number | Number of | Propagation conditions and |  |
| --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 9.2 |
|  |  | NTN-TDLC5-200 | 5.2 |
| 1 | 2 | NTN-TDLA100-200 | 2.7 |
|  |  | NTN-TDLC5-200 | 0.1 |

Table 2-6.2 Simulation results for NTN PUCCH PF2 with 30kHz SCS, 10MHz CBW,  UCI BLER <1%

| Number | Number of | Propagation conditions and |  |
| --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 11.7 |
|  |  | NTN-TDLC5-200 | 4.8 |
| 1 | 2 | NTN-TDLA100-200 | 4.5 |
|  |  | NTN-TDLC5-200 | -0.2 |

Table 2-7.1 Simulation results for NTN PUCCH PF3 with 15kHz SCS, 5MHz CBW, UCI BLER <1%

| Number | Number of | Propagation conditions and | Additional DM-RS pos |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | Pos0 | 4.2 |
|  |  |  | Pos1 | 4 |
|  |  | NTN-TDLC5-200 | Pos0 | -0.2 |
|  |  |  | Pos1 | -0.6 |
| 1 | 2 | NTN-TDLA100-200 | Pos0 | -1.7 |
|  |  |  | Pos1 | -2.5 |
|  |  | NTN-TDLC5-200 | Pos0 | -4.4 |
|  |  |  | Pos1 | -5.2 |

Table 2-7.2 Simulation results for NTN PUCCH PF3 with 30kHz SCS, 10MHz CBW, UCI BLER <1%

| Number | Number of | Propagation conditions and | Additional DM-RS pos |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | Pos0 | 7.2 |
|  |  |  | Pos1 | 6.9 |
|  |  | NTN-TDLC5-200 | Pos0 | 0 |
|  |  |  | Pos1 | -0.3 |
| 1 | 2 | NTN-TDLA100-200 | Pos0 | -0.4 |
|  |  |  | Pos1 | -0.4 |
|  |  | NTN-TDLC5-200 | Pos0 | -4.2 |
|  |  |  | Pos1 | -5.1 |

Table 2-8.1 Simulation results for NTN PUCCH PF4 with 15kHz SCS, 5MHz CBW, UCI BLER <1%

| Number | Number of | Propagation conditions and | Additional DM-RS pos |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | Pos0 | 6.8 |
|  |  |  | Pos1 | 6.8 |
|  |  | NTN-TDLC5-200 | Pos0 | 1.7 |
|  |  |  | Pos1 | 1.6 |
| 1 | 2 | NTN-TDLA100-200 | Pos0 | 0.2 |
|  |  |  | Pos1 | -0.4 |
|  |  | NTN-TDLC5-200 | Pos0 | -2.7 |
|  |  |  | Pos1 | -3.3 |

Table 2-8.2 Simulation results for NTN PUCCH PF4 with 30kHz SCS, 10MHz CBW, UCI BLER <1%

| Number | Number of | Propagation conditions and | Additional DM-RS pos |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | Pos0 | 9.5 |
|  |  |  | Pos1 | 9.5 |
|  |  | NTN-TDLC5-200 | Pos0 | 1.8 |
|  |  |  | Pos1 | 1.9 |
| 1 | 2 | NTN-TDLA100-200 | Pos0 | 2.2 |
|  |  |  | Pos1 | 1.8 |
|  |  | NTN-TDLC5-200 | Pos0 | -2.6 |
|  |  |  | Pos1 | -3.2 |

# Conclusions

Simulation results for SAN PUCCH demodulation requirements are delivered.

# References

[1]	R4-2214387, WF for NTN NR SAN demodulation, Huawei HI Silicon, RAN4#104-e

# Annex A Channel model for NTN simulation

Table A-1 NTN-TDLA100 delay profile (resolution 5ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 110 | -4.675 | Rayleigh |
| 3 | 285 | -6.482 | Rayleigh |

Table A-2 NTN-TDLC5 delay profile (resolution 5ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.394 | LOS path |
|  | 0 | -8.668 | Rayleigh |
| 2 | 60 | -21.423 | Rayleigh |
| NOTE: The first tap follows a Rician distribution with a K-factor of K1 = 8.05 dB and a mean power of 0dB. |  |  |  |