3GPP TSG-RAN WG4 Meeting #104bis-e 	R4-2215693

Electronic meeting, 10th – 19th October, 2022

Source:	Ericsson

Title:	Simulation results for FR2-2 PRACH demodulation requirements

Agenda item:	4.3.7.3.3

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the NR extended to 71GHz demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for further discussion.

# Simulation result

Table 2-1 Time error tolerance for AWGN, TDLA10-650

| PRACH | PRACH SCS | Time error tolerance |  |
| --- | --- | --- | --- |
| preamble | (kHz) | AWGN | TDLA10-650 |
| A2, B4, C2 | 120 | 0.07 us | 0.12us |
|  | 480 | 0.018 us | 0.068us |

Table 2-2 Simulation results for PRACH with Lra = 139, 120kHz SCS

| Number | Number of | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | conditions and correlation matrix (annex J) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 0 | -15.3 | -19.8 | -15.3 |
|  |  | TDLA10-650 Low | 7100 Hz | -6.5 | -9.5 | -6.7 |

Table 2-3 Simulation results for PRACH with Lra = 571, 120kHz SCS

| Number | Number of | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | conditions and correlation matrix (annex J) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 0 | -21.2 | -25.7 | -21.3 |
|  |  | TDLA10-650 Low | 7100 Hz | -13.0 | -16 | -13.1 |

Table 2-4 Simulation results for PRACH with Lra = 1151, 120kHz SCS

| Number | Number of | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | conditions and correlation matrix (annex J) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 0 | -24.1 | -28.6 | -23.9 |
|  |  | TDLA10-650 Low | 7100 Hz | -15.9 | -18.7 | -16.2 |

Table 2-5 Simulation results for PRACH with Lra = 139, 480kHz SCS

| Number | Number of | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | conditions and correlation matrix (annex J) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 0 | -15.3 | -19.7 | -15.3 |
|  |  | TDLA10-650 Low | 7100 Hz | -7.9 | -12 | -8 |

Table 2-6 Simulation results for PRACH with Lra = 571, 480kHz SCS

| Number | Number of | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | conditions and correlation matrix (annex J) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 0 | -21.1 | -25.7 | -21.1 |
|  |  | TDLA10-650 Low | 7100 Hz | -14.1 | -18.3 | -14.1 |

# Conclusions

Simulation results for FR2-2 PRACH demodulation requirement are delivered.

# References

[1]	R4-2214388, WF for NR extended to 71GHz BS demodulation, Nokia, Nokia Shanghai Bell, RAN4#104-e