3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215734

# Electronic Meeting, 10th-19th, Nov, 2022

Source: 	Samsung

Title: 	Views on improved MSD feasibility

Agenda Item:	6.6.4.1

Document for:	Discussion

- Introduction

A new WI was approved in RAN#95 meeting while further revised in RAN#96 as [1], including the objective of investigating the feasibility of Lower MSD for inter-band CA/EN-DC/DC combinations, targeting on study how the MSD could be improved and how to define the capability. In RAN4#104-e meeting discussion, example band combinations were selected, in order to carry out the analysis to study how the MSD could be improved, as well as to derive the potential low MSD threshold(s).

Particularly, in this paper, we provide analysis in form of link budget calculation on some example band combinations based on the parameter assumptions (mainly PCB isolation and antenna isolation) recorded in WF [2].

- 2. Discussion

In RAN4#104-e meeting, consensus were reached that the minimum requirements in the specification shall be kept unchanged for the lower MSD study, meanwhile example band combinations were selected to study the feasibility of MSD improvement for different MSD types, among which the 2-bands combinations are in the first priority and their corresponding interference type are listed as follow for reference.

Note, the following analysis is for the purpose to study how the MSD could be improved, as well as to derive the potential Lower MSD capability threshold(s), rather than tightening the requirement or defining another set of requirements for some specific UEs to mandatory conform to. We are against to open the discussion of tightening MSD requirements in this WI.

| Example 2-bands combinations | Interference Source and MSD value (PC3) |
| --- | --- |
| CA_n28-n40 | 3rd harmonic mixing（37.8dB） |
| CA_n41-n77 | Cross band isolation（8.3dB） |
| CA_n1-n3 | Cross band isolation ([19.7] according to the endorsed CR [9] in RAN4#104e） IMD3（23dB） |
| CA_n3-n78 | IMD2（26dB） IMD4（8dB） 2nd harmonic（23.9dB） |
| CA_n1-n78 | IMD4（8dB） |

2.1 CA_n3-n78 MSD Analysis on IMD2

The MSD trend analysis was carried out based on the architecture illustrated in [3]. The component linearity parameters and initial isolation parameters adopted in our calculation was summarized in Table 2-1 and Table 2-2 respectively.

| Component | IP2(dBm) | IP3(dBm) | IP4(dBm) | IP5(dBm) |
| --- | --- | --- | --- | --- |
| Ant. Switch | 112 | 68 | 56 | 53 |
| Triplexer | 115 | 86 | 55 | 53 |
| Duplexer | 100 | 74 | 55 | 53 |
| PA Forward | 28 | 30 | 32 | 27 |
| PA Reversed | 40 | 28 | 33 | 32 |
| LNA | 5 | -6 | -6 | -10 |

Table 2-1 Front-end component linearity parameters for MSD calculation

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 65 | PCB isolation (PA forward mixing) |
| Triplexer | 15 | Low/high/very high band isolation |
| PA (out) to PA (out) | 65 | PCB isolation (PA reverse mixing) |
| LNA (in) to PA (out) | 65 | Cross-band (B42 filter + triplexer) |
| LNA (in) to PA (out) | 65 | PCB isolation (B42 PA leakage into B3 LNA) |
| Duplexer | 50 | Tx band rejection at Rx band |

Table 2-2 Front-end component isolation

Following identical MSD derivation process as in [3], it is observed that PCB isolation, antenna isolation and duplexer isolation are the main contributors for MSD due to IMD2, the affected dominant items are marked as blue in the link budget calculation process of IMD2 presented in Table 2-3.

Table 2-3 Link budget analysis for CA_n3-n78 IMD2

As recorded in WF [2], the feasible antenna isolation used in Lower MSD study is 10-20dB, while the feasible range of PCB isolation need further discussion. Under this premise we have evaluated the MSD improvement over antenna isolation up to 20dB and PCB isolation up to 85dB which are the two contributors to all kinds of MSD improvement, as Figure 2-1 without taking into account the practical implementation. Note that the feasibility of PCB isolation is subject to the group’s further confirmation.

Figure 2-1 IMD2 MSD trend analysis over better PCB isolation and antenna isolation

Observation 1: It is observed that in terms of PC3, MSD due to IMD2 of CA_n3-n78 could be reduced to around 15dB with 85dB PCB isolation and 20dB antenna isolation, in contrast to 26dB specified MSD.

2.2 CA_n3-n78 MSD Analysis on IMD4

Identical architecture and parameters assumption as in Clause 2.1, the link budget calculation process of IMD4 is presented in Table 2-4, and the MSD improvement over antenna isolation up to 20 dB and PCB isolation up to 85 dB is presented in Figure 2-2.

Table 2-4 Link budget analysis for CA_n3-n78 IMD4

Figure 2-2 IMD4 MSD analysis over better PCB isolation and antenna isolation

Observation 2: It is observed that in terms of PC3, MSD due to IMD4 of CA_n3-n78 could be reduced to around 5dB with 85dB PCB isolation and 20dB antenna isolation, in contrast to 8dB specified MSD.

2.3 CA_n3-n78 MSD Analysis on 2nd Harmonic

Following identical MSD derivation process as in [4] [5] and architecture illustrated in [6], it is observed that PCB isolation, antenna isolation and PA harmonic rejection are the main contributors of the MSD due to harmonic, the link budget calculation process and its initial adopted RF component assumption are presented in Table 2-5, and similar as above the MSD improvement over antenna isolation up to 20 dB and PCB isolation up to 85 dB is presented in Figure 2-3.

Table 2-5 RF Component assumption and Link budget analysis for CA_n3-n78 2nd harmonic MSD

Figure 2-3 Harmonic MSD analysis over better PCB isolation and antenna isolation

Observation 3: It is observed that in terms of PC3, MSD due to 2nd harmonic of CA_n3-n78 could be reduced to around 13dB with 85dB PCB isolation and 20dB antenna isolation, in contrast to 23.9dB specified MSD.

2.4 CA_n28-n40 MSD Analysis on 3nd Harmonic mixing

Following identical MSD derivation process as in [7] [8] and architecture illustrated in [8], it is observed that PCB isolation, lower band Rx filter selectivity and Rx 3rd order harmonic rejection are the main contributors of the MSD due to harmonic mixing, the link budget calculation process and its initial adopted RF component assumption are presented in Table 2-6, based on our analysis larger antenna isolation has almost no contribution to the MSD improvement since it only affects the diversity conductive path which has less influence on Diversity MSD.

The MSD improvement over Rx 3rd harmonic rejection up to 30 dB and PCB isolation up to 85 dB is presented in Figure 2-4, without taking into account the practical implementation. Note that the antenna isolation assumption is 10dB considering antenna isolation has almost no contribution to the MSD improvement.

Table 2-6 RF Component assumption and Link budget analysis for CA_n28-n40 3rd harmonic mixing MSD

Figure 2-4 Harmonic mixing MSD analysis over better PCB isolation and Rx harmonic rejection

Observation 4: It is observed that in terms of PC3, MSD due to 3rd harmonic mixing of CA_n28-n40 could be reduced to around 25dB with 85dB PCB isolation and 20dB antenna isolation, in contrast to 37.8dB specified MSD. Note that antenna isolation has almost no contribution to the MSD improvement.

Observation 5: It is observed that in terms of PC3, MSD due to 3rd harmonic mixing of CA_n28-n40 could be reduced to around 15dB with 85dB PCB isolation and 30dB Rx harmonic rejection, in contrast to 37.8dB specified MSD.

2.5 CA_n1-n3 MSD Analysis on cross band isolation

In last RAN4 meeting, MSD due to cross band isolation for CA_n1-n3 has been re-evaluated under the worst case with maximum aggressor UL CBW and minimum victim DL CBW [10] and the newly added MSD value was captured in the agreed CR [9]. The link budget calculation process and its initial adopted RF component assumption are presented in Table 2-7 following similar MSD derivation process and RF parameter assumptions as [10], similar as above the MSD improvement trend over antenna isolation up to 20 dB and PCB isolation up to 85 dB is presented in Figure 2-5.

Table 2-7 RF Component assumption and Link budget analysis for CA_n1-n3 cross band isolation MSD

Figure 2-5 Cross band isolation analysis over better PCB isolation and antenna isolation

Observation 6: It is observed that in terms of PC3, MSD due to cross band isolation of CA_n1-n3 could be reduced to around 10dB with 85dB PCB isolation and 20dB antenna isolation, in contrast to [19.7]dB specified MSD.

2.6 Others

For CA_n41-n77, the specified MSD due to cross band isolation is 8.3dB in current spec which is not under the worst case with maximum aggressor UL CBW, it is anticipated that the MSD would roughly remain unchanged since the gap between n41 and n77 is large; For CA_n1-n78, the specified MSD due to IMD4 is 8dB which is also not significant. There are many specified MSD values under 5dB which probably would not worry operators. Therefore, it might be unnecessary to report the MSD values in case the specified MSD itself is small. In addition, when the improvement is not significant, there is no need to report it, either.

Observation 7: It is unnecessary to report the MSD values in case the specified MSD itself is small or the improvement is not significant.

For the purpose of providing sufficient information to facilitate network scheduler, it might be beneficial to have multiple Lower MSD capability thresholds, corresponding to multiple Lower MSD capability classes, the necessity and interpretation of threshold(s) are justified in our companion paper R4-2215735. Based on the analysis above, 15dB, 10dB, 5dB might be the possible candidates for multiple exact absolute thresholds with assumption that identical thresholds shared by different interference source. Below table provide more interpretation.

| Bit map | Maximum allowed actual MSD (i.e. Thresholds) | Lower MSD Capability classes | Note |
| --- | --- | --- | --- |
| 00 | - | Not supported or not reported | Not supported here generally means the actual MSD is larger than the maximum threshold, while not reported generally means either the actual MSD has almost no improvement, or the specified MSD itself is already very small, or absent. |
| 01 | 5 dB | Ⅰ | 0 ≤ Actual MSD ≤ 5 |
| 10 | 10 dB | Ⅱ | 5 ＜ Actual MSD ≤ 10 |
| 11 | 15 dB | Ⅲ | 10 ＜ Actual MSD ≤ 15 |

Proposal 1: For sake of sufficient information provided to facilitate network scheduler, 15dB, 10dB, 5dB could be considered as PC3 candidate thresholds in case multiple exact absolute thresholds would be defined with assumption that identical thresholds shared by different interference source.

- 3. Conclusion

Observation 1: It is observed that in terms of PC3, MSD due to IMD2 of CA_n3-n78 could be reduced to around 15dB with 85dB PCB isolation and 20dB antenna isolation, in contrast to 26dB specified MSD.

Observation 2: It is observed that in terms of PC3, MSD due to IMD4 of CA_n3-n78 could be reduced to around 5dB with 85dB PCB isolation and 20dB antenna isolation, in contrast to 8dB specified MSD.

Observation 3: It is observed that in terms of PC3, MSD due to 2nd harmonic of CA_n3-n78 could be reduced to around 13dB with 85dB PCB isolation and 20dB antenna isolation, in contrast to 23.9dB specified MSD.

Observation 4: It is observed that in terms of PC3, MSD due to 3rd harmonic mixing of CA_n28-n40 could be reduced to around 25dB with 85dB PCB isolation and 20dB antenna isolation, in contrast to 37.8dB specified MSD. Note that antenna isolation has almost no contribution to the MSD improvement.

Observation 5: It is observed that in terms of PC3, MSD due to 3rd harmonic mixing of CA_n28-n40 could be reduced to around 15dB with 85dB PCB isolation and 30dB Rx harmonic rejection, in contrast to 37.8dB specified MSD.

Observation 6: It is observed that in terms of PC3, MSD due to cross band isolation of CA_n1-n3 could be reduced to around 10dB with 85dB PCB isolation and 20dB antenna isolation, in contrast to [19.7]dB specified MSD.

Observation 7: It is unnecessary to report the MSD values in case the specified MSD itself is small or the improvement is not significant.

Proposal 1: For sake of sufficient information provided to facilitate network scheduler, 15dB, 10dB, 5dB could be considered as PC3 candidate thresholds in case multiple exact absolute thresholds would be defined with assumption that identical thresholds shared by different interference source.

| Bit map | Maximum allowed actual MSD (i.e. Threshold) | Lower MSD Capability classes | Note |
| --- | --- | --- | --- |
| 00 | - | Not supported or not reported | Not supported here generally means the actual MSD is larger than the maximum threshold, while not reported generally means either the actual MSD has almost no improvement, or the specified MSD itself is already very small, or absent. |
| 01 | 5 dB | Ⅰ | 0 ≤ Actual MSD ≤ 5 |
| 10 | 10 dB | Ⅱ | 5 ＜ Actual MSD ≤ 10 |
| 11 | 15 dB | Ⅲ | 10 ＜ Actual MSD ≤ 15 |

- 4. Reference

[1] RP-221496, Revised WID on Further RF requirements enhancement for NR and EN-DC in frequency range1 (FR1), Huawei, RAN#96e

[2] R4-2214452, WF on study for lower MSD, RAN4#104-e, Huawei, HiSilicon

[3] R4-160396, MSD analysis for 2UL CA_3A_42A, RAN4#78, Media Tek

[4] R4-2212093, Possible approaches to improve MSD, RAN4#104-e, Nokia, Nokia Shanghai Bell

[5] R4-1709559, Discussion on harmonic issue for LTE B3+NR 3.5G, RAN#4 Meeting NR ad-hoc #3, Huawei, HiSilicon

[6] R4-147071, 3+42 UE RF analysis, RAN4#73, Huawei, Hisilicon

[7] R4-156835, TP for TR 36.852-13: on CA_28A-40A UE RF requirements, RAN4#76bis, Huawei, HiSilicon

[8] R4-152188, MSD analysis for inter-band CA with lower band Rx harmonic mixing problem, RAN4#74bis, MediaTek Inc.

[9] R4-2215134, CR for 38.101-1 to introduce the missing MSD due to cross band isolation, RAN4#104e, Huawei, HiSilicon, Skyworks Solutions Inc, CHTTL

[10] R4-2214004 Corrections to NR-CA Cross-band Isolation MSD Test Points, RAN4#104-e, Skyworks Solutions, Inc.