3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215763

# Electronic Meeting, October 10 – October 19, 2022

Agenda Item: 6.9.3

Source: MediaTek Inc.

Title: Discussion on R18 RRM for FR1-FR1 NR-DC

Document for: Discussion

- Introduction

In R15/R16/R17, for the RRM requirement, RAN4 only defines the requirement for FR1 + FR2 NR-DC only. However, FR1 + FR1 NR-DC is also a typical case in real deployment. Thus, in R18, RAN4 agreed to introduce the requirement for FR1 + FR1 NR-DC. In the last meeting, RAN4 has a preliminary discussion for this WI. Some open issues are still not concluded yet. Our view is provided in this paper.

- Discussion

According to WF [1], the following issues will be discussed sequentially.

- General issue

- PSCell addition/release delay and conditional PSCell addition delay

- Scheduling availability

- CSSF

- HO with PSCell

- SCG activation/deactivation

2.1 General issue

In the last meeting, some companies suggest to further discuss the other R16/R17 requirements which are not mentioned in WID. Our thinking is to suggest RAN4 to only discuss the scope which is covered in current WID. If companies want to extend the discussion scope. It should be triggered in RAN plenary meeting first to discuss whether to update the WID.

Proposal 1: In R18 FR1+FR1 NR-DC, only discuss R16/R17 requirements mentioned in the WID.

2.2 PSCell addition/release delay and conditional PSCell addition delay

In this section, two sub-issues are discussed :

- Interruption requirement for PSCell addition/release

- conditional PSCell addition delay requirement

Interruption requirement

The WF of interruption requirement for PSCell addition/release is provided below.

Issue 3-2-4: Interruption requirement for PSCell addition/release for FR1+FR1 NR-DC - FFS - Option 1(Apple, Intel, Xiaomi, OPPO, MTK, QC): For FR1+FR1 NR-DC case, existing interruption length requirement for NR PSCell addition/release can be reused. - Option 2(Nokia, Ericsson, vivo): NR-DC interruption requirements need to be updated to support FR1-FR1 NR-DC. - Option 2a(vivo): When defining the interruption requirements, for the number of SCells for FR1-FR1 NR-DC, up to 3 SCell(s) in FR1 of PCell and up to 3 SCell(s) in FR1 of PSCell are configured, deconfigured, activated or deactivated.

To our understanding, option 1 and option 2 are not conflict each other. In addition, we tend to believe the existing requirement can be reused for FR1+FR1 NR-DC scenario.

Proposal 2: For FR1+FR1 NR-DC case, existing interruption length requirement for NR PSCell addition/release can be reused.

Conditional PSCell addition delay requirement

For this issue, some companies think the TUE_preparation can be further enhanced. However, we think it can be reused for FR1, i.e., no need further enhancement.

Proposal 3: For FR1+FR1 NR-DC case, existing conditional PSCell addition requirement can be reused. (section 8.9A TS38.133).

2.3 Scheduling availability

In current spec, the scheduling availability is defined in both L3 and L1 measurement. Therefore, we would like to discuss them separately below.

L3 measurement

In the last meeting, some companies propose one principle that the new requirement for FR1+FR1 ND-RC should be introduced in R18 RRM if the requirement for FR1 inter-band CA is already defined. We think this principle is clear and we are fine with it.

Proposal 4: For scheduling availability in L3 measurement, to introduce the requirement for FR1+FR1 NR-DC scenario if the existing requirement has already included FR1 inter-band CA scenario.

L1 measurement (RLM/BFD/CBD/L1-RSRP)

For L1 measurement, we suggest the similar logic as L3 measurement can be reused.

Proposal 5: For scheduling availability in L1 measurement (RLM/BFD/CBD/L1-RSRP), to introduce the requirement for FR1+FR1 NR-DC scenario if the existing requirement has already included FR1 inter-band CA scenario.

Besides, we would like to further discuss the applicability requirement (clause 3.6.9) in TS 38.133. The current spec is provided as below.

3.6.9 Applicability of requirements for scheduling availability … The scheduling availability requirements in clause 8.1.7.1, 8.1.7.2, 8.5.7.1, 8.5.7.2, 8.5.8.1, 8.5.8.2, 9.5.6.1, 9.5.6.2, 9.8.6.1, and 9.8.6.2 assumes that the UE is not configured with simultaneous UL/DL between two FR1 bands if the UE does not have the capability of supporting simultaneousRxTxInterBandCA. …

If we check the explanation of simultaneousRxTxInterBandCA in TS 38.306 as below.

| simultaneousRxTxInterBandCA Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. If this field is included in ca-ParametersNR-ForDC, it indicates the UE supports simultaneous transmission and reception between any UL/DL band pair within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. | BC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

We can know that if UE supports simultaneous transmission/reception in inter-band NR-DC, the simultaneousRxTxInterBandCA should be included in ca-ParametersNR-ForDC.

Observation 1: If UE supports simultaneous transmission/reception in inter-band NR-DC, the simultaneousRxTxInterBandCA should be included in ca-ParametersNR-ForDC.

So, based on Observation 1, whether to update the wording in clause 3.6.9 can be further discussed. For example, “… if the UE does not have the capability of supporting simultaneousRxTxInterBandCA which is included in ca-ParametersNR-ForDC”

Proposal 6: For applicability requirement (clause 3.6.9 for scheduling availability), two options are suggested:

- Option 1: current spec can be reused and no spec impact for FR1-FR1 NR-DC

- Option 2: update current spec to make it clearer, e.g., clarify simultaneousRxTxInterBandCA is included in ca-ParametersNR-ForDC.

2.4 CSSF

In the last meeting, the discussion of CSSF is triggered. The corresponding WF is provided as below for reference.

Issue 3-2-11: CSSF outside gap for FR1+FR1 NR-DC Agreement: - For CSSF, add the scenario of FR1 PCell + FR1 PSCell in CSSFoutside_gap,i. - FFS on option 1a and 2. - Option 1a(Apple, Xiaomi, OPPO, vivo, MTK, Ericsson, HW): CSSF outside gap for FR1+FR1 NR-DC is: Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR1 PSCC; CSSFoutside_gap,i for FR2 PSCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required; CSSFoutside_gap,i for inter-frequency MO with no measurement gap. FR1 + FR1 NR-DC (FR1 PCell and FR1 PScell); 1+NPCC_CSIRS; 2×( NSCC_SSB +Y+2xNSCC_CSIRS); 2x(1+ NPSCC_CSIRS) Note 2; N/A; N/A; 2x(NSCC_SSB +Y+2x NSCC_CSIRS ). - Option 2(Intel, QC): For FR1+FR1 NR-DC, further discuss whether similar rule to derive CSSFoutside_gap and CSSFwithin_gap based on FR1+FR2 NR DC can be re-used.

To our understanding, the CSSF should be updated for FR1+FR1 NR-DC and above option 1a makes sense to us.

Proposal 7: CSSF outside gap for FR1+FR1 NR-DC is as following table.

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 + FR1 NR-DC (FR1 PCell and FR1 PScell) | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS) | 2x(1+ NPSCC_CSIRS) Note 2 | N/A | N/A | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) |

2.5 HO with PSCell

For the HO with PSCell, three open issues regarding Tprocessing are under discussion without conclusion yet. The corresponding WF is provided as below.

Issue 3-2-13: HO with PSCell for FR1+FR1 NR-DC (FR1-FR1 NR-DC to FR1-FR1 NR-DC) FFS: - Option 1(Apple, Xiaomi, OPPO, vivo, Intel, MTK, QC, HW, vivo): the existing requirement of HO with PSCell in section 6.1.5.4 TS38.133 can be reused, i.e., for HO with PSCell requirement, Tprocessing= 30 ms for sequential case and Tprocessing= 25 ms for parallel case should be added for FR1-FR1 NR-DC. - Option 2(Nokia, Ericsson): HO with PSCell from NR-DC to NR-DC need to be updated to support FR1-FR1 NR-DC Issue 3-2-14: HO with PSCell for FR1+FR1 NR-DC (FR1-FR2 NR-DC to FR1-FR1 NR-DC) FFS: - Option 1(Apple, OPPO, Intel, Xiaomi, QC), HW: the existing requirement of HO with PSCell in section 6.1.5.4 TS38.133 can be reused - Option 2 (vivo, MTK, QC): Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms. - Option 3(Nokia, Ericsson): HO with PSCell from NR-DC to NR-DC need to be updated to support FR1-FR1 NR-DC Issue 3-2-15: HO with PSCell for FR1+FR1 NR-DC (FR1-FR1 NR-DC to FR1-FR2 NR-DC) FFS: - Option 1(Apple, OPPO, vivo, Intel, Xiaomi, MTK, QC, HW): - Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms. - Option 2(Nokia, Ericsson): HO with PSCell from NR-DC to NR-DC need to be updated to support FR1-FR1 NR-DC

The discussion point here is how to calculate the Tprocessing? To our understanding, the equation for Tprocessing in HO with PSCell is max(Tprocessing in PCell HO, Tprocessing in PSCell change) + 5/10 ms, where the 5 and 10 ms are for parallel and sequential case, respectively. Besides, the Tprocessing in current handover requirement and PSCell change requirement is provided as below for reference.

- Tprocessing in handover (FR1 to FR1) 6.1.1.2 NR FR1 - NR FR1 Handover … 6.1.1.2.2 Interruption time … Tprocessing is time for UE processing. Tprocessing can be up to 20ms.. - Tprocessing in PSCell change 8.11 PSCell Change … - Tprocessing = 20 ms when source and target cells are in the same FR, - Tprocessing = 40 ms when source and target cells are in different FRs..

To our understanding, for the case “FR1-FR1 NR-DC to FR1-FR1 NR-DC”, because the frequency range of PSCell is not changed before and after handover, both PCell handover’s and PSCell change’s Tprocessing is 20 ms.

For the case “FR1-FR2 NR-DC to FR1-FR1 NR-DC” and “FR1-FR1 NR-DC to FR1-FR2 NR-DC”, because the frequency range of PSCell is changed before and after handover, Tprocessing for PCell handover is 20 ms and Tprocessing for PSCell change is 40 ms.

As a result, according to above analysis, Tprocessing in HO with PSCell for various cases can be obtained in Table 1.

Table 1. The Tprocessing for various cases

| Scenario for HO with PSCell | Equation (ms) | Tprocessing (ms) |
| --- | --- | --- |
| FR1-FR1 NR-DC to FR1-FR1 NR-DC | max(20, 20) + 5/10 | 25 and 30 are for parallel and sequential case, respectively |
| FR1-FR2 NR-DC to FR1-FR1 NR-DC | max(20,40) + 5/10 | 45 and 50 are for parallel and sequential case, respectively |
| FR1-FR1 NR-DC to FR1-FR2 NR-DC | max(20, 40) + 5/10 | 45 and 50 are for parallel and sequential case, respectively |

Proposal 8: For HO with PSCell, the Tprocessing is as following table.

| Scenario for HO with PSCell | Tprocessing (ms) |
| --- | --- |
| FR1-FR1 NR-DC to FR1-FR1 NR-DC | 25 and 30 are for parallel and sequential case, respectively |
| FR1-FR2 NR-DC to FR1-FR1 NR-DC | 45 and 50 are for parallel and sequential case, respectively |
| FR1-FR1 NR-DC to FR1-FR2 NR-DC | 45 and 50 are for parallel and sequential case, respectively |

2.6 SCG activation/deactivation

For SCG activation/deactivation, two sub-issues are discussed :

- RACH based PSCell activation

- Known condition for PSCell activation

RACH based PSCell activation

The corresponding WF is provided as below.

Issue 3-2-16: RACH based PSCell activation for FR1+FR1 NR-DC Agreements: - SCG activation/deactivation requirements need to be updated to support FR1-FR1 NR-DC - FFS on option 1a and 1b - Option 1a(Apple, Xiaomi, OPPO, MediaTek, vivo, Intel, Ericsson, HW): if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms. - Option 1b(Ericsson): further study the potential enhancement in T∆

For the Tsearch, to our understanding, typically, the cell search should be completed within 3 measurement samples. In FR2, because UE is required to sweep receiving beam to find the best UE Rx beam (N=8), the total measurement is 24. However, in FR1, UE Rx beam sweeping is no longer needed, so 3 SSB bursts should be enough.

Proposal 9: For RACH based PSCell activation in SCG activation/deactivation, if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms.

For T∆, according to the current spec as below, T∆ = 1*Trs ms. Fine timing fracking is important for PSCell. Therefore, we do not think it can be skipped.

8.17.2 SCG Activation Delay Requirement … T∆ is time for fine time tracking and acquiring full timing information of the target PSCell. T∆ = 1*Trs ms.

Proposal 10: For RACH based PSCell activation in SCG activation/deactivation, not to enhance T∆.

Known condition for PSCell activation

The corresponding WF is provided as below.

Issue 3-2-17: Known condition for PSCell activation for FR1+FR1 NR-DC FFS: - Option 1(Apple, Intel, Xiaomi, OPPO, MTK, QC, HW, vivo): In FR1, the PSCell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown.

According to current spec as below, known condition is only defined for FR2. So, it should be updated for FR1.

Content extracted from clause 8.17.2 in TS 38133

In FR2, the PSCell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the SCG activation command: - the UE has sent a valid measurement report for the PSCell being activated and - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3. otherwise it is unknown.

Proposal 11: For RACH based PSCell activation in SCG activation/deactivation, Known condition for PSCell activation should be updated for FR1.

- Summary

In this paper, the discussion of FR1-FR1 NR DC is provided. We have the following proposal:

Proposal 1: In R18 FR1+FR1 NR-DC, only discuss R16/R17 requirements mentioned in the WID.

Proposal 2: For FR1+FR1 NR-DC case, existing interruption length requirement for NR PSCell addition/release can be reused.

Proposal 3: For FR1+FR1 NR-DC case, existing conditional PSCell addition requirement can be reused. (section 8.9A TS38.133).

Proposal 4: For scheduling availability in L3 measurement, to introduce the requirement for FR1+FR1 NR-DC scenario if the existing requirement has already included FR1 inter-band CA scenario.

Proposal 5: For scheduling availability in L1 measurement (RLM/BFD/CBD/L1-RSRP), to introduce the requirement for FR1+FR1 NR-DC scenario if the existing requirement has already included FR1 inter-band CA scenario.

Observation 1: If UE supports simultaneous transmission/reception in inter-band NR-DC, the simultaneousRxTxInterBandCA should be included in ca-ParametersNR-ForDC.

Proposal 6: For applicability requirement (clause 3.6.9 for scheduling availability), two options are suggested:

- Option 1: current spec can be reused and no spec impact for FR1-FR1 NR-DC

- Option 2: update current spec to make it clearer, e.g. clarify simultaneousRxTxInterBandCA is included in ca-ParametersNR-ForDC.

Proposal 7: CSSF outside gap for FR1+FR1 NR-DC is as following table.

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 + FR1 NR-DC (FR1 PCell and FR1 PScell) | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS) | 2x(1+ NPSCC_CSIRS) Note 2 | N/A | N/A | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) |

Proposal 8: For HO with PSCell, the Tprocessing is as following table.

| Scenario for HO with PSCell | Tprocessing (ms) |
| --- | --- |
| FR1-FR1 NR-DC to FR1-FR1 NR-DC | 25 and 30 are for parallel and sequential case, respectively |
| FR1-FR2 NR-DC to FR1-FR1 NR-DC | 45 and 50 are for parallel and sequential case, respectively |
| FR1-FR1 NR-DC to FR1-FR2 NR-DC | 45 and 50 are for parallel and sequential case, respectively |

Proposal 9: For RACH based PSCell activation in SCG activation/deactivation, if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms.

Proposal 10: For RACH based PSCell activation in SCG activation/deactivation, not to enhance T∆.

Proposal 11: For RACH based PSCell activation in SCG activation/deactivation, Known condition for PSCell activation should be updated for FR1.

- Reference

- R4-2214345, WF on R18 eFeRRM, Apple, RAN4 #104-e