3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215791

Electronic Meeting, October 10th – October 19th, 2022

Agenda item:	4.1.5.3.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on test tolerance values of OOB Gain for FR2 repeater

Document for:	Approval

- Introduction

In RAN4#104-e, the discussion on repeater testing part had been conducted. However, part of MU/TT values for FR2 repeater remains not concluded. In this contribution, we provide our views on TT values of OOB Gain for FR2 repeater.

2.	Discussion

In RAN4#104-e, some agreements were achieved in terms of FR2 OTA measurements. Some of the agreements are as follows:

RAN4#104-e [1] 2.2.2 TRP measurement with input antenna rotating together with repeater Agreements: - Keep the 3GPP description of providing input signal to the repeater generic and allow reasonable uncertainty for it, allowing test engineers to use appropriate solutions in various different test systems. - Further work on the drafting TP 2.2.3 TDD off power measurement Agreements: - Take same approach as BS for transmit antenna gain assumption during on/off period. - In TDD ON/OFF power measurement, input signal generator does not transmit towards repeater during the OFF-power measurement. 2.2.4 OTA test procedure Agreements: - “Measurement impact from feeding test signal by generating a signal for repeater input with repeater to be turned off. Verify measured result is enough below requirement limit” - Further work on specification drafting

In previous meetings, the uncertainty of input signal for repeater under the test has been discussed, and it was agreed that reasonable uncertainty for each test systems would be allowed and that test engineers use appropriate solutions in various different test systems.

In current draft specification TS 38.115-2, one of the requirements that the TT values are not concluded is OOB Gain.

OOB Gain is a requirement which affects other systems in outside of the passband. Therefore, it needs to define certain maximum TT values, though reasonable uncertainty for each test systems were allowed.

Observation 1: It needs to define certain maximum TT values for OTA OOB Gain.

The OOB Gain at the frequency under test is given by the difference in dB between the measured output TRP and the EIRP of the input signal, so uncertainty of input signal may affect the measurement results. Regarding OTA Output power measurement (TRP), it may also be affected by uncertainty of input signal in non ALC condition. Since both measurements of OOB Gain and OTA Output power are conducted with the input power level with Pin,p,EIRP, the uncertainty of input signals are the same. Therefore, the conditions are mostly the same between OOB Gain and OTA Output power measurement (TRP) in non ALC condition in terms of TRP measurements.

Observation 2: The conditions are mostly the same between OOB Gain and OTA Output power measurement (TRP) in non ALC condition in terms of TRP measurements.

Regarding of MU/TT values for OTA Output power, it was agreed to consider the same values with BS as starting point [2]. The TT values for OTA Output power for BS are defined in TS 38.141-2 as following:

TS 38.141-2 Table C.1-2: Derivation of test requirements (FR2 OTA transmitter tests) Test; Minimum requirement in TS 38.104 [2]; Test Tolerance (TTOTA); Test requirement in the present document. 6.2 Radiated transmit power; See TS 38.104 [2], clause 9.2; Normal conditions: 1.7 dB, 24.25GHz < f ≦ 29.5GHz 2.0 dB, 37GHz < f ≦ 43.5GHz 2.2 dB, 43.5GHz < f ≦ 48.2GHz Extreme conditions: 3.1 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.5 dB, 43.5GHz < f ≦ 48.2GHz; Formula: Upper limit + TT, Lower limit – TT. 6.3 OTA base station output power; See TS 38.104 [2], clause 9.3; 2.1 dB, 24.25GHz < f ≦ 29.5GHz 2.4 dB, 37GHz < f ≦ 43,5GHz 2.6 dB, 43.5GHz < f ≦ 48.2GHz; Formula: Upper limit + TT, Lower limit – TT.

In conclusion, we propose to consider the same values with OTA Output power (TRP) of BS for OTA OOB Gain of FR2 repeaters as baseline.

Proposal 1: RAN4 consider following values as baseline for maximum TT values of OOB Gain for FR2 repeaters.

- 2.1 dB, 24.25GHz < f ≦ 29.5GHz

- 2.4 dB, 37GHz < f ≦ 43,5GHz

- 2.6 dB, 43.5GHz < f ≦ 48.2GHz

3.	Conclusion

In this contribution, we provide views on TT values for OTA OOB Gain. The following observations and a proposal are obtained.

Observation 1: It needs to define certain maximum TT values for OTA OOB Gain.

Observation 2: The conditions are mostly the same between OOB Gain and OTA Output power measurement (TRP) in non ALC condition in terms of TRP measurements.

Proposal 1: RAN4 consider following values as baseline for maximum TT values of OOB Gain for FR2 repeaters.

- 2.1 dB, 24.25GHz < f ≦ 29.5GHz

- 2.4 dB, 37GHz < f ≦ 43,5GHz

- 2.6 dB, 43.5GHz < f ≦ 48.2GHz

References

- R4-2214369, “WF on NR repeater conformance test remaining issues”, CATT, RAN4 #104-e, August 2022

- R4-2211150, “WF on NR FR1/FR2 repeater MU and TT”, Huawei, RAN4 #103-e, May 2022